3GPP TSG-RAN3 #120	R3-233110

22nd May – 26th May 2023

Incheon, Korea

Title: 	(TP for AI/ML BLCR to TS38.300) Characteristics of the procedures for exchanging AI/ML-related information

Source: 	Ericsson, InterDigital, Deutsche Telekom

Agenda item:	12.2.1

Document Type:	Discussion and Decision

# TP for TS 38.300

Below, a draft TP is presented where the proposals made in R3-233110 are reflected

<<<<<< Start of CHANGE >>>>>>

## X.X AI/ML for NG-RAN

### X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

### X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Signalling procedures used for the exchange of AI/ML related information are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

<<<<<< End of CHANGE >>>>>>