3GPP TSG-RAN WG3 Meeting #120	R3-23xxxx

Incheon, Korea, 22nd – 26th May 2023

Agenda Item:	26.1

Source:	Ericsson, Huawei

Title:	[TP for BL CR 29.413] Remove Editor’s Note

Document for:	Discussions & Approval

# 1	Introduction

This TP follows discussions during the online discussions on Rel-18 eNPN.

# 2	Text Proposal

Change Begins

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

For the NGAP messages transferred between the Non-3GPP access network node and the AMF, the following exceptions to the specification in TS 38.413 [2] shall be applied:

<Unchanged Text Omitted>

INITIAL UE MESSAGE message:

-	the following IEs shall be ignored, when received:

-	IAB Node Indication IE

-	CE-mode-B Support Indicator IE

-	LTE-M Indication IE

-	EDT Session IE

-	NPN Access Information IE

-	RedCap Indication IE

-	RRC Establishment Cause IE: the information given within this IE is to indicate the Establishment cause for non-3GPP access as specified in TS 24.502 [7].

-	Selected PLMN Identity IE: the information given within this IE provides the selected PLMN ID for untrusted non-3GPP access as specified in TS 23.502 [4].

-	Authenticated Indication IE: the information given within this IE between the W-AGF and the AMF is to indicate that the FN-RG has been authenticated by the wireline 5G access network as specified in TS 23.316 [6].

-	Selected PLMN Identity IE: the information given within this IE contains the PLMN Identity for wireline access as specified in TS 23.316 [6], or for trusted non-3GPP access as specified in TS 23.502 [4].

-	Selected NID IE: the information given within this IE contains the NID which, together with the Selected PLMN Identity IE, indicates the SNPN identity for wireline access as specified in TS 23.316 [6], or for trusted/untrusted non-3GPP access as specified in TS 23.502 [4].

<Unchanged Text Omitted>

Change Ends