3GPP TSG-RAN WG3 #120	R3-23xxxx

# Incheon, Korea, 22nd – 26th May 2023

Agenda Item:	26.2

Source:	Ericsson

Title:	(TP to TRS_URLLC BLCR for TS 38.423): Introduction of 5G Timing Resiliency and URLLC enhancements

Document for:	Discussion, Decision

# 1	Information

This paper contains the text proposal capturing the agreement from RAN3#120.

The track changes are marked with Ericsson-RAN3#120.

# 2	Text Proposal on XnAP

************************ 1st Change********************************

#### 9.2.3.115	TSC Assistance Information

Editor’s Note: Encoding of IEs may be further refined.

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [7]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.2.3.116 | Periodicity as specified in TS 23.501 [7]. | – |  |
| Burst Arrival Time | O |  | 9.2.3.117 | Burst Arrival Time as specified in TS 23.501 [7]. | – |  |
| Survival Time | O |  | 9.2.3.152 |  | YES | Ignore |
| CHOICE RAN feedback type |  | 0..1 |  |  | YES | ignore |
| >proactive |  |  |  |  |  |  |
| >>Burst Arrival Time Window | M |  | 9.2.3.z1 |  | - |  |
| >>Periodicity Range | O |  | 9.2.3.z2 |  | - |  |
| >reactive |  |  |  |  |  |  |
| >>Capability for BAT Adaptation | M |  | 9.2.3.z3 |  | - |  |

***********************Skip to Next Change ********************************

#### 9.2.3.153	Time Synchronisation Assistance Information

This IE indicates the 5G access stratum time distribution parameters as specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Time Distribution indication | M |  | ENUMERATED (enabled, disabled, …) |  | - | - |
| Uu Time Synchronization Error Budget | C-ifEnabled |  | INTEGER (0..1000000, …) | Expressed in units of 1 ns. | - | - |
| Clock Quality Reporting Control Information | O |  | 9.2.3.x1 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifEnabled | This IE shall be present if the Time Distribution Indication IE is set to “enabled”. |

***********************Skip to Next Change ********************************

#### 9.2.3.x1	Clock Quality Reporting Control Information

This IE indicates the clock quality reporting control information as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Quality Detail Level | M |  |  |  |
| >clock quality metrics |  |  |  |  |
| >>[FFS] |  |  |  |  |
| >acceptance indication |  |  |  |  |
| >>Clock Quality Acceptance Criteria | M |  | 9.3.1.x2 |  |

#### 9.2.3.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

***********************Skip to Next Change ********************************

#### 9.2.3.z1	Burst Arrival Time Window

This IE indicates the burst arrival time window of the TSC QoS flow as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

#### 9.2.3.z2	Periodicity Range

This IE indicates the periodicity range for the TSC QoS flow as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Periodicity Range | M |  |  |  |
| >Periodicity Bound |  |  |  |  |
| >>Periodicity Lower Bound | M |  | Periodicity 9.3.1.132 |  |
| >>Periodicity Upper Bound | M |  | Periodicity 9.3.1.132 |  |
| >Periodicity List |  |  |  |  |
| >>Allowed Periodicity List |  | 1 |  |  |
| >>>Allowed Periodicity Item |  | 1..<maxnoofPeriodicities> |  |  |
| >>>>Allowed Periodicity | M |  | Periodicity 9.3.1.132 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPeriodicities | Maximum no. of allowed periodicities. Value is 8 [FFS]. |

#### 9.2.3.z3	Capability for BAT Adaptation

This IE indicates the capability for BAT adaptation for the TSC QoS flow as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

************************ The End ********************************