3GPP TSG-RAN WG3 Meeting #120	R3-233450

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TPs for SON BLCR for TS 36.300): Naming update for RA report

Source: 	Huawei

Agenda item:	10.2.3

Document Type:	other

1. Introduction

This document contains a TP to SON BLCR for TS 36.300 for RACH optimisation .

Annex: TP to TS 36.300 BL CR

22.4.3	Support for RACH Optimisation

22.4.3.1	General

The aim of this function is to support RACH Optimisation. RACH optimisation is supported by UE reported information and by RACH parameters exchange between:

-	E-UTRA cells;

-	NR cells, in case of EN-DC.

22.4.3.2	Solution description

22.4.3.2.1	E-UTRA cell case

The setting of RACH parameters that can be optimized are:

-	RACH configuration (resource unit allocation);

-	RACH preamble split (among dedicated, group A, group B, RSRP level, NRSRP level (for NB-IoT), NPRACH resource pools (for NB-IoT), EDT);

-	RACH backoff parameter value;

-	RACH transmission power control parameters.

RACH optimisation is supported by UE reported information and by PRACH parameters exchange or NPRACH parameters (for NB-IoT) between eNBs.

UEs which receive polling signalling shall report the below information:

-	Number of RACH preambles sent until the successful RACH completion;

-	Contention resolution failure;

-	For BL UE or UE in enhanced coverage or NB-IoT UE, the RSRP (NRSRP for NB-IoT) level in which the UE started the random access procedure;

-	For BL UE or UE in enhanced coverage or NB-IoT UE, an EDT fallback indication.

UE reporting of RACH information is not supported for a NB-IoT UE using the Control Plane CIoT EPS Optimisation,

22.4.3.2.2	NR cell in EN-DC case

The solution applies to an en-gNB supporting EN-DC operation. RACH optimisation is supported by UE reported information (RA report, see TS 38.300 [79]) made available at the eNB and further forwarded to the en-gNB, and by PRACH parameters exchanged (see TS 38.300 [79]) between en-gNBs and eNBs.