3GPP TSG-RAN3 Meeting #121	R3-233734

Toulouse, France, 21th Aug - 25th Aug 2023

3GPP TSG-WG SA2 Meeting #157	S2-2308248

Berlin, Germany, May 22 – 26, 2023	(revision of S2-2308198)

# Title:	LS Reply on Design of RTP Header Extension for PDU Set handling

# Response to:	LS (S2-2303974/S4-230419) from SA4

# Release:	Release 18

# Work Item:	XRM, 5G_RTP

Source:	SA WG2

To:	SA WG4, RAN WG3

Cc:	RAN WG2

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	hui DOT ni AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2308193

1. Overall Description:

SA2 thanks SA4 for the liaison (S2-2303974/S4-230419) on the design of RTP Header Extension for PDU Set handling.

Regarding the questions raised by SA4, SA2 would like to provide following replies:

Question#1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs?

Reply#1:

In SA2’s understanding, the intent of Q1 is as follows: is it possible for a single QoS flow to include both N6 packets marked with the new RTP header extension under definition in SA4 and N6 packets that are not marked with the new RTP header extension under definition in SA4?

With this understanding the SA2’s answer is as follows: Mixing of N6 packets that are marked with the new RTP header extension with N6 packets that are not marked with the new RTP header extension in the same QoS Flow can happen as long as the information for QoS Flow matching (refer to the Packet Detection Information in the Packet Detection Rule in TS 23.501 Table 5.8.5.3-1) is the same for both.

Furthermore, SA2 would like to bring to the SA4’s attention that SA2 agreed that a single QoS Flow includes either PDUs with GTP-U header extensions for PDU Sets or PDUs without such extension, but not both.

If the UPF receives a PDU that do not belong to a PDU Set based on Protocol Description for PDU Set identification, then the UPF still maps it to a PDU Set. The details are described in the attached CR.

Question#2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension?

Reply#2: Yes. Furthermore, as stated in the previous LS to SA4 (S2-2303849), for Rel-18 SA2 kindly asked SA4 to provide guidelines on End of Data Burst Indication marked by the application server via RTP header information (e.g., the timestamp, sequence number and "M" bit) or define the 3GPP specific RTP header extension if the current RTP header cannot provide such information to determine End of Data Burst Indication.

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 to take the above into account and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE

TSG-SA2 Meeting #159		October 09 – 13, 2023			Xiamen, CN