3GPP TSG-RAN WG3 Meeting #121	R3-233744

Toulouse, France, 21 – 25 Aug, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0284 | rev | 4 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR to 38.401) Introduction on MT-SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, ZTE, Nokia, Nokia Shanghai Bell, Ericsson, Intel Corporation, China Telecom, Lenovo, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MT_SDT-Core |  |  |  |  |  | Date: |  |  | 2023-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of NR Mobile Terminated – Small Data Transmission (NR MT-SDT). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture Abbreviation of MT-SDT. - Capture call flow for MT-SDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR MT-SDT is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.18.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 38.423 CR 1010 TS 38.473 CR 1140 TS 37.483 CR 0054 TS 38.420 CR 0034 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version R3-231052: Capture the agreed TP R3-230999. Rev1 R3-231158: resubmission based on latest version of specification. Rev2 R3-232533: capture the agreed TP R3-232124. Rev3 R3-233555: capture the agreed TP R3-233485. Rev4: resubmission based on latest version of spec. |  |  |  |  |  |  |  |  |

------------Start of the First Change-----------------

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

BH	Backhaul

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

EM	Element Manager

EN-DC	E-UTRA-NR Dual Connectivity

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

FTEID	Fully Qualified TEID

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

L2					Layer-2

MBS	Multicast Broadcast Service

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MN	Master Node

MgNB	Master gNB

MRB	MBS Radio Bearer

MRDC	Multi-Radio Dual Connectivity

MT-SDT	Mobile Terminated Small Data Transmission

NAS	Non-Access Stratum

NID	Network identifier

NPN	Non-Public Network

NSA	Non Standalone

OAM	Operation, Administration and Maintenance

PNI-NPN	Public Network Integrated Non-Public Network

PTP	Point to Point

PTM	Point to Multipoint

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS	Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SA	Standalone

SAP	Service Access Point

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SgNB	Secondary gNB

SM	Session Management

SMF	Session Management Function

SN	Secondary Node

SNPN	Stand-alone Non-Public Network

SRAP	Sidelink Relay Adaptation Protocol

TCE	Trace Collection Entity

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

U2N	UE-to-Network

------------Start of the Next Change-----------------

### 8.18.x	MT-SDT

The procedure for mobile terminated small data transmission in RRC Inactive is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Mobile Terminated Small Data Transmission in RRC Inactive state.

1. During the setup or modification of the bearer context as specified in 8.9.2, the gNB-CU-CP requests the gNB-CU-UP to provide MT-SDT information.

2a-0.	The gNB-CU-UP receives DL data for the UE in RRC Inactive on NG-U interface.

2a-1.	The gNB-CU-UP sends DL DATA NOTIFICATION message to the gNB-CU-CP. If determining that DL data packets are only mapped to SDT bearers, as requested in step 1, the gNB-CU-UP includes the MT-SDT information in the DL DATA NOTIFICATION message.

2b. The gNB-CU-CP receives DL NAS signalling over NGAP.

3.	After 2a or 2b, the gNB-CU-CP sends PAGING message to the gNB-DU. The MT-SDT information may be included in the PAGING message.

4.	The gNB-DU sends the Paging message to the UE. In case the MT-SDT information is received in step 3, the gNB-DU includes the MT-SDT indicator in the Paging message.

5. If the UE has been successfully reached, it initiates the RRC connection resume procedure as described in 8.6.2 or 8.9.6.2, or initiates the SDT procedure as described in 8.18.1 or from step 9 in 8.18.2 or from step 1 in 8.18.3 with the following difference:

- In case SDT procedure is initiated, the UE may indicate the RRC Resume Request is for MT-SDT, which may be without UL data.

------------End of the Change-----------------