3GPP TSG-RAN WG3 Meeting #121	R3-233816

Toulouse, France, 21 - 25 August 2023

Agenda item:	26.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.413 BL CR) Further stage 3 details for timing resiliency

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we evaluate open issues for timing resiliency according to the following WID objectives [1]:

1. 5GS network timing synchronization status and reporting [RAN3, RAN2]: a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3. c. UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2] d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

# 2	Discussion

## 2.1	RAN TSS reporting towards the AMF

The signalling framework for RAN TSS reporting towards the AMF is proposed in R3-233815 [2], including a text proposal for TS 38.401. On this basis, the following is proposed for NGAP:

Proposal 1:	In NGAP, introduce a new Timing Synchronisation Status procedure (class 1) to enable the AMF to initiate RAN TSS reporting by the gNB.

Proposal 2:	In NGAP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB to report RAN TSS to the AMF.

Proposal 3:	The (NGAP) TIMING SYNCHRONISATION STATUS REQUEST message includes the RAN TSS Request Type IE (start, stop).

Proposal 4:	The (NGAP) TIMING SYNCHRONISATION STATUS REPORT message includes the RAN Timing Synchronisation Status IE and the RAN TSS Scope IE (indicating whether the scope of the RAN TSS is “all cells within a single gNB” or “list of Cell IDs within a single gNB”).

Over F1AP, the same procedures can be introduced with only minor differences compared to NGAP, e.g. there is no need for the RAN TSS Scope IE over F1AP since the scope is implicitly “all cells” of the gNB-DU.

Proposal 5:	In F1AP, introduce two new procedures corresponding to the new NGAP procedures.

Proposal 6:	The (F1AP) TIMING SYNCHRONISATION STATUS REQUEST message includes the RAN TSS Request Type IE (start, stop).

Proposal 7:	The (F1AP) TIMING SYNCHRONISATION STATUS REPORT message includes the RAN Timing Synchronisation Status Information IE (containing all RAN TSS attributes supported by the gNB-DU).

## 2.2	Encoding of the Clock Quality Acceptance Criteria IE

SA2 refers to TS 23.501 Table 5.27.1.12-1 to list the attributes that may be contained in RAN Timing Synchronization Status (TSS) reporting.

Table 5.27.1.12-1: Information elements that gNB or UPF/NW-TT timing synchronization status information may contain (all optional)

| Information Name | Description |
| --- | --- |
| Synchronization state | Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1). |
| Clock quality |  |
| >> Traceable to GNSS | Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”. |
| >> Traceable to UTC | Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”. |
| >> Frequency stability | Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2). |
| >> Clock Accuracy | Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3). |
| Parent time source | Describes the primary source the node is currently using, represented by the values "SyncE", "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other". |
| NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process. |  |

For clock quality reporting towards the UE, the gNB receives clock quality reporting control information from the AMF including the Clock Quality Detail Level IE which can take one of the following values: “clock quality metrics” or “acceptable/not acceptable indication”.

When the clock quality detail level equals “clock quality metrics”, the gNB does not receive any additional information from the AMF. TS 23.501 indicates that the NG-RAN is locally configured which of the clock quality metrics supported by NG-RAN are provided to UE(s). That is, it is up to gNB to determine which attributes to include in the clock quality information sent to the UE, i.e. based on the implementation gNB may report all, some, or none of the attributes from TS 23.501 Table 5.27.1.12-1.

Observation-1:	When the clock quality detail level equals “clock quality metrics”, it is up to gNB implementation which of the clock quality metrics are provided to UE(s), i.e. no NGAP impacts.

When the clock quality detail level equals “acceptable/not acceptable indication”, the gNB also receives the Clock Quality Acceptance Criteria IE from the AMF. The acceptance criteria is associated with a UE and is used to decide the service status after a primary time source event (e.g., degradation, failure, recovery). According to TS 23.501, the clock quality acceptance criteria for the UE may be defined based on one or more attributes listed in Table 5.27.1.12-1. That is, the same table in SA2 can be used for defining both the clock quality acceptance criteria and for RAN TSS reporting when there is only a single acceptable value/threshold for the attribute, thus, the same encoding could be reused in RAN3 specifications.

Observation-2:	The Clock Quality Acceptance Criteria IE could reuse the same encoding as the RAN Timing Synchronisation Status IE if there is only a single acceptable value/threshold for the attribute.

However, there is an additional consideration for how to encode the Clock Quality Acceptance Criteria IE. While it is clear that a RAN TSS report has a single value per supported attribute of Table 5.27.1.12-1, the Clock Quality Acceptance Criteria IE could potentially have multiple acceptable values for two of the attributes in Table 5.27.1.12-1 (i.e., synchronization state and parent time source). For example, a time synchronization service for a UE could be “acceptable” as long as the synchronization state is not “Freerun”. This means two different values of synchronization state are tolerated (i.e., “Locked", "Holdover”). A similar case can be seen for parent time source, where a service could be considered “acceptable” as long as it is within the subset of a few sources (e.g., all parent time sources except “GNSS”). From RAN3 perspective, different alternatives can be considered to address this area:

Option A) Multiple values of clock quality acceptance criteria are allowed for the same attribute. This is only applicable for synchronization state and parent time source attributes. Different encoding options can be decided:

A.1) The RAN Timing Synchronisation Status Information IE and Clock Quality Acceptance Criteria IE use the same encoding, and the clock quality reporting control information allows to have a list of Clock Quality Acceptance Criteria IE (instead of one IE). A similar approach is already specified in TS 29.503 for the clock quality acceptance criteria encoding at the UDM. In TS 29.503, the TimeSyncData type defines clockQualityAcceptanceCriteria as 1…N ClockQualityAcceptanceCriterion. This array of ClockQualityAcceptanceCriterion can have one element of synchronization state, clock quality, or parent time source. If an array of criterion is configured, multiple instances of the synchronization state or parent time source can be stored.

A.2) The RAN Timing Synchronisation Status Information IE and Clock Quality Acceptance Criteria IE do not use the same encoding. For synchronization state and parent time source attributes, the RAN Timing Synchronisation Status Information IE uses ENUMERATED type while the Clock Quality Acceptance Criteria IE uses BIT STRING type where each bit corresponds to a value of the attribute and “1” indicates “acceptable”.

Option B)	Multiple values of clock quality acceptance criteria are not allowed for the same attribute. The RAN Timing Synchronisation Status Information IE and Clock Quality Acceptance Criteria IE use the same encoding.

From the above alternatives, option A.2 seems suitable for the synchronisation state and parent time source attributes.

Observation-3:	There can be multiple acceptable values for synchronisation state and parent time source. The clock quality acceptance criteria for these attributes can be encoded using BIT STRING where each bit represents an attribute value.

The rest of the clock quality acceptance criteria attributes only need one configuration to define the acceptable threshold for the UE’s service. That is, for the attributes Traceable to UTC, Traceable to GNSS, Clock Frequency Stability, and Clock Accuracy, there is only one value/threshold allowed per UE. Particularly for Clock Accuracy that has multiple choices in the proposed encoding, it is sufficient to have a single choice with the smallest granularity, i.e., INTEGER with 10ns granularity (pending RAN3 conclusion on the encoding of Clock Accuracy IE, see section 2.1.2) since the acceptance criteria is just a threshold for the service, it is not related to gNB implementation.

Observation-4:	Only a single value of clock quality acceptance criteria is allowed for the remaining attributes (i.e., Traceable to UTC, Traceable to GNSS, Clock Frequency Stability, Clock Accuracy). The clock quality acceptance criteria for these attributes can be encoded the same as in the RAN Timing Synchronisation Status Information IE. For Clock Accuracy, a single choice with the smallest granularity is sufficient.

The resulting encoding for the Clock Quality Acceptance Criteria IE is as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (0..100000000, …) | Clock accuracy expressed in units of 10 ns. |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

Proposal 8:	Encode the Clock Quality Acceptance Criteria IE as shown in the above table.

## 2.3	Encoding of the Clock Accuracy IE

During RAN3#120 different encodings of the Clock Accuracy IE were discussed. The description of clock accuracy in SA2 states that it is “the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock”. To allow for different implementations, the encoding of clock accuracy should support several different granularities, e.g. 10ns, 100ns, and 1ms.  In addition, an enumeration can also be supported using values from IEEE Std 1588, Table 5. Regarding the maximum value, 1 second seems sufficient. This leads to the following encoding for the Clock Accuracy IE:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >granularity 10ns |  |  |  |  |
| >>Clock Accuracy Value in 10ns | M |  | INTEGER (0..100000000, …) | Clock accuracy expressed in units of 10 ns. |
| >granularity 100ns |  |  |  |  |
| >>Clock Accuracy Value in 100ns | M |  | INTEGER (0..10000000, …) | Clock accuracy expressed in units of 100 ns. |
| >granularity 1us |  |  |  |  |
| >>Clock Accuracy Value in 1us | M |  | INTEGER (0..1000000, …) | Clock accuracy expressed in units of 1 us. |
| >enumeration |  |  |  |  |
| >>Clock Accuracy Value Enumerated | M |  | ENUMERATED (25ns, 100ns, 250ns, 1us, 2dot5 us, 10us, 25us, 100us, 250us, 1ms, 2dot5ms, 10ms, 25ms, 100ms, 250ms, 1sec, …) |  |

An additional encoding option was proposed at RAN3#120 as follows:

| >range |  |  |  |  |
| --- | --- | --- | --- | --- |
| >>Time Accuracy lower range | M |  | Integer (0..1000) | Expressed in units of 1ms |
| >>Time Accuracy high range | M |  | Integer (2000..100000) | Expressed in units of 1ms. |

The above encoding would potentially give some flexibility to the gNB implementation when signaling the value range. However, the ms level accuracy is too coarse for most timing service use cases. Also, the enumerated encoding option which maps values from IEEE Std 1588 Table 5 seems to already provide enough flexibility for implementations.

Proposal 9:	The encoding of the Clock Accuracy IE should include multiple INTEGER type choices with different granularities (e.g., 10ns, 100ns and 1000ns) and one ENUMERATED type choice (values mapping to IEEE Std 1588, Table 5).

If proposal 2 is agreeable, then it completes the encoding of all attributes in the Timing Synchronisation Status Attributes IE. Since these attributes can be provided to the UE via dedicated RRC signalling when a UE’s clock quality detail level equals “clock quality metrics”, RAN3 should inform RAN2 (and SA2) of the encoding as well as any other relevant RAN3 agreements.

Proposal 10:	Send an LS to RAN2/SA2, informing them of the encoding of the Timing Synchronisation Status Attributes IE and any other relevant RAN3 agreements.

## 2.4	RAN TSS reporting control over F1AP

SA2 describes the use of pre-configured thresholds as follows in TS 23.501 clause 5.27.1.12:

[…] gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. When the network timing synchronization status exceeds the thresholds (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) with the gNB ID, the scope of the timing synchronization status (i.e. all cells or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status (TSS) attributes as described in this clause. The gNB indicates the status change to the UEs via SIB information: […] NOTE 1: it is assumed that the gNB does not provide clock quality metrics to the UE which is better than the pre-configured thresholds. NOTE 2: It is assumed the pre-configured thresholds in the gNB(s) are sufficient to meet UE time sync performance requirement which are configured by the operator.

The pre-configured thresholds are node-level attributes, not associated with a specific UE. In addition to the pre-configured thresholds, the gNB may receive UE-specific clock quality acceptance criteria when the clock quality detail level for the UE is “acceptable/not acceptable”. The NG-RAN split architecture brings additional considerations to ensure the acceptance criteria can be evaluated that SA2 specifications do not describe. For the gNB-CU to determine if the UE’s timing service is acceptable/not acceptable, it needs the non-UE associated RAN Timing Synchronization Status (TSS) reports from the gNB-DU. The gNB-CU correlates the current gNB-DU synchronization status and UE’s clock quality acceptance criteria to determine the “acceptable/not acceptable” result. To ensure the acceptance criteria can be evaluated at the gNB-CU, different cases can be assumed:

a)  The gNB-DU may be configured with multiple pre-configured thresholds per RAN TSS attribute that match all the potential acceptance criteria the UEs time synchronization services may have for the UEs the gNB-DU may serve. For example, if the operator allows different UEs to have different acceptance criteria for the clock accuracy attribute, then the gNB-DU must be pre-configured with multiple clock accuracy thresholds corresponding to all allowed values of the clock quality acceptance criteria.

b)  The gNB-DU is configured with a single pre-configured threshold for each timing synchronization status attribute, which is the strictest value. For example, in case of clock accuracy the gNB-DU is preconfigured with the smallest value which signifies that the gNB-DU is not expected to monitor or report any values less than the preconfigured threshold. Then, the gNB-CU may configure additional thresholds via F1AP signalling that correspond to any UE-specific clock quality acceptance criteria that are more relaxed than the pre-configured thresholds to ensure the gNB-DU will generate reports when a clock quality acceptance criteria for a UE or group of UEs is exceeded or meet again.

The goal is to keep the gNB-CU updated with the current status of the synchronization plane of the gNB-DU to evaluate the service status provided to the UEs. Figure 1 illustrates it for the case b). Note, in case a), the reporting can be assumed the same (black and red arrows) if multiple pre-configured thresholds are configured at the gNB-DU.

Figure 1: Example of RAN TSS reporting towards UE1 and UE2 based on UE-specific clock accuracy acceptance criteria

To our understanding, following NOTE 2 in TS 23.501 clause 5.27.1.12, the pre-configured thresholds configured at the gNB will be the strictest ones sufficient to meet all UE’s time synchronization criteria. However, for more relaxed acceptance criteria configured for specific UEs, the pre-configured thresholds would not be enough to ensure the gNB-CU gets enough updates to evaluate the acceptance criteria.

Observation-5:	It is unclear whether RAN3 can assume multiple pre-configured thresholds per RAN TSS attribute (corresponding to each value of acceptance criteria allowed by the operator), or a single pre-configured threshold per RAN TSS attribute (corresponding to the strictest value of acceptance criteria allowed by the operator).

Since this issue impacts F1AP specification, clarification is needed from SA2.

Proposal 11:	Send an LS to SA2 to ask whether RAN3 should assume:

- multiple pre-configured thresholds per RAN TSS attribute (one for each value of per-UE acceptance criteria that the gNB-CU might receive from the AMF), or

- a single pre-configured threshold per RAN TSS attribute (e.g., the strictest value that the gNB is expected to report).

# 3	Conclusions

In this paper, we evaluated open issues for timing resiliency and made the following proposals:

For NGAP:

Proposal 1:	In NGAP, introduce a new Timing Synchronisation Status procedure (class 1) to enable the AMF to initiate RAN TSS reporting by the gNB.

Proposal 2:	In NGAP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB to report RAN TSS to the AMF.

Proposal 3:	The (NGAP) TIMING SYNCHRONISATION STATUS REQUEST message includes the RAN TSS Request Type IE (start, stop).

Proposal 4:	The (NGAP) TIMING SYNCHRONISATION STATUS REPORT message includes the RAN Timing Synchronisation Status IE and the RAN TSS Scope IE (indicating whether the scope of the RAN TSS is “all cells within a single gNB” or “list of Cell IDs within a single gNB”).

For F1AP:

Proposal 5:	In F1AP, introduce two new procedures corresponding to the new NGAP procedures.

Proposal 6:	The (F1AP) TIMING SYNCHRONISATION STATUS REQUEST message includes the RAN TSS Request Type IE (start, stop).

Proposal 7:	The (F1AP) TIMING SYNCHRONISATION STATUS REPORT message includes the RAN Timing Synchronisation Status Information IE (containing all RAN TSS attributes supported by the gNB-DU).

For both NGAP and F1AP:

Proposal 8:	Encode the Clock Quality Acceptance Criteria IE as shown in the above table.

Proposal 9:	The encoding of the Clock Accuracy IE should include multiple INTEGER type choices with different granularities (e.g., 10ns, 100ns and 1000ns) and one ENUMERATED type choice (values mapping to IEEE Std 1588, Table 5).

For LS to other groups (which can be combined in a single LS):

Proposal 10:	Send an LS to RAN2/SA2, informing them of the encoding of the Timing Synchronisation Status Attributes IE and any other relevant RAN3 agreements.

Proposal 11:	Send an LS to SA2 to ask whether RAN3 should assume:

- multiple pre-configured thresholds per RAN TSS attribute (one for each value of per-UE acceptance criteria that the gNB-CU might receive from the AMF), or

- a single pre-configured threshold per RAN TSS attribute (e.g., the strictest value that the gNB is expected to report).

A draft LS is provided in Annex A, and an NGAP TP is provided in Annex B.

# References

- RP-231106 Revised WID on NR Timing Resiliency and URLLC enhancements, Nokia, Nokia Shanghai Bell

- R3-233815 (TP for TS 38.401 BL CR) Stage 2 for timing resiliency and URLLC, Nokia, Nokia Shanghai Bell, Samsung, Qualcomm, CATT

# Annex	A:	Draft LS

3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, France, 21 – 25 August 2023

Title:	LS on timing resiliency

Release:	Release 18

Work Item:	TRS_URLLC-NR

Source:	RAN3

To:	RAN2, SA2

Cc:	-

Contact Person:

#### Name:	Sean Kelley

E-mail Address:	sean.kelley@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 is currently discussing 5GS network timing synchronization status and reporting. RAN3 has reached agreement regarding encoding details of the RAN timing synchronization status attributes as shown below.

| IE Name | Encoding |
| --- | --- |
| Synchronisation State | ENUMERATED (locked, holdover, freeRun, …) |
| Traceable to UTC | ENUMERATED (true, false, …) |
| Traceable to GNSS | ENUMERATED (true, false, …) |
| Clock Frequency Stability | BIT STRING (SIZE (16)) |
| Clock Accuracy | CHOICE Clock Accuracy. >granularity 10ns. >>Clock Accuracy Value in 10ns; INTEGER (0..100000000, …); Clock accuracy expressed in units of 10 ns.. >granularity 100ns. >>Clock Accuracy Value in 100ns; INTEGER (0..10000000, …); Clock accuracy expressed in units of 100 ns.. >granularity 1us. >>Clock Accuracy Value in 1us; INTEGER (0..1000000, …); Clock accuracy expressed in units of 1 us.. >enumeration. >>Clock Accuracy Value Enumerated; ENUMERATED (25ns, 100ns, 250ns, 1us, 2dot5 us, 10us, 25us, 100us, 250us, 1ms, 2dot5ms, 10ms, 25ms, 100ms, 250ms, 1sec, …). |
| Parent Time Source | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |

In addition, RAN3 has discussed pre-configured thresholds at the gNB based on the following text in TS 23.501 clause 5.27.1.12:

gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. […] NOTE 1: it is assumed that the gNB does not provide clock quality metrics to the UE which is better than the pre-configured thresholds.

In case of CU-DU split architecture, RAN3 assumes that the gNB-CU evaluates the per-UE clock quality acceptance criteria received from the AMF while node-level thresholds are pre-configured at the gNB-DU. RAN3 has the following question:

Question to SA2: Should RAN3 assume that the gNB-DU is pre-configured with

- multiple thresholds per timing synchronization status attribute, one for each value of per-UE clock quality acceptance criteria that the gNB-CU might receive from the AMF; or

- a single threshold per timing synchronization status attribute, e.g., the strictest value that the gNB is expected to report?

Note that in case (b), there may be additional RAN3 protocol impacts to ensure that the gNB-CU is able to accurately evaluate the clock quality acceptance criteria for UEs that are more tolerant of clock quality degradation than the pre-configured threshold per timing synchronization status attribute.

2. Actions:

To RAN2, SA2.

ACTION: 	Please take into account RAN3 agreements on the encoding details of the RAN timing synchronization status attributes.

To SA2.

ACTION: 	RAN3 respectfully asks SA2 to reply to RAN3’s question.

3. Date of Next TSG-RAN WG3 Meeting:

RAN3#121bis	from 2023-10-09	to 2023-10-13		Xiamen

RAN3#122	from 2023-11-13	to 2023-11-17		Chicago

# Annex	B: Text Proposal for TS 38.413

first modification

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |

next modification

## 8.x	Timing Synchronisation Status Reporting Procedures

### 8.x.1	Timing Synchronisation Status

#### 8.x.1.1	General

The purpose of the Timing Synchronisation Status procedure is to enable the AMF to request the NG-RAN node to start or stop reporting of RAN timing synchronisation status information as specified in TS 23.501 [9] and TS 23.502 [10]. The procedure uses non-UE associated signalling.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: Timing synchronisation status procedure: successful operation

The AMF initiates the procedure by sending a TIMING SYNCHRONISATION STATUS REQUEST message to the NG-RAN node.

#### If the RAN TSS Request Type IE is set to “start” and the NG-RAN node is able to initiate reporting of at least one RAN timing synchronisation status attribute, the NG-RAN node shall reply with the TIMING SYNCHRONISATION STATUS RESPONSE message.

#### 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1: Timing synchronisation status procedure: unsuccessful operation

If the NG-RAN node is unable to initiate reporting for any RAN timing synchronisation status attributes, the NG-RAN node shall respond with the TIMING SYNCHRONISATION STATUS FAILURE message.

#### 8.x.1.4	Abnormal Conditions

Void.

### 8.x.2	Timing Synchronisation Status Report

#### 8.x.2.1	General

The purpose of the Timing Synchronisation Status Report procedure is to enable the NG-RAN node to provide RAN timing synchronisation status information to the AMF as specified in TS 23.501 [9] and TS 23.502 [10]. The procedure uses non-UE associated signalling.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Timing synchronisation status report

The NG-RAN node initiates the procedure by sending a TIMING SYNCHRONISATION STATUS REPORT message to the AMF.

#### 8.x.2.3	Abnormal Conditions

Void.

next modification

### 9.2.x	Timing Synchronisation Status Reporting Messages

#### 9.2.x.1	TIMING SYNCHRONISATION STATUS REQUEST

This message is sent by the AMF to start or stop reporting of RAN timing synchronisation status information.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN TSS Request Type | M |  | ENUMERATED (start, stop, …) |  | YES | ignore |

#### 9.2.x.2	TIMING SYNCHRONISATION STATUS RESPONSE

This message is sent by the NG-RAN node to indicate that the request to start or stop reporting of RAN timing synchronisation status information is successful.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.3	TIMING SYNCHRONISATION STATUS FAILURE

This message is sent by the NG-RAN node to indicate that reporting of RAN timing synchronisation status information cannot be initiated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.4	TIMING SYNCHRONISATION STATUS REPORT

This message is sent by the NG-RAN node to report previously requested RAN timing synchronisation status information.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Timing Synchronisation Status Information | M |  | 9.3.1.x3 |  | YES | ignore |
| TSS Scope | M |  | 9.3.1.x6 |  | YES | ignore |

next modification

#### 9.3.1.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (0..100000000, …) | Clock accuracy expressed in units of 10 ns. |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

#### 9.3.1.x3	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided towards the AMF as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | 9.3.1.x5 |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.x5	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >granularity 10ns |  |  |  |  |
| >>Clock Accuracy Value in 10ns | M |  | INTEGER (0..100000000, …) | Clock accuracy expressed in units of 10 ns. |
| >granularity 100ns |  |  |  |  |
| >>Clock Accuracy Value in 100ns | M |  | INTEGER (0..10000000, …) | Clock accuracy expressed in units of 100 ns. |
| >granularity 1us |  |  |  |  |
| >>Clock Accuracy Value in 1us | M |  | INTEGER (0..1000000, …) | Clock accuracy expressed in units of 1 us. |
| >enumeration |  |  |  |  |
| >>Clock Accuracy Value Enumerated | M |  | ENUMERATED (25ns, 100ns, 250ns, 1us, 2dot5 us, 10us, 25us, 100us, 250us, 1ms, 2dot5ms, 10ms, 25ms, 100ms, 250ms, 1sec, …) |  |

#### 9.3.1.x6	RAN TSS Scope

This IE indicates the scope of the RAN timing synchronisation status as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RAN TSS scope | M |  |  |  |
| >all cells |  |  |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >list of cells |  |  |  |  |
| >>RAN TSS Cell List |  | 1 |  |  |
| >>>RAN TSS Cell Item |  | 1..<maxnoofCellsTSS |  |  |
| >>>>NR CGI | M |  | 9.3.1.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTSS | Maximum no. of Cell IDs in the scope of the RAN timing synchronisation status. Value is FFS. |

next modification

### 9.4.3	Elementary Procedure Definitions

** unchanged text skipped **

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFCPRelocationIndication,

AMFStatusIndication,

BroadcastSessionModificationFailure,

BroadcastSessionModificationRequest,

BroadcastSessionModificationResponse,

BroadcastSessionReleaseRequest,

BroadcastSessionReleaseRequired,

BroadcastSessionReleaseResponse,

BroadcastSessionSetupFailure,

BroadcastSessionSetupRequest,

BroadcastSessionSetupResponse,

CellTrafficTrace,

ConnectionEstablishmentIndication,

DeactivateTrace,

DistributionReleaseRequest,

DistributionReleaseResponse,

DistributionSetupFailure,

DistributionSetupRequest,

DistributionSetupResponse,

DownlinkNASTransport,

DownlinkNonUEAssociatedNRPPaTransport,

DownlinkRANConfigurationTransfer,

DownlinkRANEarlyStatusTransfer,

DownlinkRANStatusTransfer,

DownlinkUEAssociatedNRPPaTransport,

ErrorIndication,

HandoverCancel,

HandoverCancelAcknowledge,

HandoverCommand,

HandoverFailure,

HandoverNotify,

HandoverPreparationFailure,

HandoverRequest,

HandoverRequestAcknowledge,

HandoverRequired,

HandoverSuccess,

InitialContextSetupFailure,

InitialContextSetupRequest,

InitialContextSetupResponse,

InitialUEMessage,

LocationReport,

LocationReportingControl,

LocationReportingFailureIndication,

MulticastSessionActivationFailure,

MulticastSessionActivationRequest,

MulticastSessionActivationResponse,

MulticastSessionDeactivationRequest,

MulticastSessionDeactivationResponse,

MulticastSessionUpdateFailure,

MulticastSessionUpdateRequest,

MulticastSessionUpdateResponse,

MulticastGroupPaging,

NASNonDeliveryIndication,

NGReset,

NGResetAcknowledge,

NGSetupFailure,

NGSetupRequest,

NGSetupResponse,

OverloadStart,

OverloadStop,

Paging,

PathSwitchRequest,

PathSwitchRequestAcknowledge,

PathSwitchRequestFailure,

PDUSessionResourceModifyConfirm,

PDUSessionResourceModifyIndication,

PDUSessionResourceModifyRequest,

PDUSessionResourceModifyResponse,

PDUSessionResourceNotify,

PDUSessionResourceReleaseCommand,

PDUSessionResourceReleaseResponse,

PDUSessionResourceSetupRequest,

PDUSessionResourceSetupResponse,

PrivateMessage,

PWSCancelRequest,

PWSCancelResponse,

PWSFailureIndication,

PWSRestartIndication,

RANConfigurationUpdate,

RANConfigurationUpdateAcknowledge,

RANConfigurationUpdateFailure,

RANCPRelocationIndication,

RerouteNASRequest,

RetrieveUEInformation,

RRCInactiveTransitionReport,

SecondaryRATDataUsageReport,

TimingSynchronisationStatusFailure,

TimingSynchronisationStatusReport,

TimingSynchronisationStatusRequest,

TimingSynchronisationStatusResponse,

TraceFailureIndication,

TraceStart,

UEContextModificationFailure,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextReleaseRequest,

UEContextResumeRequest,

UEContextResumeResponse,

UEContextResumeFailure,

UEContextSuspendRequest,

UEContextSuspendResponse,

UEContextSuspendFailure,

UEInformationTransfer,

UERadioCapabilityCheckRequest,

UERadioCapabilityCheckResponse,

UERadioCapabilityIDMappingRequest,

UERadioCapabilityIDMappingResponse,

UERadioCapabilityInfoIndication,

UETNLABindingReleaseRequest,

UplinkNASTransport,

UplinkNonUEAssociatedNRPPaTransport,

UplinkRANConfigurationTransfer,

UplinkRANEarlyStatusTransfer,

UplinkRANStatusTransfer,

UplinkUEAssociatedNRPPaTransport,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

UplinkRIMInformationTransfer,

DownlinkRIMInformationTransfer

FROM NGAP-PDU-Contents

id-AMFConfigurationUpdate,

id-AMFCPRelocationIndication,

id-AMFStatusIndication,

id-BroadcastSessionModification,

id-BroadcastSessionRelease,

id-BroadcastSessionReleaseRequired,

id-BroadcastSessionSetup,

id-CellTrafficTrace,

id-ConnectionEstablishmentIndication,

id-DeactivateTrace,

id-DistributionSetup,

id-DistributionRelease,

id-DownlinkNASTransport,

id-DownlinkNonUEAssociatedNRPPaTransport,

id-DownlinkRANConfigurationTransfer,

id-DownlinkRANEarlyStatusTransfer,

id-DownlinkRANStatusTransfer,

id-DownlinkUEAssociatedNRPPaTransport,

id-ErrorIndication,

id-HandoverCancel,

id-HandoverNotification,

id-HandoverPreparation,

id-HandoverResourceAllocation,

id-HandoverSuccess,

id-InitialContextSetup,

id-InitialUEMessage,

id-LocationReport,

id-LocationReportingControl,

id-LocationReportingFailureIndication,

id-MulticastSessionActivation,

id-MulticastSessionDeactivation,

id-MulticastSessionUpdate,

id-MulticastGroupPaging,

id-NASNonDeliveryIndication,

id-NGReset,

id-NGSetup,

id-OverloadStart,

id-OverloadStop,

id-Paging,

id-PathSwitchRequest,

id-PDUSessionResourceModify,

id-PDUSessionResourceModifyIndication,

id-PDUSessionResourceNotify,

id-PDUSessionResourceRelease,

id-PDUSessionResourceSetup,

id-PrivateMessage,

id-PWSCancel,

id-PWSFailureIndication,

id-PWSRestartIndication,

id-RANConfigurationUpdate,

id-RANCPRelocationIndication,

id-RerouteNASRequest,

id-RetrieveUEInformation,

id-RRCInactiveTransitionReport,

id-SecondaryRATDataUsageReport,

id-TimingSynchronisationStatus,

id-TimingSynchronisationStatusReport,

id-TraceFailureIndication,

id-TraceStart,

id-UEContextModification,

id-UEContextRelease,

id-UEContextReleaseRequest,

id-UEContextResume,

id-UEContextSuspend,

id-UEInformationTransfer,

id-UERadioCapabilityCheck,

id-UERadioCapabilityIDMapping,

id-UERadioCapabilityInfoIndication,

id-UETNLABindingRelease,

id-UplinkNASTransport,

id-UplinkNonUEAssociatedNRPPaTransport,

id-UplinkRANConfigurationTransfer,

id-UplinkRANEarlyStatusTransfer,

id-UplinkRANStatusTransfer,

id-UplinkUEAssociatedNRPPaTransport,

id-WriteReplaceWarning,

id-UplinkRIMInformationTransfer,

id-DownlinkRIMInformationTransfer

FROM NGAP-Constants;

** unchanged text skipped **

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURES NGAP-ELEMENTARY-PROCEDURE ::= {

NGAP-ELEMENTARY-PROCEDURES-CLASS-1			|

NGAP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

distributionSetup				|

distributionRelease				|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

multicastSessionActivation					|

multicastSessionDeactivation				|

multicastSessionUpdate			|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

timingSynchronisationStatus		|

uEContextModification			|

uEContextRelease				|

uEContextResume								|

uEContextSuspend							|

uERadioCapabilityCheck			|

uERadioCapabilityIDMapping		|

writeReplaceWarning,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-2 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFCPRelocationIndication		|

aMFStatusIndication				|

broadcastSessionReleaseRequired	|

cellTrafficTrace				|

connectionEstablishmentIndication	|

deactivateTrace					|

downlinkNASTransport			|

downlinkNonUEAssociatedNRPPaTransport	|

downlinkRANConfigurationTransfer	|

downlinkRANEarlyStatusTransfer	|

downlinkRANStatusTransfer		|

downlinkRIMInformationTransfer	|

downlinkUEAssociatedNRPPaTransport	|

errorIndication					|

handoverNotification			|

handoverSuccess								|

initialUEMessage				|

locationReport					|

locationReportingControl		|

locationReportingFailureIndication	|

multicastGroupPaging			|

nASNonDeliveryIndication		|

overloadStart					|

overloadStop					|

paging							|

pDUSessionResourceNotify		|

privateMessage					|

pWSFailureIndication						|

pWSRestartIndication			|

rANCPRelocationIndication		|

rerouteNASRequest				|

retrieveUEInformation						|

rRCInactiveTransitionReport					|

secondaryRATDataUsageReport		|

timingSynchronisationStatusReport	|

traceFailureIndication			|

traceStart						|

uEContextReleaseRequest			|

uEInformationTransfer						|

uERadioCapabilityInfoIndication	|

uETNLABindingRelease			|

uplinkNASTransport				|

uplinkNonUEAssociatedNRPPaTransport	|

uplinkRANConfigurationTransfer	|

uplinkRANEarlyStatusTransfer				|

uplinkRANStatusTransfer			|

uplinkRIMInformationTransfer				|

uplinkUEAssociatedNRPPaTransport,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

** unchanged text skipped **

secondaryRATDataUsageReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-SecondaryRATDataUsageReport

CRITICALITY				ignore

}

timingSynchronisationStatus NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TimingSynchronisationStatusRequest

SUCCESSFUL OUTCOME		TimingSynchronisationStatusResponse

UNSUCCESSFUL OUTCOME	TimingSynchronisationStatusFailure

PROCEDURE CODE			id-TimingSynchronisationStatus

CRITICALITY				reject

}

timingSynchronisationStatusReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TimingSynchronisationStatusReport

PROCEDURE CODE			id-TimingSynchronisationStatusReport

CRITICALITY				ignore

}

traceFailureIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceFailureIndication

PROCEDURE CODE			id-TraceFailureIndication

CRITICALITY				ignore

}

** unchanged text skipped **

END

-- ASN1STOP

next modification

### 9.4.4	PDU Definitions

** unchanged text skipped **

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

** unchanged text skipped **

RANStatusTransfer-TransparentContainer,

RAN-TSS-Information,

RAN-TSS-RequestType,

RAN-TSS-Scope,

RAN-UE-NGAP-ID,

RedCapIndication,

RedirectionVoiceFallback,

RelativeAMFCapacity,

RepetitionPeriod,

ResetType,

RGLevelWirelineAccessCharacteristics,

RoutingID,

RRCEstablishmentCause,

RRCInactiveTransitionReportRequest,

RRCState,

SecurityContext,

SecurityKey,

SerialNumber,

ServedGUAMIList,

SliceSupportList,

S-NSSAI,

SONConfigurationTransfer,

SourceToTarget-TransparentContainer,

SourceToTarget-AMFInformationReroute,

SRVCCOperationPossible,

SupportedTAList,

Suspend-Request-Indication,

Suspend-Response-Indication,

TAI,

TAIListForPaging,

TAIListForRestart,

TargetID,

TargetNSSAIInformation,

TargetToSource-TransparentContainer,

TargettoSource-Failure-TransparentContainer,

TimeSyncAssistanceInfo,

TimeToWait,

TNLAssociationList,

TraceActivation,

TrafficLoadReductionIndication,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-associatedLogicalNG-connectionList,

UECapabilityInfoRequest,

UEContextRequest,

UE-DifferentiationInfo,

UE-NGAP-IDs,

UEPagingIdentity,

UEPresenceInAreaOfInterestList,

UERadioCapability,

UERadioCapabilityForPaging,

UERadioCapabilityID,

UERetentionInformation,

UESecurityCapabilities,

UESliceMaximumBitRateList,

UE-UP-CIoT-Support,

UL-CP-SecurityInformation,

UnavailableGUAMIList,

URI-address,

UserLocationInformation,

WarningAreaCoordinates,

WarningAreaList,

WarningMessageContents,

WarningSecurityInfo,

WarningType,

WUS-Assistance-Information,

RIMInformationTransfer

FROM NGAP-IEs

** unchanged text skipped **

id-RANStatusTransfer-TransparentContainer,

id-RAN-TSS-Information,

id-RAN-TSS-RequestType,

id-RAN-TSS-Scope,

id-RAN-UE-NGAP-ID,

id-RedCapIndication,

id-RedirectionVoiceFallback,

id-RelativeAMFCapacity,

id-RepetitionPeriod,

id-ResetType,

id-RGLevelWirelineAccessCharacteristics,

id-RoutingID,

id-RRCEstablishmentCause,

id-RRCInactiveTransitionReportRequest,

id-RRC-Resume-Cause,

id-RRCState,

id-SecurityContext,

id-SecurityKey,

id-SelectedPLMNIdentity,

id-SerialNumber,

id-ServedGUAMIList,

id-SliceSupportList,

id-S-NSSAI,

id-SONConfigurationTransferDL,

id-SONConfigurationTransferUL,

id-SourceAMF-UE-NGAP-ID,

id-SourceToTarget-TransparentContainer,

id-SourceToTarget-AMFInformationReroute,

id-SRVCCOperationPossible,

id-SupportedTAList,

id-Suspend-Request-Indication,

id-Suspend-Response-Indication,

id-TAI,

id-TAIListForPaging,

id-TAIListForRestart,

id-TargetID,

id-TargetNSSAIInformation,

id-TargetToSource-TransparentContainer,

id-TargettoSource-Failure-TransparentContainer,

id-TimeSyncAssistanceInfo,

id-TimeToWait,

id-TNGFIdentityInformation,

id-TraceActivation,

id-TraceCollectionEntityIPAddress,

id-TraceCollectionEntityURI,

id-TWIFIdentityInformation,

id-UEAggregateMaximumBitRate,

id-UE-associatedLogicalNG-connectionList,

id-UECapabilityInfoRequest,

id-UEContextRequest,

id-UE-DifferentiationInfo,

id-UE-NGAP-IDs,

id-UEPagingIdentity,

id-UEPresenceInAreaOfInterestList,

id-UERadioCapability,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityID,

id-UERadioCapability-EUTRA-Format,

id-UERetentionInformation,

id-UESecurityCapabilities,

id-UESliceMaximumBitRateList,

id-UE-UP-CIoT-Support,

id-UL-CP-SecurityInformation,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-W-AGFIdentityInformation,

id-WarningAreaCoordinates,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

id-WUS-Assistance-Information,

id-RIMInformationTransfer

FROM NGAP-Constants;

** unchanged text skipped **

-- **************************************************************

--

##### -- MULTICAST GROUP PAGING

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastGroupPagingIEs} },

...

}

MulticastGroupPagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY ignore	TYPE MBS-SessionID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY ignore	TYPE MBS-ServiceArea					PRESENCE optional		}|

{ ID id-MulticastGroupPagingAreaList			CRITICALITY ignore	TYPE MulticastGroupPagingAreaList	PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- TIMING SYNCHRONISATION STATUS REPORTING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- Timing Synchronisation Status Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS REQUEST

--

-- **************************************************************

TimingSynchronisationStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TimingSynchronisationStatusRequestIEs} },

...

}

TimingSynchronisationStatusRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-TSS-RequestType			CRITICALITY ignore	TYPE RAN-TSS-RequestType			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS RESPONSE

--

-- **************************************************************

TimingSynchronisationStatusResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TimingSynchronisationStatusResponseIEs} },

...

}

TimingSynchronisationStatusIEs NGAP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS FAILURE

--

-- **************************************************************

TimingSynchronisationStatusFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TimingSynchronisationStatusFailureIEs} },

...

}

TimingSynchronisationStatusFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- Timing Synchronisation Status Reporting Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- TIMING SYNCHRONISATION STATUS REPORT

--

-- **************************************************************

TimingSynchronisationStatusReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TimingSynchronisationStatusReportIEs} },

...

}

TimingSynchronisationStatusReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-TSS-Information			CRITICALITY ignore	TYPE RAN-TSS-Information		PRESENCE optional	}|

{ ID id-RAN-TSS-Scope				CRITICALITY ignore	TYPE RAN-TSS-Scope				PRESENCE optional	},

...

}

END

-- ASN1STOP

next modification

### 9.4.5	Information Element Definitions

** unchanged text skipped **

-- R

Range ::= ENUMERATED {m50, m80, m180, m200, m350, m400, m500, m700, m1000, ...}

RANNodeName ::= PrintableString (SIZE(1..150, ...))

RANNodeNameVisibleString ::= VisibleString (SIZE(1..150, ...))

RANNodeNameUTF8String ::= UTF8String (SIZE(1..150, ...))

RANPagingPriority ::= INTEGER (1..256)

RANStatusTransfer-TransparentContainer ::= SEQUENCE {

dRBsSubjectToStatusTransferList		DRBsSubjectToStatusTransferList,

iE-Extensions		ProtocolExtensionContainer { {RANStatusTransfer-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

RAN-TSS-CellList ::= SEQUENCE (SIZE(1.. maxnoofCellsTSS)) OF NR-CGI

RAN-TSS-RequestType ::= ENUMERATED {start, stop, ...}

RAN-TSS-Scope ::= CHOICE {

allCells				RAN-TSS-ScopeAllCells

listOfCells				RAN-TSS-ScopeListOfCells

choice-Extensions		ProtocolIE-SingleContainer { {RAN-TSS-Scope-ExtIEs} }

}

RAN-TSS-Scope-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

RAN-TSS-ScopeAllCells ::= SEQUENCE {

globalGNB-ID			GlobalGNB-ID,

iE-Extensions			ProtocolExtensionContainer { {RAN-TSS-ScopeAllCells-ExtIEs} } OPTIONAL,

...

}

RAN-TSS-ScopeAllCells-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RAN-TSS-ScopeListOfCells ::= SEQUENCE {

rAN-TSS-CellList		RAN-TSS-CellList,

iE-Extensions			ProtocolExtensionContainer { {RAN-TSS-ScopeListOfCells-ExtIEs} } OPTIONAL,

...

}

RAN-TSS-ScopeListOfCells-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

last modification

### 9.4.7	Constant Definitions

** unchanged text skipped **

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-TimingSynchronisationStatus				ProcedureCode ::= z1

id-TimingSynchronisationStatusReport		ProcedureCode ::= z2

** unchanged text skipped **

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAllowedCAGsperPLMN					INTEGER ::= 256

maxnoofAllowedS-NSSAIs						INTEGER ::= 8

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGSperCell							INTEGER ::= 64

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellIDforWarning						INTEGER ::= 65535

maxnoofCellinAoI							INTEGER ::= 256

maxnoofCellinEAI							INTEGER ::= 65535

maxnoofCellinTAI							INTEGER ::= 65535

maxnoofCellsforMBS							INTEGER ::= 8192

maxnoofCellsingNB							INTEGER ::= 16384

maxnoofCellsinngeNB							INTEGER ::= 256

maxnoofCellsinNGRANNode						INTEGER ::= 16384

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsTSS								INTEGER ::= FFS

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEmergencyAreaID						INTEGER ::= 65535

maxnoofEAIforRestart						INTEGER ::= 256

maxnoofEPLMNs								INTEGER ::= 15

maxnoofEPLMNsPlusOne						INTEGER ::= 16

maxnoofE-RABs								INTEGER ::= 256

maxnoofErrors								INTEGER ::= 256

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofForbTACs								INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSAreaSessionIDs					INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 64

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMBSSessions							INTEGER ::= 32

maxnoofMBSSessionsofUE						INTEGER ::= 256

maxnoofMBSServiceAreaInformation			INTEGER ::= 256

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMRBs									INTEGER ::= 32

maxnoofMultiConnectivity					INTEGER ::= 4

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNGAPIESupportInfo					INTEGER ::= 32

maxnoofNGConnectionsToReset					INTEGER ::= 65536

maxnoofNRCellBands							INTEGER ::= 32

maxnoofNSAGs								INTEGER ::= 256

maxnoofPagingAreas							INTEGER ::= 64

maxnoofPC5QoSFlows 							INTEGER ::= 2048

maxnoofPDUSessions							INTEGER ::= 256

maxnoofPLMNs								INTEGER ::= 12

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo	INTEGER ::= 8

maxnoofQosFlows								INTEGER ::= 64

maxnoofQosParaSets							INTEGER ::= 8

maxnoofRANNodeinAoI							INTEGER ::= 64

maxnoofRecommendedCells						INTEGER ::= 16

maxnoofRecommendedRANNodes					INTEGER ::= 16

maxnoofAoI									INTEGER ::= 64

maxnoofReportedCells						INTEGER ::= 256

maxnoofSensorName							INTEGER ::= 3

maxnoofServedGUAMIs							INTEGER ::= 256

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofTACs									INTEGER ::= 256

maxnoofTACsinNTN							INTEGER ::= 12

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAIforInactive						INTEGER ::= 16

maxnoofTAIforMBS							INTEGER ::= 1024

maxnoofTAIforPaging							INTEGER ::= 16

maxnoofTAIforRestart						INTEGER ::= 2048

maxnoofTAIforWarning						INTEGER ::= 65535

maxnoofTAIinAoI								INTEGER ::= 16

maxnoofTimePeriods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEsforPaging							INTEGER ::=	4096

maxnoofWLANName								INTEGER ::= 4

maxnoofXnExtTLAs							INTEGER ::= 16

maxnoofXnGTP-TLAs							INTEGER ::= 16

maxnoofXnTLAs								INTEGER ::= 2

maxnoofCandidateCells						INTEGER ::= 32

maxnoofTargetS-NSSAIs						INTEGER ::= 8

maxNRARFCN									INTEGER ::= 3279165

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

** unchanged text skipped **

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-RAN-TSS-Information									ProtocolIE-ID ::= xx1

id-RAN-TSS-RequestType									ProtocolIE-ID ::= xx2

id-RAN-TSS-Scope										ProtocolIE-ID ::= xx3

END

-- ASN1STOP