3GPP TSG RAN WG3 Meeting#121		R3-233824

Toulouse, France, August 21st – 25th, 2023

Agenda item:	13.3

Source:	KT Corp.

Title:	Discussion on mobility enhancements for mobile-IAB

Document for:	Discussion

- Introduction

In the last RAN3 #120 meeting, following was agreed and further discussion needs to be made in this meeting [1]:

The mIAB-DU’s NCGI is configured by OAM, and, e.g. to avoid CGI collision, it may be re-configured by the donor CU via F1 based on a list of NCGIs that has been configured on this donor CU by OAM or by pre-configuration. This should not affect the existing procedure of configuring NCGI of cells served by a stationary DU via OAM. The underlying assumption is that the DU´s OAM has visibility on the result of the CU-based CGI re-configuration. It needs to be further discussed how to ensure that such observability is supported. RAN3 to send an LS to SA5 including the content of the agreement. Explain the status quo in RAN3 concerning the use cases discussed. Ask SA5 how to ensure that OAM has visibility over the CGI reconfiguration decisions, as well as on feasibility and feedback of the solution and requirements agreed. To be further discussed whether any further questions need to be posed to SA5 Use cases requiring CGI re-configuration independent of CGI collision events needs to be further discussed and proven. If no other use cases than CGI collisions can be identified, the proposal above applies only to CGI collision avoidance Discussions on enhancements to legacy HO procedures to be taken at RAN3#121. If no consensus on such enhancements, the following will be agreed: For DU migration, the following baseline is enough for target CU being aware of the QoS profile for each UE traffic: The target CU for mIAB-DU migration learns the traffic profile of the UE traffic from Handover Preparation procedures for individual UEs. No further enhancements are needed.

In this contribution, above issue from the last RAN3 meeting is further discussed.

- Discussion

From the agreement from the previous meeting, m-IAB DU’s NCGI is configured by OAM and may be re-configured by the donor CU via F1 based on the list of NCGIs that has been configured on this donor CU by OAM or by pre-configuration. From this agreement, we can assume that for the vehicle mounted with m-IAB traveling only on the dedicated route such as shuttle bus, trams, etc. it is better to have the list of NCGIs pre-configured in order to improve the handover performance.

Observation 1: For the vehicles moving only on the dedicated route, OAM configuration can be applied and other cases CU configuration can be applied

Proposal 1: Mechanism for both OAM-based and CU-based configuration is specified

Above agreement explicitly indicating the case that to avoid CGI collision, it may be re-configured by the donor CU. This could be useful when dedicated route is moving across the borders and global identity containing MCC and MNC should be included to indicate which country this cell belongs to. However, for the cases when dedicated route does not travelling across borders or relatively short route (e.g. between airport terminals, Incheon International airport to Gimpo Domestic airport, etc.) the list could be further shortened or reduced in overall amount of the data stored to improve the performance of handover. In this case, having the list of PCIs for pre-configuration could be enough to deal with m-IAB travelling on the vehicle with short dedicated routes.

Observation 2: For the vehicles moving on the short dedicated route, list of full NCGIs are not necessary

Proposal 2: Allow OAM to pre-configure the list with shorter gNB identifier such as PCIs

For the case when CU-based configuration is used, we need to make sure that the target CU supports m-IAB node in advance. In this way, source CU will decide whether to send HO request to the target donor CU. For the benefit of operator, KT sees strong benefit of explicit indication of “m-IAB node” so that the target CU performs necessary admission control.

Observation 3: It is beneficial for operators to know the support of “m-IAB node” in advance to explicitly indicate “m-IAB node”

Proposal 3: Explicitly indicate “m-IAB node” and/or explicit “m-IAB-node-authorized indication” needs to be included in the HO request for the m-IAB-MT

- Conclusion

Based on the discussion in the above section, the following proposal is suggested.

Proposal 1: Mechanism for both OAM-based and CU-based configuration is specified

Proposal 2: Allow OAM to pre-configure the list with shorter gNB identifier such as PCIs

Proposal 3: Explicitly indicate “m-IAB node” and/or explicit “m-IAB-node-authorized indication” needs to be included in the HO request for the m-IAB-MT

- Reference

- R3-233701, RAN3 #121 Meeting Agenda. 3GPP RAN3 #121