3GPP TSG-RAN WG3 #121								R3-233832

Toulouse, France,21th – 25th Aug 2023

Agenda Item:	20.2

Source:	China Telecom

Title: 	Remaining Issues on Supporting of MT-SDT

Document for:	Discussion

Introduction

In RAN3#121 meeting, the following open issues on MT-SDT still need to be discussed:

- FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method.

- FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method.

- FFS on how to calculate MT-SDT Data Size for SDT DL data packets.

This paper aims to further discuss these open issues and provide our views.

Discussion

Data Size Calculation for DL SDT Data Packets

The MT-SDT data size as a useful information was agreed to be delivered over E1 and Xn interface. With this information, the receiving node and last serving node can decide whether to initiate paging message with MT-SDT indicator. In Rel-17 MO-SDT, how to calculate UL data volume in UE side was discussed in RAN2. The following agreements on the details of UL data volume calculation were made:

- RAN2#115-e meeting:

- 1.Data volume used for SDT selection criteria is calculated as the total sum of Buffer Size across SDT RBs (i.e. same approach as BSR)

- 4.For SDT procedure selection, Same data volume threshold is used for CG-SDT and RA-SDT

- RAN2#116e meeting:

- PDCP header is not considered for the SDT data volume calculation. (23/23). No spec change is needed.

- Buffered packets in PDCP/RLC entities should be counted in SDT data volume calculation. (21/23). Whether and how to avoid any buffered packets in PDCP/RLC entities at the time of SDT data volume calculation is FFS.

- RAN2#116Bis-e

- 2.For both DRBs and SRBs configured with SDT, RAN2 confirm that at the time of SDT data volume calculation, there should be no buffered packets in PDCP/RLC entities that will not be transmitted during SDT procedure.

- 3.For DRBs configured with SDT, PDCP suspend is performed upon reception of RRCRelease message including suspendConfig so that PDCP PDUs are discarded, and PDCP SDUs already stored are considered in SDT data volume calculation. No specification change is needed.

- 13.The size of CCCH message is not considered in SDT data volume calculation

According to above agreements, data volume calculation for MO-SDT only consider SDT DRBs. In view of the fact that RAN3 had concluded that the size of CCCH message is considered in data size volume calculation over Xn interface, this paper only discusses the data volume calculation for SDT DRBs. For DL user plane data packets, the similar data volume calculation mechanism can be applied in MT-SDT, i.e., the PDCP header, RLC header, and MAC header, are not considered in DL SDT data volume calculation. Since the SDAP SDUs are delivered in data forwarding procedure from the last serving gNB to the receiving gNB, there is no need to add the size of SDAP header into data volume calculation. In addition, this solution can be also applied to the support of SDT in eDRX. Therefore, we propose to not take SDAP header and PDCP header in DL SDT data volume calculation.

Proposal 1: Not take SDAP header and PDCP header in DL SDT data volume calculation

Non MT-SDT data arrival during ongoing SDT session

The issue on how to address the non-SDT data arrival during ongoing SDT session was raised and discussed in last meeting. In principle, the last serving gNB should terminate the SDT procedure in case non-SDT data is received immediately. Therefore, it is proposed to add stage 2 description on how to address the non-SDT data arrival during ongoing SDT session in TS38.300 for Rel-17 and Rel-18.

Proposal 2: It is proposed to add stage 2 description on how to address the non-SDT data arrival during ongoing SDT session in TS38.300 for Rel-17 and Rel-18.

For CP/UP split architecture, there are two options on how to inform the gNB-CU-CP about the new non-SDT data arrival during ongoing SDT session:

- Option 1: to send a new explicit IE in DL Data Notification

- Option 2: to send the existing DL Data Notification without MT-SDT information

Both solutions are workable. For Rel-17 MO-SDT, Option 2 is more straightforward and has less impact on E1 interface. In order to align with Rel-17 MO-SDT, we propose to support Option2 in Rel-18 MT-SDT. Certainly, there is nothing explicit on current specification regarding whether gNB-CU-UP needs to send a new DL Data Notification in case a new data packet arrives. Thus, in order to make specification clearer, it is proposed to procedure text on new non-SDT data arrival during ongoing SDT session in TS37.483.

Proposal 3: It is proposed to send the existing DL Data Notification without MT-SDT information when the new non-SDT data arrives during ongoing SDT session, and to add procedure text in TS37.483.

MT-SDT Oversize Data

Similar to non SDT data arrival, we propose to send the existing DL Data Notification without MT-SDT information when the MT-SDT oversize data arrives during ongoing SDT session, and to add procedure text in TS37.483. In addition, there is no need to configure the value of oversize data from gNB-CU-CP to gNB-CU-UP. We don’t think this value is UE-specific parameter. This value can be configured by OAM.

Proposal 4: It is proposed to send the existing DL Data Notification without MT-SDT information when the new MT-SDT oversize data arrives during ongoing SDT session, and to add procedure text in TS37.483.

Proposal 5: There is no need to configure the value of oversize data from gNB-CU-CP to gNB-CU-UP.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: Not take SDAP header and PDCP header in DL SDT data volume calculation

Proposal 2: It is proposed to add stage 2 description on how to address the non-SDT data arrival during ongoing SDT session in TS38.300 for Rel-17 and Rel-18.

Proposal 3: It is proposed to send the existing DL Data Notification without MT-SDT information when the new non-SDT data arrives during ongoing SDT session, and to add procedure text in TS37.483.

Proposal 4: It is proposed to send the existing DL Data Notification without MT-SDT information when the new MT-SDT oversize data arrives during ongoing SDT session, and to add procedure text in TS37.483.

Proposal 5: There is no need to configure the value of oversize data from gNB-CU-CP to gNB-CU-UP.

Reference

- RP-213583，New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR，ZTE Corporation