3GPP TSG-RAN WG3 Meeting #121	R3-233854

Toulouse, France, 21 – 25 August, 2023

Agenda item:	20.2

Source:	Nokia, Nokia Shanghai Bell, Orange

Title:	Completion of MT-SDT Open points

Document for:	Discussion and Decision

# 1	Introduction

The work item on Mobile Terminated Small Data Transmission has been agreed in [2].

The RAN3 scope has been defined as:

Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3]

- MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response;

- MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.

Note: Data transmission in DL within paging message is not in scope of this WI.

At RAN3#119bis meeting the following further agreements were taken, with associated open points indicated in blue colour:

For E1AP:

- Include an MT-SDT Information Request IE as optional IE in the E1AP: BEARER CONTEXT SETUP REQUEST/ MODIFICATION message to request the report of MT-SDT Information for bearers configured as SDT bearers

- The encoding and the name of MT-SDT information IE in E1AP DL DATA NOTIFICATION message include MT-SDT Data Size IE (Mandatory, INTEGER (1…96000, …). FFS on whether MT-SDT indicator IE is needed.

For XnAP

- RAN Paging message includes the MT-SDT Data Size IE. FFS on the MT-SDT indicator IE and the presence of MT-SDT Data Size IE

- Agree to reusing existing IE (i.e., SDT Support Request) within the XnAP Retrieve Context Request message when the UE resumes for MT-SDT, and there is no RAN3 standard impact

For F1AP

- The encoding and the name of MT-SDT information IE in F1AP: Paging message include MT-SDT indicator IE (Mandatory). FFS on MT-SDT Data Size IE.

For non-SDT data arrival

- For the issue on DL non-SDT data arrives during the ongoing MT-SDT procedure, RAN3 waits for RAN2 on whether any signaling enhancements are needed

- RAN3 acknowledges the case that DL non-SDT data arrives at the last serving gNB following the MT-SDT paging procedure before receiving UE Context Retrieval Request message. It is FFS whether it is left to gNB implementation, or reusing existing IE(s), or introducing a new IE.

At RAN3#120 the following agreements were further taken:

- In E1AP: MT-SDT indicator IE within MT-SDT information IE is not needed.

- In XnAP: MT-SDT data size calculation includes total of both SDT signalling and SDT user plane data. Try to capture it into the TP.

- In XnAP: Both MT-SDT indicator IE and MT-SDT Data Size IE are “Mandatory”.

- In F1AP: MT-SDT indicator IE is “Mandatory”.

- In F1AP: gNB-CU makes the MT-SDT decision, gNB-DU shall follow the decision.

- When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message.

In this paper, we look at the remaining open issues:

# 2	Discussion

Handling arrival of Non-SDT Data

The use case is data arriving at CU UP over a bearer which has not been configured for SDT, In this case it is common view that the UE should turn back to RRC connected mode. The anchor CU CP can generate an RRC release message towards the UE via the new gNB. There is no difference compared to release 17 handling of non-SDT data.

Actually, after the MT-SDT transaction has started, there is no difference with MO-SDT transaction and therefore same solution can apply.

Proposal 1: upon non-SDT data arrival, same solution as release 17 applies.

Currently the DL DATA NOTIFICATION message contains the list of QFIs for which data has arrived. The anchor CU CP knows the mapping of such QFIs into DRBs and can determine whether those DRBs have been configured for SDT or not. Therefore, no additional signalling is needed for this case.

Proposal 2: for non-SDT data arrival, no additional signalling is needed in the E1AP DL DATA NOTIFICATION message.

Handling Arrival of SDT data with large volume

Another use case was brought at last RAN3#120 by tdoc 2865 referred to as case number 3 which is the reception of subsequent large data:

The following special cases requiring fast termination of SDT will lead to some interactions with the corresponding gNB-CU-CP:

- Case 1: All DL SDT data has been sent and there is no newer DL SDT data arrival

  - for each SDT DRB, set the DRB Activity IE as ‘Not active’ in the BEARER CONTEXT INACTIVITY NOTIFICATION message.

- Case 2: Non-SDT data arrival during ongoing SDT session

  - existing DL DATA NOTIFICATION message could be sent without the new MT-SDT Information IE.

- Case 3: Subsequent DL SDT data becomes large enough

  - new indication in the MT-SDT Information IE in the DL DATA NOTIFICATION message?

The case 3 was left pending at last RAN3 meeting.

In this case also it is beneficial to move the UE to RRC connected. However, this case cannot be discriminated by the QFI received in the E1AP DL DATA NOTIFICATION not by any other IE. Therefore, some new indication is needed.

Proposal 3: add new IE in the DL DATA NOTIFICATION message to signal arrival of large size DL SDT data.

The next question is when and how to define large size DL SDT data?

It seems logical that the anchor CU CP is the node to decide whether it is interested to send the UE to RRC connected state in case of large DL SDT data and which threshold should be considered.

We therefore propose that the CU CP:

- configures the CU UP to report upon large DL SDT size,

- configures the CU UP with the threshold upon which the DL SDT data size should be reported by CU UP to CU CP.

Proposal 4: CU CP configures CU UP whether to report large SDT DL data size and upon which threshold.

RAN3 has already introduced one IE in the E1AP Bearer Modification Request to request the report of MT-SDT Information.

| MT-SDT Information Request | O |  | ENUMERATED (true, …) | Indicates to request the report of MT-SDT Information. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

The report of SDT information happens in the DL Data Notification:

#### 9.3.1.xxx	MT-SDT Information

This IE provides the assistant information for MT-SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Data Size | M |  | INTEGER (1..96000,…) | Indicates the total data size for all SDT bearers. Unit: byte. |

However, once MT-SDT has been initiated, a regular SDT transaction takes place, which is same as when started for MO-SDT.

In that sense, the detection of arrival of DL SDT large data size is not specific to MT-SDT but applies to all SDT transactions i.e. also the ones which started as MO-SDT.

Therefore, it is cleaner to introduce new IEs which are not tied to MT-SDT but related to SDT in general. Example is shown below:

#### 9.3.1.y	SDT Information Request

This IE provides the assistant information request for SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| DL SDT Large Data Threshold | O |  | INTEGER (1..1920000,…) | Indicates to request to report of Large DL SDT data crossing the indicated threshold. |

#### 9.3.1.z	SDT Information

This IE provides the assistant information for SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| DL SDT Data Size Threshold Crossed | O |  | ENUMERATED (true, …) |  |

Proposal 5: Introduce new DL SDT Large Data Threshold IE within an SDT Information Request IE for CU CP to control the sending of the Large DL SDT data size indication by CU UP and a new DL SDT Data Size Threshold Crossed IE within an SDT Information IE to inform CU UP when DL SDT data has arrived exceeding the configured threshold.

How to calculate MT-SDT data size for SDT DL Data Packets

When receiving a DL packet from 5GC, the CU UP will strip off the GTP-U header and get the payload. Then it will add the PDCP header to the packet. Therefore, it seems logical that the data size indicated in the DL Data Notification includes the PDCP header size for better CU CP awareness.

Ultimately, what matters here is whether PDCP header size is counted or not and both solutions could work provided that the 3GPP specification is clear on what exactly is counted.

Proposal 6: specify that MT-SDT Data Size IE includes the PDCP header size.

# 3	Conclusion

This paper has investigated the remaining open issues of MT-SDT from last RAN3 meeting. It made following proposals:

Proposal 1: upon non-SDT data arrival, the anchor gNB can release and page the UE to bring the UE to RRC connected, as in release 17.

Proposal 2: for non-SDT data arrival, no additional signalling is needed in the E1AP DL DATA NOTIFICATION message.

Proposal 3: add new IE in the existing MT-SDT Information IE of DL DATA NOTIFICATION message to signal arrival of large size DL SDT data.

Proposal 4: CU CP configures CU UP whether to report large SDT DL data size and upon which threshold.

Proposal 5: Introduce new DL SDT Large Data Threshold IE within an SDT Information Request IE for CU CP to control the sending of the Large DL SDT data size indication by CU UP and a new DL SDT Data Size Threshold Crossed IE within an SDT Information IE to inform CU CP when DL SDT data has arrived exceeding the configured threshold.

Proposal 6: specify that MT-SDT Data Size IE includes the PDCP header size.

The corresponding TP for TS 37.483 is proposed in [4].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- RP-213583, Work Item on Mobile Terminated Small Data Transmission for NR

- R3-232865, (TPs to TS 38.401, 38.473 BL CRs) Consideration on MT-SDT, Huawei

- R3-233855, (TP for TS 37.483) Completion of MT-SDT Open Points