3GPP TSG-RAN WG3#121	　　　　　　　　　　　　　　　　　R3-233869

Toulouse, France, 21 – 25th August, 2023

Agenda Item:  14.2

Source:       NEC

Title:		Rel-18 LTM issues

Document for:	decision

Introduction

This contribution discusses further Rel-18 LTM remaining issues and provide solutions in order to progress the work.

RAN3#119bis-e: The CU requests the candidate DU to provide RACH resource per candidate cell for TA acquisition in inter-DU case. For intra-DU LTM, the gNB-DU sends a DDDS frame about unsuccessfully transmitted downlink data to the gNB-CU after LTM cell switch if RLC reestablishment is configured. For inter-DU LTM, the DDDS should be sent from source gNB-DU to CU-UP when the LTM cell switch command is sent. Then the CU-UP can start forwarding the unsuccessfully transmitted data to target gNB-DU. The gNB-CU may modify or release L1/2 Triggered Mobility (LTM) candidate cells in the gNB-DU. The (candidate) gNB-DU may cancel already configured L1/2 Triggered Mobility (LTM) candidate cells and notify to the CU. The gNB-CU may use the UE Context Modification procedure to modify or release the prepared resources of candidate cells in the (candidate) gNB-DU and use the UE Context Release procedure to release the UE context in the (candidate) gNB-DU. For intra-CU-UP case, in case of CP UP separation, once CUCP receives LTM cell switch signling from (source)DU , CU CP initiates E1 bearer context modification to the CU UP including DL tunnel ID per DRB for target cell, for data transmission. For inter-CU-UP LTM, once the CU-CP receives LTM cell switch signaling from (source) DU, the CU-CP initiates E1 bearer context modification to the target CU UP including DL tunnel ID per DRB for target cell for data transmission. For inter-CU-UP LTM, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP. In case of gNB-CU-UP change, the gNB-CU triggers the source gNB-CU-UP to start data forwarding after receiving LTM cells switch signalling from DU. For inter-CU-UP LTM, Path switch procedure is performed towards the core network after detecting the UE has accessed to the target cell. For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs for which RLC is not re-established. For intra-CU inter-DU LTM, target gNB-DU sends initial DDDS using the new UL TEID to CU-UP after target gNB-DU detects the UE access (following legacy). The (candidate) gNB-DU may use the UE Context Modification Required message to release the candidate cells, and the gNB-CU shall not reject. The (candidate) gNB-DU may use the UE Context Modification Required procedure to request to cancel the prepared resources of a subset of candidate cells in it and use the UE Context Release Request procedure to request to release all candidate cells in it. Down-selection on cndidate cells configuration in one message or multiple message: Option 1: One message Option 2: multiple messages Option 3: Both options are supported. In case that a list of candidate cells is included, the DU responds to the CU with the accepted candidate cells which have the same admitted result for DRBs. FFS on the message for CU requesting “RACH resource”(UE Context Setup procedure or UE Context Modification procedure or both) FFS: For intra-DU LTM, the gNB-DU sends initial DDDS using the new UL TEID if assigned by the CU to CU-UP after target gNB-DU detects the UE access. RAN3#120: RAN3 agrees option2 and supports for multiple messages for LTM candidate cell configuration. In case of CP UP separation, once CU-CP receives LTM cell switch signaling from (source)DU, CU-CP initiates E1 bearer context modification to the CU UP including DL tunnel ID per DRB for target cell, for data transmission. For LTM with UP change, once the CU-CP receives LTM cell switch signaling from (source) DU, the CU-CP initiates E1 bearer context modification to the target CU UP including DL tunnel ID per DRB for target cell for data transmission. Notification of LTM triggering to gNB-CUs: WA: option 1 is adopted with only class 2 procedure. If more information to source DU in response message is needed, we go for option 2. E1 aspects for inter-DU LTM: For LTM with UP change, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP. For LTM with UP change after detecting the UE has accessed to the target cell, the CU-CP initializes Path switch procedure towards the core network. TA acquisition: CU can request RACH resources for early TA acquisition together with the LTM candidate cell configuration to a Candidate DU in the UE Context Setup Request or UE Context Modification Request messages (for inter-DU LTM and intra-DU LTM respectively). If the Candidate DU accepts the RACH resource request for early TA acquisition, the Candidate DU responds the CU with RACH configuration in the UE Context Setup Response or UE Context Modification Response messages. DDDS: For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs RLC is not re-established. For both intra-DU and inter-DU LTM, target gNB-DU sends initial DDDS using the new UL TEID, if new UL TEID being assigned, to CU-UP after target gNB-DU detects the UE access. Work on stage2 and stage3 details

Discussion

Co-existence between LTM and L3 mobility:

This issue was not clearly concluded in last RAN3#120, we think this issue need to have common understanding as fundamentally, at least crossing of the signalling message between DU and CU will cause significant impact on system performance of the mobility aspect.

There were summarized in three cases:

Case 1: If L3 handover command is received before LTM initiation, the (source) gNB-DU suspends to trigger the LTM command till the L3 handover completion.

Case 2: If LTM command is initiated, and a LTM cell switch notification message has sent to the gNB-CU, the gNB-CU suspends the L3 handover till the LTM completion.

Case 3: If L3 handover command is received after LTM is initiated (cross F1 signallings happen between gNB-DU and gNB-CU), the gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause.

Case 1 and Case 2 are based on the approach of “first come first serve”, this is easy.

For case 1, since it is observed that the current combination of IEs in UE CONTEXT MODIFICATION REQUEST message can be understood that it is for L3 handover, then the source DU can then suspend the LTM.

One may need to further consider, how would be the way for the source gNB-DU to know from the UE CONTEXT MODIFICATION REQUEST message that it is for L3 handover command. In the legacy e.g. inter gNB-DU handover case, following the 38.401 chapter 8.2.1.1 Figure 8.2.1.1-1 step 5 description:

TS38.401 chapter 8.2.1.1 Figure 8.2.1.1-1 step 5: 5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU, which includes a generated RRCReconfiguration message and indicates to stop the data transmission for the UE. The source gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE.

The including of the RRCconfiguration message and the data transmission indication in the F1AP: UE CONTEXT MODIFICATION REQUEST message, can let the source gNB-DU to know this is for L3 handover command, then the source gNB-DU will need to suspend to trigger the LTM command.

Observation 1: The combination of the existing IEs in F1AP: UE CONTEXT MODIFICATION REQUEST message can let the gNB-DU to know it is for L3 handover command.

(However, if this legacy way is not workable, then an explicit indicator may be needed to tell the source gNB-DU that the UE CONTEXT MODIFICATION REQUEST message is for L3 handover command.)

For case 2, as following the baseline CR for 38.401 [R3-233548] that there will be a signalling/indication to tell from the source gNB-DU to the gNB-CU that the LTM command has been triggered, the gNB-CU then can suspend the L3 handover command (if the L3 handover is still under preparation phase).

Observation 2: The newly introduced in F1AP signalling/IE to tell from the source gNB-DU to the gNB-CU that the LTM command has been triggered, the gNB-CU then can suspend the L3 handover command (if it is under preparation).

Case 3 is a crossing of signalling, which may not be a rare case, and this need to determine whether LTM or L3 handover take precedence. Since LTM is aiming for fast mobility and in fact the DU may already sent LTM command to the UE, it is then controllable to fail the L3 handover (as it is not sent to UE yet) and take LTM as precedence (as it has been sent to the UE).

There may be some ways to specify the handling of crossing the LTM and L3 handover in F1AP:

Option 1: in the crossing of LTM signalling and L3 handover signalling, gNB-DU can respond to the gNB-CU by F1AP: UE CONTEXT MODIFICATION FAILURE message. Here may be one additional thing to consider, since the failure message may have various reasons, it would be a need to let gNB-CU know the failure is due to the initiation of the LTM so the gNB-CU does not trigger UE Context Release immediately but rather to continue the whole LTM procedure (i.e. to wait signalling from the target side e.g. ACCESS SUCCESS message).

Proposed text and cause value If the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message because LTM command to the UE has been triggered, it shall reply with the UE CONTEXT MODIFICATION FAIILURE message with a cause value “LTM command triggered”

Option 2: in the crossing of LTM signalling and L3 handover signalling, both source gNB-DU and gNB-CU can implicitly take the LTM as precedence, i.e. only specify in procedure text to say the LTM take precedence. E.g.

Proposed text: Interactions with other procedure If the UE CONTEXT MODIFICATION REQUEST message is received after the gNB-DU has triggered LTM and has sent out LTM CELL CHANGE NOTIFICATION message, the gNB-DU ignore the UE CONTEXT MODIFICATION REQUEST message

The option 2 may be inappropriate since for the class 1 procedure, the gNB-DU need to respond instead of only ignoring. Therefore we propose to take Option 1.

Proposal 1: For the crossing of the LTM related signalling (DU->CU) and L3 handover related signalling (CU->DU), to give a procedure description in the F1AP.

Proposal 2: In order for the gNB-CU not to immediately trigger UE Context Release when it receives UE CONTEXT MODIFICATION FAILURE message, an explicit cause value to show the failure is due to triggering of LTM command.

Candidate cells configuration by multiple messages between CU and DU:

Last meeting has agreed to take the multiple messages approach: RAN3 agrees option2 and supports for multiple messages for LTM candidate cell configuration.

During the discussion of comparison between single message approach and multiple messages approach, there was a worry that the multiple messages approach has signalling overhead, which need to repeat e.g. the same DRB to be setup to the DU for preparing multiple candidate cells (one cell by one signalling message). IF this worry still exists, it can be avoided by giving a small enhancement in the procedure text or stage 2 text, to specify to use of same AP ID for different UE CONTEXT SETUP REQUEST message with the condition that an indication of showing for LTM purpose, then DU does not reject even if same AP ID is received.

Proposal 3: if signalling overhead in multiple messages approaches for LTM candidate cells configuration concern still exists, it can be avoided by giving a small enhancement in the procedure text or stage 2 text, to specify to use of same AP ID for different UE CONTEXT SETUP REQUEST message with the condition that an indication of showing for LTM purpose, then DU does not reject even if same AP ID is received.

Proposal

Observation 1: The combination of the existing IEs in F1AP: UE CONTEXT MODIFICATION REQUEST message can let the gNB-DU to know it is for L3 handover command.

Observation 2: The newly introduced in F1AP signalling/IE to tell from the source gNB-DU to the gNB-CU that the LTM command has been triggered, the gNB-CU then can suspend the L3 handover command (if it is under preparation).

Proposal 1: For the crossing of the LTM related signalling (DU->CU) and L3 handover related signalling (CU->DU), to give a procedure description in the F1AP.

Proposal 2: In order for the gNB-CU not to immediately trigger UE Context Release when it receives UE CONTEXT MODIFICATION FAILURE message, an explicit cause value to show the failure is due to triggering of LTM command.

Proposal 3: if signalling overhead in multiple messages approaches for LTM candidate cells configuration concern still exists, it can be avoided by giving a small enhancement in the procedure text or stage 2 text, to specify to use of same AP ID for different UE CONTEXT SETUP REQUEST message with the condition that an indication of showing for LTM purpose, then DU does not reject even if same AP ID is received.

References

[1] RP-231475 WID Further NR Mobility Enhancements

[2] R3-233557 Summary of offline discussion for CB: # MobilityEnh2_L1L2Mobilit

[3] R3-233548 (BL CR 38.401) Introduction of L1/L2 inter-cell mobility

[4] R3-233550 (BL CR 38.473) Additions for L1/L2 based inter-cell mobility