3GPP TSG-RAN WG3 Meeting # 121	R3-233885

# Toulouse, France,  21st  - 26th August, 2023

Agenda item:	14.4

Source:	Google

Title:	(TPs for SCG Selective Activation BLCR for TS 38.423 & TS 38.473) Reference SCG configuration

Document for:	Discussion & Decision

# Introduction

This paper discusses the necessity of introducing reference SCG configuration in SCG selective activation and provides a TP to the TS 38.423 BLCR.

# Discussion

On the topic of candidate and reference configuration, RAN2 made some agreements in RAN2#120, 121bis, 122 as below:

Delta configuration A UE stores the reference configuration as a separate configuration. The reference configuration is managed separately

RAN2#121:

Assume to support the following scenarios of SCG selective activation: - SN initiated intra-SN SCG selective activation - MN initiated inter-SN SCG selective activation - SN initiated inter-SN SCG selective activation It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell. In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case). R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions) For inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration). R2 understands that A target SN may include an indication in SN Addition Request Ack for each candidate target PSCell, denoting whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

RAN2#121bis-e:

For the reference configuration for SCG Selective Activation, aim at following similar design as LTM. For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format. For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC. - FFS on the following options for subsequent CPC: - Option 1: Source MN generates the execution conditions for all subsequent CPC. - Option 2: Candidate SN may generate execution conditions for subsequent CPC. For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. FFS if Candidate SN may generate/modify execution conditions for subsequent CPC Assume for now that there is only one reference configuration. The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations: - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included. - 3. The execution conditions associated with each candidate target PSCell. - a. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. - b. For SN initiated procedure, execution conditions based on events A3/A5 are supported. UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

RAN2#122:

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC. FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC The UE shall skip the condition evaluation for a candidate which is a current PScell. The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures. Will not spend specific efforts for supporting nested configurations for candidate cell configuration. Rapporteur take initiative on naming offline Terminology is “Subsequent CPAC”

It has also been captured in the latest 38.331 running CR [3] for the above agreement as below as scpac-ReferenceConfiguration-r18.

ConditionalReconfiguration-r16 ::= SEQUENCE { attemptCondReconfig-r16 ENUMERATED {true} OPTIONAL, -- Cond CHO condReconfigToRemoveList-r16 CondReconfigToRemoveList-r16 OPTIONAL, -- Need N condReconfigToAddModList-r16 CondReconfigToAddModList-r16 OPTIONAL, -- Need N ..., [[ scpac-ReferenceConfiguration-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Need FFS ]] } CondReconfigToRemoveList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

ConditionalReconfiguration field descriptions

attemptCondReconfig If present, the UE shall perform conditional reconfiguration if selected cell is a target candidate cell and it is the first cell selection after failure as described in clause 5.3.7.3.

condReconfigToAddModList List of the configuration of candidate SpCells to be added or modified for CHO, CPA or CPC.

condReconfigToRemoveList List of the configuration of candidate SpCells to be removed.

scpac-ReferenceConfiguration Includes the reference configuration for Subsequent CPAC candidates.

ConditionalReconfiguration field descriptions

#### – CG-ConfigInfo This message is used by master eNB or gNB to request the SgNB or SeNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB or SeNB to set the SCG configuration. It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, or modify an MCG or SCG. Direction: Master eNB or gNB to secondary gNB or eNB, alternatively CU to DU. CG-ConfigInfo message -- ASN1START -- TAG-CG-CONFIG-INFO-START CG-ConfigInfo ::= SEQUENCE { … skip unchanged CG-ConfigInfo-v1730-IEs ::= SEQUENCE { fr1-Carriers-MCG-r17 INTEGER (1..32) OPTIONAL, fr2-Carriers-MCG-r17 INTEGER (1..32) OPTIONAL, nonCriticalExtension CG-ConfigInfo-v1800-IEs OPTIONAL } CG-ConfigInfo-v1800-IEs ::= SEQUENCE { scpac-ReferenceConfiguration-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, nonCriticalExtension SEQUENCE[] OPTIONAL } … skip unchanged Editor’s notes: FFS on which node initially generates the reference configuration. Editor’s notes: FFS on the IE/message to transfer the reference configuration.

scpac-ReferenceConfiguration Includes the reference configuration for Subsequent CPAC candidates.

It can be seen that the reference SCG configuration is a separate configuration to the UE and RAN2 did not intend to introduce a separate inter-node RRC message for it (i.e., CG-ConfigInfo can be reused). Therefore, for the communication between the MN and candidate T-SNs during the preparation phase, CG-ConfigInfo (i.e., MN to SN Container) can be reused. Similarly for the candidate T-SN, the gNB-CU can also reuse the CG-ConfigInfo (i.e., CU to DU RRC Information) to signal the Reference SCG Configuration to the gNB-DU to allow it to prepare lower layer configurations. Although RAN2 still put an EN that the IE/message to transfer the reference configuration remains FFS, we propose to leave it in the existing MN to SN Container so that RAN3 impact can be reduced.

Proposal 1: It is not necessary to introduce additional container for the Reference SCG Configuration in the SN Addition Request message or UE Context Setup Request message (i.e., CG-ConfigInfo is reused)

Although it remains in FFS which node initially generates the Reference SCG Configuration, it is beneficial to have some initial discussion in RAN3 and agree on some text proposals if achievable. For MN-initiated (i.e., in DC) and SN-initiated procedures, it makes sense for the source SN to generate the initial one (might be updated if the SN changes). The MN may query a Reference SCG Configuration from the source SN using SN Modification Request message in the MN-initiated scenario. For SN-initiated scenario, the source SN can provide it in the SN Change Required message. Whether to add the scpac-ReferenceConfiguration in a new container or in the existing SN to MN Container (i.e., CG-Config) in the SN Modification Request Acknowledge message or SN Changed Required message may need some RAN3 discussion. However, similar to the CG-ConfigInfo, it is proposed to let RAN2 also include the scpac-ReferenceConfiguration in the CG-Config. Therefore, the existing SN to MN Container can be reused in the SN Modification Request Acknowledge message and RAN3 impact can be reduced.

Proposal 2:  Source SN may generate the Reference SCG Configuration if being queried by the MN or actively provided it in SN initiated procedure.

Proposal 3: For CPC and selective activation, add a Reference SCG Configuration Query IE (i.e., new query IE) in the SN Modification Request message

For the CPA case, it is not sure if the eNB/gNB in single connectivity is able to generate the initial Reference SCG Configuration. It may be less controversial that the eNB/gNB can request the first candidate T-SN to provide the initial Reference SCG Configuration and then signal it to other candidate T-SNs during the preparations. In such case, it is necessary to add a Reference SCG Configuration Query IE in the SN Addition Request message as in the Proposal 2.

Proposal 4: FFS if MN may be able to generate the initial Reference SCG Configuration if initially the MN is in single connectivity with the UE

Proposal 5: For CPA and selective activation, the initial Reference SCG Configuration may be generated by the first prepared T-SN

- A Reference SCG Configuration Query IE is needed for the SN Addition Request message

Considering the CU-DU split, the F1AP impact will cover the UE Context Modification procedure and, depending on whether the first candidate T-SN can provide the initial Reference SCG Configuration, the UE Context Setup procedure. If the first candidate T-SN can provide the initial Reference SCG Configuration, since both the candidate configuration and the reference configuration are required, it may be necessary to add an additional container (e.g., a new IE in the DU to DU RRC Information or a new container) for at least the UE Context Setup Response case.

Proposal 6: The source SN-CU may need to add in the UE Context Modification Request message and/or UE Context Setup Request message a Reference SCG DU Configuration Query IE (similar to GNB-DU Configuration Query IE) to obtain a Reference SCG DU Configuration

- An additional container for Reference SCG Configuration may be needed for the UE Context Setup Response message

As the progress for this agenda did not proceed much for the past few meetings, we first provide a TP for the TS 38.423 and a TP for the TS 38.473 to cover the above proposals and can further add corresponding ASN.1 changes if the proposals are agreeable during the discussion.

# Conclusion

Proposal 1: It is not necessary to introduce additional container for the Reference SCG Configuration in the SN Addition Request message or UE Context Setup Request message (i.e., CG-ConfigInfo is reused)

Proposal 2:  Source SN may generate the Reference SCG Configuration if being queried by the MN or actively provided it in SN initiated procedure.

Proposal 3: For CPC and selective activation, add a Reference SCG Configuration Query IE (i.e., new query IE) in the SN Modification Request message

Proposal 4: FFS if MN may be able to generate the initial Reference SCG Configuration if initially the MN is in single connectivity with the UE

Proposal 5: For CPA and selective activation, the initial Reference SCG Configuration may be generated by the first prepared T-SN

- A Reference SCG Configuration Query IE is needed for the SN Addition Request message

Proposal 6: The source SN-CU may need to add in the UE Context Modification Request message and/or UE Context Setup Request message a Reference SCG DU Configuration Query IE (similar to GNB-DU Configuration Query IE) to obtain a Reference SCG DU Configuration

- An additional container for Reference SCG Configuration may be needed for the UE Context Setup Response message

# References

[1] RAN2#121 Chair Notes EOM, MediaTek

[2] RAN2#121bis-e Chair Notes EOM, MediaTek

[3] [Post122][056][Mob18] 38.331 Running CR for selective activation of SCGs for NR-DC (OPPO)

# TP for TS 38.423 BL CR

<<<<<< NEXT CHANGE, SKIP UNCHANGED >>>>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the QoS Flow Level QoS Parameters IE for each QoS flow shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

<<<<<< SKIP UNCHANGED >>>>

If the Reference SCG Configuration Query IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall provide the corresponding radio configuration information within the S-NG-RAN node to M-NG-RAN node Container IE as specified in TS 37.340 [8].

<<<<<< NEXT CHANGE, SKIP UNCHANGED >>>>

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE MODIFICATION REQUEST message, it shall start the timer TXnDCprep.

The S-NODE MODIFICATION REQUEST message may contain

-	within the UE Context Information IE;

-	PDU session resources to be added within the PDU Session Resources To Be Added Item IE;

-	PDU session resources to be modified within the PDU Session Resources To Be Modified Item IE;

-	PDU session resources to be released within the PDU Session Resources To Be Released Item IE;

-	the S-NG-RAN node Security Key IE;

-	the S-NG-RAN node UE Aggregate Maximum Bit Rate IE;

-	the M-NG-RAN node to S-NG-RAN node Container IE;

-	the PDCP Change Indication IE;

-	the SCG Configuration Query IE;

-	the Requested split SRBs IE;

-	the Requested split SRBs release IE;

-	the Requested fast MCG recovery via SRB3 IE;

-	the Requested fast MCG recovery via SRB3 Release IE;

-	the Additional DRB IDs IE;

-	the MR-DC Resource Coordination Information IE;

-	the Reference SCG Configuration Query IE.

<<<<<< NEXT CHANGE, SKIP UNCHANGED >>>>

If the Reference SCG Configuration Query IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall provide corresponding radio configuration information within the S-NG-RAN node to M-NG-RAN node Container IE as specified in TS 37.340 [8].

<<<<<< NEXT CHANGE, SKIP UNCHANGED >>>>

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >Selective Activation Information Request | O |  | 9.2.3.xxx | This information is used for SCG selective activation. FFS on the details. Editor’s Note: This may be revisited based on RAN2 progress. | YES | ignore |
| >Reference SCG Configuration Query | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

<<<<<< NEXT CHANGE, SKIP UNCHANGED >>>>

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  | YES | ignore |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| CHO Information SN Modification | O |  |  |  | YES | ignore |
| >Conditional Reconfiguration | M |  | ENUMERATED (intra-MN-CHO, ...) |  | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Modification Request | O |  |  | This IE may be sent to the target SN. | YES | ignore |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. |  |  |
| Conditional PSCell Change Information Update | O |  |  | This IE may be sent to the source SN. | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | ignore |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.2.3.169 |  | YES | ignore |
| Reference SCG Configuration Query | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

<<<<<< END OF CHANGE >>>>

# TP for TS 38.473 BL CR

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

<<<<<< Skip Unchanged >>>>

If the TwoPHRModeMCG IE or the TwoPHRModeSCG IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use this value as described in TS 38.331 [8].

If the MBSInterestIndication IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

- If the Reference gNB-DU Configuration Query IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the Reference gNB-DU Configuration IE in the UE CONTEXT SETUP RESPONSE message.

<<<<<< NEXT CHANGE >>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

<<<<<< Skip Unchanged >>>>

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the TwoPHRModeMCG IE or the TwoPHRModeSCG IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use this value as described in TS 38.331 [8].

If the MBSInterestIndication IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

- If the Reference gNB-DU Configuration Query IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

<<<<<< NEXT CHANGE >>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction : gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,…) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - | - |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | 9.3.1.5 | Allocated at the target gNB-DU | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| >Reference gNB-DU Configuration Query | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to “CHO-replace”. |

<<<<<< NEXT CHANGE >>>>

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| Reference gNB-DU Configuration | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | YES | ignore |

<<<<<< NEXT CHANGE >>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, …) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| ////////////// Skip unchanged part ///////////////// |  |  |  |  |  |  |
| CPAC MCG Information |  | 0..1 |  | This IE is used at the MN for MCG configuration as specified in TS 37.340 [7] for CPAC. | YES | ignore |
| >CPAC Trigger | M |  | ENUMERATED (CPAC-preparation, CPAC-executed, …) |  | - | - |
| >PSCell ID | M |  | NR CGI 9.3.1.12 | The PSCell corresponding to the included CG-Config IE at CPAC-preparation or the selected PSCell by the UE at CPAC-executed. | - | - |
| Reference gNB-DU Configuration Query | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is FFS. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

<<<<<< END OF CHANGE >>>>