3GPP TSG-RAN WG3 Meeting # 121	R3-233886

# Toulouse, France,  21st  - 26th August, 2023

Agenda item:	14.2

Source:	Google

Title:	(TP for L1L2Mob BLCR for TS 38.401) Reference configuration and Target Configuration ID in LTM

Document for:	Discussion & Decision

# Introduction

This paper discusses the necessity of introducing reference configuration in LTM considering the CU-DU split and provides a TP to the TS 38.401 BLCR for LTM.

# Discussion

On the topic of candidate and reference configuration, RAN2 made some further agreements [1] in RAN2#121:

Agreed: Usage of reference configuration: - Candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration (FFS if done at cell switch or before the cell switch) - The complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset RLC or PDCP. - To support reconfigurations that requires reset of RLC PDCP, this should be possible (in principle same a full config) - FFS if more than RLC PDCP should be kept and how much of “replacing” need to be specified. - FFS if the reference configuration can be derived from the current UE configuration at some point of time. Potentially: R2 assumes that LTM without a separate reference configuration (if agreed) could work something like this: - Alt A: The candidate configuration (which need to be complete) is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset RLC or PDCP. (Same procedure as above) - Alt B: The candidate configuration (which can be a delta config) is applied to the current UE configuration (at the time of reconfiguration execution/cell switch), by legacy RRC reconfiguration procedure (it is assumed that the network need to coordinate if subsequent reconfigurations shall work, FFS feasibility). agree to use Model 1: One RRCReconfiguration message for each candidate target configuration RRCReconfiguration to configure target candidate cells Reference config can be empty In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration when the UE receives the LTM configuration (before the LTM cell switch). UE implementation can postpone that step to the reception of the LTM cell switch command. FFS Discuss early vs late compliance check. In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset MAC, RLC or PDCP. FFS whether we can rely on a modified version of the reconfiguration procedure with fullconfig flag set. FFS how to make sure the procedures work in case the LTM candidate configuration is a complete configuration.

RAN2#121bis-e [2]

Discuss terminology for the TS in the RRC stage-3 discussions when/if needed (not at current meeting). Whether the Reference configuration is a complete configuration or not is up to the network implementation. Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration. The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current). Confirm that only the replacement procedure (the “full config without L2 reset”) is supported for Execution of LTM cell switch. The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

It has also been captured in the latest 38.331 running CR [3] for the above agreement as below as ltm-ReferenceConfiguration-r18.

LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 OCTET STRING (CONTAINING RRCReconfiguration), OPTIONAL, -- Cond FirstLTM-Candidate ltm-CandidateToReleaseList-r18 LTM-CandidateToReleaseList-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 LTM-CandidateToAddModList-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1.. maxNrofCellsLTM-r18) OPTIONAL, -- Cond FirstLTM-Only ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF LTM-CSI-ResourceConfig OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF LTM-CSI-ResourceConfigId OPTIONAL, -- Need N ... }

LTM-CandidateConfig field descriptions

ltm-CandidateToAddModList List of LTM candidate cell configurations to add and/or modify.

ltm-CandidateToReleaseList List of LTM candidate cell configurations to remove.

ltm-CandidateNoResetL2-List FFS

ltm-ServingCellNoResetID This field is used by the UE to understand on whether no L2 reset should be performed for an LTM candidate cell upon an LTM cell switch procedure. If the value of ltm-NoResetID in an LTM candidate cell is the same as the value of ltm-ServingCellNoResetID in the serving cell of a cell group, then the UE shall not perform any L2 reset during the LTM cell switch procedure.

ltm-ReferenceConfiguration This field includes an RRCReconfiguration message used to configure a reference configuration for LTM.

Considering the CU-DU split when generating the candidate cell configuration for the UE, the reference configuration issue should be considered in RAN3. As gNB-DU is responsible for the lower layer RRC configuration, the reference configuration for initial LTM configuration has to be generated by a gNB-DU.

Proposal 1: A source gNB-DU may generate the reference configuration for initial LTM configuration.

## Intra-gNB DU LTM

As UE moves within the same gNB-DU during NR operation for LTM, the gNB-DU generates and holds the reference configuration and the gNB-CU does not need to provide the reference configuration to it. The gNB-DU can generate the lower layer RRC configuration based on the reference configuration it holds. The gNB-CU can request the gNB-DU to provide a reference configuration. The gNB-DU can update the reference configuration subsequently if necessary for the intra-gNB DU LTM.

Proposal 2: The gNB-DU takes the reference configuration into account when generating the lower layer RRC configurations.

## Inter-gNB DU LTM

When the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM, the gNB-CU has to perform UE Context Setup procedure to the candidate gNB-DU. If the gNB-CU has a reference configuration (e.g., generated and provided by the source gNB-DU), it may provide the reference configuration to the candidate gNB-DU. The candidate gNB-DU takes the reference configuration into account when generating the lower layer RRC configurations.

Proposal 3: The gNB-CU may provide a reference configuration to the candidate gNB-DU in the UE Context Setup Request message to the gNB-DU.

Otherwise, the candidate gNB-DU may generate the reference configuration for LTM configuration if not received from the gNB-CU. The gNB-CU can request the candidate gNB-DU to provide a reference configuration.

Proposal 4: The candidate gNB-DU may generate the reference configuration for LTM configuration if not received from the gNB-CU.

Some further issue may be that whether the candidate gNB-DU is allowed to generate a reference configuration to update the reference configuration received from the gNB-CU. If it is allowed, the gNB-CU needs to propagate the updated reference configuration to the source gNB-DU or other candidate gNB-DU(s) that had prepared LTM configuration(s). However, the update process may be endless if every gNB-DU is allowed to update the existing reference configuration. Therefore, it is proposed that for the inter-gNB DU LTM, the candidate gNB-DU is not allowed to update the reference configuration if received from the gNB-CU.

Proposal 5: The candidate gNB-DU is not allowed to update the reference configuration if received from the gNB-CU.

It is therefore proposed to discuss how the reference configuration will be conveyed in the UE Context Setup Request message to the gNB-DU and how to capture the supporting text in the TS 38.401 to reflect the agreements made in RAN2.

As the ltm-ReferenceConfiguration-r18 IE in [3] contains the RRCReconfiguration message, it should be encoded by the gNB-CU by combining a lower layer reference configuration from the gNB-DU and optionally a higher layer reference configuration by the gNB-CU itself. Therefore, the LTM Reference Configuration IE exchanged between the gNB-CU and the gNB-DU in F1AP should refer to a lower layer configuration (i.e., the CellGroupConfig IE). The gNB-CU stores and forwards it transparently to the candidate gNB-DU during the LTM preparation.

## Configuration ID in LTM Command

In LTM TS 38.300 Running CR, it is captured:

“The gNB decides to execute cell switch to a target cell, and transmits a MAC CE triggering cell switch by including the candidate configuration index of the target cell. The UE switches to the target cell and applies the configuration indicated by candidate configuration index.”

It is also captured in the LTM TS 38.321 Running CR that the LTM Cell Switch Command MAC CE contains a Target Configuration ID which “indicates the index of candidate target configuration to apply for LTM cell switch, corresponding to [ltm-CandidateId] as specified in TS 38.331 [5]”. As this MAC CE has to be signalled by the gNB-DU while it is assigned by the gNB-CU and included in the LTM-Candidate IE when configuring the LTM-Config to the UE.

To enable the gNB-DU to include a correct Target Configuration ID in the LTM Command after receiving the L1L2 measurement reports from the UE, the gNB-DU needs to associate the Target Configuration ID with the Requested Target Cell ID (and the (lower layer) candidate configuration). Therefore, it is proposed that the gNB-CU adds in the UE Context Setup/Modification Request message a LTM Configuration ID IE along with the SpCell ID IE for the gNB-DU to establish the corresponding association during the LTM preparation.

Proposal 6: The gNB-CU includes a LTM Configuration ID IE in the UE Context Setup Request message and the UE Context Modification Request message.

As for the inter-gNB DU LTM, the source gNB-DU needs to be informed of the prepared LTM IDs and target Cell IDs at the other candidate gNB-DUs. Similarly, the candidate gNB-DU needs to be informed of the prepared LTM IDs and target cell IDs at the source gNB-DU. Therefore, it is proposed to add a list for adding or modifying LTM ID assignment and a list for releasing LTM ID assignment in the UE Context Modification Request message so that the gNB-CU can use the UE context modification procedure to signal the LTM ID assignment to the source/candidate gNB-DU.

Proposal 7: The gNB-CU uses the UE Context Modification procedure to the source/candidate gNB-DU to signal the LTM ID assignment.

# Conclusion

Proposal 1: A source gNB-DU may generate the reference configuration for initial LTM configuration.

Proposal 2: The gNB-DU takes the reference configuration into account when generating the lower layer RRC configurations.

Proposal 3: The gNB-CU may provide a reference configuration to the candidate gNB-DU in the UE Context Setup Request message to the gNB-DU.

Proposal 4: The candidate gNB-DU may generate the reference configuration for LTM configuration if not received from the gNB-CU.

Proposal 5: The candidate gNB-DU is not allowed to update the reference configuration if received from the gNB-CU.

Proposal 6: The gNB-CU includes a LTM Configuration ID IE in the UE Context Setup Request message and the UE Context Modification Request message.

Proposal 7: The gNB-CU uses the UE Context Modification procedure to the source/candidate gNB-DU to signal the LTM ID assignment.

# References

[1] RAN2#121 Chair Notes EOM, MediaTek

[2] RAN2#121bis-e Chair Notes EOM, MediaTek

[3] [Post122][055][Mob18] 38.331 Running CR and Open issues (Ericsson)

[4] R2-2306957, 38.300 running CR for introduction of NR further mobility enhancements, MediaTek Inc., Vivo

[5] R2-2306924, 38.321 running CR for introduction of NR further mobility enhancements, Huawei, HiSilicon

# TP for TS 38.401 BL CR

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

s

Figure 8.2.1.x-1: Intra-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiateLTM configuration.

3.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID and a corresponding LTM configuration ID.

4.	If the gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer configurations for the accepted target candidate cell. The gNB-DU may generate a reference configuration for initial LTM configuration and include it in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU takes the reference configuration into account when generating the lower layer configurations. The gNB-CU may initiate another UE context modification procedure to the gNB-DU to signal the LTM ID assignment.

5.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with theLTM configuration.

FFS: whether it is DL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION REQUEST message.

6.	The gNB-DU forwards the received RRCReconfiguration message to the UE.

7.	The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

8.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS:	whether it is UL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION RESPONSE message.

9.	The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute LTM to a candidate target cell.

10.	The gNB-DU sends the LTM command to the UE including a LTM configuration ID corresponding to the candidate target cell.

Editor’s note:	The LTM command needs to be updated according to RAN2’s discussion.

11.	WA: The gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID.

FFS: How the gNB-DU detects the UE access.

12.	The gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

13.	The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

14.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

<<<<<< NEXT CHANGE >>>>>>

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID. The gNB-CU may provide a reference configuration to the candidate gNB-DU in the UE CONTEXT SETUP REQUEST message.

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell. The candidate gNB-DU shall, if supported, take the reference configuration into account when generating the lower layer configuration. The candidate gNB-DU may generate the reference configuration for LTM configuration, if not received from the gNB-CU.

5. 	After the LTM configuration preparation at the candidate gNB-DU, the gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including an assignment of a LTM configuration ID and an associated target candidate cell.

6.	The source gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message.

7.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

FFS: 	whether it is DL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION REQUEST message.

8.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

9.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

10.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS: 	whether it is UL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION RESPONSE message.

11.	The UE sends the lower layer measurement result to the source gNB-DU.

12.	The source gNB-DU decides to execute LTM to a candidate target cell.

FFS:	Notifying the LTM triggering decision to the other nodes as well.

13.	The source gNB-DU sends LTM command to the UE including a LTM configuration ID corresponding to the candidate target cell.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

14.	WA: The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID.

15.	FFS: how the target gNB-DU detects the UE access.

16.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

17.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

18.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

---------------------------------------------------------End of change -------------------------------------------------------------