3GPP TSG-RAN WG3 Meeting #121                                                                   R3-233890

Toulouse, France, August 21 – 25, 2023

Agenda item: 		13.1

Source: 		Qualcomm Incorporated

Title: 	Discussion on LSs to RAN3 on mIAB

Document for:	Discussion

# 1	Introduction

This document discusses the reply to the following LSs on mIAB:

- R3-233713/R2-2306817: LS on UE RACH-less handover for mobile IAB

- R3-233730/S2-2308158: LS On IAB-node de-authorization handling

# 2	Discussion

## LS by RAN2 on UE RACH-less handover for mobile IAB

This LS has the following content:

1. Overall Description: RAN2 has discussed the UE RACH-less handover in mobile IAB and achieved the following agreements: - RAN2#121bis meeting agreements: Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB). RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration: - Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details. - Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command). RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO. - RAN2#122 meeting agreements: RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured). For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report). for the UL grant and HO completion in RACH-less HO: - 1. Both type-1 configured grant and dynamic grant are supported - 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI). Send LS to RAN3 to check whether there are issues / feasibility concerns RAN2 would like to ask RAN3 to take those agreements into account and provide feedbacks if there are any issues or feasibility concerns. 2. Actions: To RAN3. ACTION: RAN2 respectively asks RAN3 to take the above agreements into account and provide feedbacks if there are any issues or feasibility concerns.

RAN2 is presently discussing RACH-less handover, but they have not agreed to support it. Without such RAN2 agreement, RAN3 should not spend a lot of time on the discussion for mIAB-related RACH-less HO.

Proposal 1: Before RAN2 has agreed to support RACH-less handover for mobile IAB, RAN3 should not spend a lot of time on the discussion of such a feature.

The following issues can be identified that may have RAN3 impact:

- During the UE handover preparation, the target logical DU needs to be informed whether the HO refers to a RACH-less or legacy handover, so that it can include the beam identifier and/or the configured UL grant into the CellGroupConfig, if needed.

- For network-implementation-based beam indication in the handover command, it is assumed that the source and target logical DUs share information about the UE, e.g., the physical beam the UE uses with the source logical DU may have to be known to the target logical DU. This implies that during the handover preparation procedure, a UE identification known to the source logical DU is conveyed to the target logical DU.

Proposal 2: The Reply LS to RAN2 to include the following: RAN3 has discussed RACH-less handover for mobile IAB. Based on an initial assessment, RAN3 identified the following issues which may have RAN3 impact:

(1) The target logical DU needs to know whether the handover preparation is for legacy or for RACH-less HO, so that it can include information related to RACH-less HO, if necessary,

(2) For network-implementation-based beam indication, the target logical DU needs to know for which of the source logical DU’s UEs the handover preparation is conducted, so that it can derive the beam-related information from the source logical DU.

Proposal 3: The Reply LS to RAN2 to include the following: RAN3 needs to perform a more detailed assessment in case RAN2 decides to support RACH-less handover for DU migration.

Proposal 4: Agree on draft Reply LS to RAN2 in the Annex 1.

## LS by SA2 on IAB-node de-authorization handling

This LS has the following content:

1. Overall Description: SA2 is currently discussing the MBSR (i.e., mobile IAB-node) authorization state changed from authorized to non-authorized (e.g., due to subscription data update for the MBSR). The AMF notifies the Mobile IAB-node authorization status via the N2 message. SA2 noticed that when the AMF indicates the mobile IAB node is authorized, the mobile IAB-node integrates into the network. The donor-gNB communicates with IAB-MT via RRC messages to setup the backhaul, F1-C connection (i.e., as specified in TS 38.401 clause 8.12). However, when an MBSR is no longer authorized as indicated to the gNB by AMF by N2 message, it’s not clear what is the gNB behaviour. SA2 would like to check whether this is a correct understanding: - The gNB attempts to perform handover of all the UEs served by the MBSR - When the step 1 completes, the gNB releases the F1-C Q1: is the understanding above correct? Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)? 2. Actions: To RAN3 ACTION: SA2 kindly asks RAN3 to provide input information related to above questions and assumptions.

On Q1: RAN3’s understanding is that a non-authorized mIAB-node should not serve UEs. In case the mIAB-DU already serves UEs when the AMF changes the authorization state to “non-authorized”, the mIAB-DU’s F1-C should be released. Before F1-C release, the CU should try to handover all UEs served by this mIAB-DU. This confirms SA2’s understanding of Q1.

On Q2: According to TS 23.501 clause 5.35A.4, the mIAB-MT is informed via NAS about the MBSR authorization state. It can be assumed that if this NAS-based indication is set to “non-authorized”, the mIAB-DU will not attempt F1 Setup. Therefore, with the availability of this NAS indication, there is no need to introduce a RAN-based indication to the mIAB-node on the MBSR authorization state. RAN3 may consider RAN-based signaling related to the change of the mIAB authorization state for other purposes, e.g., to inform the mIAB-DU’s CU about the change to “non-authorized” so that it can conduct UE handover before F1 is released. However, SA2’s LS does not inquire about signaling support for such purposes.

Proposal 5: The Reply LS to SA2 to include the following:

- On Q1: RAN3 agrees with SA2’s understanding on the RAN’s behaviour for the case where the MBSR’s authorization state is changed to “non-authorized”.

- On Q2: RAN3 assumes that the NAS-based indication on the MBSR authorization state as defined by SA2 is sufficient to prevent the mobile IAB-DU from establishing F1 in case the MBSR is “non-authorized”. RAN3 does not see the need to introduce a RAN-based indication for this purpose.

Proposal 6: Agree on draft Reply LS to SA2 in the Annex 2.

# Conclusion

This contribution discussed Reply LSs to RAN2 and SA2. The following proposals have been made:

Proposal 1: Before RAN2 has agreed to support RACH-less handover for mobile IAB, RAN3 should not spend a lot of time on the discussion of such a feature.

Proposal 2: The Reply LS to RAN2 to include the following: RAN3 has discussed RACH-less handover for mobile IAB. Based on an initial assessment, RAN3 identified the following issues which may have RAN3 impact:

(1) The target logical DU needs to know whether the handover preparation is for legacy or for RACH-less HO, so that it can include information related to RACH-less HO, if necessary,

(2) For network-implementation-based beam indication, the target logical DU needs to know for which of the source logical DU’s UEs the handover preparation is conducted, so that it can derive the beam-related information from the source logical DU.

Proposal 3: The Reply LS to RAN2 include the following: RAN3 needs to perform a more detailed assessment in case RAN2 decides to support RACH-less handover for DU migration.

Proposal 4: Agree on draft Reply LS to RAN2 in the Annex 1.

Proposal 5: The Reply LS to SA2 to include the following:

- On Q1: RAN3 agrees with SA2’s understanding on the RAN’s behaviour for the case where the MBSR’s authorization state is changed to “non-authorized”.

- On Q2: RAN3 assumes that the NAS-based indication on the MBSR authorization state as defined by SA2 is sufficient to prevent the mobile IAB-DU from establishing F1 in case the MBSR is “non-authorized”. RAN3 does not see the need to introduce a RAN-based indication for this purpose.

Proposal 6: Agree on draft Reply LS to SA2 in the Annex 2.

# Annex 1

3GPP TSG RAN WG3#119	R3-23xxxx

Toulouse, France, August 21 – 25,2023

Title:	[Draft] Reply LS on UE RACH-less handover for mobile IAB

Response to:	LS in R3-233713 (R2-2306817) on UE RACH-less handover for mobile IAB

Release:	Rel-18

Work Item:	NR_mobile_IAB-Core

Source:	Qualcomm [to be: RAN3]

To:	RAN2

Cc:

Contact person:

Name:	Georg Hampel

Tel. Number:

E-mail Address:	ghampel@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 would like to thank RAN2 for their LS on UE RACH-less handover for mobile IAB (R3-233713/R2-2306817).

RAN3 has discussed RACH-less handover for mobile IAB. Based on an initial assessment, RAN3 identified the following issues, which may have RAN3 impact:

(1) The target logical DU needs to know whether the handover preparation is for legacy or for RACH-less HO.

(2) For network-implementation-based beam indication, the target logical DU needs to know for which of the source logical DU’s UEs the handover preparation is conducted.

RAN3 needs to perform a more detailed assessment in case RAN2 decides to support RACH-less handover for DU migration.

RAN3 asks RAN2 to take the above feedback into account.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks RAN2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	 #121-bis-e – October 9 to 13, 2023    Xiamen, China

TSG-RAN3 Meeting	 #122 – November 13 to 17, 2023      Chicago, IL, USA

# Annex 2

3GPP TSG RAN WG3#119	R3-23xxxx

Toulouse, France, August 21 – 25,2023

Title:	[Draft] Reply LS on IAB-node De-authorization handling

Response to:	LS in R3-233730 (S2-2308158) on IAB-node De-authorization handling

Release:	Rel-18

Work Item:	NR_mobile_IAB-Core

Source:	Qualcomm [to be: RAN3]

To:	RAN2

Cc:

Contact person:

Name:	Georg Hampel

Tel. Number:

E-mail Address:	ghampel@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 would like to thank SA2 for their LS on IAB-node De-authorization handling (R3-233730/S2-2308158).

SA2 provided the following questions:

However, when an MBSR is no longer authorized as indicated to the gNB by AMF by N2 message, it’s not clear what is the gNB behaviour.

SA2 would like to check whether this is a correct understanding:

- The gNB attempts to perform handover of all the UEs served by the MBSR

- When the step 1 completes, the gNB releases the F1-C

Q1: is the understanding above correct?

Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)?

RAN3 has discussed the questions Q1 and Q2, and converged on the following answers:

On Q1: RAN3 agrees with SA2’s understanding on the RAN’s behaviour for the case where the MBSR’s authorization state is changed to “non-authorized”.

On Q2: RAN3 assumes that the NAS-based indication on the MBSR authorization state as defined by SA2 is sufficient to prevent the mobile IAB-DU from establishing F1 in case the MBSR is “non-authorized”. RAN3 does not see the need to introduce a RAN-based indication for this purpose.

RAN3 asks SA2 to take the above feedback into account.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	 #121-bis-e – October 9 to 13, 2023    Xiamen, China

TSG-RAN3 Meeting	 #122 – November 13 to 17, 2023      Chicago, IL, USA