3GPP TSG-RAN3 #121	R3-2333905

21st August – 25th August 2023

Toulouse, France

Agenda Item:	14.2

Source:	Ericsson

Title:	(TP for LTM BL CR to TS 38.401) Solutions for LTM

Document for: Discussion, Decision

# 1	Introduction

In the previous meeting the issue of L1/L2 triggered mobility was discussed and some agreements were made. In this contribution we will further discuss the main components of the L1/L2 triggered mobility and put forward our proposals.

# 2	Discussion

In the previous meeting the following agreements were captured:

RAN3 agrees option2 and supports for multiple messages for LTM candidate cell configuration.

WA: option 1 is adopted with only class 2 procedure. If more information to source DU in response message is needed, we go for option 2.

For LTM with UP change, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP.

For LTM with UP change after detecting the UE has accessed to the target cell, the CU-CP initializes Path switch procedure towards the core network.

CU can request RACH resources for early TA acquisition together with the LTM candidate cell configuration to a Candidate DU in the UE Context Setup Request or UE Context Modification Request messages (for inter-DU LMT and intra-DU LTM respectively).

If the Candidate DU accepts the RACH resource request for early TA acquisition, the Candidate DU responds the CU with RACH configuration in the UE Context Setup Response or UE Context Modification Response messages.

For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs RLC is not re-established.

For both intra-DU and inter-DU LTM, target gNB-DU sends initial DDDS using the new UL TEID, if new UL TEID being assigned, to CU-UP after target gNB-DU detects the UE access.

In the following we will look into the open points, analyse them and provide our views and proposals.

## 2.1	Execution of L1/L2 triggered mobility serving cell change

In the previous meetings the issue of the execution of LTM serving cell change was discussed repeatedly and it has been recognized that two main different approaches have come up as the key alternatives for the inter-DU case and further interaction between RAN2 and RAN3 took place about this issue.

Below we briefly summarize the history and the agreements relevant to this topic.

After offline discussions in RAN2 regarding a relevant LS by RAN3 no on-line agreement was reached. The proposals in the summary were according to below:

Proposal 5: [18/27] RAN2 reply RAN3 that: 1. Approach 1 is the baseline and should be supported; 2. RAN2 understands followings about Approach 2: - Firstly, it may delay the sending of the LTM cell switch command with the risk of source link failure. - Secondly, it may reduce LTM cell switch latency/interruption by enable the exchange of dynamic information in MAC CE (if any). RAN2 will further discuss whether to include such information in the MAC CE, which requires approach 2. RAN2 will keep RAN3 informed on this. Observation 1: It is better to ask RAN3 to clarify, since there are two different understandings on approach 2 “the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE”: - Approach 2a: The request and feedback can be done early before the source DU cell’s switch decision (e.g. during LTM preparation or early sync phase), which will NOT defer the triggering LTM cell switch command to the UE - Approach 2b: Those information (in MAC CE) has to be requested immediately before triggering LTM cell switch command (e.g. during LTM execution phase), which defers the triggering LTM cell switch command to the UE

Below we present the approaches.

In approach 1 the serving DU informs the CU only after sending the LTM cell switch command to the UE and it can therefore not retrieve any dynamic information just before the execution. In Figure 1, approach 1 for LTM cell switch execution is illustrated, and we have put it into the context of the overall LTM procedure.

Figure 1. Approach 1 for LTM cell switch execution in the context of the overall LTM procedure. The serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch.

Regarding approach 2, in the RAN2 email discussion summary, it was suggested that there may be two understandings of approach 2:

-	Approach 2a: The retrieval of dynamic information about the target cell to be included in the LTM cell switch MAC CE is done early before the source DU cell’s switch decision, which will NOT defer the triggering of LTM cell switch command to the UE

-	Approach 2b: The retrieval of dynamic information about the target cell to be included in the LTM cell switch MAC CE is done immediately before triggering LTM cell switch command, which defers the triggering of LTM cell switch command to the UE

We have illustrated these two variants in Figure 2 and Figure 3 below.

Figure 2. Approach 2a for LTM cell switch execution in the context of the overall LTM procedure. Early before the LTM cell switch “decision”, the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE.

Figure 3. Approach 2b for LTM cell switch execution in the context of the overall LTM procedure. Immediately after the LTM cell switch “decision”, the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE.

Essentially by looking at the different approaches, we see that the main difference lies in which order the serving DU transmits the LTM cell switch command to the UE and informs the target DU via the CU about the execution. So besides the timing of inter-node signalling during execution, another difference is when the candidate DU becomes aware of which UL TCI state the UE will use and can start listening for the UE.

As we should aim for the fastest possible LTM execution in the UE, we think that even if there are likely cases when the candidate DU is informed about the LTM execution by the CU before the UE arrives, in approach 1 the candidate DU needs to be prepared for and listen for the UE in the target cell already after the LTM candidate cell configuration was performed (e.g. during the UL / DL pre-sync).

- In approach 1, due to race conditions in this approach, the candidate DU typically needs to be prepared for the UE arriving in target cell before the serving DU transmits the LTM cell switch command.

This preparation can even be performed during UL or DL pre-sync.

We think that the discussion about signalling approaches is rather about the inter-node signalling support for transferring the following information between the serving DU and the candidate DU and it should be up to the network implementation when to trigger the transfer of this information (e.g. during UL or DL pre-sync or just before the LTM cell switch command MAC CE is sent to the UE).

Once more we want to highlight that as we explained above all approaches are feasible and we propose that RAN3 remains open to all approaches until more feedback is received from RAN2.

- Since all approaches are feasible, RAN3 remains open to all approaches until feedback is received from RAN2.

## 2.2	Notification of LTM triggering to gNB-CUs

Another issue that was discussed in the previous meeting has to do with notification of LTM triggering to gNB-CUs and is related to the following agreement.

Intra-CU UP case: CU will start data transmission after LTM cells switch signaling from DU including target cell ID.

The following two options were discussed:

- Option 1: New F1 message, class class 1 or class 2, such as “LTM CELL CHANGE NOTIFICATION”.

- Option 2: Legacy message, i.e., “UE CONTEXT MODIFICATION REQUIRED” message

And the following WA was agreed:

WA: option 1 is adopted with only class 2 procedure. If more information to source DU in response message is needed, we go for option 2.

We note that both options are feasible. On the other hand, we see also the benefits of introducing a new procedure that can be only class 2, which would be more future proof and less prone to errors. As it seems for now there is no need for a response message, since the objective is that the CU is notified about the LTM cell switch, and the target cell ID. Based on that we propose that the WA is turned to agreement.

- We propose to turn the WA to an agreement that a new Class-2 procedure is introduced to notify gNB-CU about the LTM cell switch.

## 2.3	E1 aspects

Regarding E1 aspects the following agreements were made in the previous meeting:

For LTM without UP change case, propose to turn the following WF to an agreement:

In case of CP UP separation, once CUCP receives LTM cell switch signaling from (source)DU , CU CP initiates E1 bearer context modification to the CU UP including DL tunnel ID per DRB for target cell, for data transmission.

For LTM with UP change, once the CU-CP receives LTM cell switch signaling from (source) DU, the CU-CP initiates E1 bearer context modification to the target CU UP including DL tunnel ID per DRB for target cell for data transmission.

For LTM with UP change, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP.

For LTM with UP change after detecting the UE has accessed to the target cell, the CU-CP initializes Path switch procedure towards the core network.

In R3-233453 a TP for inter-DU LTM with UP change procedure was agreed. The procedure used for inter-DU LTM with the change of gNB-CU-UP within a gNB is shown below.

Figure 8.2.1.z LTM with the change of gNB-CU-UP

0. The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1. The gNB-CU-CP decides to initiate LTM configuration.

2. The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4 - 5. F1 UE context setup procedure is performed to setup one or more bearers in the gNB-DU.

6 - 7.The gNB-CU-CP sends the RRC Reconfiguration message to the UE. Details are FFS.

8.  The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a candidate target cell.

9. WA: The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

10-11. The gNB-CU-CP performs the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers.

12-13.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UP/DL status to the target gNB-CU-UP.

14.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

15.  The target gNB-DU detects the UE in the target cell.

16. The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

17 - 19. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

20-21.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-DU.

In principle, the call flow shown above can work and is not erroneous. Nevertheless, we note that the impact on CU-UP is substantial. Namely, the CU-UP will need to produce UL TNL per DRB for every configured candidate cell and signal them to CU-CP. For that reason, we investigated alternative ways that would lead to a minimization of the impact on CU-UP.

- The impact on CU-UP needs to be minimized.

In our quest to minimize the CU-UP impact we explored two different options.

One way would be that the steps 2 and 3 are executed together with steps 12 and 13. Namely after step 9 where the DU informs the CU-CP about triggering LTM to the target cell, the CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the CU-UP. The CU-CP decides flow-to-DRB mapping and sends the generated SDAP and PDCP configuration to the CU-UP. The CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, and DL TNL address information for NG-U.

After these 2 steps, the CU-CP sends a BEARER CONTEXT MODIFICATION REQUEST message containing the DL TNL address information for F1-U, and PDCP status. And in its turn the CU-UP responds with a BEARER CONTEXT MODIFICATION RESPONSE message.

By performing steps 2 and 3 in the call flow above together with steps 12 and 13, the CU-UP will need to provide UL TNL address information only for the target cell.

- One option that can minimize the impact on CU-UP when performing LTM is that the steps 2 and 3 are executed together with steps 12 and 13.

Another alternative would be that in step 3 the CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing only one UL TNL address information for F1-U to all the candidate cells but only the target cell will be allowed to use it. In that case the BEARER CONTEXT SETUP REQUEST message indicates to CU-UP not to initiate sending downlink packets until the UE successfully accesses the target cell. Based on the above we make the following proposal.

- One more option that can minimize the impact on CU-UP when performing LTM is that the CU-UP provides only one UL TNL address which will only be used by the target cell after successful execution of LTM cell switch.

## 2.4	TA management

RAN2 has agreed the following for L1/L2-triggered mobility:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch).

R2 assumes that Early TA RACH option 3 (with RAR from candidate cell) is not needed in Rel-18.

RAN1#121bis-e agreements on TA, as received in the RAN1 LS R1-2304276

Agreement

When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case.

Agreement

When reception of RAR is configured, support RAR is received from serving cell in inter-DU case.

- FFS: RA response window related issues

In RAN2#122 the following was agreed concerning TA acquisition

- For early TA acquisition for candidate Cells

For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network).

RAN2 doesn’t see a need for a solution with RAR in for Rel-18.

Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18

The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed).

In the previous RAN3 meeting the following agreements were made:

CU can request RACH resources for early TA acquisition together with the LTM candidate cell configuration to a Candidate DU in the UE Context Setup Request or UE Context Modification Request messages (for inter-DU LMT and intra-DU LTM respectively).

If the Candidate DU accepts the RACH resource request for early TA acquisition, the Candidate DU responds the CU with RACH configuration in the UE Context Setup Response or UE Context Modification Response messages.

In the following, we will discuss the RAN3 issues related to the solution for TA management where the TA value for the LTM candidate cell is calculated by the C-DU based on the transmitted preamble and is received by the UE in the LTM cell switch command and propose a way forward.

Below the signalling flow for inter-DU LTM configuration with TA establishment configuration and triggering is shown

Figure 4 signalling flow for inter-DU LTM configuration with TA establishment configuration and triggering.

### 2.4.1	TA establishment triggering

RAN1 has agreed that once the UE receives the RACH configuration for the TA establishment procedure (for one or more LTM candidate cells) the TA establishment is triggered by the reception of a PDCCH order from the serving cell which needs to indicate the LTM candidate cell configuration to which the UE needs to transmit a PRACH preamble for the TA establishment.

Since the PDCCH order is transmitted by the S-DU, it makes sense to consider that it is indeed the S-DU which triggers the initiation of the TA establishment procedure at the UE. This is done without the need to involve the CU and/or the C-DU(s) also because it is the S-DU which receives the latest CSI measurements on LTM candidate cell(s).

- The S-DU triggers the initiation of TA establishment at the UE without the CU or C-DU(s) involvement.

### 2.4.2	TA value in LTM cell switch

At the network side, the handling of the TA value may depend on whether approach 1 or approaches 2a and 2b described above is used for the LTM cell switch.

Assuming approach 1, in which the S-DU triggers the LTM cell switch without first requesting the C-DU (via the CU), the C-DU needs to provide the TA value to the S-DU just after it calculates it. Thus, the C-DU receives the preamble at an LTM candidate cell, calculates a TA value associated to that preamble and transmits the TA value to the CU, which provides the TA value to the S-DU.

Assuming approach 2a or 2b, in which the S-DU contacts the C-DU (via the CU) before triggering LTM cell switch, a similar solution could be adopted, though approach 2 opens for another possibility in which the C-DU response may include the TA value.

- When TA establishment is triggered the C-DU receives the preamble, calculates the TA value and provides it to the CU (which forwards it to the S-DU).

Regarding the handling of TA value irrespective of which procedure will be decided, the UE Context Modification (DU initiated) procedure could be used to signal it. We note that when the C-DU sends the TA values, there might be overlapping configurations, it can send the mapping between preamble and TA values to the CU and then to the S-DU, as agreed by RAN2:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

Based on that we believe that the best solution is the definition of a new message which is more future proof and would be less error prone. A new message would be non-UE associated and thus a better fit for the purpose of the procedure in question.

- Regarding the handling of the TA value we consider that the best way forward is the introduction of a new non-UE associated procedure.

### 2.4.3	TA acquisition failure and fallback solutions

RAN1 has made some agreements concerning the fallback procedure when a preamble transmission attempt by the UE fails:

***************************************************************************************************************************

Agreement

For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

- If reception of RAR is not configured/indicated (without RAR)

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- […]

- UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- Send LS to RAN2 and RAN3 to check the feasibility about this agreement

- Note: Definition of candidate cells is up to RAN2

***************************************************************************************************************************

In the TA establishment solution, there needs to be a way to acknowledge the successful reception of the preamble. In our view, the simplest solution is that the S-DU transmits the PDCCH order, assumes the UE transmits the preamble, and monitors the reception from the C-DU (via the CU) of the TA value for the LTM candidate cell. The reception of the TA value at the S-DU acknowledges that the preamble reception (assuming approach 1), triggered by the PDCCH order, has been successfully received at the C-DU. When the S-DU transmits the PDCCH order it could start a timer or monitor a time window, so that the expiry of this timer or timer windows triggers the absence of the TA value and indicates the failure, which may trigger the S-DU to transmit a sub-sequent PDCCH order with power ramping (fallback). However some of these details could be left to network implementation.

- When TA establishment without RAR is configured, the reception of the TA value at the S-DU indicates the successful reception of the preamble at the C-DU.

- When TA establishment without RAR is configured, the absence of the TA value at the S-DU indicates a failed attempt of preamble reception at the C-DU.

An example of signalling flow with the fallback is shown below:

Figure 5 signalling flow for fallback when RAR is not configured

### 2.4.4	TA validity and time alignment timer(s)

In a legacy RA procedure, the UE transmits a preamble and receives the TA value in the RAR, based on which the UE starts a Time Alignment timer with a value configured in RRC. However, when RAR is not configured for TA establishment, the TA value is received at the S-DU for LTM candidate cell and is only to be provided to the UE when the LTM cell switch is triggered. Thus, as the UE does not receive the TA value at the moment or close to the moment the TA value is calculated, it makes sense to leave the monitoring of its validity to the network. For example, in the network implementation the S-DU could start something like a Time Alignment timer and while that is running the S-DU assumes that the received TA value is valid and can be provided to the UE in the LTM cell switch command.

- When TA establishment without RAR is configured, there is no time alignment timer at the UE for the LTM candidate cell(s) as the TA value validity is monitored by the network.

## 2.5	Early DL sync

RAN1 has agreed that an indication of a beam or a TCI state (e.g. a TCI state index or some form of TCI identifier) will be included in the LTM cell switch command (i.e. MAC CE indicating the UE to perform LTM cell switch to an LTM candidate cell), with the intention that the UE identifies a TCI state of the LTM candidate cell during LTM cell switch. In addition, RAN1 has also agreed that the UE may receive the indication of a TCI state of an LTM candidate cell even before the LTM cell switch command is received, in a procedure which is sometimes called DL pre-sync with LTM candidate cell(s), since the TCI state ID provided even before the LTM cell switch command.

As in the case of early UL pre-sync, the CU may request a C-DU to provide the configuration associated to DL pre-sync either when it is requesting the C-DU to configure LTM candidate cell(s) in a UE CONTEXT SETUP REQUEST and/or afterwards, for an already configured LTM candidate, in a UE CONTEXT MODIFICATION REQUEST message.

The CU can receive the indication of the TCI of an LTM candidate cell in the UE Context Setup Response message and then can forward this indication to the serving DU via the UE Context Modification procedure. So legacy procedures can be used for early DL sync. Thus, we propose the following.

- Legacy procedures can be used for early DL sync configuration. The CU can request DL pre-sync configuration either in a UE CONTEXT SETUP REQUEST or a UE CONTEXT MODIFICATION REQUEST. In response, the CU receives the DL pre-sync configuration of in the UE Context Setup Response or the CONTEXT MODIFICATION RESPONSE message and then can forward this indication to the serving DU via the UE Context Modification procedure.

An example of a signaling flow between CU, S-DU and C-DU is provided below for the case in which the TCI state ID for an LTM candidate is included in a MAC CE received by the UE for pre-activating a TCI state before the reception of an LTM cell switch command.

Figure 6 DL pre-sync and provision of TCI state ID of a candidate before an LTM cell switch command

## 2.6	Handover collision avoidance between LTM and L3 handover

The discussion on the co-existence of L3 handover and LTM continued in the previous meeting also.

The following options have been discussed:

-	Option 1: OAM configured priority.

-	Option 2: Network decides the priority based on scenario (intra-gNB-CU or inter-gNB-CU) and some assistance information (the measurement results, candidate target cells).

-	Option 3: Flexible priority. The handover triggered first take the high priority.

We think that by fixing the priority between L3 mobility and LTM will be a suboptimal solution. It is obvious that it can not be known a priori in each case which is the best cell for mobility and how to attain best performance. Having a fixed priority will inevitably lead to inferior performance.

- The priority of mobility should not be fixed.

We also note that LTM is triggered based on L1 measurements that are periodic while L3 mobility is triggered based on L3 measurements that are event based. This entails that LTM will be triggered faster in most cases and the possibility of co-existence of L3 mobility and LTM does not seem very prominent. As discussed in the previous meeting the following cases were mentioned for option 3:

- Case 1, L3 handover triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority.

- Case 2, LTM triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority,

- Case 3, LTM and L3 handover are triggered almost simultaneously (cross signallings on F1).

In case 1, the DU may wait to send the LTM cell switch command until L3 handover is completed. Along the same lines in case 2, the CU will not send a HO command as long as LTM is under execution. Similarly in case 3, the DU will respond the UE Context Modification Request received from CU with UE Context Modification Failure and after LTM cell switch is completed the CU can re-initiate the L3 handover.

In the Chair’s notes we find also the following cases:

-	Case 1: The DU receives the L3 HO command while the DU is about to trigger LTM switching the UE to a different target cell.

-	Case 2: The CU receives the LTM notification from the DU after the CU has started preparing (inter-CU) L3 HO but has not triggered L3 HO.

-	Case 3: The DU receives the L3 HO command while the DU has already triggered the LTM. In this case, the UE may have already switched to another cell, and the L3 HO is no longer applicable.

In case 1, the DU refrains from triggering LTM switching and it proceeds with the L3 HO. In case 2 and 3 the UE might have already switched to another cell, so in case 2 the CU does not proceed with the L3 HO, while in case 3 the DU responds with UE Context Modification Failure to indicate that the L3 HO command is not accepted.

Also the following agreement made in RAN2 about the LTM and L3 HO coexistence:

While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

The agreement assumes that it is be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO), e.g. avoid sending LTM switch command and L3 HO command simultaneously.

Based on the above we believe the best way forward in order to assure best performance is to leave the issue of co-existence of L3 mobility and LTM up to network implementation.

- Leave the issue of co-existence of L3 mobility and LTM up to network implementation.

The stage-2 signaling flows for LTM execution (Approach 1), Early UL sync, and Early DL sync are provided in the Annex. Correspondingly, the F1AP TP is provided in [2].

- RAN3 to agree the stage-2 signaling flows in the Annex and F1AP TP in [2].

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In approach 1, due to race conditions in this approach, the candidate DU typically needs to be prepared for the UE arriving in target cell before the serving DU transmits the LTM cell switch command.

Observation 2	The impact on CU-UP needs to be minimized.

In this paper we propose the followings.

Proposal 1	Since all approaches are feasible, RAN3 remains open to all approaches until feedback is received from RAN2.

Proposal 2	We propose to turn the WA to an agreement that a new Class-2 procedure is introduced to notify gNB-CU about the LTM cell switch.

Proposal 3	One option that can minimize the impact on CU-UP when performing LTM is that the steps 2 and 3 are executed together with steps 12 and 13.

Proposal 4	One more option that can minimize the impact on CU-UP when performing LTM is that the CU-UP provides only one UL TNL address which will only be used by the target cell after successful execution of LTM cell switch.

Proposal 5	The S-DU triggers the initiation of TA establishment at the UE without the CU or C-DU(s) involvement.

Proposal 6	When TA establishment is triggered the C-DU receives the preamble, calculates the TA value and provides it to the CU (which forwards it to the S-DU).

Proposal 7	Regarding the handling of the TA value we consider that the best way forward is the introduction of a new non-UE associated procedure.

Proposal 8	When TA establishment without RAR is configured, the reception of the TA value at the S-DU indicates the successful reception of the preamble at the C-DU.

Proposal 9	When TA establishment without RAR is configured, the absence of the TA value at the S-DU indicates a failed attempt of preamble reception at the C-DU.

Proposal 10	When TA establishment without RAR is configured, there is no time alignment timer at the UE for the LTM candidate cell(s) as the TA value validity is monitored by the network.

Proposal 11	Legacy procedures can be used for early DL sync configuration. The CU can request DL pre-sync configuration either in a UE CONTEXT SETUP REQUEST or a UE CONTEXT MODIFICATION REQUEST. In response, the CU receives the DL pre-sync configuration of in the UE Context Setup Response or the CONTEXT MODIFICATION RESPONSE message and then can forward this indication to the serving DU via the UE Context Modification procedure.

Proposal 12	The priority of mobility should not be fixed.

Proposal 13	Leave the issue of co-existence of L3 mobility and LTM up to network implementation.

Proposal 14	RAN3 to agree the stage-2 signaling flows in the Annex and F1AP TP in [2].

# 4	References

RP-223520, Revised WID on Further NR mobility enhancements, MediaTek Inc., Apple

R3-233906, (TP for LTM BL CR to TS 38.473) F1AP impacts for LTM

# Annex A: TP to BL CR for TS 38.401 to support LTM

#### 8.2.1.Y	Inter-gNB-DU LTM

##### 8.2.1.Y-x	Approach 1 for inter-DU LTM Execution

Figure 8.2.1.Y-x: Approach 1 for inter-DU LTM Execution

The figure above shows in detail the steps for the execution of Inter-gNB-DU L1/L2 Triggered Mobility. The steps pertaining to the configuration of Inter-gNB-DU L1/L2 Triggered Mobility are not shown in detail.

- Configuration of candidate target cells and TA management

- The UE sends the lower layer measurement result to the source gNB-DU.

- DL pre-sync with the candidate target cells

- UL pre-sync with the candidate target cells

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

- The source gNB-DU sends LTM command to the UE.

- The source gNB-DU signals the gNB-CU about the initiation of the L1/L2 triggered mobility command to the UE including the Target cell ID.

- UE executes the LTM cell switch procedure.

- A Random Access procedure is performed at the target gNB-DU or the first UL transmission is sent to the target gNB-DU.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

- The UE responds to the target gNB-DU with an RRCReconfigurationComplete message.

- The target gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received RRCReconfigurationComplete message.

##### 8.2.1.Y-y	Early UL sync

Figure 8.2.1.Y-y: Early UL sync for inter-DU LTM

Early UL sync comprises the following steps:

- The UE sends a MeasurementReport message (L3 measurements) to the source gNB-DU containing measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 triggered mobility configuration and TA establishment

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells and also request for TA establishment.

- If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s) and the RACH configuration for TA establishment.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 triggered mobility configuration.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU sends PDCCH order to the UE for triggering TA establishment, indicating a RACH resources/ preamble from the RACH configuration for TA establishment.

- The UE sends to the candidate gNB-DU a RACH preamble according to RACH configuration for TA establishment as indicated in the PDCCH order from the S-DU.

- The candidate gNB-DU sends to the gNB-CU the TA value of the target candidate cell associated to the received RACH preamble. The reception in the source gNB-DU of the TA value from the gNB-CU indicates the successful reception of the RACH preamble in the C-DU, otherwise a RACH preamble re-transmission may be triggered by the S-DU.

FFS: introduce new message or reuse legacy message for the non-UE-associated signalling which includes the TA value.

- The gNB-CU sends to the source gNB-DU the TA value of the candidate target cell associated to the received RACH preamble.

FFS: introduce new message or reuse legacy message for the non-UE-associated signalling which includes the TA value.

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

- The source gNB-DU sends LTM command including the TA value of the target candidate cell to the UE.

- The UE transmits PUSCH/PUCCH to the candidate DU based on TA value of the target candidate cell.

##### 8.2.1.Y-z	Early DL sync

Figure 8.2.1.Y-z: Early DL sync for inter-DU LTM

DL sync comprises the following steps:

- The UE sends a MeasurementReport message (L3 measurements) to the source gNB-DU containing measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 triggered mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells and request the configuration associated to DL pre-sync.

- If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s) and a TCI state configuration per accepted candidate target cell for DL pre-sync.

4a.	 The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU, including a TCI state configuration per accepted candidate target cell.

4b.	 Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the source gNB-DU sends a UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU with the updated version of the UE’s current CellGroupConfig including a TCI state configuration per accepted candidate target cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message including the updated version of the UE’s current CellGroupConfig including a TCI state configuration per accepted candidate target cell.

- The source gNB-DU forwards the received RRCReconfiguration message including the updated version of the UE’s current CellGroupConfig including a TCI state configuration per accepted candidate target cell to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU sends a UL RRC MESSAGE TRANSFER message to the gNB-CU, which includes the RRCReconfigurationComplete message.

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source DU sends a TCI pre-activation command to the UE.