3GPP TSG-RAN WG3 Meeting #121	R3-233911

Toulouse, FR, 21st – 25th Aug, 2023

Agenda item:		12.2.1

Source:				Intel Corporation

Title:	Discussion on Prediction Time and Accuracy

Document for:		Discussion and Decision

# Introduction

During RAN3 #120 meeting, following agreements were made for prediction time in AI/ML information request procedure:

The requested prediction time includes the specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future. The encoding needs to be further discussed.

FFS on the encoding and exact values of the specific time point. Down-select the options:

- INTEGER TYPE

- ENUMERATED TYPE

FFS on the benefits of the predicted information over a period of time.

The specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future, can be configured for both one-time reporting and periodic reporting.

In this contribution, we mainly focus on the benefit of including a period of time for predicted information and encoding of the specific time point agreed in RAN3 #120 meeting.

# Discussion

## Prediction Time

### Encoding of Prediction Time

In the RAN3 #120 meeting, the encoding of prediction time is expected to be down-selected from below two options:

- Option 1: INTEGER type

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested prediction time |  |  | INTEGER (0..4095, ...) | Unit: seconds |

- Option 2: ENUMERATED type

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested prediction time |  |  | ENUMERATED(0s, 5s, 10s, 20s, 40s, 80s, 160s, 320, 640, …) FFS on the values | Unit: seconds |

Compared with ENUMERATED type, INTEGER type within a reasonable future is more flexible. First, it may limit the usage at the requesting NG-RAN node for only several specific timing. Moreover, the ENUMERATED type with discrete values of larger granularity may also limit AI/ML algorithm implementation, since the AI/ML algorithms at requested NG-RAN node should be designed to only generate predicted information at the discrete timing.

Without the flexibility of requesting prediction information at any time, the requesting NG-RAN node may not be able to make accurate decision for its own real-time decisions. For example, if one NG-RAN node wants to use AI/ML model to make decision at Timing N, compared with requesting the prediction information at neighbouring NG-RAN node at discrete values (i.e. may not be Timing N), requesting for exact Timing N may allow the requesting NG-RAN node generate more accurate inference result by considering the situation at the target NG-RAN node for the corresponding timing.

Observation 1: Encoding with INTEGER type for the requested prediction time is more flexible and more helpful to make accurate inference decisions than ENUMERATED type.

Proposal 1: Use INTEGER type for the specific time point of the requested prediction time. FFS on the range.

### Time Interval and Validity Time

It is FFS on the benefits of the predicted information over a period of time.

FFS on the benefits of the predicted information over a period of time.

For one-time reporting, the requesting NG-RAN node may need to send AI/ML information request message to the neighbouring NG-RAN node whenever there’s a need to use AI/ML model to generate inference results. However, considering inference processing time, it may be very difficult for the neighbouring NG-RAN nodes to generate multiple requested prediction information in a short time very frequently.

Observation 2: It is very difficult for requested NG-RAN node to frequently generate prediction information in a short time.

Time Interval in AI/ML Information Request message

One compromise solution is to introduce time interval for one-time reporting. However, this also means neighbouring NG-RAN nodes need to provide a “valid” value over the requested period, so that the source NG-RAN node can trust the prediction information from neighbouring NG-RAN nodes. This “valid” value can be a generalized value over the requested period. Though prediction information over a period may be less accurate than the prediction information at an instant time, if it is tolerable at the source NG-RAN node, there’s no harm to introduce time interval in the AI/ML information request message. Even the source NG-RAN node requires a higher accuracy, it can still set the time interval to “zero” if the exact prediction information at an instant time stamp is needed.

Observation 3: Though the accuracy of generalized value over the requested time interval may degrade, the source NG-RAN node can set time interval to “zero”.

Validity Time in AI/ML Information Update message

Alternatively, if the time interval is not provided in the AI/ML Information request message, validity time can be provided by the requested NG-RAN node in the AI/ML information update message, indicating the duration that a requesting NG-RAN node can use/trust the received prediction information. In this option, there’s no limitation for AI/ML algorithm implementation, the requested NG-RAN node can provide such information based on its availability.

Upon receiving one prediction information with its corresponding validity time, the source NG-RAN node does not need to resend AI/ML Information request message, hence the overload situation can be avoided.

For periodic reporting, the configured report periodicity can be used for the purpose of time interval. Therefore, at least for one-time reporting, time interval or validity time can be helpful.

Proposal 2: At least for one-time reporting, RAN3 to consider below two options to indicate the period of prediction information:

Option 1: Introduce a time interval in AI/ML Information request message, where the requested NG-RAN node provides a generalized prediction information over the requested period.

Option 2: Introduce a validity time in AI/ML Information update message, where the requesting NG-RAN node uses the received value over the corresponding period and only requests an update of prediction information after the time expires.

## Confidence Level

For the predicted data received from a neighbouring NG-RAN node, it is important for the receiving NG-RAN node to know whether the predicted data can be trusted or not, i.e. the confidence level. If the confidence level of the predicted data is low, the NG-RAN node may be better not to use such information for model inference or model training. Alternatively, the NG-RAN node may also take the confidence level as the weight of using the predicted data for the corresponding AI/ML algorithm. The confidence level of the predicted data can be transmitted together with the predicted information in the Predicted Information Update message over Xn interface.

Proposal 3: Confidence level of the requested predicted data should also be provided together with the predicted information in the Predicted Information Update message over Xn interface.

# Conclusion

In this contribution, we discussed the remaining issue on prediction time and accuracy of AI/ML information procedure.

We propose the following observations and proposals:

Observation 1: Encoding with INTEGER type for the requested prediction time is more flexible and more helpful to make accurate inference decisions than ENUMERATED type.

Proposal 1: Use INTEGER type for the specific time point of the requested prediction time. FFS on the range.

Observation 2: It is very difficult for requested NG-RAN node to frequently generate prediction information in a short time.

Observation 3: Though the accuracy of generalized value over the requested time interval may degrade, the source NG-RAN node can set time interval to “zero”.

Proposal 2: At least for one-time reporting, RAN3 to consider below two options to indicate the period of prediction information:

Option 1: Introduce a time interval in AI/ML Information request message, where the requested NG-RAN node provides a generalized prediction information over the requested period.

Option 2: Introduce a validity time in AI/ML Information update message, where the requesting NG-RAN node uses the received value over the corresponding period and only requests an update of prediction information after the time expires.

Proposal 3: Confidence level of the requested predicted data should also be provided together with the predicted information in the Predicted Information Update message over Xn interface.