3GPP TSG-RAN WG3 Meeting #121                                                      	R3-233925

Toulouse, Aug 21st – Aug 25th, 2023

Agenda item:	10.2.5

Source:	Qualcomm Incorporated

Title:	SON enhancements for NR-U

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the open issues on SON enhancements for NR-U based on the discussion in the previous RAN3 meetings.

# Discussion

  - DL LBT issues at target gNB during handover execution

FFS whether to enable a UE-based solution or a NW-based solution to inform the source gNB of a handover about DL LBT issues occurring at the target gNB, during a handover execution

Suppose a handover failed due to continuous DL LBT failures at the target gNB (e.g., LBT kept failing for MSG2 or MSG4), three options were discussed (one UE-based approach and two NW-based approaches) on how to inform the source gNB:

A) UE-based approach: the UE adds in RLF report an indication of DL LBT failure causing absence of SSB transmissions during handover

B) NW-based approach 1: the target gNB sends to source gNB an indication of DL LBT failure during handover execution

C) NW-based approach 2: the target gNB logs the waiting time in DL and sends a network-side failure report

Regarding Option A) i.e., the UE-based approach, we don’t think absent SSB transmissions at UE during handover execution can exactly mean the existence of DL LBT issues at NW side, for example, there is a possibility that the UE can’t detect the DL SSB due to beam quality is poor. Therefore, option A (UE based approach) is not appropriate to detect DL LBT failures.

Observation 1: Option A (UE based approach) is not appropriate to detect DL LBT failures because absent SSB transmissions at UE during handover execution doesn’t always mean the existence of DL LBT issues at gNB side

Regarding Option B), a concern was raised that the source gNB (performing root cause analysis) has to collect information from two sources (RLF from UE and also a notification from peer node) and in principle the two inputs can arrive at the source gNB at very different times. In our view, this can be left to gNB implementation on how to do the correlation.

Proposal 1: Target gNB can send an indication that DL LBT failures were encountered during handover execution to source gNB post a handover failure.

  - Distinguishing true mobility failures vs. LBT related mobility failures

Enhancements of RLF reports and RA reports are beneficial to separate mobility related errors from the LBT-related ones

One solution proposed to distinguish true mobility failures vs. LBT failures is to add some LBT related time information in RLF Report e.g., time between each LBT start and LBT success or time between each LBT start and a subsequent LBT failure or waiting time. It was argued that the network can decide how the report should be used while doing MRO analysis, for example, if long time duration is spent for LBT, it may mean that the failure is mainly caused by channel occupancy and not due to bad mobility thresholds.

We are not convinced that knowing this exact time can help much in MRO analysis and requesting UE to compute this time during each LBT attempt is too much processing at the UE. We already send an indication that HOF is due to consistent LBT failure in RLF Report and information concerning LBT failures in RA procedure (e.g., number of LBT failures that is being discussed) and that should be sufficient.

Observation 2: gNB can't optimize much knowing the exact timing related information related to LBT e.g., time between LBT start to LBT success/failure. Also requesting the UE to compute this time during each LBT attempt is too much processing at the UE.

Proposal 2: The already agreed indication that HOF is due to consistent LBT failure and information concerning LBT failures in RA procedure during handover is sufficient to distinguish true mobility failures with LBT-related mobility failures

  - MLB for NR-U

FFS whether to enhance RLF report to include information determining the outcome of the channel access procedure, with focus on EDT UL

FFS on whether the EDT UL sent over Xn can also consider the EDT UL reported by UEs.

There is no need for the UE to report the average of the applied EDT UL in RLF report

The EDT in UL included in the XnAP RESOURCE STATUS UPDATE message reflects the average of the configured maximum EDT UL

FFS on the introduction in F1 of EDT in UL in RESOURCE STATUS UPDATE message.

maxEnergyDetectionThreshold-r16 should already be sent as part of CellGroupConfigInfo from gNB-DU to gNB-CU. Therefore, we don’t think there is a need to explicitly send the EDT UL over F1AP

Observation 3: maxEnergyDetectionThreshold-r16 should already be sent from gNB-DU to gNB-CU as part of CellGroupConfigInfo.

Proposal 3: There is no need to enhance F1AP RESOURCE STATUS UPDATE to signal maxEnergyDetectionThreshold-r16

Further, we think the WA on exchanging Radio Resource Status per NR-U channel can be made into ana agreement.

WA: introduce an optional load metric on Radio Resource Status per NR-U Channel in XnAP RESOURCE STATUS UPDATE message and in F1AP RESOURCE STATUS UPDATE message.

Proposal 4: Convert the following WA into agreement:

WA: introduce an optional load metric on Radio Resource Status per NR-U Channel in XnAP RESOURCE STATUS UPDATE message and in F1AP RESOURCE STATUS UPDATE message.

# Conclusion

DL LBT issues at target gNB during handover execution

Observation 1: Option A (UE based approach) is not appropriate to detect DL LBT failures because absent SSB transmissions at UE during handover execution doesn’t always mean the existence of DL LBT issues at gNB side

Proposal 1: Target gNB can send an indication that DL LBT failures were encountered during handover execution to source gNB post a handover failure.

Distinguishing true mobility failures vs. LBT related mobility failures

Observation 2: gNB can't optimize much knowing the exact timing related information related to LBT e.g., time between LBT start to LBT success/failure. Also requesting the UE to compute this time during each LBT attempt is too much processing at the UE.

Proposal 2: The already agreed indication that HOF is due to consistent LBT failure and information concerning LBT failures in RA procedure during handover is sufficient to distinguish true mobility failures with LBT-related mobility failures.

MLB for NR-U

Observation 3: maxEnergyDetectionThreshold-r16 should already be sent from gNB-DU to gNB-CU as part of CellGroupConfigInfo.

Proposal 3: There is no need to enhance F1AP RESOURCE STATUS UPDATE to signal maxEnergyDetectionThreshold-r16

Proposal 4: Convert the following WA into agreement:

WA: introduce an optional load metric on Radio Resource Status per NR-U Channel in XnAP RESOURCE STATUS UPDATE message and in F1AP RESOURCE STATUS UPDATE message.

# 4. References