3GPP TSG-RAN WG3 Meeting #121                                                      	R3-233928

Toulouse, Aug 21st – Aug 25th, 2023

Agenda item:	11.3

Source:	Qualcomm Incorporated

Title:	Support for QoE in NR-DC

Document for:	Discussion and Decision

# Introduction

In this paper, we continue the discussion on how to support QoE/RVQoE in NR-DC based on agreements and open issues last meeting.

# Discussion

  - Which SRB to use for QoE configuration in NR-DC?

### s-based QoE

The following was agreed in a previous meeting:

MN is responsible to configure the s-based QoE to UE.

But the BL CR to 37.340 shows otherwise as follows:

For signalling-based and management-based QoE activation, the MN:

- Allocates the measurement configuration application layer ID;

- Determines whether the MN or the SN sends the QoE configuration to the UE;

- Notifies the SN accordingly.

In our view, there is no use case for MN asking the SN to send the s-based QoE configuration via SRB3. Also RAN2 agreed that the s-based QoE configuration should be always sent via SRB1. Therefore, we make the following proposal:

Proposal 1: For s-based QoE, MN always sends the QoE configuration via SRB1. Revise the BL CR to 37.340 to reflect this.

### m-based QoE received by the MN

If the m-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE

Similar to s-based QoE, we think there is no use case for MN to send the m-based QoE configuration (received from OAM) via SRB3. We therefore propose the following changes to the previous agreement:

Proposal 2: Revert the previous agreement:

If the m-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE

and agree to this instead:

If the m-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and should send the QoE configuration to the selected UEs via SRB1

### m-based QoE received by the SN

Support the following scenarios for m-based QoE configuration received in the SN:

- The SN wants to configure the UE by using SRB3

- The SN wants to configure the UE, by sending the configuration in a transparent container to the MN, which then sends it to the UE via SRB1

In case the SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

- The MN shall send the configuration information to the UE, or

- The SN should send the configuration to the UE directly, or

- The SN should send the configuration information to the UE via the MN (inside a container)

FFS whether for management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE via SRB3.

FFS whether for management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE transparently for the MN via SRB1.

In our view, when SN receives a m-based QoE configuration from OAM, these can be the sequence of steps:

- SN asks MN whether it can send the QoE configuration to the UE

- If the MN has also received the m-based QoE configuration from OAM,

  - it replies to SN that it will configure (or has already configured) the UE with this m-based QoE configuration and asks SN to not send this QoE configuration to the UE

- Else

  - MN replies to SN to send the m-based QoE configuration to the UE

- SN can autonomously decide whether it wants to use SRB3 or use SRB1 (via a container to MN) to configure the UE

As outlined above, we think that there is no need for MN to indicate the QoE configuration preference to SN and it can autonomously decide which SRB to use for QoE configuration (e.g., it uses SRB3 if available, else it may use SRB1).

There is no need to ask permission from MN to use SRB1; SN can simply send the QoE configuration in a container explicitly over Xn or over inter-node RRC message and MN will include this as SN RRCReconfiguration within a MN RRCReconfiguration like it’s done today for any SN configuration messages.

Proposal 3: SN can autonomously decide whether it wants to use SRB3 or use SRB1 (via a container to MN) to configure the UE with m-based QoE (after getting the confirmation from MN that SN should send the QoE configuration to the UE)

Proposal 4: Modify the previous agreement as follows:

In case the SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

- The MN shall send the configuration information to the UE, or

- The SN should send the configuration to the UE directly, or

- The SN should send the configuration information to the UE via the MN (inside a container)

Proposal 5: For management based QoE, there is no need for SN to explicitly indicate to the MN whether it wants to configure the UE directly via SRB3 or via a container and SRB1.

  - QoE/RVQoE reporting in NR-DC

The network can explicitly instruct a UE in NR-DC to switch the reporting leg

As the baseline, QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over the same leg.

WA: QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over different legs.

RAN3 understanding is that the UE can be explicitly instructed whether to report over SRB5 or SRB4.

There might be a use case (e.g., due to overload) where a node might want to switch the SRB to use for QoE reporting but still wants the RVQoE reports to be reported via the same SRB (as it might be the consumer and might want to avoid retrieving the RVQoE reports over Xn to avoid latency). So as an enhancement, we are OK if QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over different SRBs.

Observation 1: In case of overload, a node might want to switch the SRB to use for QoE reporting but still wants the RVQoE reports to be reported via the same SRB (as it might be the node providing the bearers and might want to avoid retrieving the RVQoE reports over Xn to avoid latency).

Proposal 6: QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over different SRBs by the UE, if explicitly instructed by the network

To achieve the above, the UE has to be explicitly instructed which SRB to use for QoE reports and RVQoE reports via two separate flags in QoE configuration.

Proposal 7: UE is explicitly instructed which SRB to use for QoE reports and RVQoE reports via two separate flags in QoE configuration. Send LS to RAN2 to confirm.

When a management-based QoE configuration is received directly by the SN from the OAM, the SN can explicitly indicate to the MN whether it is going to receive the QoE/RVQoE reports via the SRB5 or SRB4, whether it is transparently via SRB4 needs further discussion.

Considering a multi-vendor scenario (where MN and SN are from different vendors), MN shouldn’t be able to read the SN configured RVQoE configuration and the associated RVQoE reports i.e., it should pass transparently through the MN. Therefore, we think UE should be able to provide the SN-configured RVQoE reports either directly via SRB5 or transparently via SRB4.

Proposal 8: If the SN provides the RVQoE configuration to the UE (either via SRB3 or via container over SRB1), UE should be able to provide the RVQoE reports either directly via SRB5 or transparently via SRB4.

Similarly, UE can provide the MN-configured RVQoE reports either directly via SRB4 or transparently via SRB5. But to keep it simple, we propose that UE should always provide the MN-configured RVQoE reports directly via SRB4 (there is no need to report transparently via SRB5)

Further it can be up to RAN2 to decide which message to use (measurementReportAppLayer or ULInformationMRDC) to send the QoE/RVQoE report in the above scenarios.

Proposal 9: If the MN provides the RVQoE configuration to the UE (via SRB1), UE should always provide the MN-configured RVQoE reports directly via SRB4 (there is no need to report transparently via SRB5)

  - RVQoE configuration in NR-DC

FFS whether, in a UE in NR-DC, each QoE configuration can have more than one corresponding RVQoE configuration.

In our understanding, a UE in NR-DC should be configured with only one RVQoE configuration per QoE Reference. It doesn’t make sense (would be duplicate) if MN and SN tries to send two different RVQoE configurations for the same QoE Reference and then MN and SN coordinate later to “fix” this. We therefore have the following proposal:

Proposal 10: A UE in NR-DC should be configured with only one RVQoE configuration per QoE Reference

RAN3 has made the following agreements:

When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. FFS whether the indication is explicit or implicit.

In the response, the non-RVQoE-configuring node:

1. Can indicate to the RVQoE-configuring node that it does not want to receive the RVQoE reports.

2. Can indicate whether it prefers to receive the RVQoE reports directly from the UE.

3. Can indicate to the RVQoE-configuring node its preferred RVQoE configuration parameters, based on which the RVQoE-configuring node should modify the RVQoE configuration.

We think that an explicit indication would be a cleaner way and therefore propose the following:

Proposal 11: When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node explicitly indicates that to the non-RVQoE-configuring node

FFS whether the non-RVQoE-configuring node receives an RVQoE report and determines that the RVQoE-configuring node provides the bearer(s) for the application session, the non-RVQoE-configuring node indicates that to the RVQoE-configuring node. Whether in an explicit way or implicit way needs further discussion.

FFS whether the non-RVQoE-configuring node can update its preferred RVQoE configuration parameters and reporting leg preference, and it can request from the RVQoE-configuring node to update these parameters at the UE.

The above open issue is the reverse scenario of what was agreed last time. Consider a scenario where MN sends RVQoE configuration to the UE and indicates UE to use SRB5 for RVQoE reporting. SN (non-RVQoE configuring node) upon receiving the RVQoE report determines that the MN (RVQoE configuring node) provides the bearer(s) for the application session. Then the SN (non-RVQoE-configuring node) needs to indicate that to the MN (RVQoE-configuring node and can further indicate its preferred RVQoE configuration parameters so that the MN (RVQOE configuring node) can take those into consideration when the RVQoE configuration is modified.

Proposal 12: When the non-RVQoE-configuring node receives an RVQoE report and determines that the RVQoE-configuring node provides the bearer(s) for the application session, the non-RVQoE-configuring node indicates that to the RVQoE-configuring node and can further indicate its preferred RVQoE configuration parameters.

Proposal 13: In response, the RVQoE-configuring node can change the SRB used to report the RVQoE and update the RVQoE configuration.

# Conclusion

Which SRB to use for QoE configuration in NR-DC?

Proposal 1: For s-based QoE, MN always sends the QoE configuration via SRB1. Revise the BL CR to 37.340 to reflect this.

Proposal 2: Revert the previous agreement:

If the m-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE

and agree to this instead:

If the m-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and should send the QoE configuration to the selected UEs via SRB1

Proposal 3: SN can autonomously decide whether it wants to use SRB3 or use SRB1 (via a container to MN) to configure the UE with m-based QoE (after getting the confirmation from MN that SN should send the QoE configuration to the UE)

Proposal 4: Modify the previous agreement as follows:

In case the SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

- The MN shall send the configuration information to the UE, or

- The SN should send the configuration to the UE directly, or

- The SN should send the configuration information to the UE via the MN (inside a container)

Proposal 5: For management based QoE, there is no need for SN to explicitly indicate to the MN whether it wants to configure the UE directly via SRB3 or via a container and SRB1.

QoE/RVQoE reporting in NR-DC

Observation 1: In case of overload, a node might want to switch the SRB to use for QoE reporting but still wants the RVQoE reports to be reported via the same SRB (as it might be the node providing the bearers and might want to avoid retrieving the RVQoE reports over Xn to avoid latency).

Proposal 6: QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over different SRBs by the UE, if explicitly instructed by the network

Proposal 7: UE is explicitly instructed which SRB to use for QoE reports and RVQoE reports via two separate flags in QoE configuration. Send LS to RAN2 to confirm.

Proposal 8: If the SN provides the RVQoE configuration to the UE (either via SRB3 or via container over SRB1), UE should be able to provide the RVQoE reports either directly via SRB5 or transparently via SRB4.

Proposal 9: If the MN provides the RVQoE configuration to the UE (via SRB1), UE should always provide the MN-configured RVQoE reports directly via SRB4 (there is no need to report transparently via SRB5)

RVQoE configuration in NR-DC

Proposal 10: A UE in NR-DC should be configured with only one RVQoE configuration per QoE Reference

Proposal 11: When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node explicitly indicates that to the non-RVQoE-configuring node

Proposal 12: When the non-RVQoE-configuring node receives an RVQoE report and determines that the RVQoE-configuring node provides the bearer(s) for the application session, the non-RVQoE-configuring node indicates that to the RVQoE-configuring node and can further indicate its preferred RVQoE configuration parameters.

Proposal 13: In response, the RVQoE-configuring node can change the SRB used to report the RVQoE and update the RVQoE configuration.

# 4. References