3GPP TSG-RAN WG3 Meeting #121                                                      	R3-233929

Toulouse, Aug 21st – Aug 25th, 2023

Agenda item:	11.4

Source:	Qualcomm Incorporated

Title:	Other open topics related to QoE

Document for:	Discussion and Decision

# Introduction

In this paper, we continue the discussion on other open topics related to QoE based on open issues identified last meeting.

# Discussion

  - LS from SA4 on Buffer Level

SA4 sent the following LS R3-233813 to RAN3:

SA4 replied to RAN2 on their LS on buffer level threshold-based RVQoE reporting in S4-230684, confirming RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. However, in SA4#124 meeting, SA4 has further discussed this problem and came to an understanding that buffer level on its own is not an appropriate information for RAN to be able to assist the application due to a variety of reasons, e.g.

- A higher buffer level may help reduce the probability of a playback stall, but may also increase the playback latency, so it’s a trade-off that needs to be balanced by applications. Different proprietary application mechanisms attempting to strike this balance may result in different application behavior for same buffer level or same content.

- Also, different applications (e.g., low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level.

Hence based on this, SA4 would like to update the reply sent in S4-230684 and does not confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. SA4 is considering appropriate alternatives for this purpose. SA4 requests RAN2 and RAN3 to take this information in account, and that SA4 will keep RAN2 and RAN3 updated on progress in this direction.

Although it might be true that buffer level on its own might give the “complete picture”, we think Rel-18 can at least define a framework for threshold-based reporting of RVQoE metrics. If SA4 provides an alternative solution say in Rel-19, threshold-based reporting mechanism can be extended to include this alternative solution. Also, it is not clear why it would matter if different applications have very different behaviour of maintaining the buffer level. For example, if both APP1 and APP2 reports buffer level = X, it should mean the same buffer level irrespective of how its’ maintained.

Proposal 1: Send a reply LS to SA4 on threshold-based reporting of buffer level asking the following questions:

- Q1: Although it might be true that buffer level on its own might give the “complete picture”, is there any issue from SA4 to at least support a framework for application-layer triggering of threshold-based triggers for RVQoE in Rel-18? If SA4 later provides an alternative solution to buffer level, the threshold-based reporting mechanism can be extended to include this alternative solution in Rel-19.

- Q2: It is not clear to RAN3 why it would matter if “different applications (e.g., low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level”. For example, if both APP1 and APP2 reports buffer level = X, it should mean the same buffer level irrespective of how its’ maintained right?

  - Threshold-based triggers for reporting buffer level

Assuming SA4 can support application-layer triggering of threshold-based triggers for RVQoE in Rel-18, we have the following discussion.

RAN3 made the following agreements:

If a UE is configured with periodic RVQoE reporting that automatically starts at the beginning of the application session or immediately upon reception of RVQoE configuration, it cannot be configured with a threshold-based trigger at the same time

The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold.

Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

To provide flexibility to the network, we think the two types of events for threshold based reporting of buffer level (when buffer level is below threshold or when buffer level is above threshold) can be allowed to be configured simultaneously. Depending on the optimization goal, either or both the events can be configured at a given time.

Proposal 2: The two types of events for threshold-based reporting of buffer level (i.e., when the buffer level is below a threshold or above a threshold) can be configured simultaneously.

FFS whether to introduce TTT (time to trigger) for threshold-based triggers.

In addition to the threshold, we think gNB should also be able to configure a “Time-to-Trigger” (TTT) for buffer level reporting. The motivation behind this is same as the TTT defined for RRM measurements i.e., to avoid reporting if the event was met only “momentarily”. UE APP should ensure that the buffer level continuously meets the threshold for TTT duration and report only then.

RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options:

- Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports)

- Option 2: Periodically based on a gNB configured reporting periodicity

- Option 3: A certain number of times based on gNB configured report amount

Although Option 1 is a possible option, it might need gNB to reconfigure the threshold multiple times to get the RVQoE report each time. To provide more flexibility, we think that the gNB can configure a ReportAmount and ReportInterval in addition to the threshold. UE will report RVQoE periodicially as indicated by the ReportInterval for the ReportAmount number of times.

Proposal 3: In addition to the thresholds configured to support threshold-based reporting of buffer level, gNB may configure the following to UE:

- Time-to-Trigger (TTT)

- ReportAmount

- ReportInterval

Proposal 4: UE will start reporting the buffer level only when the buffer level is smaller than a threshold for a certain TTT duration

Proposal 5: Upon meeting the threshold, UE will report RVQoE periodicially as indicated by the ReportInterval for the ReportAmount number of times unless the session ends.

Proposal 6: In the LS to SA4 on threshold-based reporting of buffer level, also include Proposals 2-5 and check whether it is feasible from SA4 perspective.

# Conclusion

LS from SA4 on Buffer Level

Proposal 1: Send a reply LS to SA4 on threshold-based reporting of buffer level asking the following questions:

- Q1: Although it might be true that buffer level on its own might give the “complete picture”, is there any issue from SA4 to at least support a framework for application-layer triggering of threshold-based triggers for RVQoE in Rel-18? If SA4 later provides an alternative solution to buffer level, the threshold-based reporting mechanism can be extended to include this alternative solution in Rel-19.

- Q2: It is not clear to RAN3 why it would matter if “different applications (e.g., low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level”. For example, if both APP1 and APP2 reports buffer level = X, it should mean the same buffer level irrespective of how its’ maintained right?

Threshold-based triggers for reporting buffer level

Proposal 2: The two types of events for threshold-based reporting of buffer level (i.e., when the buffer level is below a threshold or above a threshold) can be configured simultaneously.

Proposal 3: In addition to the thresholds configured to support threshold-based reporting of buffer level, gNB may configure the following to UE:

- Time-to-Trigger (TTT)

- ReportAmount

- ReportInterval

Proposal 4: UE will start reporting the buffer level only when the buffer level is smaller than a threshold for a certain TTT duration

Proposal 5: Upon meeting the threshold, UE will report RVQoE periodicially as indicated by the ReportInterval for the ReportAmount number of times unless the session ends.

Proposal 6: In the LS to SA4 on threshold-based reporting of buffer level, also include Proposals 2-5 and check whether it is feasible from SA4 perspective.

# 4. References