3GPP TSG-RAN WG3 Meeting #121	R3-233934

Aug 21 – 25 2023

Agenda item:	 10.2.2

Source:		Samsung

Title:		(TP for SON BLCR for 38.413) MRO for inter-system handover for voice fallback

Document for:		Discussion and Decision

1	Introduction

This contribution discussed the stage 3 impacts on MRO enhancement for inter-system handover for voice fallback.

2	Discussion

The following scenarios were agreed to be considered on MRO enhancement for inter-system handover for voice fallback at RAN3#117 meeting:

Consider Case 1-2 for MRO enhancements for inter-system inter-RAT handover for voice fallback:

-	Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell.

-	Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

At RAN3#117bis-e meeting, it was agreed to introduce stage 2 description of failure type definition.

Introduce stage 2 descriptions of failure type definition for inter-system inter-RAT HO from NR to E-UTRA for voice fallback. The detailed descriptions are FFS.

At RAN3#119 meeting, the stage 2 TP for capturing the definition was agreed in R3-230993.

At RAN3#119bis-e meeting, the detection mechanism was agreed to stage 2:

Stage-2 description of the detection mechanism will be introduced in TS 38.300 (based on R3-231585).

At RAN3#120 meeting, the following working assumption was agreed:

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.

RAN2#122 agreements:

Agreements:

1	Introduce a new indication in the LTE RLF report for the case an RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback.

2	UE to log the time until reconnection during RRC connection establishment to the acceptable cell and reconnection cell ID in is absent, which will reuse the legacy field.

2.1	Stage 3 Impact

At last RAN3#120 meeting, RAN3 agreed the following working assumption:

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.

For RLF in Case 1, the signalling flow is shown in Figure 1 below.

Figure 1: RLF of Case 1

RLF happens shortly after successful HO from gNB to eNB1. A suitable E-UTRA cell in eNB2 is selected. The UE tries RRC Connection Setup procedure in eNB2.

eNB2 forwards the received RLF Report to eNB1 using RLF Indication message. eNB2 can know the failed PCell ID from RLR report container. So no change is needed for RLF INDICATION message.

eNB1 detects the failure reason after receiving RLF Indication message in step 5. The main question is that whether any change in the message in step 6 and step 8 are needed.

The information included in Inter-system HO Report is copied below.

===

9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |
| >Too early Inter-system HO |  |  |  |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |
| >Inter-system Unnecessary HO |  |  |  |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |
| >>…… |  | 1 |  |  |

===

We can see only two Handover Report Type i.e. Too early and Inter-system Unnecessary HO are included. The too early Inter-system HO is from LTE to NR. These two are not proper to indicate the mobility failure from NR for voice fallback.

There was comment that the source gNB can know inter-system mobility during voice fallback via the Voice Fallback Indicator in the RLF Report from the UE. However, RLF Report is in LTE RRC format. gNB cannot decode it. Therefore a new handover report type is needed otherwise the source gNB cannot differentiate the failure for Voice Fallback from other failures.

Observation 1: “Too early Inter-system HO” and “Inter-system Unnecessary HO” type included in Inter-system HO Report are not approriate to indicate Inter-system Mobility Failure for Voice Fallback.

Observation 2: The gNB cannot know the indicator of Inter-system Mobility Failure for Voice Fallback from LTE RLF Report.

Proposal 1: Turn the Working Assumption to agreement: Define a new handover report type in the Inter-system HO Report over S1 and NG.

In case of Handover Report Type is Mobility Failure from NR for Voice Fallback, the following information are needed

- Source Cell ID

- Failure Cell ID

- Suitable Cell ID

- UE RLF Report Container

Similar as the Handover Report Type of“Too early Inter-system HO” and “Inter-system Unnecessary HO”, Source Cell ID and Failure Cell ID are needed which are madantory. For case 1, a suitable E-UTRA is selected. If the source gNB knows this suitable cell, the source gNB can know that the handover failure could be avoided if the handover target cell is this suitable cell. Suitable Cell ID could be optional.

Source Cell ID, Failure Cell ID, Suitable Cell ID are included in RLF Report. But RLF Report is in LTE RRC format. Maybe gNB cannot decode it. That’s why Source Cell ID, Failure Cell ID have been introduced for “Too early Inter-system HO”. With the same reason, Source Cell ID , Failure Cell ID , Suitable Cell ID for Mobility Failure from NR for Voice Fallback are needed.

Proposal 2: Include Source Cell ID , Failure Cell ID , Suitable Cell ID, UE RLF Report Container in case Handover Report Type is Inter-system Mobility Failure for Voice Fallback.

Based on Proposal 1 and Proposal 2, the corresponding TP for TS38.413 is attached in the Annex as well.

Proposal 3: Agree the TP for TS38.413 in Annex A.

3	Conclusion

In this contribution, the stage 3 impacts on MRO for inter-system inter-RAT handover for voice fallback are discussed. We have the following observations and proposals.

Observation 1: “Too early Inter-system HO” and “Inter-system Unnecessary HO” type included in Inter-system HO Report are not approriate to indicate Inter-system Mobility Failure for Voice Fallback.

Observation 2: The gNB cannot know the indicator of Inter-system Mobility Failure for Voice Fallback from LTE RLF Report.

Proposal 1: Turn the Working Assumption to agreement: Define a new handover report type in the Inter-system HO Report over S1 and NG.

Proposal 2: Include Source Cell ID , Failure Cell ID , Suitable Cell ID, UE RLF Report Container in case Handover Report Type is Inter-system Mobility Failure for Voice Fallback.

Proposal 3: Agree the TP for TS38.413 in Annex A.

Reference

[1] Chairman notes for RAN3#120 meeting

Annex: TP for TS38.413

9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |
| >Too early Inter-system HO |  |  |  |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |
| >Inter-system Unnecessary HO |  |  |  |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |
| >>Candidate Cell List |  | 1 |  |  |
| >>>Candidate Cell Item |  | 1..<maxnoofCandidateCells> |  |  |
| >>>>CHOICE Candidate Cell Type | M |  |  |  |
| >>>>>Candidate CGI |  |  |  |  |
| >>>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |
| >>>>>Candidate PCI |  |  |  |  |
| >>>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |
| >>>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNARFCN) | RF Reference Frequency as defined in TS 38.104 [39], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |
| >Inter-system Mobility Failure for Voice Fallback |  |  |  |  |
| >>Source Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for the HO. |
| >>Failure Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the target cell for the HO. |
| >>Suitable Cell ID | O |  | E-UTRA CGI 9.3.1.9 | CGI of a suitable E-UTRA cell |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. Value is 32 |
| maxNARFCN | Maximum value of NR carrier frequency, defined in TS 38.331 [18] |

**************************** Next Change *************************

9.4.5	Information Element Definitions

****** skip unchanged part******

id-EarlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-TAI,

id-HFCNode-ID-new,

id-GlobalCable-ID-new,

id-IntersystemMobilityFailureforVoiceFallback,

maxnoofAllowedAreas,

****** skip unchanged part******

InterSystemHOReport ::= SEQUENCE {

handoverReportType		InterSystemHandoverReportType,

iE-Extensions			ProtocolExtensionContainer { { InterSystemHOReport-ExtIEs} }			OPTIONAL,

...

}

InterSystemHOReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InterSystemHandoverReportType ::= CHOICE {

tooearlyIntersystemHO				TooearlyIntersystemHO,

intersystemUnnecessaryHO			IntersystemUnnecessaryHO,

choice-Extensions		ProtocolIE-SingleContainer { { InterSystemHandoverReportType-ExtIEs} }

}

InterSystemHandoverReportType-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-IntersystemMobilityFailureforVoiceFallback	CRITICALITY ignore	TYPE IntersystemMobilityFailureforVoiceFallback		PRESENCE mandatory		},

...

}

IntersystemUnnecessaryHO ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

earlyIRATHO				ENUMERATED {true, false, ...},

candidateCellList		CandidateCellList,

iE-Extensions			ProtocolExtensionContainer { { IntersystemUnnecessaryHO-ExtIEs} }			OPTIONAL,

...

}

IntersystemUnnecessaryHO-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemMobilityFailureforVoiceFallback::= SEQUENCE {

sourcecellID			NGRAN-CGI,

failurecellID			EUTRA-CGI,

SuitablecellID			EUTRA-CGI					OPTIONAL,

uERLFReportContainer	UERLFReportContainer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { IntersystemMobilityFailureforVoiceFallback -ExtIEs} }			OPTIONAL,

...

}

IntersystemMobilityFailureforVoiceFallback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- J