3GPP TSG-RAN WG3 #121	R3-233950

Toulouse, France, 21 – 25 Aug 2023

Title: 	Discussion on the support of enhancement for NR XR

Source: 	Samsung

Agenda item:	25.2.

Document Type:	Discussion

1. Introduction

XR Enhancements for NR was approved in RP-230786. The following objectives related to XR enhancement WI have RAN3 impact:

Discard operation of PDU Sets for DL and UL (RAN2, RAN3)

Specify the enhancements for XR Awareness:

- Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

- Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3);

- Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3)

-2. Discussion

2.1 XR Awareness

Issue 1 Signalling for semi-static information, i.e. PDU Set QoS parameters

According to SA2 agreements, A PDU Set is comprised of one or more PDUs carrying an application layer payload. PDU Set QoS Parameters are associated with a QoS flow. The NG-RAN use PDU Set QoS parameters to perform PDU Set based QoS handling. For example, to perform the configuration of scheduling and link layer functions per PDU set.

The NG-RAN performs PDU set based QoS handling according to one or more of the following information received from the SMF.

- PDU Set Delay Budget (PSDB).

- PDU Set Error Rate (PSER)

- PDU Set Integrated Handling Information (PSIHI)

PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the UPF. For DL, it is the duration between the reception time of the first PDU at UPF and the time when all PDUs have been successfully received at the UE. For UL, it is the duration between the reception time of the first PDU at UE and the time when all PDUs have been successfully received at the UPF. Similar as the PDB usage, when the NG-RAN receives the PSDB, the NG-RAN subtracts CN PDB to get the AN PSDB.

PSER defines an upper bound for the rate of PDU Sets that have been processed by the sender of RLC but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP). Thus, the PSER defines an upper bound for a rate of non-congestion related PDU Set losses. In this Release, a PDU Set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

PSIHI indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side.

All the PDU Sets within one QoS flow should apply the same PSER, PSDB and PSIHI. The value of the PSDB, PSER is the same in UL and DL. The granularity of these parameters is QoS flow. Therefore, they should be included in QoS Flow Level QoS parameters IE transmitted in NGAP, E1AP and F1AP.

- Agree to include PDU Set QoS Parameters in QoS Flow Level QoS Parameters in NGAP, XnAP, E1AP and F1AP.

The following is the example of specifying PDU Set QoS parameters in E1AP.

9.3.1.26	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.27 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.29 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.30 | This IE shall be present for GBR QoS Flows and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [20]. This IE applies to Non-GBR flows only and is ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU Session. | - |  |
| Paging Priority Indicator (PPI) | O |  | 9.3.1.55 |  | - |  |
| RDI | O |  | ENUMERATED (enabled, …) | Indicates whether Reflective QoS flow to DRB mapping should be applied. | - |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow. | YES | ignore |
| MCG Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.3.1.30 | This IE contains M-Node offered GBR QoS Flow Information. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1..1800, …) | Indicates the Reporting Frequency for RAN part delay for Qos monitoring. Units: second | YES | ignore |
| QoS Monitoring Disabled | O |  | ENUMERATED (true, ...) | Indicates to stop the QoS monitoring. | YES | ignore |
| Data Forwarding Source IP Address | O |  | Transport Layer Address 9.3.2.4 | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| PDU Set QoS Information | O |  | 9.3.1.x |  | YES | ignore |

9.3.1.x	PDU Set QoS Information

This IE defines PDU Set QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSDB | O |  | 9.3.1.47 FFS |  |  |  |
| PSER | O |  | 9.3.1.48 FFS |  | - | - |
| PSIHI | O |  | ENUMERATED (True, …) |  | - | - |

Issue 2 User plane impact

To support PDU Set based QoS handling, assistance information related to PDU set is introduced in the user plane. The NG-RAN determines which PDUs are belong to the same PDU set based on the assistance information included in the GTP-U header.

- PDU Set Sequence Number.

- Indication of End PDU of the PDU Set.

- PDU Sequence Number within a PDU Set.

- PDU Set Size in bytes.

- PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

- Indication of End of Data Burst.

Indication of End of Data Burst is defined for UE power saving purpose. We list this IE here since it is also included in the GTP-U header. In addition, we think the PDU set size should be optional, because NG-RAN can derive whether all of PDUs in the same PDU set are received based on the PDU set SN, PDU SN within a PDU set, and the indication of End PDU of the PDU set. Therefore, RAN3 should discuss when and how to use the PDU set size, and how to indicate the PDU set size in bytes in GTP-U header.

- RAN3 to discuss when and how to use the PDU set size, and how to indicate the PDU set size in bytes in GTP-U header.

Issue 3 Mobility impacts

Due to the introduction of PDU Set, we need to discuss the impact on the mobility procedure. We identified two aspects related to RAN3.

- Mobility in the non-homogenous deployment

- Mobility with PDU set integrated handling

According to SA2 agreements, when sending at least one PDU Set QoS parameters to the NG-RAN, it means the SMF requests the NG-RAN to active PDU set QoS handling for XR service. If the NG-RAN doesn’t active PDU Set handling, the NG-RAN can reject the PDU Set QoS parameters and notify the rejection to the SMF. Then the SMF can configure the UPF to deactivate the inserting of PDU set assistance information in the GTP-U header.

When the NG-RAN is split NG-RAN, all the split NG-RAN entities participate in the PDU set handling. e.g. the CU-UP performs PDU Set  identify according to the GTP-U header, the DU performs scheduling according to the PDU Set QoS parameter. Each split NG-RAN entity can provide an indication whether an PDU Set QoS parameter is accepted or not, then the CU-CP can draw a conclusion based on the indication from the CU-UP and the DU.

During the mobility, the source node provides QoS information per QoS flow, including the PDU Set QoS parameter. The target node can indicate the acceptance of PDU Set QoS in the response message. i.e. in homogenous deployment, when the receiving node receives the PDU Set QoS parameters, the receiving node can notify the sender with an indication of whether the PDU Set QoS parameter is accepted or not. But in the non-homogenous deployment, the PDU Set QoS parameters are not always included in the Request message. The receiving node is unable to provide the indication based on the PDU Set QoS parameter. More detail analysis is in the below. We split mobility in non-homogenous area into two scenarios:

- Scenario 1: mobility from PDU Set handling non-supporting to supporting

Figure 1: mobility from PDU Set handling non-supporting node to supporting node

The source node does not support PDU Set handling and treats the XR QoS flow without PDU Set concept. There is no PDU Set QoS parameters in the Handover Request message. Even if the target node supports PDU set handling, the target CU-CP is not aware PDU Set QoS parameters. Therefore, the target CU-CP could not report the acceptance of PDU Set QoS parameters in the Handover Request Ack or in the Path Switch Request message.

The problem is as long as the UE moves to an unsupported location once, neither the access network nor the core network can active PDU set handling function. To solve the problem, the target CU-CP needs to indicate PDU Set handling support explicitly in the Path Switch Request message. Even the target CU-CP has not received PDU set QoS flow parameter from the source CU-CP. The target CU-CP generates this indication based on the similar indication sending from the CU-UP and the DU.

When the SMF knows the target CU-CP support PDU Set handling from the indication, the SMF actives the PDU set handling in the UPF in step 4. The SMF also knows the target CU-CP has not receive the PDU Set QoS parameter. The SMF includes the PDU Set QoS parameters in the Path Switch Request Ack message in step 5. The SMF can alternatively trigger a separated procedure to transmit the PDU Set QoS parameters to the target CU-CP, which introducing some delay.

- Agree to include PDU Set Support indication in E1AP-Bearer Context Setup Response message, F1AP-UE Context Setup Response message.

- Agree to include PDU Set Support indication in the Path Switch Request message.

- Agree to include PDU Set QoS parameters in the Path Switch Request Ack message.

- Scenario 2: mobility from XR supporting to XR non-supporting

Figure 2: mobility from PDU Set handling supporting node to non-supporting node

The source CU-CP supports PDU Set handling and includes PDU Set QoS parameters in the Handover Request message. The target cannot support PDU Set handling, e.g. the CU-CP doesn’t recognize PDU Set QoS parameters. In the Handover Request Ack message, the acceptance indication for PDU Set QoS parameters is not included. From the absence of the acceptance indication, the source CU-CP is aware the target doesn’t support PDU Set Handling.

The extension header for the PDU set consumes many GTP-U resources. If there is data forwarding, the data forwarded from source (supporting) to target (non-supporting) need not contain PDU Set assistance information. Therefore, the source CU-CP can notify “the target not support PDU Set handling” when the source CU-CP configures data forwarding address to the source CU-UP. When receives this notification, the source CU-UP forwards data without PDU set assistance information.

- Agree to include the indication of the target not support PDU Set handling in E1AP-Bearer Context Modification Request message. With this indication, the gNB-CU-UP needn’t include PDU Set related information in the GTP-U header.

The second impact introduced by the PDU Set QoS parameter is mobility with PSIHI. PSIHI indicates whether all PDUs of the PDU Set are needed for the application layer. PSIHI is sent to the NG-RAN so that the NG-RAN can discard the PDU set as a whole in case some PDU is missing or some PDU can not be received by the receiver. Thus the radio resource can be saved and the efficiency of XR data transmission is improved.

Considering the high latency requirement for the XR service, it is better not to introduce any interruption for the data transmission during the mobility. If possible, the handover can be triggered after all the PDUs in one data burst are transmitted to the UE in the source node. But currently it is impossible to achieve zero interruption. In case the whole PDUs within one PDU set can’t be transmitted in the source node when the handover triggers, the remaining PDUs in the same PDU set will be transmitted from the UPF to the target node. If the HSIHI is set, when one PDU has been lost in the source node, the remaining PDUs of that PDU set needn’t be transmitted and can be discard by the target node.

Therefore, during the mobility, the source node can indicate whether there is PDU missing for a PDU set to the target node. There are two messages can be used, one is the Handover Request message, another is the SN Status Transfer message. Considering the NG-RAN node can be split node, when the source node transmits the Handover Request message, the source CU-CP hasn’t got any packet transmission status from the source CU-UP, thus the SN Status Transfer message is the proper message to convey this information. The source CU-UP reports the discard PSSN in the Bearer Modification Response message. The source CU-CP includes the discard PSSN in the SN Status Transfer message sending to the target CU-CP. The target CU-CP includes the discard PSSN in the Bearer Modification Request message sending to the target CU-UP.

Figure 3: PSHIH impacts to mobility procedure

- Agree to include discard PSSN in E1AP-Bearer Context Modification Request message, E1AP-Bearer Context Modification Response message. The gNB-CU-UP discards the whole PDU set indicated by this PSSN.

- Agree to include discard PSSN in Xn-SN Status Transfer message.

- Agree to include discard PSSN in NG-UL/DL RAN SN Status Transfer message.

2.2: UE Power saving

Current TSC Assistance Information (TSCAI) contains Periodicity, Burst Arrival Time, Survival Time. In addition, N6 jitter range with the DL Periodicity should be included in the TSCAI.

In NG interface, the TSCAI is included in the TSC Traffic Characteristics IE. In the semantics description, this IE is present in case of GBR QoS flows and is ignored otherwise. According to SA2 agreement, TSCAI is applied to both GBR and non-GBR QoS flow, as discussed in the last meeting, there are two options to modify our specification:

- Option 1: To define a new TSCAI IE

- Option 2: Remove the semantics description in the TSC Traffic Characteristics IE

We slightly prefer the option 1.

In E1AP, XnAP and F1 AP, there is no semantics description for TSC Traffic Characteristics IE, therefore, the existing IE can be used.

- Agree to add N6 jitter range IE in TSCAI IE in NGAP, E1AP, XnAP and F1AP.

- Agree to define a new TSC Traffic Characteristics IE in NGAP.

2.3 ECN marking

ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. According to TS23.501, ECN marking in the IP header is supported in either the NG-RAN or in the UPF.

- ECN marking in the NG-RAN. The SMF sends an indication for a QoS flow to active the ECN marking in the NG-RAN. If this indication is absent, the NG-RAN will not active the ECN marking.

- ECN marking in the UPF. SMF indicates to NG-RAN to report the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header. If there is no UL packet when report for DL and/or UL needs to be provided, NG-RAN may generate an UL Dummy GTP-U Packet for such a reporting.

In addition, NG-RAN should also report the congestion level per QoS flow (e.g., low/medium/high) to CN, and CN can make more suitable policy (e.g., PCC rule) to improve network performance. The following is the example of specifying congestion level in GTP-U header, Bit 1 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) can be used to indicate if the congestion level indication is present (1) or not (0).

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 |  | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  | QMP | DL Delay Ind. |  | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag |  | QoS Flow Identifier |  |  |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) |  | New IE Flag 6 |  | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 |  | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Congestion Level |  |  |  |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

UL PDU SESSION INFORMATION (PDU Type 1) Format

3. Conclusion

Based on the discussion in this paper, we propose the following:

- Agree to include PDU Set QoS Parameters in QoS Flow Level QoS Parameters in NGAP, XnAP, E1AP and F1AP.

- RAN3 to discuss when and how to use the PDU set size, and how to indicate the PDU set size in bytes in GTP-U header.

- Agree to include PDU Set Support indication in E1AP-Bearer Context Setup Response message, F1AP-UE Context Setup Response message.

- Agree to include PDU Set Support indication in the Path Switch Request message.

- Agree to include PDU Set QoS parameters in the Path Switch Request Ack message.

- Agree to include the indication of the target not support PDU Set handling in E1AP-Bearer Context Modification Request message. With this indication, the gNB-CU-UP needn’t include PDU Set related information in the GTP-U header.

- Agree to include discard PSSN in E1AP-Bearer Context Modification Request message, E1AP-Bearer Context Modification Response message. The gNB-CU-UP discards the whole PDU set indicated by this PSSN.

- Agree to include discard PSSN in Xn-SN Status Transfer message.

- Agree to include discard PSSN in NG-UL/DL RAN SN Status Transfer message.

- Agree to add N6 jitter range IE in TSCAI IE in NGAP, E1AP, XnAP and F1AP.

- Agree to define a new TSC Traffic Characteristics IE in NGAP.

Below two TPs capture the proposals in above.

- The TP for TS 37.483 is in R3-233951, captures all the proposals in above related to E1.

- The TP for TS38.413 TS38.423is in R3-233952, captures the proposals in above related to mobility in NG and Xn.

- Agree the TP in R3-233951 and R3-233952.

4. Reference

[1] TS 23.501 System architecture for the 5G System (5GS)

[2] TR 38.835 Study on XR enhancements for NR

[3] RP-230786