3GPP TSG-RAN WG3 Meeting #121	R3-233962

Toulouse, France, 21 – 25 Aug, 2023

Title: 	(TP to eNS_Ph3 BLCR for TS 38.300) Enhanced network slicing phase 3

Source: 	Huawei

Agenda item:	26.3

Document Type:	Other

# 1. Introduction

The last RAN3#120 meeting concludes the following open issues [1] for further discussion.

For RAN impact on supporting Network Slice Service continuity scenario:

Potential signaling impact on Alternative S-NSSAI in PDU session Modify for backwards compatibility aspects (receiver so far is assumed to not accept a slice modification of ongoing slice)

Opt1: Reusing current S-NSSAI IE

Opt2: Adding a new S-NSSAI IE in the PDU Session Setup/Modification message and Initial UE context setup request message

Potential signaling impact on Alternative S-NSSAI I in PDU session Setup for RAN usage of resources or NS-AoS check

Opt1: Reusing current S-NSSAI IE

Opt2: Adding a new S-NSSAI IE in the PDU Session Setup/Modification message and Initial UE context setup request message

Mobility scenario

Adding Alternative S-NSSAI in HO request message for slice restoration and more generally how to handle restoration e.g. original slice becomes available again after the congestion.

For Support of partially Allowed NSSAI:

UE moves from non-supporting TA to supporting TA, PDU session modify and Path switch

This contribution mainly focuses on the remaining open issues of the Alternative S-NSSAI and stage 2 TP.

# 2. Discussion

## 2.1 Open issues of the Alternative S-NSSAI

- NGAP signalling impacts

As discussed at previous meeting, there are two candidate options for both the PDU session resource setup and modify procedure.

- Option 1: Introduce the explicit Alternative S-NSSAI

- Option 2: Reusing the legacy S-NSSAI per PDU session

Whether to allow the NG-RAN aware of the mapping of native S-NSSAI and the Alternative S-NSSAI was discussed. As a result, a LS [2] was agreed to list three approaches on the RAN resources of the Alternative S-NSSAI. Basically all three approaches have no signalling impact on RAN side, though the final decision can be determined by SA2.

Hence, for the PDU session resource setup procedure, there is no need to introduce the explicit Alternative S-NSSAI, since the mandatory S-NSSAI is sufficient. That is, if the UE is requesting a new PDU Session establishment with both the S-NSSAI and its Alternative S-NSSAI, then the AMF could indicate the Alternative S-NSSAI reusing the legacy S-NSSAI IE in the PDU Session Resource Setup Request message.

- For PDU session resource setup procedure, the existing S-NSSAI per PDU session can be used to indicate the Alternative S-NSSAI.

For the PDU session resource modify procedure, the corresponding detailed stage 3 call flows are excerpted as follows, based on the latest TS 23.502 [3].

4.3.3.2 UE or network requested PDU Session Modification (non-roaming and roaming with local breakout) <Skip the irrelevance> 3a. For UE or AN initiated modification or AMF initiated modification, the SMF responds to the AMF through Nsmf_PDUSession_UpdateSMContext Response ([N2 SM information (PDU Session ID, QFI(s), QoS Profile(s), [Alternative QoS Profile(s)], Session-AMBR], [CN Tunnel Info(s)]), N1 SM container (PDU Session Modification Command (PDU Session ID, QoS rule(s), QoS rule operation, QoS Flow level QoS parameters if needed for the QoS Flow(s) associated with the QoS rule(s), Session-AMBR, [Always-on PDU Session Granted], [Port Management Information Container], [Non-3GPP QoS Assistance Information Container]))). See clause 5.7 of TS 23.501 [2] for the QoS Profile, Alternative QoS Profile and QoS rule and QoS Flow level QoS parameters. Alternative QoS Profile is only valid for AN initiated modification. … The N2 SM information carries information that the AMF shall provide to the (R)AN. It may include the QoS profiles and the corresponding QFIs to notify the (R)AN that one or more QoS flows were added, or modified. It may include only QFI(s) to notify the (R)AN that one or more QoS flows were removed. The SMF may indicate for each QoS Flow whether redundant transmission shall be performed by a corresponding redundant transmission indicator. If the SMF decides to activate redundant transmission in step 2a, the SMF includes the allocated additional CN Tunnel Info in the N2 SM information. If the SMF decides to perform redundant transmission for new QoS Flow with two I-UPFs in step 2a, the SMF includes the allocated CN Tunnel Info of the two I-UPFs in the N2 SM information. If the PDU Session Modification was triggered by the (R)AN Release in step 1e the N2 SM information carries an acknowledgement of the (R)AN Release. If the PDU Session Modification was requested by the UE for a PDU Session that has no established User Plane resources, the N2 SM information provided to the (R)AN includes information for establishment of User Plane resources. For Network Slice Replacement if the SMF determines that the PDU Session is to be retained, the S-NSSAI in N2 SM information is set to Alternative S-NSSAI.

It is specified that the SMF shall set the S-NSSAI in N2 SM information to the Alternative S-NSSAI. Note that in the TS 38.413, the S-NSSAI IE is already included in the PDU Session Resource Modify Request message, with the following procedure texts.

- For each PDU session, if the S-NSSAI IE is included in the PDU Session Resource Modify Request Item IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall replace the previously provided S-NSSAI by the received S-NSSAI for the concerned PDU session and use it as specified in TS 23.502 [10].

Hence, it is not necessary to introduce the Alternative S-S-NSSAI in the NGAP signalling additionally because using existing PDU Session Management messages can already work. If the AMF determines to send updates to the SMF of an established PDU Session, then the value of the Alternative S-NSSAI can be indicated in the PDU Session Resource Modify Request message.

During the last RAN3 meeting, some companies argued that the PDU session resource modify procedure was introduced to support EPS to 5GS handover scenarios, not for the S-NSSAI value change in NR. Given that the TS 23.502 [3] is referred, there seems no need to update the procedure texts either.

- For PDU session resource modification procedure, the existing S-NSSAI per PDU session can be used to indicate the Alternative S-NSSAI.

- Network Slice Replacement during handover

In the recent version of TS 23.501 [4], it specifies the details of Network Slice Replacement during the handover procedure as below.

During a handover procedure, if an S-NSSAI has to be replaced with an Alternative S-NSSAI, the handover procedure (including any PDU session associated with the S-NSSAI to be replaced) shall continue unaffected by the Network Slice Replacement. Any Network Slice Replacement for the S-NSSAI shall not take place during the handover.

As quoted above, the Network Slice Replacement could happen after the handover is completed since these two procedures are independent from each other. After the handover procedure, the SMF can decide whether to modify the PDU session by replacing with the Alternative NSSAI, or release the PDU session. Hence, there is no need to introduce the Alternative S-NSSAI in the XnAP signalling.

- No need to introduce the Alternative S-NSSAI in the XnAP signalling since the Network Slice Replacement shall not take place during the handover procedure.

## 2.2 TP for TS 38.300

In the TS 38.300, the following aspects can be added for the new features introduced by SA2.

- Partially Allowed NSSAI

In section 16.3.1, the slice availability is described where the slice may be available only in part of the network. The Partially Allowed NSSAI indicates the slices where the UE can use in certain TAs in the current registration area. And the BLCR already provides the signalling details that the AMF can provide the Partially Allowed NSSAI to the NG-RAN for handover/redirection purpose. Then these aspects can be described in this section.

In addition, in Section 16.3.4.3, it is specified that the Allowed NSSAI can be signalled by the AMF in the Initial Context Setup Request message. The new introduced Partially Allowed NSSAI together with the Target NSSAI can be specified as well.

- Support of Network Slice Replacement

As discussed in [2], there are several approaches can be used to prepare NG-RAN resources to serve the Alternative S-NSSAI, when the native S-NSSAI becomes unavailable or congested in the CN. From RAN perspective, it should be the RRM policy defined in TS 28.541, to avoid any RAN resources congestion in such case. At least, some stage2 descriptions are needed in section 16.3.3.2 Handling of Slice Resources.

- Agree the stage 2 TP in the Annex.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: For PDU session resource setup procedure, the existing S-NSSAI per PDU session can be used to indicate the Alternative S-NSSAI.

Proposal 2: For PDU session resource modification procedure, the existing S-NSSAI per PDU session can be used to indicate the Alternative S-NSSAI.

Proposal 3: No need to introduce the Alternative S-NSSAI in the XnAP signalling since the Network Slice Replacement shall not take place during the handover procedure.

Proposal 4: Agree the stage 2 TP in the Annex.

The corresponding TP for TS 38.300 is provided in the Annex.

# 4. Reference

- Chairman notes, TSG-RAN WG3 Meeting #120 meeting.

- R3-233432, Response to Reply LS on Support of network slices which have area of service not matching deployed tracking areas, RAN3.

- 3GPP TS 23.502, Procedures for the 5G System (5GS); stage 2 (Release 18).

- 3GPP TS 23.501, System architecture for the 5G System (5GS); stage 2 (Release 18).

# 5. Annex – TP for TS 38.300

Change Begins

### 16.3.1	General Principles and Requirements

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Access control

-	By means of the unified access control (see clause 7.4), operator-defined access categories can be used to enable differentiated handling for different slices. NG-RAN may broadcast barring control information (i.e. a list of barring parameters associated with operator-defined access categories) to minimize the impact of congested slices.

Slice Availability

-	Some slices may be available only in part of the network. The NG-RAN supported S-NSSAI(s), NSAG(s) and NSAG related information such as NSAG associated Cell Reselection Priority and/or NSAG associated RACH resources are configured by OAM. Awareness in the NG-RAN of the slices supported in the cells of its neighbours may be beneficial for inter-frequency mobility in connected mode. In order to support the NSAG, the NG-RAN provides the AMF with the NSAG information per TA in the appropriate NG interface management procedures, as specified in TS 38.413 [26]. Awareness in the NG-RAN of the NSAG information supported in the list(s) of neighbour cells may be configured by OAM, or exchanged with neighbour NG-RAN nodes.It is assumed that the slice availability does not change within the UE's registration area (see TS 23.501 [3]).

-	The NG-RAN and the 5GC are responsible to handle a service request for a slice that may or may not be available in a given area. Admission or rejection of access to a slice may depend by factors such as support for the slice, availability of resources, support of the requested service by NG-RAN.

-	The NG-RAN may be signalled with the Partially Allowed NSSAI from the AMF when one or more slice(s) of such S-NSSAI is not available in the UE’s current TA so that the CN registration area includes TA where the requested S-NSSAI is not supported as specified in TS 23.501 [3]. The NG-RAN can decide to use the Partially Allowed NSSAI for RRM policies e.g., to redirect or handover the UE to TAs where the Partially Allowed NSSAI is supported.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 16.3.3.2	Handling of Slice Resources

The NG-RAN node may use Multi-Carrier Resource Sharing or Resource Repartitioning to allocate resources to a slice during the procedures described in 16.3.4 to support the slice service continuity in case of slice resources shortage.

In Multi-Carrier Resource Sharing the RAN node can setup the dual connectivity or carrier aggregation with different frequency and overlapping coverage where the same slice is available.

The Resource Repartitioning allows a slice to use resources from the shared pool or/and prioritized pool when its own dedicated or prioritized resources are not available and the use of unused resources in the prioritized pool is as specified in TS 28.541 [49].

Slice RRM policies/restrictions associated with Resource Repartitioning are configured from O&M.

Measurements of RRM policy utilization according to resource types defined in TS 28.541 [49] are reported from RAN nodes to O&M and may lead O&M to update the configuration of the Slice RRM policies/restrictions.

The NG-RAN node may be configured with RRM policy defined in TS 28.541 [49] to support the Network Slice Replacement defined in TS 23.501 [3].

Editor’s Note:	 The above may be reformulated based on SA2 reply.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 16.3.4.3	UE Context Handling

Following the initial access, the establishment of the RRC connection and the selection of the correct AMF, the AMF establishes the complete UE context by sending the Initial Context Setup Request message to the NG-RAN over NG-C. The message contains the Allowed NSSAI/Target NSSAI/Partially Allowed NSSAI and additionally contains the S-NSSAI(s) as part of the PDU session(s) resource description when present in the message. Upon successful establishment of the UE context and allocation of PDU session resources to the relevant network slice(s) when present, the NG-RAN responds with the Initial Context Setup Response message.

Figure 16.3.4.3-1: Network Slice-aware Initial Context Setup

Change Ends