3GPP TSG-RAN WG3 Meeting #121	R3-234038

Toulouse, France, August 21st – 25th 2023

Agenda Item:	13.5

Source:	Ericsson

Title:	(TP for mIAB BL CR for TS 38.470) Discussion of SA2 FS_VMR Solutions

Document for:	Agreement

# Introduction

SA2 sent an LS to RAN3 in S2-2207070 during the RAN3#117-bis-e seeking clarification of several issues related to the SA2 work on VMR. After RAN3 replied in R3-226048, SA2 has sent a further LS in R3-230032 asking for additional clarifications for some of the issues. Finally, RAN3 sent replies with partial answers in R3-231011 and R3-233469.

In this paper we discuss the remaining issues stemming from the LS. A TP for mIAB BL CR for TS 38.470 is provided in the Annex.

# Discussion

We discuss the following:

- Indicating the mIAB-MT’s serving cell ID to mIAB-DU’s donor

- Stage-2 impact.

## Indicating the mIAB-MT’s serving cell ID to mIAB-DU’s donor

One open issue captured in the RAN3#119-bis-e meeting notes is how the mIAB-DU’s CU acquires the mIAB-MT’s serving cell ID to be included in the newly defined IAB-MT User Location Information IE:

RAN3 to select between the following options for IAB-DU’s donor CU to obtain the IAB-MT’s serving cell ID in case IAB-MT and IAB-DU are connected to different IAB-donors

- Option A, the cell ID of IAB-MT’s serving cell is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU.

- Option B, the cell ID of IAB-MT’s serving cell is passed from IAB-DU to IAB-DU’s donor CU.

With respect to the above, we prefer Option B, because Option B is independent of the existence of an Xn connection between the mIAB-MT’s CU and the mIAB-DU’s CU.

Proposal 1: The mIAB-DU’s donor CU obtains the mIAB-MT’s serving cell ID from the mIAB-DU.

## Stage-2 impact

RAN3 has previously agreed that, when available, the mobile IAB-MT’s UE ID (i.e., the GPSI) can be included in the F1AP TRP INFORMATION RESPONSE message, so that the donor can CU transfer this information to LMF. However, it is not described how the donor CU should interpret this information. Therefore, it needs to be clarified in stage 2 that the positioning function allows the gNB-DU to provide the Mobile IAB-MT UE ID (i.e., the GPSI) and location information to the donor CU of a mobile TRP. A TP for mIAB BL CR for TS 38.470 is provided in the Annex.

Proposal 2: Agree the TP for mIAB BL CR for TS 38.470 is provided in the Annex.

# Conclusion

This paper presents our view on SA2 questions about VMR. The following is proposed:

Proposal 1: The mIAB-DU’s donor CU obtains the mIAB-MT’s serving cell ID from the mIAB-DU.

Proposal 2: Agree the TP for mIAB BL CR for TS 38.470 is provided in the Annex.

# Annex: TP for mIAB BL CR for TS 38.470

-------------------------------------------Start of changes-------------------------------------------

### 5.2.11	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU. With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements. With this function, the gNB-DU can provide the Mobile IAB-MT UE ID (i.e., GPSI) and location information to the gNB-CU for a mobile TRP.

The function allows the gNB-CU to

-	transfer the positioning assistance data to gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

-	request the gNB-DU to configure SRS transmissions for UE.

-	request the gNB-DU to configure PRS transmissions.

-	request the gNB-DU to configure measurement gap or PRS processing window.

-	request the gNB-DU to activate or deactivate preconfigured measurement gap or PRS processing window.

-	request the gNB-DU to broadcast positioning system information.

-------------------------------------------End of changes-------------------------------------------