3GPP TSG-RAN WG3 Meeting #121	R3-234068

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	10.2.3

Source:	Huawei

Title:	(TPs for SON BLCR for TS 36.300, TS38.300 and TS 38.473): Remaining issues for RACH optimisation

Document for:	other

- Introduction

In the RAN3#120 meeting, the following agreements and open issues were made for RACH enhancements:

Not consider the NG and S1 forwarding of RA report.

Include the “PSCell list” optionally in addition to the “RACH Report Container” in ACCESS AND MOBILITY INDICATION over Xn.

RACH report information enhancements:

Option 1: Include the feature priority

Option 2: Include the RACH partition configuration related information (e.g., start preamble / number of preambles in the RA partition)

Option 3: Include the time between RACH access that led to the generation of a RA Report and reporting of the RA Report

Option 4: The network controls the UE to report RA information

Option5: Mobility information

FFS whether and how to forward the RA Report in case the MN that retrieves the RA Report is not connected to any SN associated to the PSCells received with the RA Report.

LS to RAN2 on RA Report?

This paper addresses the remaining issues and presents our further analysis and corresponding proposals.

2	Discussion

2.1	RA report information enhancement

Five options were discussed for RA report enhancement for the RA configuration retrieval:

RACH report information enhancements:

Option 1: Include the feature priority

Option 2: Include the RACH partition configuration related information (e.g., start preamble / number of preambles in the RA partition)

Option 3: Include the time between RACH access that led to the generation of a RA Report and reporting of the RA Report

Option 4: The network controls the UE to report RA information

Option5: Mobility information

Optino 1 and option 2 should be reported by the UE together to make the function workable. Reporting the RA pramameters used by th UE when RA report is collectred will not mandate the RAN node to store the RA configurations for at most 48 hours after a RA procedure is initiated. Amd it is the most efficient and straightforward solution.

Option 4 proposes that the network indicates the UE to report the RA report only related to the latest RA configuration. However, this option may have significant impact on the UE than option 1 and option 2 have. Since the UE does not know the certain RA configuration ( and the latest RA configuration) and also does not remember which RA configuration relates to which RA report.

Option 3 and option 5 may have similar impact on the network to store and retireve the RA configuration used when the RA report is collected. The disadvantage of option 5 is that the RAN node is mandated to store the RA configurations for at most 48 hours. Considering that the gNB-DU usually is deployed distributedly and not on a cload platform, Storing the RA configurations for at most 48 hours for all served cells may cause unacceptable storage consumption in the gNB-DU. And as the RA report records every successful RA attempt of a UE, the enquiring processing load in the gNB-DU may also be considerable.

The network impact from option 5 is still not clear and needs further study.

With option 3, the network only to store the latest RA configuration. The network may determine whether the received RA report is still usefule or already out of date by the timer indicated in the RA report.

Proposal 1: RAN3 to agree on option 1 and option 2. And if not, adopt option 3 as a compromise solution.

2.2	SN RA reporting forwarding in inter-MN handover case

RAN2 agreed that the UE reports a list of unique PSCell IDs outside the NR RA report container to help the forwarding of SN NR RA reports between RAN nodes. And requiring the UE further report the PCell ID associated to each PScell in the RA report was not agreed by RAN3 at last meeting. As a compromise solution, The PScell ID list are also transferred to the SN together with the SN RA report over Xn. This solution assume that the RAN node receving the SN RA report may be also have direct Xn interface to other SN ndoes indicated by the PScell IDs. Then the following issue was raised and marked as FFS:

FFS whether and how to forward the RA Report in case the MN that retrieves the RA Report is not connected to any SN associated to the PSCells received with the RA Report.

One possible way is to send the SN RA report to the source MN indicated in the UHI. Because the use case of the FFS implies that the SN RA report are collected by the UE when serving by the source MN before a inter-MN without SCG handover.

- Proposal 2: The MN that retrieves the SN RA Report may forward it to the source MN indicated in the UHI.

Propoal 1 and proposal 2 are related to RAN2. A LS is needed to inform them about RAN3 progress.

Proposal 3: To send a LS to RAN2 to inform them about RAN3 progress in proposal 1 and proposal 2.

Accordling to the current specification, For E-UTRAN nodes, the Access and Mobility Indication is only sent from eNB to en-gNB. However, it could be necessary to support SN RA report forwarding between eNBs. As The possiblility of the MN that retrieves the RA Report is not connected to any SN associated to the PSCells is high. And there is no EN-DC X2 interface between en-gNBs.

Proposal 4: To enable SN RA report forwarding between eNBs by the Access and Mobility Indication procedure on X2 interface.

The corresponding TPs for above proposals are in the annex. It is proposed to proceed the TPs. Besides there is a TP to stage 2 BLCR for TS 38.320 should be treated as well which was noted due to some online comments.

Proposal 5: To proceed the satge 2 TPs for TS 36.300, TS 38.300 and TS 38.420 BL CRs and the stage 3 TP for TS 36.423 BL CR in the nnex and [1].

3	Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: RAN3 to agree on option 1 and option 2. And if not, adopt option 3 as a compromise solution.

Proposal 2: The MN that retrieves the SN RA Report may forward it to the source MN indicated in the UHI.

Proposal 3: To send a LS to RAN2 to inform them about RAN3 progress in proposal 1 and proposal 2.

Proposal 4: To enable SN RA report forwarding between eNBs by the Access and Mobility Indication procedure on X2 interface.

Proposal 5: To proceed the satge 2 TPs for TS 36.300, TS 38.300 and TS 38.420 BL CRs and the stage 3 TP for TS 36.423 BL CR in the nnex and [1].

The draft LS and related TPs are provided in the annex and [1].

4.	Reference

[1]	R3-234069	(TP for SON BLCR for TS 38.420): Introduction of RACH Indication

Annex 1 draft LS to RAN2

3GPP TSG-RAN WG3 Meeting #121

Toulouse, France, 21 – 25 Aug, 2023

Title:	Reply LS on RACH enhancement

Response to:	R2-2306848

Release:	Rel-18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact person:	Hongzhuo Zhang

Zhanghongzhuo(at)Huawei(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 would like to thank RAN2 for the reply LS in R2-2306848.

RAN3 discussed how to forward the SN RA report to the correct SN indicated by the PScell IDs in the RA Report. RAN3 thinks that without the PCell ID associated to each PScell where the SN RA report is collected by the UE, the RAN node may not be able to forward the SN RA report to the correct SN in case that there is no X2/Xn interface between the RAN node received the report and the RAN node indicated by the PSCell ID. Because inter MN handover or MN to eNB/gNB handover may happen before the SN RA report is retrieved.

RAN3 agrees that in case the MN that retrieves the RA Report is not connected to any SN associated to the PSCells received with the RA Report it may forward the RA Report to the source MN indicated in the UHI.

RAN3 also discussed the potential RACH Report enhancements and would like RAN2 to consider including the following information:

- the feature priority

- the RACH partition configuration related information (e.g., start preamble / number of preambles in the RA partition)

# 2 	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to take the above into consideration and provide feedback if any.

3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#121-bis 	 2023-10-09 - 2023-10-13		Xiamen, CN

RAN3#122  	2023-11-13 -  2023-11-17 		Chicago, US

Annex 2-TP to TS 36.300

/*************Start of change*******************/

22.4.3	Support for RACH Optimisation

22.4.3.1	General

The aim of this function is to support RACH Optimisation. RACH optimisation is supported by UE reported information and by RACH parameters exchange between:

-	E-UTRA cells;

-	NR cells, in case of EN-DC.

22.4.3.2	Solution description

22.4.3.2.1	E-UTRA cell case

The setting of RACH parameters that can be optimized are:

-	RACH configuration (resource unit allocation);

-	RACH preamble split (among dedicated, group A, group B, RSRP level, NRSRP level (for NB-IoT), NPRACH resource pools (for NB-IoT), EDT);

-	RACH backoff parameter value;

-	RACH transmission power control parameters.

RACH optimisation is supported by UE reported information and by PRACH parameters exchange or NPRACH parameters (for NB-IoT) between eNBs.

UEs which receive polling signalling shall report the below information:

-	Number of RACH preambles sent until the successful RACH completion;

-	Contention resolution failure;

-	For BL UE or UE in enhanced coverage or NB-IoT UE, the RSRP (NRSRP for NB-IoT) level in which the UE started the random access procedure;

-	For BL UE or UE in enhanced coverage or NB-IoT UE, an EDT fallback indication.

UE reporting of RACH information is not supported for a NB-IoT UE using the Control Plane CIoT EPS Optimisation,

22.4.3.2.2	NR cell in EN-DC case

The solution applies to an en-gNB supporting EN-DC operation. RACH optimisation is supported by UE reported information (RA report, see TS 38.300 [79]) made available at the eNB and further forwarded to the en-gNB, and by PRACH parameters exchanged (see TS 38.300 [79]) between en-gNBs and eNBs.

In EN-DC, when the UE performs a successful random-access procedure in the secondary en-gNB, the secondary en-gNB may inform the potential availability of RA Report in the UE to the master eNB via a RACH indication. The eNB may then retrieve the RA Report from the UE based on the RACH indication received via X2AP signalling from the secondary en-gNB.

When an eNB retrieves RA reports for SN, it may forward the RA reports to the secondary en-gNBs indicated by the PSCell IDs associated with the RA report. The eNB may also forward the RA reports to the source master eNB as indicated in the UHI if available.

/*************End of change*******************/

Annex 3-TP to TS 38.300

/*************Start of change*******************/

15.5.3	Support for RACH Optimization

RACH optimization is supported by UE reported information made available at the NG RAN node as specified in TS 38.331 [12] and by PRACH parameters exchange between NG RAN nodes.

The contents of the RA Report comprise of the following:

-	Contention detection indication per RACH attempt;

-	Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts;

-	Indication whether the selected SSB is above or below the configured RSRP threshold per RACH attempt;

-	2-step RACH information as specified in clause 5.7.10.4 of TS 38.331 [12].

SN RA Reports

The UE operating in NR-DC, may also support collection of RA Reports in the PSCells of the SN.

RA Report retrieval in case of dual connectivity:

In MR-DC, when UE performs a successful random-access procedure in the SN, the SN may inform the potential availability of RA Report in the UE to the MN via a RACH indication. The MN may then retrieve the RA Report from the UE based on the RACH indication received via XnAP signalling from the SN.

When a MN retrieves RA reports for SN from a UE, it may forward the RA reports to the corresponding SN indicated by the PSCell IDs associated with the RA report.

/*************End of change*******************/

Annex 4-TP to TS 36.423

Start of the change

### 8.3.16	Access and Mobility Indication

#### 8.3.16.1	General

The purpose of the Access and Mobility Indication procedure is to transfer Access and Mobility related information between E-UTRAN nodes.

#### 8.3.16.2	Successful Operation

Figure 8.3.16.2-1: Access and Mobility Indication. Successful operation – eNB-initiated

Figure 8.3.16.2-2: Access and Mobility Indication. Successful operation – eNB-initiated

The eNB initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message sent to the en-gNB.

The eNB1 initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message sent to the eNB2.

If the PSCell List Container IE is included in the ACCESS AND MOBILITY INDICATION message, the eNB2 may use it to identify the en-gNB to which the RA Report Container IE should be forwarded.

#### 8.3.16.3	Abnormal Conditions

Not applicable.

Start of the next change

9.1.2.50	ACCESS AND MOBILITY INDICATION

This message is sent by the eNB to the eNB/en-gNB to transfer access and mobility related information.

Direction: eNB  en-gNB, eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR RA Report List |  | 0..1 |  |  | YES | ignore |
| >NR RA Report List Item |  | 1 .. <maxnoofRAReports> |  |  | EACH | ignore |
| >>NR RA Report Container | M |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| >>UE Assistant Identifier | O |  | en-gNB UE X2AP ID 9.2.100 |  | – |  |
| >>PSCell List Container | O |  | OCTET STRING | [FFS – Name to be decided by RAN2] IE as defined in subclause [FFS – to be decided by RAN2] in TS 36.331 [9]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |

End of the change