3GPP TSG-RAN WG3 Meeting #121	R3-234073

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	10.2.5

Source:	Huawei

Title:	(TP for MDT BL CR for TS 38.300): SON for NR-U

Document for:	Other

1	Introduction

In the last RAN3 #120 meeting, the following agreements have been achieved in SON for NR-U:

There is no need for the UE to report the average of the applied EDT UL in RLF report. The EDT in UL included in the XnAP RESOURCE STATUS UPDATE message reflects the average of the configured maximum EDT UL. TP for XnAP is in R3-233491.

Meanwhile, there is still an FFS on whether to introduce UL EDT in RESOURCE STATUS UPDATE message over F1 interface. Besides, it’s still FFS how to handle the issue of handover execution failures with DL LBT issues.

In this paper, we provide our views about above issues and related TPs.

2	Discussion

The first issue is whether the gNB-DU should send UL EDT to CU in RESOURCE STATUS UPDATE message. A reference is in the clause 9.2.2 of TS 38.473, in the messages of UE CONTEXT SETUP RESPONSE, UE CONTEXT MODIFICATION RESPONSE and UE CONTEXT MODIFICATION REQUIRED, the gNB-DU sends the RRC information to CU, which includes the CellGroupConfig. Then as defined in TS 38.331, the CellGroupConfig includes the energyDetectionConfig. Therefore, we think that the gNB-CU is already informed with the UL EDT by the gNB-DU and it is not essential to introduce redundant signalling exchange over F1 interface.

The gNB-CU is already informed with the UL EDT by the gNB-DU and there is no need to transfer the UL EDT in MLB related messages.

Another remaining issue is how to handle the HO execution failure with DL LBT issues. The root cause is that the target node suffers from LBT failure when it attempts to send RACH related signalings. Currently, there are two candidiate solutions, the network based solution and the UE based solution respectively.

For the UE based solution, the UE needs to detect and record whether there is SSB absence during HO in NR-U channels. It is up to RAN2 to discuss and decide whether it is feasible or not. RAN3 could send a LS to RAN2 for consultation.

For the network based solution, the target node is required to record and inform the source node whether there is LBT failures during HO execution. We think that the network solution could be considered only when RAN2 cannot support the UE based solution considering the introduced signalling overhead and complexity to the network.

To consult RAN2 on the feasibility of the UE based solution for DL LBT issue detection and not pursue the network based solution before any conclusion for the UE based soluion in RAN2

In the end,  the MLB for NR-U is almost completed in RAN3, it is proposed to proceed the following stage 2 TP to TS 38.300:

#### 15.5.1.2	Load reporting for intra-RAT and intra-system inter-RAT load balancing

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

The following load related information should be supported which consists of:

-	Radio resource usage (per-cell and per SSB area PRB usage: DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, DL/UL total PRB usage, and DL/UL scheduling PDCCH CCE usage; PRB usage for slice(s): DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, and DL/UL Total PRB allocation);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell, per SSB area and per slice: UL/DL available capacity);

-	HW capacity indicator (offered throughput and available throughput over E1, percentage utilisation over F1);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs;

-	NR-U channel load (channel occupancy time percentage DL/UL, energy detection threshold DL/UL).

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

To agree on the stage 2 TP to the BLCR of TS 38.300.

3	Conclusion

In this contribution, we have discussed the remaining issuss about SON for NR-U and propose the following.

- The gNB-CU is already informed with the UL EDT by the gNB-DU and there is no need to transfer the UL EDT in MLB related messages.

- To consult RAN2 on the feasibility of the UE based solution for DL LBT issue detection and not pursue the network based solution before any conclusion for the UE based soluion in RAN2

- To agree on the stage 2 TP to the BLCR of TS 38.300.

The stage 2 TP is provided in the annex.

Annex1	TP for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 15.5.1.2	Load reporting for intra-RAT and intra-system inter-RAT load balancing

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

The following load related information should be supported which consists of:

-	Radio resource usage (per-cell and per SSB area PRB usage: DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, DL/UL total PRB usage, and DL/UL scheduling PDCCH CCE usage; PRB usage for slice(s): DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, and DL/UL Total PRB allocation);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell, per SSB area and per slice: UL/DL available capacity);

-	HW capacity indicator (offered throughput and available throughput over E1, percentage utilisation over F1);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs;

-	NR-U channel load (channel occupancy time percentage DL/UL, energy detection threshold DL/UL).

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>