3GPP TSG-RAN WG3 Meeting #121	R3-234087

Toulouse, France, 21 – 25 Aug, 2023

Title: 	(TP to TS 37.480 BL CR) Consideration on remaining issues for MT-SDT

Source: 	Huawei

Agenda item:	20.2

Document Type:	Other

# Introduction

At RAN3#120 meeting, the following agreement were made.

In E1AP: MT-SDT indicator IE within MT-SDT information IE is not needed. In XnAP: MT-SDT data size calculation includes total of both SDT signalling and SDT user plane data. Try to capture it into the TP. In XnAP: Both MT-SDT indicator IE and MT-SDT Data Size IE are “Mandatory”. In F1AP: MT-SDT indicator IE is “Mandatory”. In F1AP: gNB-CU makes the MT-SDT decision, gNB-DU shall follow the decision. When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. 2 FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method. When large size of new DL data is coming through SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method. To be FFS: How to calculate MT-SDT Data Size for SDT DL data packets.

In this document we are going to discuss the remaining MT-SDT issues based on the above agreements, and provide the TP for 37.480 BLCR.

# Discussion

  - non-SDT data arrival in E1AP: DL DATA NOTIFICATION message

In case that non-SDT data or signalling is arrived during the ongoing SDT session, gNB-CU-CP shall first perform state transition to RRC connected state procedure, e.g. sending RRC resume message to the UE, and then transmit it in RRC connected state[1]. For non-SDT signalling, the gNB-CU-CP can realize it by itself while for non-SDT data, in last RAN3 meeting, we got an agreement with FFS for letting gNB-CU know the non-SDT data arrival as below:

When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. 2 FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method.

On FFS point, we think that either excluding MT-SDT information IE or introducing a new indicator (i.e. non-SDT data arrival) works for gNB-CU-CP to determine that state transition to RRC connected state shall be initiated for this UE. Although the solution of excluding MT-SDT information IE has less spec impact, introducing a new indicator is clearer and can avoid confusion especially in case of different version of gNB-CU-UPs deployment. We slightly prefer the solution of introducing new indicator to indicate non-SDT data arrival.

Proposal 1: When new DL data comes through non-SDT bearer during SDT session, the gNB-CU-UP shall send a DL DATA NOTIFICATION message with a new indicator to indicate non-SDT data arrival.

  - oversize SDT data arrival indication in E1AP: DL DATA NOTIFICATION message

In case that large size of new SDT data arrived during the ongoing SDT session, those data would be suitable to be transmitted over RRC connected state [1] and in last RAN3 meeting, we got an agreement with FFS on this case as below:

When large size of new DL data is coming through SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method. To be FFS: How to calculate MT-SDT Data Size for SDT DL data packets.

Upon receiving the large size SDT data from the core network, there is currently no way for the gNB-CU-UP to indicate such information to the gNB-CU-CP, considering that some implementations may still be able to transmit these oversized SDT data via the ongoing SDT procedure, some other implementations have to move the UE to RRC_Connected, it is needed for the gNB-CU-UP to provide a new indicator to indicate oversize SDT data arrival.

Proposal 2: When large size of new DL data is coming through SDT bearer during SDT session, the gNB-CU-UP shall send DL DATA NOTIFICATION message with a new indicator to indicate oversize SDT data arrival.

# Conclusion

Based on the discussion above, the following proposals are made:

Proposal 1: When new DL data comes through non-SDT bearer during SDT session, the gNB-CU-UP shall send a DL DATA NOTIFICATION message with a new indicator to indicate non-SDT data arrival.

Proposal 2: When large size of new DL data is coming through SDT bearer during SDT session, the gNB-CU-UP shall send DL DATA NOTIFICATION message with a new indicator to indicate oversize SDT data arrival.

Based on these proposals, the corresponding TP to TS37.480 is provided in section 4.

# TP to TS 37.480 BL CR

-----Start of the Change----

### 5.1.2	E1 bearer context management function

The establishment of the E1 bearer context is initiated by the gNB-CU-CP and accepted or rejected by the gNB-CU-UP based on admission control criteria (e.g., resource not available).

The modification of the E1 bearer context can be initiated by either gNB-CU-CP or gNB-CU-UP. The receiving node can accept or indicate failure to carry out the modification request. The E1 bearer context management function also supports the release of the bearer context previously established in the gNB-CU-UP. The release of the bearer context is triggered by the gNB-CU-CP either directly or following a request received from the gNB-CU-UP.

This function is used to setup and modify the QoS-flow to DRB mapping configuration. The gNB-CU-CP decides flow-to-DRB mapping and provides the generated SDAP and PDCP configuration to the gNB-CU-UP. The gNB-CU-CP also decides the Reflective QoS flow to DRB mapping. The function is also used to send to the gNB-CU-UP the alternative QoS Parameters Sets when available for a QoS flow. For each PDU Session Resource to be setup or modified, the S-NSSAI, shall be provided in the E1 bearer context setup procedure and may be provided in the E1 bearer context modification procedure by gNB-CU-CP to the gNB-CU-UP.

This function is also used to setup and modify the EPS bearer/E-RAB to DRB mapping configuration for the case of eNB-CP and eNB-UP separation. The eNB-CP decides EPS bearer/E-RAB-to-DRB mapping and provides the E-UTRAN/NR PDCP configuration to the eNB-UP.

This function is used for the gNB-CU-CP to send the security information to the gNB-CU-UP.

This function is used for the gNB-CU-CP to send to the gNB-CU-UP transport layer information to be used for data forwarding e.g., during handovers.

This function is used for the gNB-CU-CP to send the parameters for header compression for certain traffic types e.g., IP, Ethernet to the gNB-CU-UP.

This function is used for the gNB-CU-CP to send the uplink data compression parameters to the gNB-CU-UP for certain data radio bearer(s).

This function is used for the gNB-CU-UP to notify the event of DL data arrival detection to the gNB-CU-CP. With this function, the gNB-CU-UP requests gNB-CU-CP to trigger paging procedure over F1 or Xn to support RRC Inactive state. RRC Inactive state is not supported when this function is used between an eNB-CP and an eNB-UP.

This function is used for the gNB-CU-UP to notify the gNB-CU-CP that a DL packet including a QFI value not configured by the gNB-CU-CP or an UL packet including a QFI value in the SDAP header of the default DRB not configured by the gNB-CU-CP is received for the first time. The gNB-CU-CP can take further action if needed.

This function is used for the gNB-CU-UP to notify the gNB-CU-CP that non-SDT data is received or SDT data crossed the data volume threshold is received during SDT procedure, the gNB-CU-CP can take further action if needed.

This function is used for the gNB-CU-UP to notify the event of user inactivity to the gNB-CU-CP. With this function, the gNB-CU-UP indicates that the inactivity timer associated with a bearer, a PDU session or a UE expires, or that user data is received for the bearer, the PDU session or the UE whose inactivity timer has expired. The gNB-CU-CP consolidates all the serving gNB-CU-UPs for the UE and takes further action.

This function is used for the gNB-CU-UP to report data volume to the gNB-CU-CP.

This function is used for the gNB-CU-CP to notify the suspension and resumption of bearer contexts to the gNB-CU-UP. Suspension and resumption of bearer contexts are not applicable to eNB-CP/eNB-UP and ng-eNB-CU-CP/ng-eNB-CU-UP.

This function also allows to support CA based packet duplication as described in TS 38.300 [6], i.e. one data radio bearer should be configured with at least two GTP-U tunnels between gNB-CU-UP and a gNB-DU.

This function is used to support the enhanced mobility operations as described in TS 38.300 [6] in the gNB-CU-UP.

-----End of the Change----