3GPP TSG-RAN WG3 Meeting #121	R3-234097

Toulouse, France, 21st – 25th Aug, 2023

Title: 	Further discussion on the support for QoE in NR-DC

Source: 	Huawei

Agenda item:	11.3

Document Type:	discussion and decision

1. Introduction

In the last meeting, RAN3 discussed the QoE measurement in NR-DC and reached some agreements. In this paper we provide our views on the remaining issues and make further proposals.

2. Discussion

2.1 Configuration of QoE measurement

In RAN3#119, RAN3 achieved the following agreements for the management based QoE measurement.

For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

The MN shall send the configuration information to the UE, or

The SN should send the configuration to the UE directly, or

The SN should send the configuration information to the UE via the MN (inside a container).

Then in RAN3#119b, RAN3 had the following agreements for the management based QoE measurement.

Support the following scenarios for m-based QoE configuration received in the SN:

The SN wants to configure the UE by using SRB3.

The SN wants to configure the UE, by sending the configuration in a transparent container to the MN, which then sends it to the UE via SRB1.

According to the above agreements, it is still not clear, after receiving the request from SN, whether MN can configure the QoE measurement for UE and send the configuration information to UE.

If the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure.

The MN is responsible for RRC ID allocation for m-based sessions configured by the MN or SN, and notifies the allocated RRC ID(s) to the SN.

According to agreements of RAN3, if the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure and MN assigns an RRC ID for this QoE measurement. If SRB3 is not supported, SN sends the SN RRC message including the QoE measurement configuration to UE via MN. It will increase the delay of QoE measurement and increase the interaction between MN and SN. Therefore we think SN can request MN to send the QoE measurement configuration to UE. In this case, SN needs to send the QoE measurement configuration container to MN and then MN can directly send the configuration to UE.

- SN can request MN to send the QoE measurement configuration to UE, which can be rejected by MN. When SN sends the request message of management based QoE to MN, SN can include QoE measurement configuration container.

In the last meeting of RAN3#120, RAN3 has the following FFS:

FFS whether:

- For management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE via SRB3.

- For management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE transparently for the MN via SRB1.

In the last meeting, some companies argued that, the FFS should be supported directly, as we already had the following agreement

For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether: The MN shall send the configuration information to the UE, or The SN should send the configuration to the UE directly, or The SN should send the configuration information to the UE via the MN (inside a container).

We however, think there is no usage for MN to know whether SN wants to configure the UE via SRB3 or transparently for the MN via SRB1. From the perspective of MN, it only sees one SN RRC message.

In our understanding, MN only needs to know whether SN wants MN to configure the QoE measurement for UE. MN can know it based on whether the QoE measurement configuration container is included in the request message. If the QoE measurement configuration container is included, MN knows SN wants MN to configure. Otherwise MN knows SN wants to configure itself and MN only need to send the RRC ID to SN.

- SN does not need to explicitly indicate to the MN whether SN wants to configure QoE via SRB3 or to configure transparently for MN via SRB1.

RAN3 also has the following remaining issue.

In NR-DC, a node can configure the UE with an m-based QoE configuration only if it has received this configuration from the OAM, and if it serves the UE by a cell within the area scope.

In R17 QoE, the network will configure the QoE measurement when UE is within the area scope of this QoE measurement. In R17 NR-DC, MN/SN does not know the serving cells of SN/MN respectively. Therefore we think the simpler solution is that only one node checks the area scope. The OAM directly sends the QoE measurement configuration to each RAN, and it is the receiving node to decide whether it wants to configure the QoE measurement for UE. Therefore we suggest the node receiving the management based QoE measurement checks the area scope.

- It is the node which receives the QoE measurement from OAM to check the area scope.

For the QoE measurement configurations received by SN from OAM, in our understanding, SN may request more than one QoE measurements in one message and may request SRB4 for all the QoE reporting. In this case, MN may reject parts of SRB4 request due to the overload of MN. Therefore we think MN should explicitly inform SN which request of SRB4 is rejected.

- MN should explicitly inform SN which request of SRB4 is rejected

2.2 RAN visible QoE

- In the last meetings, RAN3 has the following agreements and FFS on the RAN visible QoE.

For UEs in NR-DC, the node that configured the UE with a QoE measurement configuration can generate the corresponding RVQoE measurement configuration.

- FFS whether, in a UE in NR-DC, each QoE configuration can have more than one corresponding RVQoE configuration.

- In order to avoid the complexity of UE, we think only one node is allowed to configure the RAN visible QoE measurement corresponding to each QoE measurement configuration. In the last meetings, RAN3 has confirmed that the network does not know in advance which of the two nodes carried the application session. For the first RVQoE configuration, it is blindly configured by MN or SN. It is straightforward that it is only the node which sends the QoE measurement configuration container can configure the RAN visible QoE.

- Only the node which sends the QoE measurement configuration container to the UE can configure the RAN visible QoE measurement corresponding to this QoE measurement.

- In the last meeting, RAN3 has the following agreements:

When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. FFS whether the indication is explicit or implicit.

- If the node decides the peer node carries the application session based on the first RAN visible QoE report, it can decide whether need to send the RAN visible QoE report to the peer node based on the interests of the peer node.

- In some cases, only the node which configures the RAN visible QoE knows the available RAN visible QoE metrics. Therefore it also needs to forward the available RAN visible QoE metrics to the peer node.

- The node which configures the RAN visible QoE can ask the peer node whether the peer node is interested in receiving the RVQoE reports and which RAN visible QoE metrics are needed, and sends the available RAN visible QoE metrics to the peer node.

- The next issue is whether the QoE reports container and RVQoE reports can be sent over different legs

As the baseline, QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over the same leg.

WA: QoE reports and RVQoE reports pertaining to the same QoE reference can be sent over different legs.

- In our understanding, the use case of RVQoE leg switching is that the services are only served by the node which is different from the current node receiving the RAN visible QoE report. In this case, it is beneficial to switch the leg of RAN visible QoE report. Considering we have introduced the switch of the leg of QoE reporting container, we can use the same command to switch these two legs together. It means it is not necessary to introduce an independent switch indication for the RAN visible QoE reporting. If different legs are supported, it will increase the number of signalling in Uu.

- Not need to introduce an independent switch indication for the RAN visible QoE reporting. The legs of QoE results container reporting and RAN visible QoE reporting are switched together.

2.3 Others

- In the last meeting, RAN3 has the following agreements:

If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. Whether the SN-configured QoE/RVQoE configuration information can be passed to the MN in case of SN release needs to be further discussed.

In R17 QoE, the UE releases the QoE measurement when the UE enters to the RRC_IDLE. In NR-DC, the UE will release all the SCG configuration when the SCG is released. But in order to keep the continuity of services, the MN can transmit the services which are served by the SN. For example, the MN can reconfigure or add DRBs for these services. For these cases, we need to keep the QoE measurement continuity because the services are still on-going, the UE can also report the QoE results to the MN so that MCE can still obtain the QoE results and MN can use the RAN visible QoE results to optimize the scheduling.

Some companies may argue that it is strange that UE sends the QoE results to MN when MN does not configure this QoE measurement for UE. In the last meeting, RAN3 has agreed that the network can switch the report leg of the QoE reporting. It means that UE can send the QoE reporting to MN even if MN does not configure the QoE measurement for UE. Therefore we think the network can use the same principle to switch the QoE reporting in the SN release.

- In order to forward the QoE results to MCE by MN after the SN release and optimize the scheduling based on the RAN visible QoE results, MN needs to know the MCE address and the RAN visible QoE configuration.

- For the QoE measurement configured by the SN, continue the QoE measurement if the services are still on-going in the SN release case. MN receives the QoE/RAN visible QoE configuration information from SN and informs UE whether to keep the QoE measurement.

- Corresponding stage 2 TPs to 37.340 and 38.423 reflecting the proposals above could be seen in the annex and [1] respectively.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- When SN sends the request message of management based QoE to MN, SN can include QoE measurement configuration container.

- SN does not need to explicitly indicate to the MN whether SN wants to configure QoE via SRB3 or to configure transparently for MN via SRB1.

- It is the node which receives the QoE measurement from OAM to check the area scope.

- MN should explicitly inform SN which request of SRB4 is rejected

- Only the node which sends the QoE measurement configuration container to the UE can configure the RAN visible QoE measurement corresponding to this QoE measurement.

- The node which configures the RAN visible QoE can request the peer node whether the peer node is interested in receiving the RVQoE reports and which RAN visible QoE metrics are needed, and sends the available RAN visible QoE metrics to the peer node.

- Not need to introduce an independent switch indication for the RAN visible QoE reporting. The legs of QoE results container reporting and RAN visible QoE reporting are switched together.

- For the QoE measurement configured by the SN, continue the QoE measurement if the services are still on-going in the SN release case. MN receives the QoE/RAN visible QoE configuration information from SN and informs UE whether to keep the QoE measurement.

4. Reference

- R3-234098, (TP for 38.423) on QoE measurement enhancements, Huawei

5. Annex TP to 37.340

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

BFD	Beam Failure Detection

CHO	Conditional Handover

CLI	Cross Link Interference

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DC	Intra-E-UTRA Dual Connectivity

DCP	DCI with CRC scrambled by PS-RNTI

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

MCG	Master Cell Group

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR-DC	NR-NR Dual Connectivity

QMC	QoE Measurement Collection

QoE	Quality of Experience

RLM	Radio Link Monitoring

SCG	Secondary Cell Group

SMTC	SS/PBCH block Measurement Timing Configuration

SN	Secondary Node

V2X	Vehicle-to-Everything

# 13	Other aspects

## 13.1	Interference avoidance for in-device coexistence

In-Device Coexistence (IDC) solution as described in TS 36.300 [2] is extended to address EN-DC operation. Only FDM solution, where the list of NR carriers suffering from IDC problems is signalled in IDC indication, is supported in this version of the specifications. The requirement on RRM/RLM/CSI measurements in different phases of IDC interference defined in TS 36.300 [2] is applicable except that for NR serving cell, the requirements in TS 38.133 [8] and TS 38.101-1 [12], TS 38.101-2 [13], TS 38.101-3 [14] apply.

## 13.2	Sidelink

NR Sidelink Communication, V2X Sidelink Communication and NR Sidelink Discovery cannot be configured in MR-DC in this release.

## 13.3	SCG UE history information

The MN stores and correlates the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwards UE History Information and optional UE History Information from the UE to its connected SNs. The resulting information is then used by SN for dual-connectivity operation. The SN is in charge of collecting SCG UE history information and providing the collected information to the MN.

If the UE stays in a PSCell for a duration exceeding the maximum value of the Time Stay parameter, the SN may store the PSCell information with consecutive entries using the same PSCell identity. The total stay time in this PSCell is the sum of stay time for all consecutive PSCell with the same identity.

The SN shall provide the collected SCG UE history information, if available, to the MN in the following procedures:

-	the SN Release, and SN initiated SN Change procedures

-	the MN initiated SN Modification procedure if requested by the MN in this procedure

-	the SN initiated SN modification procedure upon PSCell change if subscribed in the SN Addition procedure

When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via Handover Request message for CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source PSCell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving Handover Request message from the source NG-RAN node to receiving RRC Reconfiguration Complete message from the UE.

## 13.x	Application Layer Measurement Collection

### 13.x.1	Overview

The QoE Measurement Collection function as described in TS 38.300 [3] is extended to address NR-DC operation. The requirements on the gNB provided in TS 38.300 [3] apply to the MN, together with additional requirements on MN and SN provided in following sub-clauses.

### 13.x.2	QoE Measurement Configuration

#### 13.x.2.1	QoE Measurement Collection Activation and Reporting

For a UE in NR-DC, the MN and the SN may coordinate QoE measurement collection activation and reporting as follows:

For signalling-based and management-based QoE activation, the MN:

- Allocates the measurement configuration application layer ID;

- Determines whether the MN or the SN sends the management-based QoE configuration to the UE;

- Notifies the SN accordingly.

For management-based QoE activation received directly by the SN from the OAM, the SN may perform UE selection. For a selected UE, the SN indicates to the MN QoE reference of the management-based QoE session, QoE measurement configuration container and whether it is going to receive the corresponding reports via the MN (using SRB4) or using SRB5. The MN indicates the allocated measurement configuration application layer ID to the SN.

The network indicates to the UE whether to send QoE reports via SRB4 or SRB5, per QoE reference. The SRB for QoE reporting can be changed during the application session. The command for changing the SRB used for reporting may be sent to the UE by the node that configured that specific QoE configuration.

The MN should inform the SN that a UE is configured with a management-based QoE/RAN visible QoE measurement configuration.

If the MN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the SN, then, if the MN decides that the SN forwards the reports directly to the MCE, the MN should indicate to the SN the QoE reference, the MCE IP address and the measConfigAppLayerId.

If the SN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the MN, then, if the SN decides that the MN forwards the reports directly to the MCE, the SN should indicate to the MN the QoE reference and the MCE IP address.

Only the node which sends the QoE measurement configuration container to the UE can configure the RAN visible QoE measurement corresponding to this QoE measurement.

If one node receives the RAN visible QoE report from UE and the services corresponding to this QoE measurement is served by the peer node, it can send the received RAN visible QoE report and the available RAN visible QoE metrics to the peer node, the peer node responds with the RAN visible metrics that it wants or indication that it does not want to receive the RAN visible QoE report or whether it prefers to receive the RAN visible QoE report directly from UE.

### 13.x.3	QoE Measurement Continuity for Mobility

Editor’s note: Content for this clause is FFS. QMC continuity during mobility in NR-DC should be discussed after the baseline solution for QMC in NR-DC is in place.

At SN change without MN change, the QoE configuration information is passed from the source SN to the target SN via the MN. If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. For the QoE measurement configured by the SN in the SN release, MN receives the QoE/RAN visible QoE configuration information from SN and informs UE whether to keep the QoE measurement.