3GPP TSG-RAN3 Meeting #121	R3-234100

# Toulouse, France, 21st – 25th Aug, 2023

Agenda Item:	17.2

Source: 	Huawei

Title: 		Further discussion on multiple TACs

Document for:	Discussion

Introduction

In the last meeting, we achieved a WA with a related ‘to be continue’ issue as follows

WA: Do not exchange multiple TACs over Xn for NTN. How to describe which TAC should be used in semantic description?

In this paper, we provide some further considerations regarding the TAC issue.

Discussion

In the last meeting, RAN3 agreed that Uu cell ID is expected to be used in Xn setup and Configuration Update procedures, but also minute that ‘the usage of mapped cell ID is not precluded’. With the agreement achieved, it seems RAN3 has solved most controversial issues, and the only ‘to be continued’ issue is how to exchange TAC in Xn setup and Configuration update procedures.

One WA was achieved to not exchange multiple TACs over Xn for NTN. We suggest to turn this WA into an agreement and finish the related discussions. Note that, OAM is anyway present and signalling only reduce the OAM effort, the gNB knows the relationship between PCI/Uu cell ID and mapped cell IDs/TACs. This allows the nodes to check whether a certain TAC is in mobility restriction list (MRL). Alternatively, information of neighbouring with the peer nodes can be exchanged via exchanging cell ID and TAC via Xn setup/configuration update. However, we note that exchanging neighbouring information via Xn is only one alternative way, which cannot replace the work of OAM. The neighbouring are anyway validated by OAM before any operation (TS 38.300). This is especially the case for NTN, where Xn may not always be deployed, and there is currently no signalling to exchange neighbouring information via NG.

In addition, introducing a new multiple TAC list requires frequent update of TAC information, and hence lead to frequent NG-RAN configuration update signalling. As a result, OAM effort is inevitable, and there is no need to do further enhancements like exchange multiple TAC list over Xn.

Turn the ‘WA: Do not exchange multiple TACs over Xn for NTN’ into an agreement

- About the fact that the TAC included in Served Cell Information NR is a mandatory IE, it was agreed in RAN2 in R17 that, the network may broadcast Multiple TACs per PLMN in a NR NTN cell. We need to decide which TAC should be used in this case. In our view, in the context of NTN, since OAM anyway maintains the neighboring relationship between different nodes, we can transfer any one of the TAC that is broadcast in NTN cell. Correspondingly, we propose relevant changes for XnAP as shown in the annex.

The TAC transferred in Served Cell Information NR IE can be whatever TAC that is broadcast in NR NTN cell as defined in RAN2

Summary

Based on the discussion, we have following observations and proposals:

Proposal 1: Turn the ‘WA: Do not exchange multiple TACs over Xn for NTN’ into an agreement.

- Proposal 2:  The TAC transferred in Served Cell Information NR IE can be whatever TAC that is broadcast in NR NTN cell as defined in RAN2

Annex- TP to 38.423

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code In case of NTN, the TAC is one of the TAC defined in trackingAreaList-r17 as specified in TS 38.331 [10]. | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs contained in the SIB1 message as specified in TS 38.331[10], associated to the NR Cell Identity in the NR CGI IE. | – |  |
| >PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| <Unchanged text omitted> |  |  |  |  |  |  |