3GPP TSG-RAN WG3 Meeting #121	R3-234101

Toulouse , France，21st Aug – 25th Aug, 2023

Title: 	Discussion on common signalling in CHO

Source: 	Huawei

Agenda item:	17.2

Document Type:	Discussion

1. Introduction

In RAN2 #122, an LS was sent to RAN3 to ask whether it is feasible that the target cell’s servingCellConfigCommon is provided independently to the source cell for the purpose of inter-gNB handover in Rel-18 with or without specification changes, which is copied as follows:

1. Overall Description: For mobility enhancement in Rel-18 NR NTN, RAN2 has discussed the solution for source cell to broadcast in advance servingCellConfigCommon from the target cell (as common (C)HO signalling) in order to reduce signalling overhead of handover command which is sent using dedicated (per UE) signalling. In this way, the target´s cell servingCellConfigCommon does not need to be included in each RRCReconfiguration message. During RAN2’s discussion, it was indicated this might have RAN3 impact, e.g., for the inter-gNB handover case, the target cell’s servingCellConfigCommon might need to be transferred to the source cell using different means than what’s available in the legacy specification. To facilitate RAN2’s future work, RAN2 would like to ask RAN3 to answer the following question. In case target cell’s servingCellConfigCommon needs to be broadcasted in the source cell, whether it is feasible that the target cell’s servingCellConfigCommon is provided independently to the source cell for the purpose of inter-gNB handover in Rel-18 with or without specification changes?

This paper provides the considerations about these issues.

2. Discussion

In past RAN2 meetings, common configuration is proposed to reduce signaling overhead of handover. Specifically, cell-specific configurations (e.g., servingCellConfigCommon) of target cells can be broadcast to UE by source cell rather than using the dedicated signaling, which significantly reduces the size of RRCReconfiguration message. To achieve this, target cell needs to transfer its configuration to the source cell, which is difficult to achieve according to current spec. The reasons are explained as follows.

One possible way to transfer servingCellConfigCommon between gNBs is by using Xn interface. Specifically, source gNB and target gNB can exchange their configuration information during Xn setup procedure. However, as raised by some companies in RAN3, the availability of Xn interface is not guaranteed in NTN scenario, especially for transparent NTN framework.

Observation 1: Transferring servingCellConfigCommon via Xn is feasible, but there can be very rare deployment of Xn interface.

The other possible way is to transfer this information between gNBs via NG interface. However, in current version of spec, source gNB and target gNB only transfer the UE associated information during handover preparation procedure, and the non-UE associated cell-specific configurations like servingCellConfigCommon are not exchanged via NG interface in usual cases. In our view, it is not appropriate for source gNB to obtain target cell’s servingCellConfigCommon via NG.

Observation 2: Transferring servingCellConfigCommon via NG is not an appropriate solution.

Furthermore, broadcast servingCellConfigCommon of target cell in serving cell will cause increasing power consumption of UE. Generally, UE will receive the system information upon accessing the serving cell. However, once the serving cell broadcasts servingCellConfigCommon of target cell, the corresponding SIB broadcast in the current serving cell may update frequently. For example, once the neighboring cells of the current serving cell change (e.g., add/delete), the current serving cell has to update the corresponding SIB, and all UEs served by the current serving cell need to read the SIB, resulting in increasing UE power consumption.

Observation 3: Broadcasting servingCellConfigCommon of target cell in serving cell will cause increasing power consumption of UE.

Based on the observations above, we should inform RAN2 about the possible issues identified by RAN3.

LS back to RAN2, informing RAN2 the potential issues identified by RAN3.

- A draft LS out is shown in the annex.

3 Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: Transferring servingCellConfigCommon via Xn is feasible, but there can be very rare deployment of Xn interface.

Observation 2: Transferring servingCellConfigCommon via NG is not an appropriate solution.

Observation 3: Broadcasting servingCellConfigCommon of target cell in serving cell will cause increasing power consumption of UE.

- LS back to RAN2, informing RAN2 the potential issues identified by RAN3.

4. Annex

3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, France, 21st – 25th Aug, 2023

Title:	[draft] Reply LS on common signalling in (C)HO

Response to:	LS R2-2306922

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:	-

Contact person:	Jingcong Sun

Sunjingcong@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN3 thanks to RAN2 for common signalling in (C)HO.

RAN3 thinks transferring servingCellConfigCommon via Xn is feasible, but also there can be very rare deployment of Xn interface considering the long distance between gNBs in NTN scenario. Then RAN3 notes that usually source gNB and target gNB only transfer the UE associated information during handover preparation procedure in NG, which means it is not appropriate to transfer cell-specific configurations like servingCellConfigCommon. Also RAN3 thinks broadcasting servingCellConfigCommon of target cell in serving cell can lead to more power consumption of UE, which needs RAN2 to evaluate.

# 2	Actions

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above feedback into account.

# 3	Dates of next RAN3 meetings

RAN3#121-bis	                2023-10-09 - 2023-10-13 Xiamen, CN

RAN3#122                                  	                                              2023-11-13 - 2023-11-17 Chicago, US