# 3GPP TSG-RAN WG3 Meeting #121	R3-234114

# 21-25 August 2023, Toulouse, France

Agenda Item:	16.4

Source:	CATT

Title:	(TP for SL relay 38.401 and 38.473) Discussion on Multi-path Support for SL relay

Document for:	Discussion

- Introduction

In previous meetings, there are some agreements have been achieved.

WA: For indirect path addition, the Remote UE Local ID IE and the T420-like timer are sent to the gNB-DU.

The details of T420-like timer are pending to RAN2.

RAN3 reuses the existing F1AP signaling to achieve the operations for ‘Path to be modified’ by means of modifying the configurations associated to the path(s).

In this contribution, we discuss the RAN3 impacts for multi-path for Scenario 1. Scenario 2 may or may not have stage-3 impact for authorized information but it still under discussing in RAN2, so we can wait for them progress.

- Discussion

2.1 LS from RAN2

LS to RAN3 on mode 1 scheduling in inter-DU multi-path case

For Scenario-1, mode-1 scheduling for remote UE is supported at least for intra-DU case, with the SR/BSR and grant sent on the direct path; whether it is supported for inter-DU case is up to R3, but R2 do not intend to make specification changes to support this case, and for specification purposes RAN2 intend to model it as a single MAC entity at the UE.

Based on the LS, it can be found that whether to support inter-DU mode-1 scheduling is up to RAN3. The SL-BSR reported from UE contains destination index, LCG ID and buffer size.

However, LCG ID is configured by SL-LogicalChannelConfig-r16 in SL-ConfigDedicatedNR-r16. For inter-DU MP, SL-ConfigDedicatedNR-r16 is generated by the indirect path DU.

SL-LogicalChannelConfig-r16 ::=            SEQUENCE {

sl-Priority-r16                            INTEGER (1..8),

sl-PrioritisedBitRate-r16                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

sl-BucketSizeDuration-r16                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

sl-ConfiguredGrantType1Allowed-r16         ENUMERATED {true}                                                        OPTIONAL,   -- Need R

sl-HARQ-FeedbackEnabled-r16                ENUMERATED {enabled, disabled }                                          OPTIONAL,   -- Need R

sl-AllowedCG-List-r16                      SEQUENCE (SIZE (0.. maxNrofCG-SL-1-r16)) OF SL-ConfigIndexCG-r16

OPTIONAL,   -- Need R

sl-AllowedSCS-List-r16                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                        OPTIONAL,   -- Need R

sl-MaxPUSCH-Duration-r16                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1}

OPTIONAL,   -- Need R

sl-LogicalChannelGroup-r16                 INTEGER (0..maxLCG-ID)                                                   OPTIONAL,   -- Need R

sl-SchedulingRequestId-r16                 SchedulingRequestId                                                      OPTIONAL,   -- Need R

sl-LogicalChannelSR-DelayTimerApplied-r16  BOOLEAN                                                                  OPTIONAL,   -- Need R

...

}

-- TAG-SL-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

Observation 1: SL-BSR includes destination index, LCG ID and buffer size while LCG ID is configured by sl-LogicalChannelGroup-r16 which is generated by indirect path DU.

To support inter-DU mode 1, there are two options can be considered.

- Option 1: Direct path DU sends SL-BSR to indirect path DU, indirect DU generates PC5 resource base on SL-BSR.

- Option 2: Indirect path sends SL-LogicalChannelConfig-r16 of remote UE to direct path DU, direct path DU generates PC5 resource base on SL-LogicalChannelConfig-r16

For option 1, direct path DU sends SL-BSR to CU via F1AP UE context setup or modification response message (step 1), CU further sends the SL-BSR to indirect path DU via UE context setup or modification request message (step 2). After PC5 resource allocating, indirect path DU sends the allocated PC5 resource to direct path DU via CU (step 3 and step 4) and direct path DU schedules remote UE via DCI. Note that the MAC entity and PHY entity of remote UE is located at direct path and DCI is sent by PHY entity hence the allocated PC5 resource should be sent back to direct path and direct path schedules PC5 resource via DCI.

For option 2, CU may store the SL-LogicalChannelConfig-r16 which is configured by indirect path DU. It can send the SL-LogicalChannelConfig-r16 to direct path DU proactively (step 1). In case of CU does not store SL-LogicalChannelConfig-r16, indirect path DU should inform SL-LogicalChannelConfig-r16 to CU (step 2). After PC5 resource allocating in direct path DU, direct path DU sends the resource by DCI via direct path.

Both options are feasible. Transferred BSR in option 1 may shorter than SL-LogicalChannelConfig-r16 in option 2 while option 2 may only need one F1AP message(s) from CU to direct path i.e., transfer SL-LogicalChannelConfig-r16 or two F1AP message(s) in case CU does not store low layer configuration.

Observation 2: There are four steps are needed for option 1 while may be only one or two messages is needed for option 2. On the contrary, SL-BSR in option 1 may shorter than SL-LogicalChannelConfig-r16 in option 2 which can reduce the load of F1AP message.

It seems that option 2 would be faster and save more signaling than option 1 and the size of SL-LogicalChannelConfig-r16 may not too heavy for interface. We are open to discuss both options but slightly prefer option 2.

Proposal 1: RAN3 support mode 1 scheduling in inter-DU multi-path case. There are two options can be considered as follow. We slightly prefer option 2.

-	Option 1: direct path DU sends BSR to indirect path DU, indirect DU generates PC5 resource base on BSR.

-	Option 2: indirect path sends SL-LogicalChannelConfig-r16 to direct path DU, direct path DU generates PC5 resource base on SL-LogicalChannelConfig-r16

2.2 Path release

In last meeting, a CHIOCE structure has been introduced for path addition. For path modification, we agreed to use legacy “path to be modified”. The left issue is path release for intra-DU.

- Release case: CU decides to release indirect path, it will send UE context release command to DU. However, DU may not know which path should be release when DU receives UE context release message.

There are two options to enhance UE context release command:

- Option 1: relay UE L2ID and/or PCell ID

- Option 2: path type: direct path/indirect path/both paths

- Option 3: Path Switch Configuration IE for direct path release, PC5/Uu RLC release for indirect path release via UE context modification procedure

- Option 4: path ID

We slightly prefer Option 1 because it would be further proof in case multi path supports two indirect paths or multi-hops in the further compared with option 2. Once the relay UE L2ID present, the relay UE located indirect path should be released. For option 3, the name is about path switch, it is not very suitable for path release scenario for multi path. Also, this procedure is path release but it has to convey some useful less information e.g., Remote UE Local ID (M) and T420 (M). Option 4 is required to allocate path ID in path addition procedure, it would be more complex than option 1 from specification design perspective. One relay UE located in two paths may only exits for multi-hops but it can be addressed by include each relay UE in path release information.

| Target Relay UE ID | M |  | BIT STRING (SIZE(24)) | Corresponds to the targetRelayUE-Identity contained in the CellGroupConfig IE, defined in TS 38.331 [8] |
| --- | --- | --- | --- | --- |
| Remote UE Local ID | M |  | 9.3.1.267 |  |
| T420 | M |  | ENUMERATED (ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000) | Corresponds to the t420 contained in the CellGroupConfig IE, defined in TS 38.331 [8] |

The stage-3 impact for option 1 as follow, the CHOICE structure can be added in UE context release command message which is align with path addition procedure.

#### 9.3.1.x	Path Release Information

This IE provides information for path release.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Path Addition Information | M |  |  |  |
| >Indirect Path Release |  |  |  |  |
| >>Target Relay UE ID list |  | 0 .. <maxnoofTargetRelayUE> |  |  |
| >>>Target Relay UE ID | M |  | BIT STRING (SIZE(24)) | Corresponding to SL- targetRelayUEIdentity-r17, defined in TS 38.331 [8] |
| >Direct Path Release |  |  |  |  |
| >>PCell ID | M |  | NR CGI 9.3.1.12 |  |

Proposal 2: Add a CHOICE structure in UE context release command message for path release.

2.3 RRC reconfiguration complete in multi-path

The relevant agreements achieved in RAN2#121bis as below

Agreements:

When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1.

When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition.

-	when primary RLC entity of split SRB1 is on direct path

-	when non-split SRB1 is configured on direct path

And further discussed in RAN2#122

WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.  This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

They can be classed into are three cases:

- If non-split SRB1 is configured to remote UE, the RRC reconfiguration complete message is sent via the non-split SRB1 configured path.

- If split SRB1 is configured without duplication and primary RLC entity of split SRB1 is configured on direct path, RRC reconfiguration complete message is sent via the direct path i.e., primary RLC entity located path.

- If split SRB1 is configured with duplication, RRC reconfiguration complete messages are sent via both paths.

Observation 2: RRC reconfiguration complete messages should be sent via primary path in case of split SBR1 without duplication. When split SRB1 with duplication is configured, the remote UE sends the RRC Reconfiguration Complete message to gNB via both paths.

Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress.

In RAN2#121, there are two solutions were agreed as below. In case of primary RLC entity is not configured in the indirect path or non-split SRB1 is not configured in the indirect path and without SRB1 duplication, a PC5-RRC trigger sending from remote UE to target relay UE is needed.

Agreement:

UP-based approach (excluding SL-RLC1) in Option 1 is excluded for relay UE to enter RRC_CONNECTED.

Agreements:

For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case.

Observation 3: In case of primary RLC entity or SRB1 is not configured in the indirect path and without SRB1 duplication, a PC5-RRC trigger sending from remote UE to target relay UE is needed to trigger relay UE entry RRC-CONNECTED state.

Based on observation 2 and observation 3, TS38.401 can be updated as below to remove two Editor’s Notes.

9.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. In case of split SRB1 without duplication, remote UE sends RRCReconfigurationComplete message to the gNB-DU via primary path i.e., primary RLC entity located path.

In case of split SRB1/SRB2 is not configured in the indirect path and target Relay UE in RRC-IDLE/INACTIVE state, a PC5-RRC trigger sending from remote UE to target relay UE is needed to trigger relay UE entry RRC-CONNECTED state.

Proposal 3: Update the description of multi-path support procedure as Annex1.

- Conclusion

Observation 1: BSR includes destination index, LCG ID and buffer size while LCG ID is configured by sl-LogicalChannelGroup-r16 which is generated by indirect path DU.

Proposal 1: RAN3 support mode 1 scheduling in inter-DU multi-path case. There are two options can be considered:

-	Option 1: direct path DU sends BSR to indirect path DU, indirect DU generates PC5 resource base on BSR.

-	Option 2: indirect path sends SL-LogicalChannelConfig-r16 to direct path DU, direct path DU generates PC5 resource base on SL-LogicalChannelConfig-r16

Proposal 2: Add a CHOICE structure in UE context release command message for path release.

Observation 2: RRC reconfiguration complete messages should be sent via primary path in case of split SBR1 without duplication. When split SRB1 with duplication is configured, the remote UE sends the RRC Reconfiguration Complete message to gNB via both paths.

Observation 3: In case of primary RLC entity or SRB1 is not configured in the indirect path and without SRB1 duplication, a PC5-RRC trigger sending from remote UE to target relay UE is needed to trigger relay UE entry RRC-CONNECTED state.

Proposal 3: Update the description of multi-path support procedure as Annex1.

- Reference

[1] RAN2#121bis, Chair notes.

- Annex: TP for 38.401

  - 8.xx Overall procedures for multi-path support

    - 8.xx.1 Inter-DU direct path addition on top of indirect path

The signalling flow for inter-DU direct path addition is shown in Figure 8.xx.1-1.

Figure 8.xx.1-1: Signalling procedure of inter-DU direct path addition on top of indirect path

- The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the direct path to remote UE under a different gNB-DU (i.e., gNB-DU1).

3.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU1, which contains at least the direct path configuration.

4.	The gNB-DU1 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

5.	The gNB-CU sends an RRCReconfiguration message to the relay UE to update the indirect path configuration if necessary.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for remote UE by including the RRCReconfiguration message to the gNB-DU2. The contents in the RRCReconfiguration message may include at least direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).

7.	The gNB-DU2 sends the RRCReconfiguration message to the remote UE.

8.	The gNB-DU2 sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The remote UE performs random access procedure at the gNB-DU1.

10.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU2. In case of split SRB1 without duplication, remote UE sends RRCReconfigurationComplete message to the gNB-DU via primary path i.e., primary RLC entity located path.

11.	The gNB-DU2 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

    - 8.xx.2 Inter-DU indirect path addition on top of direct path

The signalling flow for inter-DU indirect path addition is shown in Figure 8.xx.2-1.

Figure 8.xx.2-1 Signalling procedure of inter-DU indirect path addition on top of direct path

1. The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under a different gNB-DU (i.e., gNB-DU2).

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU2 and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU2, which contains the indirect path configuration at least.

5.	The gNB-DU2 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU1. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s).

7.	gNB-DU1 sends the RRCReconfiguration message to the remote UE.

8.	The remote UE establishes PC5 connection with relay UE.

9.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. In case of split SRB1 without duplication, remote UE sends RRCReconfigurationComplete message to the gNB-DU via primary path i.e., primary RLC entity located path.

In case of (split) SRB1 is not configured in the indirect path and target Relay UE in RRC-IDLE/INACTIVE state, a PC5-RRC trigger sending from remote UE to target relay UE is needed to trigger relay UE entry RRC-CONNECTED state.

10.	The gNB-DU1 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

    - 8.xx.3	Intra-DU direct path addition on top of indirect path

The signaling flow for intra-DU direct path addition is shown in Fig. 8.xx.3-1.

Figure 8.xx.3-1: Signalling procedure of intra-DU direct path addition on top of indirect path

- The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the direct path to remote UE under the same gNB-DU.

3.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the remote UE to the gNB-DU, which contains at least the direct path configuration.

4.	The gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message.

5.	The gNB-CU sends an RRCReconfiguration message to the relay UE to update the indirect path configuration if necessary.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to the gNB-DU. The contents in the RRCReconfiguration message may include at least direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).

7.	The gNB-DU sends the RRCReconfiguration message to the remote UE.

8.	The remote UE performs random access procedure at the gNB-DU.

9.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. In case of split SRB1 without duplication, remote UE sends RRCReconfigurationComplete message to the gNB-DU via primary path i.e., primary RLC entity located path.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

    - 8.xx.4	Intra-DU indirect path addition on top of direct path

The signaling flow for intra-DU indirect path addition is shown in Fig. 8.xx.4-1.

Figure 8.xx.4-1: Signalling procedure of intra-DU indirect path addition on top of direct path

1. The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under the same gNB-DU.

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the remote UE to the gNB-DU, which contains the indirect path configuration at least.

5.	The gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s).

7.	gNB-DU sends the RRCReconfiguration message to the remote UE.

8.	The remote UE establishes PC5 connection with relay UE.

9.	The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. In case of split SRB1 without duplication, remote UE sends RRCReconfigurationComplete message to the gNB-DU via primary path i.e., primary RLC entity located path.

In case of (split) SRB1 is not configured in the indirect path and target Relay UE in RRC-IDLE/INACTIVE state, a PC5-RRC trigger sending from remote UE to target relay UE is needed to trigger relay UE entry RRC-CONNECTED state.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

- Annex2: TP for 38.473

8.3.3	UE Context Release (gNB-CU initiated)

8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

<<<Unchanged part is skipped>>>

If the Positioning Context Reservation Indication IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall not release the positioning context including the SRS configuration for the UE.

If the Path Release Information IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider it as a direct or indirect path release procedure for SL Relay Multi-path scenario.

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

9.2.2.5	UE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the UE-associated logical F1 connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | ignore |
| SRB ID | C- ifRRCContainer |  | 9.3.1.7 | The gNB-DU sends the RRC message on the indicated SRB. | YES | ignore |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Include it if RRCReestablishmentRequest is not accepted | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| Positioning Context Reservation Indication | O |  | ENUMERATED (True,...) |  | YES | ignore |
| CG-SDT Kept Indicator | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Path Release Information | O |  | 9.3.1.x |  | YES | ignore |

#### 9.3.1.x	Path Release Information

This IE provides information for path release.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Path Release Information | M |  |  |  |
| >Indirect Path Release |  |  |  |  |
| >> Relay UE ID list |  | 0 .. <maxnoofRelayUE> |  |  |
| >>> Relay UE ID | M |  | BIT STRING (SIZE(24)) | Corresponding to SL- targetRelayUEIdentity-r17, defined in TS 38.331 [8] |
| >Direct Path Release |  |  |  |  |
| >>PCell ID | M |  | NR CGI 9.3.1.12 |  |