# 3GPP TSG-RAN WG3 Meeting #121	R3-234117

# 21-25 August 2023, Toulouse, France

Agenda Item:	26.1

Source:	CATT

Title:	Discussion on RAN impact for NPN enhancement in Rel-18

Document for:	Discussion

1. Introduction

Agree to add Selected NID IE in the top level of the INITIAL UE MESSAGE message.

To be continued: inclusion of Selected NID in S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST in XnAP.

In this paper, we further analysis the detail impacts in RAN3 for above open issues.

2. Discussion

In legacy, SCG (re)selection at the SN is based on roaming and access restriction information (i.e., Mobility Restriction List) in SN.

TS 37.340:

11.1 Roaming and Access Restrictions The principles for conveying roaming and access restriction info for EN-DC are described in TS 36.300 [2]. For MR-DC with 5GC, SCG (re)selection at the SN is based on roaming and access restriction information in SN. If roaming and access restriction information is not available at the SN, the SN shall consider that there is no restriction for SCG (re)selection. Therefore, MN needs to convey the up-to-date roaming and access restriction information to SN via XnAP messages.

For PLMN case, SCG (re)selection at the SN is based on Mobility Restriction List in SN. Serving PLMN and equivalent PLMNs are included in MRL which means that SN can select PLMN based on serving PLMN and equivalent PLMNs. In our understanding, this principle is also applicable for SNPN case.

It is consider that the equivalent SNPNs IE is introduced in NPN Mobility Information IE contained in the Mobility Restriction List IE, SN can select NID among serving SNPN and equivalent SNPNs in Mobility Restriction List.

The equivalent SNPNs IE is introduced NPN Mobility Information IE contained in the Mobility Restriction List IE over XnAP and NGAP.

Observation 1: SN has enough information i.e., serving SNPN and equivalent SNPNs in Mobility Restriction List for NID selection.

However, in 38.423, the selected PLMN is introduced in S-NODE ADDITION REQUEST/ S-NODE MODIFICATION REQUEST but it states that the S-NG-RAN node may use it for RRM purposes. It is not clear what the meaning of RRM purpose but the fact is that MN can send the select PLMN to SN.

If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES |
| --- | --- | --- | --- | --- | --- |

Observation 2: Based on the specification, the MN is able to provide selected PLMN of SCG to SN.

For compromise in SNPN, we can assume that the MN provides the selected NID to SN for recommendation. SN is able to select a NID other than the selected NID provided by MN. It is similar as legacy handover that source node provides target cell to target node but target node can select other target cell for UE access.

Proposal 1: MN provides selected NID to SN for recommendation. SN is able to select other NID to serve UE.

If proposal 1 is agreeable, we think the description should also be updated. The “RRM purpose” is not very clear for us.

If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for the PLMN selection in SN.

Proposal 2: If proposal 1 is agreeable, the description of select PLMN should be updated. The S-NG-RAN node may use it for the PLMN selection in SN.

3. Conclusions

Based on the discussion in section 2 the followings are proposed:

Observation 1: SN has enough information i.e., serving SNPN and equivalent SNPNs in Mobility Restriction List for NID selection.

Observation 2: Based on the specification, the MN is able to provide selected PLMN of SCG to SN.

Proposal 1: MN provides selected NID to SN for recommendation. SN is able to select other NID to serve UE.

Proposal 2: If proposal 1 is agreeable, the description of select PLMN should be updated. The S-NG-RAN node may use it for the PLMN selection in SN.