3GPP TSG-RAN WG3 Meeting #121	R3-234133

Toulouse, France, 21 – 25 August, 2023

Agenda item:	10.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on signalling based logged MDT override protection

Document for:	Discussion and Decision

# 1	Introduction

The present paper provides a discussion on signalling based logged MDT override protection as per the following WID objective:

- Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

RAN3#119bis-e made the following agreement:

RAN3 confirms that the scenarios for inter-RAT signalling based logged MDT protection includes the following:

Scenario 1: Inter-system inter-RAT: EPC –> 5GC

Scenario 2: Intra-system Inter-RAT and intra-5GC: LTE –> NR

RAN3#120 further agreed:

There is no requirement that NR signalling based logged MDT has a lower priority than E-UTRAN signalling based logged MDT.

# 2	Discussion

The starting point for the present discussion is the agreement taken at last meeting that there is no requirement that NR signalling based logged MDT has a lower priority than E-UTRAN signalling based logged MDT. However this doesn’t preclude work in Rel-18 to enable operators e.g. to configure higher priority to E-UTRAN s-based logged MDT. This may be suitable e.g. in deployments where NG-RAN networks are well deployed and prioritized whenever available but E-UTRAN still is used for full coverage. Furthermore, we believe this need, originating from deployment aspects, is best handled by overall network configuration, e.g. per target NR cell or gNB, and will not need configuration per UE.

Proposal 1: Support configuration of priority to E-UTRAN s-based logged MDT per target NR cell or gNB.

A further question is then whether a correspondingly failed s-based NR MDT configuration needs to be signalled to the AMF by enhancement of error signalling. On our side we don’t see how the AMF could make use of this information, and the OAM will already be aware that the UE is configured with E-UTRAN s-based logged MDT and the configured priority situation.

Proposal 2: No need for error signalling/new cause on NGAP in case of failed activation of NR MDT.

We believe the above can be supported without further RAN3 impact based on RAN2#121’s agreement to further work on the following solution:

Solution 2: Override protection by cross-RAT signaling but no cross-RAT reporting of LTE logged MDT report from NR to LTE.

With solution details as follows:

1	Extend the LTE LoggedMeasurementConfiguration to include Logged MDT type indication information

2	NR signaling is needed to inform the gNB that signaling based MDT is configured by E-UTRA.

3	Try to reuse R17 NR signaling by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA

Based on this agreement, RAN3 can assume that the UE that is configured with EUTRA s-based logged MDT will indicate sigLogMeasConfigAvailable set to true in NR RRCSetupComplete and RRCResumeComplete messages. Because cross-RAT reporting upload is not supported, RAN3 could further assume that such UE will not set the Logged MDT data availability indicator while connecting to NR cell in NG-RAN.

In that case the gNB has the needed information to understand, depending on configured information as per proposal 1 above, that the ongoing EUTRA logged MDT configuration shall not be replaced by any new MDT logged configuration, neither s-based nor m-based logged MDT.

Proposal 3: If priority of E-UTRAN s-based logged MDT has been configured as per proposal 1: If the UE indicates sigLogMeasConfigAvailable set to true in NR RRCSetupComplete or RRCResumeComplete messages, while not indicating availability of Logged MDT data, the gNB shall not configure the UE with s-based or m-based logged MDT.

This will efficiently protect the EUTRAN-configured s-based logged MDT configuration, without any further standardization needed.

# 3	Conclusion

We have made the following proposals:

Proposal 1: Support configuration of priority to E-UTRAN s-based logged MDT per target NR cell or gNB.

Proposal 2: No need for error signalling/new cause on NGAP in case of failed activation of NR MDT.

Proposal 3: If priority of E-UTRAN s-based logged MDT has been configured as per proposal 1: If the UE indicates sigLogMeasConfigAvailable set to true in NR RRCSetupComplete or RRCResumeComplete messages, while not indicating availability of Logged MDT data, the gNB shall not configure the UE with s-based or m-based logged MDT.