3GPP TSG-RAN WG3 Meeting #121	R3-234135

Toulouse, France, 21 – 25 August, 2023

Agenda item:	24.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TS 38.473) Beam deactivation decision and signalling for energy saving

Document for:	Text Proposal

# 1	Introduction

It was agreed in RAN3#119bis-e that for inter-node beam activation, the F1AP GNB-CU CONFIGURATION UPDATE procedure shall be reused.

In this contribution, we look at the F1AP signalling options for the inter-node beam deactivation procedure.

# 2	Discussion

It was agreed for the Rel-17 Capacity and Coverage Optimization feature that the SSB beam deactivation is a gNB-DU level decision, and corresponding signalling was introduced to communicate this decision to the gNB-CU for actions like forwarding to a neighbouring gNB over XnAP. In Rel-17, SSB beam deactivation information is communicated by gNB-DU to gNB-CU over F1AP using the Coverage Modification Notification IE as part of the for Coverage and Capacity Optimization (CCO) functionality. The IE definition is as below.

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. |
| >>SSB Coverage Modification List |  | 0..1 |  |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. |

It is to be noted that the SSB Coverage State IE may be used to indicate if a given SSB beam is deactivated (value ‘0’).  This leads us to the possibility of reusing this IE for the purpose of inter-node beam deactivation for NES too.

However, RAN3 first needs to decide whether beam switch-off decision for Rel-18 energy saving is taken by the:

- Option 1: The DU - following legacy principles for CCO;

- Option 2: The CU - following legacy principles for cell switch-off.

The exact traffic pattern and energy consumption is known by the DU, and e.g. switch-off using gradual reduction of PSS/SSS power level by can trigger graceful handover to other frequency layers. We therefore expect that the energy saving potential is higher for a beam switch-off decision taken by the gNB-DU.

Proposal 1: Beam switch-off decision to be taken by the gNB-DU due to higher energy saving potential.

Following such beam switch-off decision, the corresponding F1 signalling could then use the F1 Coverage Modification Notification IE copied above, already conveyed by the gNB-DU Configuration Update message, or introduce a new IE with similar function in the same message, e.g. as follows:

| Cells with SSBs Deactivated List |  | 0.. <maxCellingNBDU> |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >NR CGI | M |  | 9.3.1.12 |  | - |  |
| >SSBs deactivated List |  | 1 .. < maxnoofSSBAreas > |  |  | EACH | ignore |
| >>SSB Index | O |  | INTEGER (0..63) | Identifier of the deactivated SSB beam | - |  |

Considering that an energy saving decision relates to both coverage and capacity it may not in practice be obvious for the implementation which IE to use if a new IE is introduced. Also, if separate algorithms are implemented in the gNBs for CCO and energy saving, race conditions between the two algorithms could occur. Our preference would therefore be to enhance the legacy IE with a cause value, e.g.:

| >>>>Coverage Modification Cause | O |  | ENUMERATED(CCO, Energy Saving, …) |  |
| --- | --- | --- | --- | --- |

It could also be considered to use the same cause values agreed in Rel-17 (coverage, cell edge capacity) for coverage modification signalling over Xn.

Proposal 2: Use legacy F1 Coverage Modification Notification IE to notify beam switch-off decision in the gNB-DU.

A corresponding TP to F1AP is provided in annex of this paper.

In the same way, it seems reasonable to indicate the beam switch-off decision using legacy coverage modification signalling over Xn, enhanced with energy saving cause value.

Proposal 3: Indicate the beam switch-off decision using legacy coverage modification signalling over Xn, enhanced with energy saving cause value

A corresponding TP to XnAP can be found in annex of [1] submitted to this meeting.

# 3	Conclusion

We have made the following proposals:

Proposal 1: Beam switch-off decision to be taken by the gNB-DU due to higher energy saving potential.

Proposal 2: Use legacy F1 Coverage Modification Notification IE to notify beam switch-off decision in the gNB-DU.

Proposal 3: Indicate the beam switch-off decision using legacy coverage modification signalling over Xn, enhanced with energy saving cause value

# References

[1]		R3-234134, (TP to TS 38.423) Support of network energy saving techniques, Nokia, Nokia Shanghai Bell

# Annex	- TP for BLCR to TS 38.473

<<< start of changes >>>

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. |
| >>SSB Coverage Modification List |  | 0..1 |  |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. |
| >>>>Coverage Modification Cause | O |  | ENUMERATED(CCO, Energy Saving, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<< end of changes >>>