3GPP TSG-RAN WG3 Meeting #121	R3-234154

Toulouse, France, 21 - 25 August, 2023

Agenda item:	25.2

Source: 	Huawei, Qualcomm Inc

Title: 	(TP for NR_XR_enh BL CRs) Discussion on the enhancement for XR awareness

Document for:	Discussion and Decision

1. Introduction

During the RAN#99 meeting, the WID objectives are determine for R18 XR. There are some enhancements related to RAN3, as quoted below.

Specify the enhancements related to capacity: - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

In this contribution, we analyse the RAN3 impacts for XR awareness. We give the proposals on RAN3 enhancements for supporting provision of the XR specific information that has been agreed by the other WGs. Some simple TPs are provided.

2. Discussion

  - PDU set QoS parameters

SA2 has agreed to introduce some new QoS parameters about PDU set for XR, including PSDB, PSER and PSIHI, see the quoted text from TS 23.501 [1].

### 5.7.7 PDU Set QoS Parameters #### 5.7.7.1 General PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. The followings are the PDU Set specific QoS characteristics: 1. PDU Set Delay Budget (PSDB). 2. PDU Set Error Rate (PSER). 3. PDU Set Integrated Handling Information (PSIHI).

For a QoS Flow supporting PDU Set based QoS handling, these new QoS parameters are included in the QoS profile and sent to NG-RAN nodes. NGAP, XnAP, F1AP and E1AP need to be enhanced to support this. The QoS Flow Level QoS parameters IE can be extended to include the PDU set QoS parameters. Possible modifications are provided in the annexed TPs.

Proposal 1: RAN3 to add the PDU set QoS parameters (PSDB, PSER and PSIHI) in NGAP, XnAP, F1AP and E1AP.

  - Dynamic PDU set information

Apart from the PDU set QoS parameters, SA2 and SA4 also defined some user plane information to support PDU set based handling. The specific information is described in TS 26.522 [2].

### 4.4.2 RTP Header Extension for PDU Set Marking #### 4.4.2.4 Semantics The semantics of the fields of the RTP Header Extension for the marking of PDU Set and End of Bursts are defined as follows: - End PDU of the PDU Set [E] (1 bit): This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set. - End of Data Burst [EDB] (3 bits): The EDB field is 3 bits in length and indicates the end of a Data Burst. The 3 bits encode the End of Data Burst indication as per the encoding and guidelines provided in Clause 4.4.2.6.1. - PDU Set Importance [PSI] (4 bits): The PDU Set Importance field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15. - PDU Set Sequence Number [PSSN] (10 bits): The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set. - PDU Sequence Number within a PDU Set [PSN] (6 bits): The sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender. - PDU Set Size [PSSize] (24 bits): The PDU Set Size indicates the total size of all PDUs of the PDU Set to which this PDU belongs. This field is optional and subject to an SDP signaling offer/answer negotiation, where the Application Server may indicate whether it will be able to provide the size of the PDU Set for that RTP stream. If not enabled, the field should not be present. If enabled, but the Application Server is not able to determine the PDU Size for a particular PDU Set, it should set the value to 0 in all PDUs of that PDU Set. The PSSize shall indicate the size of a PDU Set including RTP/UDP/IP header encapsulation overhead of its corresponding PDUs. The PSSize is expressed in bytes.

After receiving the N6 PDU with RTP header extension, the UPF is supposed to be able to provide the above information to NG-RAN via GTP-U header. At least NG-U header shall be enhanced to support new fields for the information.

Furthermore, in case of CU-DU split, considering the DU is responsible for scheduling, DU should be aware of the PDU set information when performing scheduling to guarantee the PDU set level QoS requirements. Thus, F1-U should also be enhanced. Similar for the DC split bearer case, the PDU set information is also needed by the Secondary Node, so Xn-U is impacted as well.

Proposal 2: RAN3 to add the following fields which are defined in RTP extension header to the GTP-U headers for the DL NG-U packets, F1-U packets and Xn-U packets:

- End PDU of the PDU set [E];

- End of Data Burst [EDB];

- PDU Set Importance [PSI];

- PDU Set Sequence Number [PSSN];

- PDU Sequence Number within a PDU Set [PSN];

- PDU Set Size [PSSize];

Besides, to help DU or SN identify the PDU sets, QFI is needed because the PDU set information is provided per QoS flow. The user plane protocol applied for Xn-U and F1-U in TS 38.425 should be enhanced to include QFI in the DL packets. Accordingly, TPs for TS38.415 and TS38.425 are provided in Annex part to show the possible changes.

Proposal 3: RAN3 to add QFI to the GTP-U extension header for the DL F1-U packets and Xn-U packets.

  - UL assistance information

During the past RAN2#122 meeting, it has been agreed that UE can report some XR-specific assistance information to RAN for UL, including the jitter range and BAT (Burst Arrival Time). The agreements are quoted below.

- 1. UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter. - 2: Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side. - 2. UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information. - 3 RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

According to the RAN2 agreements, the UL assistance information is going to be delivered utilizing the existing UAI framework.

Observation 1: RAN2 has agreed to add the XR traffic assistance information in UAI.

Currently, UAI is delivered over the Uu interface via RRC messages. In case of the gNB is CU-DU split, the XR related UL assistant information can be forwarded to the gNB-DU, and gNB-DU may consider the information for determining the DRX/CG configurations. And, in the UE HO case, the source gNB can provide such XR related UL assistant information to the target gNB. In F1 and Xn interface, the UAI already been included as RRC container, which is transparently forwarded. In F1AP, it is transferred by CU to DU RRC Information IE, and in XnAP by the RRC Context IE. The corresponding descriptions can be found in TS 38.473 [4] and TS 38.423 [5].

38.473: 9.3.1.25 CU to DU RRC Information This IE contains the RRC Information that are sent from gNB-CU to gNB-DU. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. >>>>>>Unrelated parts are skipped<<<<<<. UEAssistanceInformation; O; OCTET STRING; Includes the UEAssistanceInformation message, as defined in TS 38.331 [8].; YES; ignore. CG-Config; O; OCTET STRING; Includes the CG-Config message, as defined in TS 38.331 [8].; YES; ignore.

38.423: #### 9.1.1.1 HANDOVER REQUEST This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover. Direction: source NG-RAN node target NG-RAN node. >RRC Context; M; OCTET STRING; …, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB.; –. * Note: ueAssistanceInformation is included in HandoverPreparationInformation message, see TS 38.331 [6].

Observation 2: The UAI message is included as a container in F1AP and XnAP messages and will be transferred transparently over F1 and Xn interface.

With the above observations, we find that the XR-specific UAI enhancement has no impact on RAN3 specifications.

Proposal 4: For the UE provision of XR assistance information, no enhancement is needed from RAN3 point of view.

  - 5GC Assistance Information to NG-RAN for power saving via TSCAI

In TS 23.501, clause 5.37.8, SA2 captured following normative text.

#### 5.37.8.1	General

The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

-	Indication of End of Data Burst.

The UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity are provided by the CN to NG RAN via TSCAI.

SA2 has agreed to introduce the UL and/or DL Periodicity as well as the N6 jitter information in TSCAI for XR. And from the TS38.413, the UL and DL Periodicity already included in the TSCAI as quoted below.

9.3.1.130 TSC Traffic Characteristics This IE provides the traffic characteristics of TSC QoS flows. IE/Group Name; Presence; Range; IE type and reference; Semantics description. TSC Assistance Information Downlink; O; TSC Assistance Information 9.3.1.131. TSC Assistance Information Uplink; O; TSC Assistance Information 9.3.1.131. #### 9.3.1.131 TSC Assistance Information This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]). IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Periodicity; M; 9.3.1.132; -. Burst Arrival Time; O; 9.3.1.133; -. Survival Time; O; 9.3.1.221; YES; ignore.

To convey the information over NG, Xn and F1 interface, the TSC Assistance Information IE defined for NGAP, XnAP, F1AP and E1AP shall be extended to include the N6 Jitter Information field. We provide TPs for these modification.

Besides, RAN3 should further discuss about the detailed design for the new added filed. According to RAN1’s study in TR 38.838 [7], jitter is modelled as a Truncated Gaussian distribution with mean value of 0. Thus the jitter range can be expressed as a single parameter such as an integer, e.g., jitter = 4 can mean the jitter range is [-4, 4] ms. At least [-1, 1] ~ [-8, 8] ms should be considered according to RAN1’s study.

As for the granularity, since the jitter range information is used for power saving, mainly for DRX configuration, and note that drx-onDuration is configured in units of 1ms, so it may be sufficient to provide jitter with 1ms granularity. It is also possible to have a finer granularity such as 1us, the same as the periodicity information, so that the information is precise and can be more useful for scheduling.

Observation 3：SA2 conclusion requires that UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity are provided by the CN to NG RAN via TSCAI, and the existing signalling already allow the UL and/or DL Periodicity to be included in the TSCAI.

Proposal 5: RAN3 to add N6 DL jitter information to the TSC Assistance Information to enable 5GC/SMF providing power saving assistance information to NG-RAN for XR traffic. FFS on the details for the new added IEs.

  - Congestion information report

According to SA2 XRM conclusion, the congestion information report is necessary for two main purpose: ECN marking for L4S, and the congestion information exposure.

L4S is supposed to be supported by 5GS for application layer rate adaptation for XR. ECN marking for L4S will be enabled on a per QoS flow basis in the uplink and downlink direction. Either the NG-RAN or the UPF will be responsible to perform the ECN marking, as per described in clause 5.37.3 of TS 23.501 (quoted as below).

In addition, the network exposure of 5GS information also requires the NG-RAN to report the congestion information to the PSA UPF, as per described in clause 5.37.4 of TS 23.501 (also quoted as below).

### 5.37.3 Support of ECN marking for L4S to expose the congestion information In 5G System, ECN marking for L4S may be supported. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. ECN marking for the L4S in the IP header is supported in either the NG-RAN (see clause 5.37.3.2 and TS 38.300 [27]), or in the PSA UPF (see clause 5.37.3.3). NOTE 1: Whether NG-RAN or PSA UPF based ECN marking for L4S is used is decided by SMF based on operator's network configuration and policies. In the case of ECN marking for L4S by PSA UPF, the NG-RAN is instructed to perform congestion information monitoring. >>>>>>>>>>unrelated parts are skipped<<<<<<<<<< ### 5.37.4 Network Exposure of 5GS information 5GS and XR/media services cooperate to provide a better user experience using External Network Exposure. Based on the AF request, the 5GS can expose the following information based on the QoS Monitoring as defined in clause 5.33.3 and/or clause 5.45: - For the UL and/or DL congestion information monitoring (see clause 5.45.3), based on the PCC rule from PCF, the SMF requests the NG-RAN to report the information via GTP-U header to PSA UPF This NG-RAN reported information is common to support congestion information exposure and to support ECN marking for L4S in PSA UPF as described in clause 5.37.3.3. In the case of congestion information exposure, the PSA UPF exposes the UL and/or DL congestion information via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18. It can be applied to a Non-GBR or GBR QoS flow.

As clearly stated in TS23.501, NG-RAN may be asked to report the congestion information for UL or DL direction to UPF for either the network exposure or the UPF based ECN marking for L4S. The congestion information should be common for the two purpose, and it can be a congestion level. To align with SA2’s conclusion, the N3 GTP-U header extension will be used to carry such information. Considering the case of CU-DU split, it shall be supported to convey the congestion information from DU to CU, since DU hosts the scheduling procedures for the air interface and is much clear about the network congestion. Similarly, if the UE is dual connected, the Xn-U also need to be enhanced to allow the node providing air interface of MCG/SCG to report the congestion information to the node hosting the PDCP entity. More details on the design of congestion information, e.g., the granularity should be discussed by RAN3.

Proposal 6: The congestion information need to be carried in GTP-U header for UL N3-U, UL F1-U, and UL Xn-U packets. FFS on the granularity of the congestion information.

Another issue is about how to inform NG-RAN to perform the congestion information report or ECN marking for L4S.

TS 23.501: ### 5.45.3 Congestion information monitoring The NG-RAN may be required to provide the UL and/or DL QoS Flow congestion information (i.e. a percentage of congestion level for exposure). The UPF may be required to monitor the UL and/or DL QoS Flow congestion information reported from the NG-RAN. QoS monitoring request to the NG-RAN and NG-RAN reporting for UL and/or DL QoS Flow congestion information to PSA UPF is as defined in 5.37.3. The PSA UPF reports the received UL and/or DL QoS Flow congestion information to the target NF as instructed by the QoS Monitoring request from the SMF. Only one of ECN marking for L4S (in the case of ECN marking for L4S in RAN as described in clause 5.37.3) or QoS monitoring of congestion information may be requested to NG-RAN for a QoS Flow. They are mutually exclusive, therefore, measurements of Congestion information on a QoS Flow are not provided in QoS Monitoring reports if SMF enables ECN marking for L4S in RAN (see clause 5.37.3).

As agreed by SA2, whether NG-RAN or UPF based ECN marking is used is decided by SMF. So naturally, there should be an indication from SMF to NG-RAN to indicate that ECN marking for L4S or congestion information report shall be performed. It can be further discussed whether to have separate indications for ECN marking for L4S and congestion information report.

Proposal 7: RAN3 to discuss how to support the ECN marking for L4S indication or the congestion information report indication to NG-RAN.

3. Conclusion

In this contribution, we analyzed RAN3 impacts for XR awareness. The following observations and proposals were made:

PDU set QoS parameters:

Proposal 1: RAN3 to add the PDU set QoS parameters (PSDB, PSER and PSIHI) in NGAP, XnAP, F1AP and E1AP.

Dynamic PDU set information:

Proposal 2: RAN3 to add the following fields which are defined in RTP extension header to the GTP-U headers for the DL NG-U packets, F1-U packets and Xn-U packets:

- End PDU of the PDU set [E];

- End of Data Burst [EDB];

- PDU Set Importance [PSI];

- PDU Set Sequence Number [PSSN];

- PDU Sequence Number within a PDU Set [PSN];

- PDU Set Size [PSSize];

Proposal 3: RAN3 to add QFI to the GTP-U extension header for the DL F1-U packets and Xn-U packets.

UL assistance information:

Observation 1: RAN2 has agreed to add the XR traffic assistance information in UAI.

Observation 2: The UAI message is included as a container in F1AP and XnAP messages and will be transferred transparently over F1 and Xn interface.

Proposal 4: For the UE provision of XR assistance information, no enhancement is needed from RAN3 point of view.

Power saving assistance information via TSCAI:

Observation 3：SA2 conclusion requires that UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity are provided by the CN to NG RAN via TSCAI, and the existing signalling already allow the UL and/or DL Periodicity to be included in the TSCAI.

Proposal 5: RAN3 to add N6 DL jitter information to the TSC Assistance Information to enable 5GC/SMF providing power saving assistance information to NG-RAN for XR traffic. FFS on the details for the new added IEs.

Congestion information report:

Proposal 6: The congestion information need to be carried in GTP-U header for UL N3-U, UL F1-U, and UL Xn-U packets. FFS on the granularity of the congestion information.

Proposal 7: RAN3 to discuss how to support the ECN marking for L4S indication or the congestion information report indication to NG-RAN.

4. Reference

[1]. 3GPP TS 23.501, System architecture for the 5G System (5GS); Stage 2.

[2]. 3GPP TS 26.522, 5G Real-time Media Transport Protocol Configurations.

[3]. RAN2-122 Meeting Report.

[4]. 3GPP TS 38.473, NG-RAN; F1 application protocol (F1AP).

[5]. 3GPP TS 38.423, NG-RAN; Xn application protocol (XnAP).

[6]. 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification.

[7]. 3GPP TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

Annex A: TP for NR_XR_enh BL CR of TS 38.413

Start of Change

#### 9.3.1.12	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.18 |  | - |  |
| Allocation and Retention Priority | M |  | 9.3.1.19 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE shall be present for GBR QoS flows and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [9]. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1.. 1800, …) | Indicates the reporting frequency for RAN part delay for QoS monitoring. Units: second | YES | ignore |
| PDU set QoS Parameters | O |  | 9.3.1.x | Indicate the QoS Characteristics for a QoS flow which supports PDU set based QoS handling. | YES | ignore |

Next  Change

#### 9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  | - |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  | - |  |
| Survival Time | O |  | 9.3.1.221 |  | YES | ignore |
| N6 Jitter Information | O |  | 9.3.1.y | Indicates the jitter information associated with the Periodicity in downlink, as defined in TS 23.501[9]. | YES | ignore |

Next  Change

#### 9.3.1.x	PDU set QoS Parameters

This IE indicates the QoS Characteristics for a QoS flow which supports PDU set based QoS handling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.135 | Upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [9]. |
| PDU Set Error Rate | O |  | Packet Error Rate 9.3.1.81 | Upper bound for a rate of non-congestion related PDU Set losses, as defined in TS 23.501 [9]. |
| PDU Set Integrated Handling Information | O |  | ENUMERATED(true, false, …) | Indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side, as defined in TS 23.501 [9]. |

#### 9.3.1.y N6 Jitter Information

This IE indicates the jitter information associated with the Periodicity in downlink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| N6 Jitter Information | O |  | FFS | FFS |

End of Change

Annex B: TP for NR_XR_enh BL CR of TS 38.473

Start of Change

#### 9.3.1.45	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow, or to a DRB, or to a BH RLC channel, or to a Uu Relay RLC channel, or to a PC5 Relay RLC channel.

NOTE:	For a BH RLC channel, the listed mandatory IEs and the GBR QoS Flow Information IE are applicable, where GBR QoS Flow Information IE may be present if BH RLC channel conveys the traffic belonging to a GBR QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.49 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.47 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.48 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.46 | This IE shall be present for GBR QoS Flows only and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Details in TS 23.501 [21]. This IE applies to non-GBR flows only and is ignored otherwise. | - |  |
| PDU Session ID | O |  | INTEGER (0 ..255) | As specified in TS 23.501 [21]. | YES | ignore |
| UL PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.22 | The PDU session Aggregate Maximum Bit Rate Uplink which is associated with the involved PDU session. | YES | ignore |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| PDCP Terminating Node DL Transport Layer Address | O |  | Transport Layer Address 9.3.2.3 | DL Transport Layer Address of node terminating PDCP. Included for MN-terminated SCG bearers and SN-terminated MCG bearers. | YES | ignore |
| PDU set QoS Parameters | O |  | 9.3.1.x | Indicate the QoS Characteristics for a QoS flow which supports PDU set based QoS handling. | YES | ignore |

Next  Change

#### 9.3.1.142	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [21]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.143 | Periodicity as specified in TS 23.501 [21]. | - |  |
| Burst Arrival Time | O |  | 9.3.1.144 | Burst Arrival Time as specified in TS 23.501 [21]. | - |  |
| Survival Time | O |  | 9.3.1.231 |  | YES | ignore |
| N6 Jitter Information | O |  | 9.3.1.y | Indicates the jitter information associated with the Periodicity in downlink, as defined in TS 23.501[21]. | YES | ignore |

Next  Change

#### 9.3.1.x	PDU set QoS Parameters

This IE indicates the QoS Characteristics for a QoS flow which supports PDU set based QoS handling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.145 | Upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [21]. |
| PDU Set Error Rate | O |  | Packet Error Rate 9.3.1.52 | Upper bound for a rate of non-congestion related PDU Set losses, as defined in TS 23.501 [21]. |
| PDU Set Integrated Handling Information | O |  | ENUMERATED(true, false, …) | Indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side, as defined in TS 23.501 [21]. |

#### 9.3.1.y	N6 Jitter Information

This IE indicates the jitter information associated with the Periodicity in downlink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| N6 Jitter Information | O |  | FFS | FFS |

End of Change

Annex C: TP for NR_XR_enh BL CR of TS 38.423

Start of Change

#### 9.2.3.5	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | – |  |
| >Non Dynamic 5QI |  |  |  |  |  |  |
| >>Non dynamic 5QI Descriptor | M |  | 9.2.3.8 |  | – |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.2.3.9 |  | – |  |
| Allocation and Retention Priority | M |  | 9.2.3.7 |  | – |  |
| GBR QoS Flow Information | O |  | 9.2.3.6 | This IE shall be present for GBR QoS flows and is ignored otherwise. | – |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Reflective QoS is specified in TS 23.501 [7]. This IE applies to Non-GBR bearers only and is ignored otherwise. | – |  |
| Additional QoS flow Information | O |  | ENUMERATED (more likely, …) | If this IE is set to "more likely", this indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR flows only and is ignored otherwise. | – |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1.. 1800, …) | Indicates the Reporting Frequency for RAN part delay for Qos monitoring. Unit: second | YES | ignore |
| QoS Monitoring Disabled | O |  | ENUMERATED(true, ...) | Indicates to stop the QoS monitoring. | YES | ignore |
| PDU set QoS Parameters | O |  | 9.2.3.x | Indicate the QoS Characteristics for a QoS flow which supports PDU set based QoS handling. | YES | ignore |

Next  Change

#### 9.2.3.115	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [7]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.2.3.116 | Periodicity as specified in TS 23.501 [7]. | – |  |
| Burst Arrival Time | O |  | 9.2.3.117 | Burst Arrival Time as specified in TS 23.501 [7]. | – |  |
| Survival Time | O |  | 9.2.3.152 |  | YES | ignore |
| N6 Jitter Information | O |  | 9.2.3.y | Indicates the jitter information associated with the Periodicity in downlink, as defined in TS 23.501[7]. | YES | ignore |

Next  Change

#### 9.2.3.x	PDU set QoS Parameters

This IE indicates the QoS Characteristics for a QoS flow which supports PDU set based QoS handling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Delay Budget | O |  | Extended Packet Delay Budget 9.2.3.113 | Upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [7]. |
| PDU Set Error Rate | O |  | Packet Error Rate 9.2.3.13 | Upper bound for a rate of non-congestion related PDU Set losses, as defined in TS 23.501 [7]. |
| PDU Set Integrated Handling Information | O |  | ENUMERATED(true, false, …) | Indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side, as defined in TS 23.501 [7]. |

#### 9.2.3.y	N6 Jitter Information

This IE indicates the jitter information associated with the Periodicity in downlink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| N6 Jitter Information | O |  | FFS | FFS |

End of Change

Annex D: TP for NR_XR_enh BL CR of TS 37.483

Start of Change

#### 9.3.1.26	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.27 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.29 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.30 | This IE shall be present for GBR QoS Flows and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [20]. This IE applies to Non-GBR flows only and is ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU Session. | - |  |
| Paging Priority Index | O |  | INTEGER (1.. 8, …) | This IE is not used in this version of the specification. | - |  |
| RDI | O |  | ENUMERATED (enabled, …) | Indicates whether Reflective QoS flow to DRB mapping should be applied. | - |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow. | YES | ignore |
| MCG Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.3.1.30 | This IE contains M-Node offered GBR QoS Flow Information. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1..1800, …) | Indicates the Reporting Frequency for RAN part delay for Qos monitoring. Units: second | YES | ignore |
| QoS Monitoring Disabled | O |  | ENUMERATED (true, ...) | Indicates to stop the QoS monitoring. | YES | ignore |
| Data Forwarding Source IP Address | O |  | Transport Layer Address 9.3.2.4 | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| PDU set QoS Parameters | O |  | 9.3.1.x | Indicate the QoS Characteristics for a QoS flow which supports PDU set based QoS handling. | YES | ignore |

Next  Change

#### 9.3.1.76	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [20]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.77 |  | - |  |
| Burst Arrival Time | O |  | 9.3.1.78 |  | - |  |
| Survival Time | O |  | 9.3.1.103 |  | YES | ignore |
| N6 Jitter Information | O |  | 9.3.1.y | Indicates the jitter information associated with the Periodicity in downlink, as defined in TS 23.501[20]. | YES | ignore |

Next  Change

#### 9.3.1.x	PDU set QoS Parameters

This IE indicates the QoS Characteristics for a QoS flow which supports PDU set based QoS handling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.79 | Upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [20]. |
| PDU Set Error Rate | O |  | Packet Error Rate 9.3.1.48 | Upper bound for a rate of non-congestion related PDU Set losses, as defined in TS 23.501 [20]. |
| PDU Set Integrated Handling Information | O |  | ENUMERATED(true, false, …) | Indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side, as defined in TS 23.501 [20]. |

#### 9.3.1.y	N6 Jitter Information

This IE indicates the jitter information associated with the Periodicity in downlink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| N6 Jitter Information | O |  | FFS | FFS |

End of Change

Annex E: TP for NR_XR_enh BL CR of TS 38.415

Start of Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NextGen Radio Access Network (NG-RAN); Overall description; Stage 2".

[3]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[4]	3GPP TS 37.324: "E-UTRA and NR; Service Data Application Protocol (SDAP) specification".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[7]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[X] 	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations ".

Next Change

#### 5.5.2.1	DL PDU SESSION INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective DL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 |  | 4 | 3 |  |  | 2 |  | 1 | 0 |  |  |
| PDU Type (=0) |  |  |  |  | QMP |  |  | SNP |  | MSNP |  | Spare | 1 |
| PPP | RQI | QoS Flow Identifier |  |  |  |  |  |  |  |  |  |  | 1 |
| PPI |  |  |  | Spare |  |  | PDU Set Flag |  | EDB Flag |  | PSS Flag |  | 0 or 1 |
| DL Sending Time Stamp |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL QFI Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL MBS QFI Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| End PDU of the PDU Set | End of Data Burst |  |  |  |  | PDU Set Importance |  |  |  |  |  |  | 0 or 1 |
| PDU Set Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| PDU Set Sequence Number |  |  | PDU Sequence Number within a PDU Set |  |  |  |  |  |  |  |  |  |  |
| PDU Set Size |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL PDU SESSION INFORMATION (PDU Type 0) Format

Next  Change

#### 5.5.3.x1	PDU Set Flag

Description: This parameter indicates the presence of the PDU set parameters The PDU set parameters includes the End PDU of the PDU Set Number, PDU Set Importance, PDU Set Sequence Number, and PDU Sequence Number within a PDU Set.

Value range: {0= PDU set parameters not present, 1= PDU set parameters present}.

Field length: 1 bit.

#### 5.5.3.x2	EDB Flag

Description: This parameter indicates the presence of the End of Data Burst.

Value range: {0= End of Data Burst not present, 1= End of Data Burst present}.

Field length: 1 bit.

#### 5.5.3.x3	PSS Flag

Description: This parameter indicates the presence of the PDU Set Size.

Value range: {0= PDU Set Size not present, 1= PDU Set Size present}.

Field length: 1 bit.

#### 5.5.3.x4	End PDU of the PDU Set

Description: This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set, as specified in TS 26.522[X].

Value range: {0= not the last PDU of the PSU Set, 1= the last PDU of the PDU Set}.

Filed length: 1 bit.

#### 5.5.3.x5	End of Data Burst

Description: This field indicates the end of a Data Burst. The 3 bits encode the End of Data Burst indication as per the encoding and guidelines provided in TS 26.522[X].

Value range: FFS.

Filed length: 3 bits.

#### 5.5.3.x6	PDU Set Importance

Description: This field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow as specified in TS 26.522[X]. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15.

Value range: {0..15}.

Filed length: 4 bits.

#### 5.5.3.x7	PDU Set Sequence Number

Description: The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set, as specified in TS 26.522[X].

Value range: {0..210-1}.

Filed length: 10 bits.

#### 5.5.3.x8	PDU Sequence Number within a PDU Set

Description: The sequence number of the current PDU within the PDU Set, as specified in TS 26.522[X]. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..26-1}.

Filed length: 6 bits.

#### 5.5.3.x9	PDU Set Size

Description: The filed indicates the total size of all PDUs of the PDU Set to which this PDU belongs, as specified in TS 26.522[X].

Value range: {0..224-1}.

Filed length: 3 octets.

End of Change

Annex F: TP for NR_XR_enh BL CR of TS 38.425

Start of Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[3]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[4]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[5]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[6]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 23.501: "System Architecture for the 5G System".

[9]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description".

[X] 	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations ".

Next Change

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU.

The following shows the respective DL USER DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

| Bits |  |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 |  | 5 |  |  | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  |  |  |  | PDU Set Flag |  | DL Discard Blocks | DL Flush | Report polling | 1 |
| Spare | EDB Flag |  |  | PSS Flag |  | Request OutofSeq Report | Report Delivered |  | User data existence flag | Assistance Info. Report Polling Flag | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Spare |  |  |  |  | QoS Flow Identifier |  |  |  |  |  |  | 0 or 1 |
| End PDU of the PDU Set |  | End of Data Burst |  |  |  |  |  | PDU Set Importance |  |  |  | 0 or 1 |
| PDU Set Sequence Number |  |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| PDU Set Sequence Number |  |  |  |  | PDU Sequence Number within a PDU Set |  |  |  |  |  |  |  |
| PDU Set Size |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

Next Change

#### 5.5.3.x1	PDU Set Flag

Description: This parameter indicates the presence of the PDU set parameters. The PDU set parameters includes the QoS Flow Identifier, End PDU of the PDU Set, PDU Set Importance, PDU Set Sequence Number, and PDU Sequence Number within a PDU Set.

Value range: {0= PDU set parameters not present, 1= PDU set parameters present}.

Field length: 1 bit.

#### 5.5.3.x2	EDB Flag

Description: This parameter indicates the presence of the End of Data Burst.

Value range: {0= End of Data Burst not present, 1= End of Data Burst present}.

Field length: 1 bit.

#### 5.5.3.x3	PSS Flag

Description: This parameter indicates the presence of the PDU Set Size.

Value range: {0= PDU Set Size not present, 1= PDU Set Size present}.

Field length: 1 bit.

#### 5.5.3.x4	End PDU of the PDU Set

Description: This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set, as specified in TS 26.522[X].

Value range: {0= not the last PDU of the PSU Set, 1= the last PDU of the PDU Set}.

Filed length: 1 bit.

#### 5.5.3.x5	End of Data Burst

Description: This field indicates the end of a Data Burst. The 3 bits encode the End of Data Burst indication as per the encoding and guidelines provided in TS 26.522[X].

Value range: FFS.

Filed length: 3 bits.

#### 5.5.3.x6	PDU Set Importance

Description: This field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow as specified in TS 26.522[X]. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15.

Value range: {0..15}.

Filed length: 4 bits.

#### 5.5.3.x7	PDU Set Sequence Number

Description: The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set, as specified in TS 26.522[X].

Value range: {0..210-1}.

Filed length: 10 bits.

#### 5.5.3.x8	PDU Sequence Number within a PDU Set

Description: The sequence number of the current PDU within the PDU Set, as specified in TS 26.522[X]. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..26-1}.

Filed length: 6 bits.

#### 5.5.3.x9	PDU Set Size

Description: The filed indicates the total size of all PDUs of the PDU Set to which this PDU belongs, as specified in TS 26.522[X].

Value range: {0..224-1}.

Filed length: 3 octets.

#### 5.5.3.x10	QoS Flow Identifier (QFI)

Description: When present this parameter indicates the QoS Flow Identifier of the QoS flow to which the transferred packet belongs.

Value range: {0..26-1}.

Field length: 6 bits.

End of Change