3GPP TSG-RAN WG3 Meeting #121	R3-234155

Toulouse, France, 21 - 25 August, 2023

Agenda item:	25.2

Source: 	Huawei, Qualcomm Inc.

Title: 	Discussion on incoming LS from other WGs

Document for:	Discussion and Decision

- 1. Introduction

In this meeting, RAN3 received some LS in regarding to the XR from SA2. Some may requires discussion and/or impact in RAN3. We provide some discussion on some LSs which we think worth to be discussed by RAN3 WG.

- 2. Discussion

  - Non-homogeneous support of PDU Set based handling in NG-RAN

SA2 has sent an LS in S2-2308252 [1] to RAN3 to trigger the discussion on non-homogeneous deployments for supporting PDU Set based handling. The content is quoted below.

1. Overall Description: SA2 has discussed the issue of non-homogeneous deployments of NG-RAN nodes supporting PDU Set based handling and agreed the attached CR vs. TS 23.501. 2. Actions: To RAN3/CT4: ACTION: SA2 kindly asks RAN3/CT4 to take the above into account in their work and provide feedback as deemed necessary.

In this contribution, we will analyse the potential RAN3 impacts to support the non-homogeneous deployments and give our view.

SA2 raised that there are scenarios where NG-RAN nodes are deployed non-homogeneously from the perspective of whether support PDU Set based handling, since it is optional for NG-RAN nodes to support PDU Set based QoS guarantee or not for XR traffics. SA2 described the scenario in the CR S2-2308246 [2] submitted to SA2#156 meeting. We copy the following text from the CR.

#### 5.37.5.x Non-homogenous support of PDU set based handling in NG-RAN By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [xx]. Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking. In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set.

From the above CR, we can see that SMF would like to know whether PDU Set based handling is supported by the serving NG-RAN node, so that it can configure UPF to activate/deactivate for PDU Set identification and marking. This will save the processing overhead in UPF and the N3 GTP-U header overhead when PDU Set based handling is not supported.

According to SA2’s CR, NG-RAN is expected to provide an indication to SMF upon receiving PDU Set QoS parameters from CN or from the source NG-RAN in case of handover. The indication reveals whether the NG-RAN node regarding whether PDU Set based handling is supported.

Observation 1: SMF needs to know whether the NG-RAN node supports PDU Set based handling.

To let SMF know whether the NG-RAN node supports PDU Set based handling, an indication from NG-RAN to SMF can be used, as mentioned by SA2. The indication can be explicit indication or implicit indication (e.g. by reject the PDU set related QoS parameters, with criticality “reject”).

Proposal 1: For non-homogeneous support of PDU Set based handling in NG-RAN, RAN3 to specify explicit/implicit indicator from NG-RAN to SMF to indicate whether the NG-RAN node supports PDU Set based handling.

  - Design of RTP Header Extension for PDU Set handling

SA2 has sent an LS in S2-2308248 [3] to RAN3, which replies SA4’s question on the design of RTP Header extension for PDU Set handling.

The first question asked by SA4 is about the possibility to include both PDUs marked with PDU set RTP header extension and unmarked PDUs within a single QoS flow. We quoted SA2’s reply as following.

Question#1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs? Reply#1: …Mixing of N6 packets that are marked with the new RTP header extension with N6 packets that are not marked with the new RTP header extension in the same QoS Flow can happen as long as the information for QoS Flow matching (refer to the Packet Detection Information in the Packet Detection Rule in TS 23.501 Table 5.8.5.3-1) is the same for both. Furthermore, SA2 would like to bring to the SA4’s attention that SA2 agreed that a single QoS Flow includes either PDUs with GTP-U header extensions for PDU Sets or PDUs without such extension, but not both. If the UPF receives a PDU that do not belong to a PDU Set based on Protocol Description for PDU Set identification, then the UPF still maps it to a PDU Set. The details are described in the attached CR.

It can be seen the N6 packets which are marked with the new RTP header extension for PDU Set based handling and the other N6 packets which are not marked with the new RTP header extension may be mapped to the same QoS flow at UPF. However, the difference among the packets would be eliminated on N3 interface. For a N6 packet without RTP layer PDU Set header extension, UPF will still map it to a PDU set based on its own implementation and add the PDU set information in GTP-U header extension. So, for one QoS flow in NG-U, there is no mix up of packets with and without fields for PDU set information in GTP-U header extension. Consequently, we see there is  no RAN3 impact.

Observation 2: There is no RAN3 impact regarding the mix up of PDUs with and without PDU set RTP header extension.

The second question asked by SA4 is about the information to help UPF decide which PDUs belong to a data burst. The question and reply are quoted as well.

Question#2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension? Reply#2: Yes. Furthermore, as stated in the previous LS to SA4 (S2-2303849), for Rel-18 SA2 kindly asked SA4 to provide guidelines on End of Data Burst Indication marked by the application server via RTP header information (e.g., the timestamp, sequence number and "M" bit) or define the 3GPP specific RTP header extension if the current RTP header cannot provide such information to determine End of Data Burst Indication.

From SA2’s reply, we can see that End of Data Burst Indication is required by UPF. SA2 expect some helpful information can be provided via the RTP header extension, such as a specific End of Data Burst Indication. This is not new and it was already agreed and captured in TS 26.522 [4], see the following quoted text.

### 4.4.2 RTP Header Extension for PDU Set Marking #### 4.4.2.4 Semantics The semantics of the fields of the RTP Header Extension for the marking of PDU Set and End of Bursts are defined as follows: … - End of Data Burst [EDB] (3 bits): The EDB field is 3 bits in length and indicates the end of a Data Burst. The 3 bits encode the End of Data Burst indication as per the encoding and guidelines provided in Clause 4.4.2.6.1.

The same field shall be added to N3 GTP-U header. It has been discussed in our correlated contribution [3]. Since there is no new information introduced so far, we see no additional impact on RAN3 from this LS.

Observation 3: End of Data Burst Indication is already agreed. No new impact on RAN3 for identifying data bursts.

With the above observations, we think no action is needed for the LS from RAN3 perspective.

Proposal 2: No action is needed for RAN3 regarding the LS about RTP header extension.

  - LS on TSCAI for XR

SA2 has sent an LS in S2-2308197 [5] to RAN3, which replies RAN2’s question on the TSCAI. The core content is copied below:

Question: RAN2 has discussed the provision of assistance information from CN to RAN for XR services and is wondering whether TSCAI can be provided for both GBR and non-GBR QoS flows in case of XR.

SA2 Answer: for either GBR or non-GBR QoS flow, TSCAI can be provided to the RAN.

Question: RAN2 has discussed the provision of assistance information from CN to RAN for XR services and is wondering whether TSCAI can be provided for both GBR and non-GBR QoS flows in case of XR.

SA2 Answer: for either GBR or non-GBR QoS flow, TSCAI can be provided to the RAN.

However, in the current NGAP specification, TSCAI is provided as part of the TSC Traffic Characteristics IE, which can be included in the PDU Session Resource Setup Request Transfer IE or the PDU Session Resource Modify Request Transfer IE.

As described in TS 38.413, the usage of the TSC Traffic Characteristics IE is restricted to only GBR QoS flows. It will be ignored by the NG-RAN for the case of non-GBR QoS flows if present. The description is copied as following.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

To align with SA2 feedback, such restriction shall be removed for XR since Rel-18. A corresponding TP is provided in the Annex part.

Proposal 3: Remove the semantics description for TSC Traffic Characteristics IE in TS38.413.

- 3. Conclusion

In this contribution, we analyzed RAN3 impacts for some SA2 Liaisons, the following observation and proposal were made:

- Supporting non-homogeneous deployment for PDU Set based handling.

Observation 1: SMF needs to know whether the NG-RAN node supports PDU Set based handling.

Proposal 1: For non-homogeneous support of PDU Set based handling in NG-RAN, RAN3 to specify explicit/implicit indicator from NG-RAN to SMF to indicate whether the NG-RAN node supports PDU Set based handling.

- Design of RTP Header Extension for PDU Set handling

Observation 2: There is no RAN3 impact regarding the mix up of PDUs with and without PDU set RTP header extension.

Observation 3: End of Data Burst Indication is already agreed. No new impact on RAN3 for identifying data bursts.

Proposal 2: No action is needed for RAN3 regarding the LS about RTP header extension.

- TSCAI for XR

Proposal 3: Remove the semantics description for TSC Traffic Characteristics IE in TS38.413.

- 4. Reference

[1]. S2-2308252, Non-homogeneous deployment of PDU Set based handling.

[2]. S2-2308246, PDU Set QoS handling: non-homogenous support (Alternative 2).

[3]. S2-2308248, LS Reply on Design of RTP Header Extension for PDU Set handling.

[4]. 3GPP TS 26.522, 5G Real-time Media Transport Protocol Configurations.

[5]. S2-2308197, LS reply on TSCAI for XR.

- Annex: TP for NR_XR_enh BL CR of TS 38.413

CHANGE STARTS

#### 9.3.4.1	PDU Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 | This IE shall be present when at least one Non-GBR QoS flow is being setup and is ignored otherwise. | YES | reject |
| UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. | YES | reject |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of UL PDUs for split PDU session. | YES | reject |
| Data Forwarding Not Possible | O |  | 9.3.1.63 | This IE may be present in case of HANDOVER REQUEST message and is ignored otherwise. | YES | reject |
| PDU Session Type | M |  | 9.3.1.52 |  | YES | reject |
| Security Indication | O |  | 9.3.1.27 |  | YES | reject |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Setup Request List |  | 1 |  |  | YES | reject |
| >QoS Flow Setup Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 |  | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| Common Network Instance | O |  | 9.3.1.120 |  | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.64 | This IE may be present in case of inter-system handover and intra-system handover. | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission. | YES | ignore |
| Additional Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of redundant UL PDUs for split PDU session. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Redundant PDU Session Information | O |  | 9.3.1.136 |  | YES | ignore |
| MBS Session Setup Request List | O |  | 9.3.1.211 |  | YES | ignore |

NEXT CHANGE

#### 9.3.4.3	PDU Session Resource Modify Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 |  | YES | reject |
| UL NG-U UP TNL Modify List |  | 0..1 |  |  | YES | reject |
| >UL NG-U UP TNL Modify Item |  | 1..<maxnoofMultiConnectivity> |  | This IE(s) are included only for modification of an existing tunnel. | - |  |
| >>UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. | - |  |
| >>DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the NG-RAN node. | - |  |
| >>Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission. | YES | ignore |
| >>Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the NG-RAN node for the redundant transmission. | YES | ignore |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Add or Modify Request List |  | 0..1 |  |  | YES | reject |
| >QoS Flow Add or Modify Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | O |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 |  | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| QoS Flow to Release List | O |  | QoS Flow List with Cause 9.3.1.13 |  | YES | reject |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of UL PDUs for split PDU session. | YES | reject |
| Common Network Instance | O |  | 9.3.1.120 |  | YES | ignore |
| Additional Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of redundant UL PDUs for split PDU session. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission of the Redundant QoS Flow(s). | YES | ignore |
| Security Indication | O |  | 9.3.1.27 |  | YES | ignore |
| MBS Session Setup or Modify Request List | O |  | 9.3.1.212 |  | YES | ignore |
| MBS Session To Release List | O |  | 9.3.1.215 |  | YES | ignore |

CHANGE ENDS