3GPP TSG-RAN WG3 Meeting #121	R3-234170

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	13.3

Source: 	Lenovo

Title: 	Mobility enhancements for mobile IAB-node and its served UE

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, enhancement for mobile IAB-node mobility is discussed, including aspects related to mobile IAB-node indication and configuration for IAB-DU’s parameters.

- Discussion

mIAB-node indication/mIAB-node authorized in HO request

RAN3 119bis-e: To be continued: Whether explicit mIAB-node indication and/or explicit mIAB-node-authorized indication needs to be included in the HO request for the mIAB-MT, e.g., so that the target CU can perform admission control.

Based on the current specification for R16 IAB-node as follow, explicit IAB Node Indication and IAB Authorized IE are included in the HANDOVER REQUEST in XnAP and NGAP separately.

TS 38.423: If the IAB Node Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node. In addition: TS 38.413: If the IAB Authorized IE is contained in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, consider that the handover is for an IAB node.

Similar to design of R16, for mobile IAB, an explicit mIAB Node Indication is contained in the XnAP HANDOVER REQUEST message from source IAB-donor to target IAB-donor, and an explicit mIAB Authorized IE is contained in the NGAP HANDOVER REQUEST message from AMF to target IAB-donor. In addition, the mIAB Node Indication may be used to indicate the mIAB-node is authorized.

Proposal 1: An explicit mIAB Node Indication is contained in the XnAP HANDOVER REQUEST message from source IAB-donor to target IAB-donor.

Proposal 2: An explicit mIAB Authorized IE is contained in the NGAP HANDOVER REQUEST message from AMF to target IAB-donor.

Configuration for IAB-DU’s parameters

RAN3 117-e: RAN3 to discuss whether a mobile IAB node may be configured with multiple configurations, each corresponding to a different target donor, that can be activated upon fulfillment of certain condition(s). The details of the configurations are FFS. RAN3 117bis-e: RAN3 to discuss which of the OAM-configured and network-configured parameters may be pre-configured at a mobile IAB-node, after a baseline procedure for IAB-DU migration is developed.

Pre-configuration at the mobile IAB-node may have some advantages to reduce the latency during IAB-DU migration or full migration, especially for the case of regular trajectory or predicable trajectory of mobile IAB-node. In addition, only some relative-static paraments could be considered to be pre-configured at the mobile IAB-node, otherwise, it’s meaningless and the pre-configured parameters may be already outdated by the time they are used which led to extra overhead. As an example, at least the TNL address(es) of IAB-DU and target IAB-donor-CU, and NCGI of IAB-DU can be pre-stored at the mobile IAB-node to accelerate the F1 setup between second logical IAB-DU and target IAB-donor-CU.

Proposal 3: The TNL address(es) of IAB-DU and target IAB-donor-CU, and NCGI of IAB-DU can be pre-configured at the mobile IAB-node in case of determined trajectory.

Fixed IAB-node handover

RAN2 119-e: The method of not broadcasting “iab-Support” indication, is sufficient to prevent other IAB-node from accessing mobile IAB (without further spec impact). RAN2 121bis-e: R2 assumes that a mobile IAB node is not required to receive the system information of neighbour cells for reporting of measurements (i.e. it will not refrain from reporting measurements of cells that are not broadcasting the “mobile iab Support” indication, and this is acc to current R2 TS).

For the mobile IAB-node, it should have no descendent IAB-nodes and only serve for UEs as defined in the WID. In RAN2 119 e-meeting, RAN2 has agreed that the method of not broadcasting “iab-Support” indication is sufficient to prevent other IAB-node from accessing mobile IAB-node. However, the “iab-Support” indication is only applied for IAB-node integration procedure, and in last RAN2 121bis e-meeting, RAN2 has assumed not to refrain reporting measurements of the mobile IAB cells. So, it needs to avoid that a source IAB-donor-CU to configure measurement to an IAB-node with a potential cell belonging to a mobile IAB-node, and the source IAB-donor-CU to trigger handover of the IAB-node to the mobile IAB-node cell. Therefore, the source IAB-donor-CU of a fixed IAB-node should be aware of whether the potential target cell belongs to a mobile IAB-node via XnAP message.

Proposal 4: Source IAB-donor-CU of a fixed IAB-node should be aware of whether potential target cell belongs to a mobile IAB-node via Xn message.

Supporting mobile IAB-node indication

Because legacy R16/R17 IAB-donors or parent IAB-nodes may not support the functionalities and characters that will be introduced by the R18 mobile IAB, it’s not a good choice for the R18 mobile IAB-node to always camp on a legacy R16/R17 IAB-donor or parent IAB-node. Therefore, to assist the mobile IAB-node access to a suitable IAB-donor or parent IAB-node, a “mobile-IAB-Support” IE can be introduced in SIB1 just as the “iab-Support” IE in R16. And in RAN2 meeting, RAN2 assumes "supporting mobile-IAB" indication is provided by Rel-18 Mobile IAB capable parent cell. Therefore, IAB-donor-CU needs to configure whether each cell of fixed parent node IAB-DU or IAB-donor-DU is mobile-IAB-supported or not.

Proposal 5: IAB-donor-CU configures whether each cell of fixed parent node IAB-DU or IAB-donor-DU is mobile-IAB-supported.

# Conclusion

This contribution aims to discuss enhancements for mobility of an IAB-node together with its served UEs. And following observations and proposals are concluded.

Proposal 1: An explicit mIAB Node Indication is contained in the XnAP HANDOVER REQUEST message from source IAB-donor to target IAB-donor.

Proposal 2: An explicit mIAB Authorized IE is contained in the NGAP HANDOVER REQUEST message from AMF to target IAB-donor.

Proposal 3: The TNL address(es) of IAB-DU and target IAB-donor-CU, and NCGI of IAB-DU can be pre-configured at the mobile IAB-node in case of determined trajectory.

Proposal 4: Source IAB-donor-CU of a fixed IAB-node should be aware of whether potential target cell belongs to a mobile IAB-node via Xn message.

Proposal 5: IAB-donor-CU configures whether each cell of fixed parent node IAB-DU or IAB-donor-DU is mobile-IAB-supported.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN3 117-119bis meetings.