3GPP TSG-RAN WG3 Meeting #121	R3-234172

Toulouse, FR, 21st – 25th August 2023

Agenda item:	 10.2.1

Source:		Lenovo

Title:		SON enhancements for SHR and SPR

Document for:		Discussion and Decision

1	Introduction

In this paper, we further discuss SON enhancements for SHR and SPR, based on agreements achieved so far and left issues raised in previous RAN3 meetings.

2	Discussion

  - 2.1 inter-RAT SHR from NR to LTE

RAN2#122 meeting discussed issues raised from RAN3 in the LS [1] and provided an LS to RAN3 [2]. Due to limited time and high workload in RAN2, RAN2 has made the agreements that:

=>	intra-NR SHR and Inter-RAT SHR from LTE to NR will be deprioritized in RAN2 for R18.

=>	SPR except the critical issues will not be further enhanced from this meeting until the end of R18.

In previous RAN3 meetings, we have spent much time discussing intra-NR SHR and inter-RAT SHR from LTE to NR, and we provided information and suggestions to RAN2 for further confirmation from RAN3 point of view. However, RAN2 would deprioritize intra-NR SHR and inter-RAT SHR from LTE to NR in R18. Without RAN2’s work and support, it is obviously not necessary for RAN3 to further discuss issues related to intra-NR SHR and inter-RAT SHR from LTE to NR. From this meeting, RAN3 should not further enhance intra-NR SHR and inter-RAT SHR from LTE to NR.

Proposal 1: Intra-NR SHR and inter-RAT SHR from LTE to NR should not be further discussed in R18.

We would focus on left issues for inter-RAT SHR from NR to LTE. For inter-RAT SHR from NR to LTE, only T310 and T312 related triggers are to be considered. Based on RAN3#119 meeting’s agreements, the receiving node forwards the inter-RAT SHR to source NR node when T310/T312 triggers the inter-RAT SHR.

The FFS issue is in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, whether to support correlation of NR RLF and LTE SHR, and if yes, whether any UE assistance is needed to support this correlation.

Similar as intra-NR SHR specified in R17, we think correlation of NR RLF and LTE SHR is needed since it is beneficail for MRO analysis and optimisation at network side to avoid mistakes. The solutions for network correlation are as below:

- Option A: source gNB performs correlation based on target C-RNTI. In Option A, target C-RNTI is needed in inter-RAT SHR even source gNB can’t understand target C-RNTI, target C-RNTI is just used as a reference to enable correlation, i.e. source gNB can identify that the two reports are for a same handover procedure for a same UE when a same target C-RNTI is included in both inter-RAT SHR and LTE RLF Report.

- Option B: source gNB performs correlation based on source C-RNTI. In Option B, source C-RNTI is needed in inter-RAT SHR. Since target node would explicitly transfer source C-RNIT to source node when forwarding RLF report to source node, source gNB can correlate inter-RAT SHR and LTE RLF Report, based on the source C-RNTI which is received in the inter-RAT SHR and explicitly received from target eNB.

The spec impact of Option A is to include target C-RNTI in inter-RAT SHR, and the spec impact of Option B is to include source C-RNTI inter-RAT SHR. Besides reports correlation, source C-RNTI can also be used by the source gNB for UE context retrieval, Option B is preferred due to more benefits.

Proposal 2: When T310/T312 triggers inter-RAT SHR from NR to LTE, source gNB correlates inter-RAT SHR and LTE RLF Report with source C-RNTI in case that there is a RLF occuring shortly after a successful inter-RAT HO from NR to LTE, and the source C-RNTI is added in the inter-RAT SHR.

  - 2.2 SPR

Based on RAN2’s LS [2], we know that SPR except the critical issues will not be further enhanced in RAN2 until the end of R18.

In previous RAN3 meetings, we have discussed whether to have an indication on whether the PSCell change was MN-initiated or SN-initiated and made an WA. From technical point of view, since old MN knows whether the PSCell change was MN-initiated or SN-initiated when it identifies the UE context e.g. based on the “C-RNTI in old PCell” and “Time between PSCell change and SPR retrieval” in the SPR, an indication reported from the UE is not needed. Since network based solution can be used to understand whether the PSCell change was MN-initiated or SN-initiated, it is not a critical issue for RAN2 to introduce such an indication in the SPR.

Proposal 3: Indication on whether the PSCell change was MN-initiated or SN-initiated is not needed in the SPR.

Another left issue for SPR is which node decides the trigger condition of T310/T312 for MN-initiated PSCell change/CPC, there are three options on table as below:

- Option 1: MN autonomously decides the T310/T312 SPR triggers

- Option 2: Source SN node decides the T310/T312 SPR triggers

- Option 3: MN decides the T310/T312 SPR triggers based on source SN inputs (e.g., after getting assistance from SN regarding the configured T310/T312 timer values)

RAN3#119-bis meeting agreed that if the trigger is T312/T310, the objective of SPR is to optimize PSCell change configuration and associated mobility thresholds, optimize lower layer issues of source PSCell (e.g., optimize T310/T312 timer values), and, T310/T312 related SPR triggers can also be optimized to ensure UE doesn’t unnecessarily collect SPR or only rarely collects SPR.

For Option 2, after old MN receives SPR, old MN performs initial analysis to identify the node(s) to which the SPR is to be forwarded, and forwards the SPR to the source SN which should perform the SPR optimization, i.e. old MN needs to forward SPR to source SN for root cause analysis after optimizing PSCell change configuration, then source SN may optimize T310/T312 timer values or T310/T312 SPR triggers.

For Option 1 or Option 3, after old MN receives SPR, old MN performs failure cause analysis, old MN may need to optimize the T310/T312 SPR triggers and optimize PSCell change configuration. Optionally, old MN may indicate source SN to optimize T310/T312 timer values without forwarding SPR. The MRO analysis and optimisation of Option 1 or Option 3 are simplier than that of Option2.

Both Option 1 and Option 3 can work, i.e. for MN-initiated classic PSCell change or CPC, the MN node decides the T310/T312 trigger threshold for SPR, with or without input of T310/T312 timer values from source SN. To compare Option 1 with Option 3, since T310/T312 trigger threshold of SPR is a percentage value rather than an absolute value, MN can decide the percentage value (e.g. 40%, or 60%) by itself without knowing the exact timer value of T310/T312, which means that it is not necessary of MN to achieve the configured timer value of T310/T312 from source SN. In such a way, signalling coordination between MN and source SN can be avoided. Therefore, Option 1 is preferred, and Option 3 is acceptable.

Proposal 4: For MN-initiated classic PSCell change or CPC, the MN node decides the T310/T312 trigger threshold for SPR.

3	Conclusion

This paper mainly discussed SON enhancements for SHR and SPR, the proposals were achieved as following:

Proposal 1: Intra-NR SHR and inter-RAT SHR from LTE to NR should not be further discussed in R18.

Proposal 2: When T310/T312 triggers inter-RAT SHR from NR to LTE, source gNB correlates inter-RAT SHR and LTE RLF Report with source C-RNTI in case that there is a RLF occuring shortly after a successful inter-RAT HO from NR to LTE, and the source C-RNTI is added in the inter-RAT SHR.

Proposal 3: Indication on whether the PSCell change was MN-initiated or SN-initiated is not needed in the SPR.

Proposal 4: For MN-initiated classic PSCell change or CPC, the MN node decides the T310/T312 trigger threshold for SPR.

Reference

[1] R3-232140, LS on intra-system inter-RAT SHR and SPR, HW

[2] R2-2306896, Reply LS on SHR and SPR, HW