3GPP TSG-RAN WG3 Meeting #121	R3-234174

Toulouse, FR, 21st – 25th August 2023

Agenda item:	 10.2.2

Source:		Lenovo

Title:		MRO for fast MCG recovery and voice fallback

Document for:		Discussion and Decision

1	Introduction

In this paper, we further discuss MRO for fast MCG recovery and voice fallback, based on agreements achieved so far and issues left in previous RAN3 meetings.

2	Discussion

  - 2.1 MRO for fast MCG recovery

RAN3#120 meeting agreed to define Case c (near-failure successful fast MCG recovery case) in R18, and the solution can be further discussed. For Case c, fast MCG recovery is successfully performed even the elapsed timer T316 is near to be expired when the response message for fast MCG recovery is received. Last RAN3 meeting discussed [1] whether the network based solution or UE based solution should be used for Case c, but didn’t achieve consensus:

- Network based solution: SN reports to MN the elapsed time between receiving the MCGFailureInformation and receiving confirmation from the UE of the reception of the reconfiguration from the MN.

- UE based solution: UE sends the elapsed value of T316 to networks.

We can find that RAN2#122 meeting also agreed to support Case c, and now RAN2 is having a on-going email [2] to discuss the solutions for Case c, for example, whether to introduce a new trigger condition for SHR e.g. to generate SHR when the elapsed timer of T316 is higher than a threshold, and what information is needed to be reported by the UE. We can wait for RAN2 progress on the solutions for Case c.

Observation: Wait for RAN2 progress on the solutions for Case c.

Previous RAN3 meeting also discussed an issue on whether to transfer T316 from the MN to the SN in order to support MRO for fast MCG recovery for pre-Rel 18 UEs. Obviously since MRO for fast MCG recovery is a feature to be supported from Rel-18, we think there is no need for pre-Rel 18 UEs to support the Rel-18 MRO for fast MCG recovery, and thus T316 value does not need to be transferred from the MN to the SN.

Proposal 1: No need for pre-Rel 18 UEs to support the Rel-18 MRO for fast MCG recovery.

  - 2.2 MRO for voice fallback

Until now, Case 1 and Case 2 are agreed for MRO consideration:

-	Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRAN cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRAN cell.

-	Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

As summarized in [3], for Case 1, it can be split into two cases:

-	Case 1a: a suitable E-UTRAN cell2 is selected after HOF of inter-system inter-RAT handover from source NR cell1 to target E-UTRAN cell1;

-	Case 1b: a suitable E-TURAN cell2 is selected, after a RLF occurs shortly after a successful handover from source NR cell1 to target E-UTRAN cell1.

From RAN3 point of view, we would further discuss how to report and deliver RLF report for inter-system inter-RAT HO from NR to E-UTRAN for voice fallback.

For Case 1a, when the UE is back to a NR cell3, UE reports NR RLF report to NR cell3, then NR cell3 forwards the NR RLF report to source NR cell1, e.g. the NR cell3 may transfer the UE RLF Report Container to the source NR cell1 directly via the existing FAILURE INDICATION message. If there is no direct Xn interface, the NR cell3 may transfer the UE RLF Report Container to the source NR cell1 via the existing UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message. It is obvious that Case 1a can be supported without any stage 3 spec impact.

For Case 2, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in a NR cell 2, after HOF of inter-system inter-RAT handover. Similar as Case 1a, the existing FAILURE INDICATION message or UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message can be reused to transfer the UE RLF Report Container to the source NR node. Also, we can find Case 2 can be supported without any stage 3 spec impact.

For Case 1b, the UE may report LTE RLF report which is enhanced to include an indication concerning that the last failed inter-system inter-RAT HO was triggered due to voice fallback to E-UTRAN cell2. E-UTRAN cell2 forwards the LTE RLF report to target E-UTRAN cell1, and target E-UTRAN cell1 decodes the LTE RLF report and explicitly transfers failure related information which is decoded from the LTE RLF report to source NR cell1 over S1 and NG interfaces, e.g. including source cell ID and failure cell ID. Current S1 and NG interfaces do not support to transfer these information. Last RAN3 meeting, we made a WA to introduce a new handover report type e.g. ‘Inter-system Mobility Failure for Voice Fallback’in the Inter-system HO Report over S1 and NG, this WA should be turned into agreements.

Proposal 2: Turn WA into agreement: Define a new handover report type in the Inter-system HO Report over S1 and NG.

3	Conclusion

This paper mainly discussed MRO for fast MCG recovery and voice fallback, the observation and proposals were achieved as following:

Observation: Wait for RAN2 progress on the solutions for Case c.

Proposal 1: No need for pre-Rel 18 UEs to support the Rel-18 MRO for fast MCG recovery.

Proposal 2: Turn WA into agreement: Define a new handover report type in the Inter-system HO Report over S1 and NG.

Reference

[1] R3-233336, Summary_of_offline_disc_CB # SONMDT2_MRO, ZTE

[2] R2-23xxxxx, Summary of [Post122][584][R18 SON/MDT] Open issues on fast MCG recovery, CMCC

[3] R3-233026, Update of MRO scenarios, Lenovo, Qualcomm, Huawei