3GPP TSG-RAN WG3 Meeting #121	R3-234177

Toulouse, FR, 21st – 25th August 2023

Agenda item:	 10.2.5

Source:		Lenovo

Title:		(TP for SON BLCR to TS38.423) Discussion on MRO for NR-U

Document for:		Discussion and Decision

1	Introduction

In RAN3#117-e meeting [1], some agreements for MRO for NR-U were achieved:

- Add to RLF report indications concerning Measured RSSI and HOF due to consistent LBT failure.

- Send an LS to RAN2 requesting:

to support latest Measured RSSI and Indication of HOF due to consistent LBT failure in RLF report

to evaluate addition in RLF report of: Energy Detection Threshold, LBT configuration parameter lbt-FailureRecoveryConfig, Channel Occupancy in UL, Time duration for LBT during SpCell change

to support “Indication of consistent LBT failure” in RA report

- Keep existing failure type definition and detection to indicate RLF or HOF or PSCell change failure due to consistent LBT failure.

In RAN3#118 meeting [2], RAN3 replied RAN2’s LS in [3] on possibility on LBT-FailureRecoveryConfig.

RAN3#119 bis-e meeting further agreed [4]:

- Enhancements of RLF reports and RA reports are beneficial to separate mobility related errors from the LBT-related ones.

- RLF Report and RA report can be enhanced to include information concerning the LBT failures in RA procedures, the granularity and implementation details needs to be further discussed based on progress in RAN2.

In this paper, we would further discuss the details of MRO for NR-U.

2	Discussion

In NR-U, before operating in the unlicensed spectrum, both the UE and the gNB would perform LBT to guarantee that the radio resource is not occupied by others e.g. Wifi, data/signalling transmission can be allowed if the wireless channnel is available. For UL, consistent LBT failure is detected per UL BWP by counting LBT failure indications from the lower layers to the MAC entity. If consistent LBT failure has been triggered in all UL BWPs configured with PRACH occasions on same carrier in the serving cell, the MAC entity indicates consistent LBT failure to upper layers and then RLF occurs.

Considering mobility in NR-U, one possible case is that after the UE receives the HO command , the UE performs uplink LBT but the shared spectrum of the target PCell is occupied, so consistent uplink LBT failure may happen before or during RACH procedure with the target PCell. In such case, HO failure may happen when T304 expires. To enable network understand that HO failure is caused by LBT failure, last RAN3 meeting agreed to enhance RLF report i.e. adding an indication concerning HOF due to consistent LBT failure in the RLF report. Similar as RLF report, for a SCG failure caused by consistent LBT failure, it is beneficial to add an indication concerning SCG failure due to consistent LBT failure in the SCG Failure Information message.

RAN3#117e meeting agreed to reuse the existing failure type definition and detection to indicate RLF/HOF or PSCell change failure due to consistent LBT failure. Similar as legacy MRO, the RLF report due to consistent LBT failure may be transferred between network nodes for root cause analysis, e.g. from the receiving node to the node last serving the UE, which means that the Failure Indication procedure over Xn, or Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG may be reused.

Observation: RLF report due to consistent LBT failure should be transferred at the network side for root cause analysis.

Another possibility is that before or during RACH procedure to the target PCell, uplink LBT in the UE side is successful, but downlink LBT fails in the network side, for example, the target PCell fails to send response e.g. RAR/MSG4/MSG B during the RACH procedure in the case that unlicensed channel resources in target PCell are unavailble. When T304 expires, from UE point of view, it does not know LBT fails in the target PCell, and it may trigger RLF report as legacy for the failed handover, i.e. no indication concerning HOF due to consistent LBT failure is included in the RLF report.

The receiving node which receives the RLF report from the UE may transfer the RLF report to the source node for MRO analysis. Since there is no indication concerning HOF due to consistent LBT failure in the RLF report, the receiving node may execute failure cause analysis and modification as legacy e.g. update trigger threshold for the handover procedure. However, the fact may be that the main failure cause is inappropriate LBT related configuration rather than handover configuration, in such a case, modifying handover configuration at network side is not essential and needs to be avoided.

It is FFS on whether to enable a UE-based solution or a NW-based solution to inform the source gNB of a handover about DL LBT issues occurring at the target gNB during a handover execution. As summarized in [5], there are two options on the table:

- Option 1: (UE-based solution) the UE adds in RLF report an indication of DL LBT failure causing absence of SSB transmissions during handover.

- Option 2: (NW-based solution) the target gNB sends to source gNB an indication of DL LBT failure during handover execution.

For Option1, it should be decided in RAN2, since it is a UE based solution. On the other hand, we don’t think absent SSB transmissions during handover execution can exactly mean the existence of DL LBT issues at network side, for example, there is a possibility that the UE can’t detect the DL SSB due to beam quality is quite poor.

In Option 2, the target node can indicate the source node that DL LBT failure occurred in the target cell, e.g. when the target node fails to respond the UE during the RACH procedure due to unlicensed channel resources in target cell are unavailble. Currently, HANDOVER REPORT message is transferred from target node to source node for MRO for a RLF case rather than a HOF case, i.e. HANDOVER REPORT message can’t be reused to indicate the source node that HOF happens due to DL LBT failure in the target cell. Therefore, it seems better to introduce a new Xn message from target node to source node to indicate DL LBT failure. In such a way, the source node may make proper failure analysis after receiving the DL LBT failure indication from the target node and the RLF report from the receiving node. On the other hand, target node may optimize LBT configuration when DL LBT failure occurred, or when the source node informs the target node to do optimization for LBT configuration if the source node decides there is a LBT issue in the target node. In Option 2, even the source gNB has to collect RLF report from UE and indication of DL LBT failure from target node at different time for MRO analysis, it is NW implementation to do the correlation, there is no any specific issue. For Option 2, there is no need to involve other WGs, and the spec impact to XnAP is small.

Compared Option 1 and Option2, Option 2 is simpler and more straightforward. Therefore, Option 2 as a NW-based solution is preferred.

Proposal 1: In case that HOF happens due to DL LBT failure at target node, the target node can send an indication concerning DL LBT failure to the source node for MRO analysis e.g. via a new introduced Xn message.

For the case that handover failure occurred due to LBT failure, to enable network better know the uplink channel situation and detailed information of LBT process, it is beneficial to include number of LBT failures for the last RA procedure in the RLF report, since RAN2#121 meeting agreed that “Log the last successful RA procedure related information in the RA report”, and the agreements can also be applied to RLF report.

For the last BWP of the last RA procedure, the entire RA-InformationCommon should be included, wherein for each preamble transmission attempt included in the perRAAtttemptInfoList, the number of preamble transmissions blocked due to LBT failure can be included in the RLF report.

Proposal 2: The number of preamble transmissions blocked due to LBT failure per RACH attempt for the last BWP of the last RA procedure should be included in the RLF report.

For the other BWPs (i.e. except the last BWP) in which consistent LBT failure happens in the last RA procedure, besides basic BWP information which is used to identify the BWP (e.g. locationAndBandwidth, subcarrierSpacing, and absoluteFrequencyPointA), the number of LBT failures per BWP can be included in the RLF report.

Proposal 3: The number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure should be included in the RLF report.

On the other hand, after the UE receives the HO command, it is possible that LBT with target PCell is successful before or during RACH procedure towards target PCell, but handover fails when T304 expires. Time information during handover procedure, e.g. time duration for UL LBT before per RACH attempt for the last BWP of the last RA procedure and the time elapsed since the last HO execution until successful LBT, is useful to decide how RLF report is used for MRO analysis, for example, if too long time which is close to timer period of T304 is spent for UL LBT, it may mean that the failure is mainly caused by channel occupancy rather than coverage issue even though LBT during handover procedure is successful, network may not perform a coverage optimization after receiving the failure information. Therefore, it is beneficial to include time information during handover procedure in the RLF report, e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT.

Proposal 4: Time information during handover procedure, e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT, should be included in the RLF report.

Similar as enhancements for RLF report, to enable network better know whether RACH configuration is configured properly, it is beneficial to include number of LBT failures (i.e. the number of preamble transmissions blocked due to LBT failure per RACH attempt for the last BWP of the last RA procedure, and the number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure), and time information for UL LBT before per RACH attempt in the RA report.

Proposal 5: The number of preamble transmissions blocked due to LBT failure per RACH attempt for the last BWP of the last RA procedure should be included in the RA report.

Proposal 6: The number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure should be included in the RA report.

Proposal 7: Time duration for UL LBT before per RACH attempt should be included in the RA report.

3	Conclusion

In this contribution, MRO for NR-U is discussed. We have the following proposals:

Observation: RLF report due to consistent LBT failure should be transferred at the network side for root cause analysis.

Proposal 1: In case that HOF happens due to DL LBT failure at target node, the target node can send an indication concerning DL LBT failure to the source node for MRO analysis e.g. via a new introduced Xn message.

Proposal 2: The number of preamble transmissions blocked due to LBT failure per RACH attempt for the last BWP of the last RA procedure should be included in the RLF report.

Proposal 3: The number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure should be included in the RLF report.

Proposal 4: Time information during handover procedure, e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT, should be included in the RLF report.

Proposal 5: The number of preamble transmissions blocked due to LBT failure per RACH attempt for the last BWP of the last RA procedure should be included in the RA report.

Proposal 6: The number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure should be included in the RA report.

Proposal 7: Time duration for UL LBT before per RACH attempt should be included in the RA report.

Reference

[1] Chairman notes for RAN3#117-e meeting

[2] Chairman notes for RAN3#118 meeting

[3] R3-226809, Response LS on Possibility on LBT-FailureRecoveryConfig, Nokia

[4] Chairman notes for RAN3#119bis-e meeting

[5] R3-231872, SoD on NR-U: CB # SONMDT5_NRU, Ericsson

Annex: TP for TS38.423

## 8.1	Elementary procedures

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| RACH Indication | RACH INDICATION |
| LBT Failure | LBT FAILURE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.x	LBT Failure

#### 8.4.x.1	General

The purpose of the LBT failure procedure is to inform that DL LBT failure happens in NG-RAN node2.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2: LBT Failure procedure

NG-RAN node2 initiates the procedure by sending the LBT FAILURE message to NG-RAN node1, when NG-RAN node2 detects that one or more DL LBT failures occurred in NG-RAN node2.

Editor’s note: The procedure text can be updated further based on the agreements.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.3.x	LBT FAILURE

This message is sent by the NG-RAN node2 to the NG-RAN node1 to indicate that one or more DL LBT failures occurred in NG-RAN node2.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node1. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- LBT FAILURE

--

-- **************************************************************

LbtFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ LbtFailure-IEs}},

...

}

LbtFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace 											ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-ProcedureCode41-NotToBeUsed									ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-rachIndicaion												ProcedureCode ::= xx

id-lbtFailure										ProcedureCode ::= yy

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>