3GPP TSG-RAN WG3 #121	R3-234203

Toulouse, France, 21st – 25th August 2023

Source:	CATT

Title:	(TP to 38.423) Discussion on the duration for which the prediction is

Agenda Item:	12.2.1

Document for:	Discussion and decision

- Introduction

This discussion paper focuses on the issue on the duration for which the prediction is.

- Discussion

Last meeting we agreed to introduce an IE (or IE group) for the client node to describe the time for which the prediction is. However, the encoding remained FFS, as well as the open issue whether the length of that time should be provided as well.

For the encoding aspect, it is sufficient enough to use a relative time and encoding it as an integer to reflect start point for the (first) prediction. Due to the precision requirement for start time is not very high, there is no need to introduce an absolute time, and relative time is easier to encode (as an integer). If the start point is set to e.g., 10s, it means the start point shall be set as 10s latter than receiving the request message.

Proposal 1: The IE structure “Requested Prediction Time” contains an integer to indicate the start time (of the first report message), which reflects the time difference compared with the time point that the client node receives the request message.

For the length aspect, it is beneficial to introduce it as an optional IE in Requested Prediction Time IE. It is a common case for network implementation to follow SA5’s definition that most metrics are averaged between time windows rather than measured only at a given time point, e.g:

##### 5.1.1.2.1 DL Total PRB Usage a) This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink for any purpose. b) SI c) This measurement is obtained as: , where is the DL total PRB usage, which is percentage of PRBs used, averaged during time period with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for DL traffic transmission shall be included; is total number of PRBs available for DL traffic transmission during time period ; and is the time period during which the measurement is performed. d) A single integer value from 0 to 100. e) RRU.PrbTotDl, which indicates the DL PRB Usage for all traffic f) NRCellDU g) Valid for packet switched traffic h) 5GS i) One usage of this measurement is for monitoring the load of the radio physical layer.

Therefore it is better to support that approach for predictions. The IE structure can be designed as the below:

| Requested Prediction Time |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Offset to Prediction time | M |  | INTEGER (0..4095) | Time interval between (the start time of) the prediction time in seconds [FFS]. | – |  |
| >Averaging Window Length | O |  | INTEGER (0..4095) | Unit: second [FFS]. | – |  |

The presence of the “Averaging Window Length” IE is designed as optional so that:

-	If it is absent, the predictions is for only one point of time, e.g. the prediction included in the first report message is for the point of time that is “Offset to Prediction time” later than receiving the request message.

-	If it is present, the predictions is for a time duration, e.g. the prediction included in the first report message is for the duration from “Offset to Prediction time” to “Offset to Prediction time” + “Averaging Window Length” later than receiving the request message.

What’s more, the relationship between this window length and the reporting periodicity is not essential. The predication mechanism makes sense no matter whether the window length is longer or shorter than the report periodicity. A shorter-than-periodicity window length means that the requested node is only interested in part of the reporting periodicity, similar to the case that each prediction is only for one point of time. A longer-than-periodicity window length does result in that the time duration for which the prediction in a later report is overlaps with the time duration for which the prediction in a prior report is, but their meaning is different due to they are predicted at different time.

Proposal 2: The IE structure “Requested Prediction Time” contains an optional IE to indicate the window length of the duration for which the (first) prediction is.

Proposal 3: The relationship between the window length and the reporting periodicity is not essential. The requesting node can choose a proper value for the window length.

In addition, it is beneficial for some case that the multiple predictions for consecutive periods are reported within a report message, so that predictions are enough for the client node to make decision at any given point of time. One example is depicted in Figure 1:

Figure 1: Why reporting the prediction for only one period is not sufficient enough.

As shown in Figure 1, the client receives the prediction for 10:00:00–10:00:10 at 09:59:59, and uses it to assist decision making, e.g. whether to hand over a UE, from 10:00:00 to 10:00:10. However, decision making may happen at any time. If it happens at 10:00:08, the end of that duration (i.e. 10:00:10) is too near, making the prediction not suffienct enough. What is accually sufficient enough may be a prediction covering the duration of 10:00:08–10:00:13.

For simplicity, we propose introducing an integer to indicate how many predictions (of the concerned characteristic) should be contained in one report message, each for a period consecutive one by one.

Proposal 4: The IE structure “Requested Prediction Time” contains an optional IE to indicate how many predictions (of the concerned characteristic) should be contained in one report message, each for a period consecutive one by one.

- Conclusion

Proposal 1: The IE structure “Requested Prediction Time” contains an integer to indicate the start time (of the first report message), which reflects the time difference compared with the time point that the client node receives the request message.

Proposal 2: The IE structure “Requested Prediction Time” contains an optional IE to indicate the window length of the duration for which the (first) prediction is.

Proposal 3: The relationship between the window length and the reporting periodicity is not essential. The requesting node can choose a proper value for the window length.

Proposal 4: The IE structure “Requested Prediction Time” contains an optional IE to indicate how many predictions (of the concerned characteristic) should be contained in one report message, each for a period consecutive one by one.

Based on the proposal, we draft a TP in the Annex (ASN.1 part is not included, as its baseline is not included in the baseline CR either).

- Annex: TP for TS 38.423 on R3-233535

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported, except the cases for predictions. | YES | ignore |
| Requested Prediction Time |  | 0..1 |  |  | YES | ignore |
| >Offset to Prediction time | M |  | INTEGER (0..4095) | Time interval between (the start time of) the prediction time in seconds [FFS]. | – |  |
| >Averaging Window Length | O |  | INTEGER (0..4095) | Unit: second [FFS]. | – |  |
| >Number of Windows | O |  | INTEGER (1..16) |  | – |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item(FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status List |  | 0..1 |  |  | – |  |
| >>>Predicted Radio Resource Status |  | 1 .. <maxnoofTimeWindows> | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs List |  | 0..1 |  |  | – |  |
| >>>Predicted Number of Active UEs |  | 1 .. <maxnoofTimeWindows> | 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections List |  | 0..1 |  |  | – |  |
| >>>Predicted RRC Connections |  | 1 .. <maxnoofTimeWindows> | 9.2.2.56 |  | – |  |
| UE Associated Info Result |  | 0..1 |  |  |  |  |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >> UE Assistant Identifier (FFS on the name) | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1 |  |  |
| >>UE Performance | O |  | 9.2.3.Y |  |  |  |
| >>UE trajectory | O |  | FFS |  |  |  |
| Energy Cost | O |  | FFS | It represents the actual measurement of the Energy Cost (FFS) |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |
| maxnoofTimeWindows | Maximum no. of time windows per reported character. Value is 16. |

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////