3GPP TSG-RAN WG3 Meeting#121   	 R3-234212

# Toulouse, FR, 21st - 25th Aug, 2023

Title:	Multicast reception in RRC_INACTIVE

Source:	ZTE

Agenda Item:	15.3

Document for:	Discussion

Introduction

- In this paper, we will discuss several issues for multicast reception in RRC_INACTIVE based on agreements of SA2/RAN2 and latest progress in RAN3, including:

section 2, the enhancement of NG/Xn/F1 group paging message

section 3, whether to notify gNB-DU the session states change or not

section 4, CU or DU to initiate multicast reception in RRC_INACTIVE, and the signaling interaction for obtaining PTM config in RRC Release from gNB-DU

Impacts from Group paging enhancement

# NG group paging

In RAN3#119bis meeting, whether NGAP group paging message needs to be enhanced to carry session activation indication was discussed without agreement. Companies suggested to wait for SA2 and RAN2 discussion. Well, the latest progress of RAN2 and SA2 are as follows:

- RAN2#121bis - =>Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS. - =>For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data. RAN2#122 =>Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE. SA2#TS23247-i20 Editor's note: How NG-RAN notifies the UE that the MBS session is activated and whether the MBS session is allowed to be received in RRC-INACTIVE state will be decided by RAN WGs.

It can be seen that RAN2 has introduced an new indication in group paging to inform Rel-18 UEs having a valid configuration to receive multicast in RRC_INACTIVE. And SA2 prefer this question should be decided by RAN WGs.

Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon a new indication in group paging when session is activated.

UE will be back to RRC_CONNECTED when session is released without any additional enhancement.

Due to different UE behaviors for session activation and session release, it is necessary to distinguish the purpose of CN group paging, e.g., for session activation or session release. A new indication about cause of CN group paging (multicast session activation or release) needs to be introduced in the NGAP : MULTICAST GROUP PAGING message, otherwise UE in RRC_INACTIVE might be triggered to resume to RRC_CONNECTED unnecessarily.

Due to different UE behaviors for session activation and session release, it is necessary to distinguish the purpose of CN group paging, e.g., for session activation or session release.

An indication about cause of CN group paging to indicate multicast session activation is introduced in the NGAP: MULTICAST GROUP PAGING message.

# F1 group paging

Since gNB-DU is responsible for execution of multicast group paging, to help gNB-DU distinguish whether an indication is added for multicast reception in RRC_INACTIVE, MULTICAST GROUP PAGING message from gNB-CU to gNB-DU should be enhanced too. But on the enhancement details of F1 group paging we should wait for RAN2 progress.

An indication information per tmgi is introduced in F1 group paging message which informs that this session is activated and to be received in RRC_INACTIVE, details are FFS.

# Xn group paging

Considering RAN2 has agreed to introduce a new indication in group paging for multicast reception in RRC_INACTIVE, whether Xn group paging need to be enhanced should be discussed, in following scenarios:

- Scenario 1: Source gNB is not configured to receive this multicast in RRC_INACTIVE and target gNB is not configured to receive this multicast in RRC_INACTIVE.

- Scenario 2: Source gNB is not configured to receive this multicast in RRC_INACTIVE and target gNB is configured to receive this multicast in RRC_INACTIVE.

- Scenario 3: Source gNB is configured to receive this multicast in RRC_INACTIVE and target gNB is not configured to receive this multicast in RRC_INACTIVE.

- Scenario 4: Source gNB is configured to receive this multicast in RRC_INACTIVE and target gNB is configured to receive this multicast in RRC_INACTIVE.

- For scenario 1 ( S-gNB not configured and T-gNB not configured), source and target gNB may be a Rel-17 gNB, or though Rel-18 gNB but is not configured to receive this multicast session in RRC_INACTIVE. In this scenario, the usefulness of group paging is only to move UEs into RRC_CONNECTED state as legacy.

- For scenario 2 ( S-gNB not configured and T-gNB configured), source may be a Rel-17 gNB, or though Rel-18 gNB but is not configured to receive this multicast session in RRC_INACTIVE. Target gNB is a Rel-18 gNB which is configured to receive this multicast session in RRC_INACTIVE. In this scenario, the usefulness of group paging is only to move UEs into RRC_CONNECTED state as legacy.

- For scenario 3 ( S-gNB configured and T-gNB not configured), source is a Rel-18 gNB which is configured to receive this multicast session in RRC_INACTIVE. Target gNB may be a Rel-17 gNB, or though Rel-18 gNB but is not configured to receive this multicast session in RRC_INACTIVE. In this scenario, the usefulness of group paging is only to move UEs into RRC_CONNECTED state as legacy.

- For scenario 4 ( S-gNB configured and T-gNB not configured), source and target gNBs are all Rel-18 gNB which is configured to receive this multicast session in RRC_INACTIVE. In this scenario, the usefulness of group paging is to notify UE to continue receiving multicast in RRC_INACTIVE.

If S-gNB is not configured to receive multicast in RRC_INACTIVE, RRC_INACTIVE UEs will always move back to RRC_CONNECTED.

If S-gNB is configured to receive multicast in RRC_INACTIVE, whether to move UEs back to RRC_CONNECTED depends on T-gNB.

The configuration for multicast reception in RRC_INACTIVE of S-gNB will affect T-gNB’s group paging procedure.

- To assist target gNB initiate group paging procedure, an indication per tmgi is introduced in Xn group paging message which informs this session is configured to receive in RRC_INACTIVE, as for the details of the indication, we suggest to wait for RAN2’s progress.

An indication per tmgi is introduced in Xn group paging message which informs that this session is activated and to be received in RRC_INACTIVE, details are FFS.

Session states on F1AP

In RAN2#121 meeting, the agreements about PTM configuration used for multicast reception in RRC_INACTIVE are as follows:

- => If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured. - =>When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration. Other dedicated RRC messages will not be used to provide PTM configuration for MBS multicast for INACTIVE.

It can be seen that, PTM configuration for multicast reception in RRC_INACTIVE can be configured to UE via RRCRelease message, before session activation.

PTM configuration for multicast reception in RRC_INACTIVE can be configured to UE via RRCRelease message, before session activation.

From the above analysis, gNB may deliver PTM configuration for inactive session via RRCRelease message. That is, gNB-DU should consider whether keeping PTM configuration for inactive session which is transitions from active to inactive.

To be aligned with RAN2 agreements, gNB-DU may keep PTM configuration for inactive session.

- In RAN2#122 meeting, the agreements about multicast session deactivation notification are as follows:

- =>MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE.

To better assist gNB-DU in making decision about inactive session configuration, it is necessary to notify gNB-DU the session states (e.g., introducing session state indication in MULTICAST CONTEXT MODIFICATION REQUEST message)

To assist gNB-DU in keeping configuration for inactive session and notifying MC session deactivation in MCCH, gNB-CU needs to notify gNB-DU the session states.

CU or DU, the same old story

There is no doubt that it is CU to determine UE's RRC states. However, when and who to initiate the RRC_INACTIVE, CU or DU, we think it deserves a discussion.

Multicast reception in RRC_INACTIVE was decided to be part of Rel-18 simply because it is a useful tool to handle the network congestion in case of large number of MCPTT UEs. That is, by releasing UE to RRC_INACTIVE and continuing the reception, UEs in RRC_CONNECTED with unicast services can still be served.

Multicast reception in RRC_INACTIVE is to handle network congestion.

To minimize the service interruption, the congestion should be detected/solved as soon as possible.

With this background, we come back to CU or DU things.

Initiating multicast reception in RRC_INACTIVE

- # gNB-CU

- Since gNB-CU decides the RRC state, it knows the associated multicast sessions for each UE, it is aware of the congestion in each cell, it seems natural for the CU to initiate multicast reception in RRC_INACTIVE, in one DU or one cell. Or is it?

- In current spec gNB-CU, is only aware of the level of cell congestion through reported resource status from gNB-DU. However, there are certain drawbacks:

the granularity of such report is quite limited, if CU seeks a more precise action to release which UE and enable the RRC_INACTIVE reception in which cell.

Due to the nature of open loop tuning, gNB-CU might make multiple adjustments about initiating multicast reception in RRC_INACTIVE, according to the change of reported resource status.

The granularity of such report is quite limited, if CU seeks a more precise action to release which UE and enable the RRC_INACTIVE reception in which cell.

gNB-CU may need to make multiple adjustments about initiating multicast reception in RRC_INACTIVE according to the update of reported resource status from gNB-DU.

#gNB-DU

- Since gNB-DU hosts RLC, MAC and PHY layers of the gNB, gNB-DU is better aware of the resource usage of each cell. That further means gNB-DU is able to know the cause of cell congestion clearly, and quickly and accurately solve congestion problems for itself.

- To help DU find the best solution to handle the congestion (e.g., start multicast reception in RRC_INACTIVE), DU might be able to make a better suggestion to DU.

gNB-DU is better aware of the resource usage of cells and UEs, which can help DU to find the best solution to handle the congestion. And multicast reception in RRC_INACTIVE is one of the solution.

- Compared with these solutions, initiate multicast reception in RRC_INACTIVE by gNB-DU can solve cell congestion faster and more accurately.

Initiating multicast reception in RRC_INACTIVE by gNB-DU can solve cell congestion faster and more accurately.

To be aligned with the role split between CU and DU, we think it is better to take the the initiation from DU as a suggestion, that is to say, CU makes the final decision on which UE to be released, and in which cell to enable the multicast reception in RRC_INACTIVE.

DU initiates multicast reception in RRC_INACTIVE (as a suggestion).

CU makes the final decision on which UE to be released, and in which cell to enable the multicast reception in RRC_INACTIVE.

- How to send the initiation from gNB-DU to gNB-CU for multicast reception in RRC_INACTIVE? There are two possible solutions:

- Solution 1: UE associated F1 signaling

- Solution 2: Multicast session associated F1 signaling

- For solution 1, gNB-DU can initiate multicast reception in RRC_INACTIVE by UE Context Release Request procedure. The related information can be carried to gNB-CU, such as the UE release cause (be configured to receive multicast in RRC_INACTIVE), the PTM configuration (which is used for multicast reception in RRC_INACTIVE). According to the information from gNB-DU, gNB-CU can initiate UE Context Release procedure and include the PTM configuration in RRCRelease message.

- For solution 2, gNB-DU can require to initiate multicast reception in RRC_INACTIVE by a multicast session associated F1 procedure. The related information can be carried to gNB-CU, such as a cell list (which cells are configured to receive multicast in RRC_INACTIVE), the PTM configuration (which is used for multicast reception in RRC_INACTIVE).

- For the same multicast session, the requirement from gNB-DU to initiate multicast reception in RRC_INACTIVE for all UEs joined this session is no difference, and the PTM configuration used in RRC_INACTIVE is the same. Thus, compared with solution 2, solution 1 seems inefficient and unnecessary.

Initiating multicast reception in RRC_INACTIVE via UE associated F1 signaling is  inefficient and unnecessary.

Initiating multicast reception in RRC_INACTIVE via session associated F1 signaling by gNB-DU.

How to obtain PTM configuration from gNB-DU

To support carrying PTM configuration for multicast reception in RRC_INACTIVE via RRCRelease message, gNB-CU needs to obtain the latest PTM configuration for inactive/active sessions. Considering the PTM configuration for different UEs for the same session is the same, using session related procedure to obtain the configuration from gNB-DU is more appropriate than using UE related procedure. Two scenarios should be considered.

- scenarios 1: gNB-CU initiates to obtain the PTM configuration from gNB-DU

- scenarios 2: gNB-DU initiates to update the PTM configuration to gNB-CU

- # scenarios 1: gNB-CU initiates to obtain the PTM configuration from gNB-DU

- For a multicast session, if gNB-CU wants to obtain the PTM configuration for multicast reception in RRC_INACTIVE when establishing the multicast context, CU can told DU this information in MULTICAST CONTEXT SETUP REQUEST message. DU can confirm to actively inform CU when updating PTM configuration.

- And, if CU wants to obtain the PTM configuration for multicast reception in RRC_INACTIVE after establishing multicast context, CU can trigger multicast context modification procedure to inform DU. According CU’s notification, DU may carry the latest configuration in MULTICAST CONTEXT MODIFICATION RESPONSE MESSAGE for an inactive/active multicast session.

- Fig 1. gNB-CU initiates to obtain the PTM configuration from gNB-DU

- gNB-CU sends MULTICAST CONTEXT SETUP/MODIFICATION REQUEST message to notify gNB-DU that this session is allowed to be received in RRC_INACTIVE, and may carry the PTM configuration which is delivered via MCCH message.

- gNB-DU replies MULTICAST CONTEXT SETUP/MODIFICATION RESPONSE message and may carry the latest multicast configuration.

- 3/4/5. gNB-CU initiates UE context release procedure to deliver the RRCRelease message carried latest PTM configuration for multicast reception in RRC_INACTIVE when UE transitions to RRC_IINACTIVE.

gNB-DU, in MULTICAST CONTEXT SETUP/MODIFICATION RESPONSE message, may carry the latest multicast configuration if the RRC_INACTIVE reception is enabled.

- # scenarios 2: gNB-DU initiates to update the PTM configuration to gNB-CU

- Except that CU triggers to obtain multicast configuration from DU, we also need to ensure DU can actively inform CU when updating PTM configuration for an inactive/active session. A possible solution is to introduce the multicast context modification required procedure, therefore DU is able to trigger this procedure to inform CU the updated configuration.

- FIG 2. gNB-DU initiates to update the PTM configuration to gNB-CU

- gNB-DU sends MULTICAST CONTEXT MODIFICATION REQUIRED message to provide the updated PTM configuration for a multicast session.

- gNB-CU sends MULTICAST CONTEXT MODIFICATION CONFIRM message to inform gNB-DU the successful modification.

- 3/4/5. gNB-CU initiate UE context release procedure to deliver the RRCRelease message carried PTM configuration for multicast reception in RRC_INACTIVE when UE transitions to RRC_IINACTIVE.

gNB-DU, in the new MULTICAST CONTEXT MODIFICATION REQUIRED message, may provide the updated PTM configuration, if the RRC_INACTIVE reception is enabled.

The signaling interaction for multicast reception in RRC_INACTIVE

- Through the discussion of the first two sections, an example signaling interaction for multicast reception in RRC_INACTIVE is as follows:

- FIG 3. The signaling interaction for multicast reception in RRC_INACTIVE

- gNB-DU suggests and gNB-CU confirms, that initiation multicast reception in RRC_INACTIVE by MULTICAST CONTEXT MODIFICATION/SETUP procedure.

- gNB-CU obtains (or gNB-DU updates) the PTM configuration carried in RRC_RRCRelease via MULTICAST CONTEXT MODIFICATION/SETUP procedure.

- 3/4/5. gNB-CU initiates UE context release procedure to deliver the RRCRelease message carried PTM configuration for multicast reception in RRC_INACTIVE when UE transitions to RRC_IINACTIVE.

Conclusion

We have the following recommendations:

# impacts from Group paging enhancement

- An indication about cause of CN group paging to indicate multicast session activation is introduced in the NGAP: MULTICAST GROUP PAGING message.

- An indication information per tmgi is introduced in F1 group paging message which informs that this session is activated and to be received in RRC_INACTIVE, details are FFS.

- An indication per tmgi is introduced in Xn group paging message which informs that this session is activated and to be received in RRC_INACTIVE, details are FFS.

- # Session states on F1AP

- To assist gNB-DU in keeping configuration for inactive session and notifying MC session deactivation in MCCH, gNB-CU needs to notify gNB-DU the session states.

- # Initiating multicast reception in RRC_INACTIVE

DU initiates multicast reception in RRC_INACTIVE (as a suggestion).

CU makes the final decision on which UE to be released, and in which cell to enable the multicast reception in RRC_INACTIVE.

Initiating multicast reception in RRC_INACTIVE via session associated F1 signaling by gNB-DU.

- # How to obtain PTM configuration from gNB-DU

gNB-DU, in MULTICAST CONTEXT SETUP/MODIFICATION RESPONSE message, may carry the latest multicast configuration if the RRC_INACTIVE reception is enabled.

gNB-DU, in the new MULTICAST CONTEXT MODIFICATION REQUIRED message, may provide the updated PTM configuration, if the RRC_INACTIVE reception is enabled.

With the proposals above, we provide the corresponding TPs in the following sections.

- Annex A. TP to TS 38.473

-----------------Start of the Changes-------------------

    - 8.14.x	Multicast Context Required (gNB-DU initiated)

      - 8.14.x.1	General

The purpose of the Multicast Context Modification procedure is to modify an established MBS session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

      - 8.14.x.2	Successful Operation

Figure 8.14.x.x: Multicast Context Modification Required procedure. Successful operation

The MULTICAST CONTEXT MODIFICATION REQUIRED message is initiated by the gNB-DU.

Upon reception of the MULTICAST CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall perform the modifications, and, if successful, report the update in the MULTICAST CONTEXT MODIFICATION CONFIRM message.

      - 8.14.x.3	Unsuccessful Operation

Figure 8.14.x.x: Multicast Context Modification Required procedure. Unsuccessful operation

In case none of the requested modifications of the multicast context can be successfully performed, the gNB-CU shall respond with the MULTICAST CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

      - 8.14.x.4	Abnormal Conditions

Not applicable.

-----------------Next of the Change------------------

- 9.2.14	Multicast Context Management messages

// skip unchanged part

- 9.2.14.2	MULTICAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a multicast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| MBS Multicast CU to DU RRC Information | O |  | FFS |  | YES | ignore |
| Multicast MRBs To Be Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRBs to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

-----------------Next of the Change------------------

- 9.2.14.3	MULTICAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Multicast DU to CU RRC Information | O |  | FFS |  | YES | ignore |
| Multicast MRB Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | Reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

-----------------Next of the Change------------------

- 9.2.14.7	MULTICAST CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to modify multicast context information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS Multicast CU to DU RRC Information | O |  | FFS |  | YES | ignore |
| Session State Indication | O |  | 9.3.1.xxx |  | YES | ignore |
| Multicast MRB To Be Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | O |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 0 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | O |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

-----------------Next of the Change------------------

- 9.2.14.8	MULTICAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Multicast DU to CU RRC Information | O |  | FFS |  | YES | ignore |
| Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Multicast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

-----------------Next of the Change------------------

      - 9.2.14.x	MULTICAST CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-DU to provide the updated PTM configuration for a multicast session.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS Multicast DU to CU RRC Information | M |  | FFS |  | YES | ignore |

-----------------Next of the Change------------------

      - 9.2.14.x	MULTICAST CONTEXT MODIFICATION CONFIRM

- This message is sent by the gNB-DU to inform gNB-DU the successful modification.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |

-----------------Next of the Change------------------

- 9.3.1	Radio Network Layer Related IEs

// skip unchanged part

- 9.3.1.xxx Session State Indication

This IE provides the multicast session state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Session State Indication | M |  | ENUMERATED (inactive, …) |  |

-----------------Next of the Change------------------

- 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

// skip unchanged part

MulticastContextModificationRequest,

MulticastContextModificationResponse,

MulticastContextModificationFailure,

MulticastContextModificationRequired,

MulticastContextModificationConfirm,

MulticastDistributionSetupRequest,

MulticastDistributionSetupResponse,

MulticastDistributionSetupFailure,

MulticastDistributionReleaseCommand,

MulticastDistributionReleaseComplete,

// skip unchanged part

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-MulticastContextModificationRequired,

// skip unchanged part

multicastDistributionSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionSetupRequest

SUCCESSFUL OUTCOME		MulticastDistributionSetupResponse

UNSUCCESSFUL OUTCOME	MulticastDistributionSetupFailure

PROCEDURE CODE			id-MulticastDistributionSetup

CRITICALITY				reject

}

multicastDistributionRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionReleaseCommand

SUCCESSFUL OUTCOME		MulticastDistributionReleaseComplete

PROCEDURE CODE			id-MulticastDistributionRelease

CRITICALITY				reject

}

multicastContextModificationRequired F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextModificationRequired

SUCCESSFUL OUTCOME		MulticastContextModificationConfirm

UNSUCCESSFUL OUTCOME	MulticastContextModificationFailure

PROCEDURE CODE			id-MulticastContextModificationRequired

CRITICALITY				reject

}

-----------------Next of the Change------------------

- 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

// skip unchanged part

MBS-Session-ID,

MBS-ServiceArea,

MBSMulticast-CUtoDURRCInformation,

MBSMulticast-DUtoCURRCInformation,

Session-State-Indication,

// skip unchanged part

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticast-CUtoDURRCInformation,

id-MBSMulticast-DUtoCURRCInformation,

id-Session-State-Indication,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastF1UContextReferenceCU,

// skip unchanged part

#### -- MULTICAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP REQUEST

--

-- **************************************************************

MulticastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupRequestIEs}},

...

}

MulticastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory  }|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID						PRESENCE mandatory  }|

{ ID id-MBS-ServiceArea					CRITICALITY reject TYPE	MBS-ServiceArea							PRESENCE optional   }|

{ ID id-MBSMulticast-CUtoDURRCInformation	CRITICALITY ignore TYPE	MBSMulticast-CUtoDURRCInformation			PRESENCE optional   }|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI								PRESENCE mandatory  }|

{ ID id-MulticastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	MulticastMRBs-ToBeSetup-List		PRESENCE mandatory  },

...

}

MulticastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetup-ItemIEs} }

MulticastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	MulticastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

MulticastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupResponseIEs}},

...

}

MulticastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MBSMulticast-DUtoCURRCInformation	CRITICALITY ignore TYPE	MBSMulticast-DUtoCURRCInformation			PRESENCE optional   }|

{ ID id-MulticastMRBs-Setup-List			CRITICALITY reject TYPE MulticastMRBs-Setup-List					PRESENCE mandatory	}|

{ ID id-MulticastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetup-List 	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

MulticastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Setup-ItemIEs} }

MulticastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetup-ItemIEs} }

MulticastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Setup-Item			CRITICALITY reject	TYPE MulticastMRBs-Setup-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE MulticastMRBs-FailedToBeSetup-Item	PRESENCE mandatory},

...

}

// skip unchanged part

#### -- MULTICAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

MulticastContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationRequestIEs}},

...

}

MulticastContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea						PRESENCE optional  }|

{ ID id-MBSMulticast-CUtoDURRCInformation	CRITICALITY ignore TYPE	MBSMulticast-CUtoDURRCInformation			PRESENCE optional   }|

{ ID id-Session-State-Indication	CRITICALITY ignore TYPE	Session-State-Indication			PRESENCE optional   }|

{ ID id-MulticastMRBs-ToBeSetupMod-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeModified-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-List	PRESENCE optional  },

...

}

MulticastMRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetupMod-ItemIEs} }

MulticastMRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeModified-ItemIEs} }

MulticastMRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeModified-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

MulticastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeReleased-ItemIEs} }

MulticastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

MulticastContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationResponseIEs}},

...

}

MulticastContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-MBSMulticast-DUtoCURRCInformation	CRITICALITY ignore TYPE	MBSMulticast-DUtoCURRCInformation			PRESENCE optional   }|

{ ID id-MulticastMRBs-SetupMod-List			CRITICALITY reject TYPE MulticastMRBs-SetupMod-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeSetupMod-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-Modified-List			CRITICALITY reject TYPE MulticastMRBs-Modified-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeModified-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeModified-List	PRESENCE optional  }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional  },

...

}

MulticastMRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-SetupMod-ItemIEs} }

MulticastMRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-SetupMod-Item				CRITICALITY		reject	TYPE MulticastMRBs-SetupMod-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetupMod-ItemIEs} }

MulticastMRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetupMod-Item	CRITICALITY		ignore	TYPE MulticastMRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Modified-ItemIEs } }

MulticastMRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Modified-Item				CRITICALITY		reject	TYPE MulticastMRBs-Modified-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeModified-ItemIEs} }

MulticastMRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeModified-Item	CRITICALITY 	ignore	TYPE MulticastMRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

// skip unchanged part

#### -- MULTICAST CONTEXT MODIFICATION REQUIRED (gNB-DU initiated) ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

MulticastContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationRequiredIEs}},

...

}

MulticastContextModificationRequiredIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-MBS-Session-ID					     CRITICALITY reject 	TYPE	MBS-Session-ID						PRESENCE mandatory  }|

{ ID id-MBS-ServiceArea						CRITICALITY reject TYPE	MBS-ServiceArea							PRESENCE optional   }|

{ ID id-MBSMulticast-DUtoCURRCInformation	CRITICALITY ignore TYPE	MBSMulticast-DUtoCURRCInformation			PRESENCE optional   }|

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

MulticastContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationConfirmIEs}},

...

}

MulticastContextModificationConfirmIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-MBS-Session-ID						CRITICALITY reject 	TYPE	MBS-Session-ID						PRESENCE mandatory  }|

{ ID id-MBS-ServiceArea						CRITICALITY reject TYPE	MBS-ServiceArea							PRESENCE optional   }|

...

}

-----------------Next of the Change------------------

- 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

// skip unchanged part

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticast-CUtoDURRCInformation,

id-MBSMulticast-DUtoCURRCInformation,

id-Session-State-Indication,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastF1UContextReferenceCU,

// skip unchanged part

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

MBSMulticast-CUtoDURRCInformation ::= SEQUENCE {

FFS

...

}

MBSMulticast-DUtoCURRCInformation ::= SEQUENCE {

FFS

...

}

Session-State-Indication ::= ENUMERATED {inactive, ...}

-----------------Next of the Change------------------

- 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup

// skip unchanged part

id-PDCMeasurementTerminationCommand			ProcedureCode ::= 79

id-PDCMeasurementFailureIndication 			ProcedureCode ::= 80

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= 81

id-MulticastContextModificationRequired		ProcedureCode ::= FFS

// skip unchanged part

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-MBSMulticast-CUtoDURRCInformation				ProtocolIE-ID ::= FFS

id-MBSMulticast-DUtoCURRCInformation				ProtocolIE-ID ::= FFS

id-Session-State-Indication							ProtocolIE-ID ::= FFS

-----------------End of the Changes-------------------

- Annex A. TP to TS 38.413

-----------------Start of the Changes-------------------

- 9.2.4.2	MULTICAST GROUP PAGING

This message is sent by the AMF and is used to notify involved UEs about the activation of a multicast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | ignore |
| MBS Service Area | O |  | 9.3.1.208 |  | YES | ignore |
| Group Paging Cause | O |  | ENUMERATED (session activation, ...) |  | - | ignore |
| Multicast Group Paging Area List |  | 1 |  |  | YES | ignore |
| >Multicast Group Paging Area Item |  | 1..<maxnoofPagingAreas> |  |  | - |  |
| >>Multicast Group Paging Area | M |  | 9.3.1.216 |  | - |  |
| >>UE Paging List |  | 0..1 |  |  | - |  |
| >>>UE Paging Item |  | 1..<maxnoofUEsforPaging> |  |  | - |  |
| >>>>UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| >>>>Paging DRX | O |  | 9.3.1.90 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingAreas | Maximum no. of paging areas for multicast group paging. Value is 64. |
| maxnoofUEsforPaging | Maximum no. of UEs allowed within one paging area for multicast group paging. Value is 4096. |

-----------------Next of the Change------------------

- 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

// skip unchanged part

MBS-AreaSessionID,

MBS-ServiceArea,

MBS-SessionID,

Group-Paging-Cause,

MBS-DistributionReleaseRequestTransfer,

MBS-DistributionSetupRequestTransfer,

// skip unchanged part

id-MBS-AreaSessionID,

id-MBS-ServiceArea,

id-MBS-SessionID,

id-Group-Paging-Cause,

id-MBS-DistributionReleaseRequestTransfer,

id-MBS-DistributionSetupRequestTransfer,

// skip unchanged part

-- **************************************************************

--

#### -- MULTICAST GROUP PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST GROUP PAGING

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastGroupPagingIEs} },

...

}

MulticastGroupPagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY ignore	TYPE MBS-SessionID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY ignore	TYPE MBS-ServiceArea					PRESENCE optional		}|

{ ID id-Group-Paging-Cause					CRITICALITY ignore	TYPE Group-Paging-Cause					PRESENCE optional		}|

{ ID id-MulticastGroupPagingAreaList			CRITICALITY ignore	TYPE MulticastGroupPagingAreaList	PRESENCE mandatory	},

...

}

-----------------Next of the Change------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

// skip unchanged part

id-MBS-ServiceArea,

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-Group-Paging-Cause,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-SessionTNLInfo5GC,

// skip unchanged part

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Group-Paging-Cause ::= ENUMERATED {session activation, ...}

-----------------Next of the Change------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

// skip unchanged part

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-Group-Paging-Cause									ProtocolIE-ID ::= FFS

-----------------End of the Changes-------------------