3GPP TSG-RAN WG3 Meeting #121	R3-234233

Toulouse, France, 21st – 25th August 2023

Agenda Item:	15.2

Source:	Ericsson

Title:	Further thoughts on MBS reception for RAN sharing scenarios

Document for:	Discussions & Approval

# 1	Introduction

This document continues discussions on MBS reception for RAN sharing scenarios.

# 2	Discussion

## 2.1	On identification of identical content in case of location dependent MBS sessions

We had discussions last meeting on the topic how to determine the per 5GC provide Area Session IDs which provide the same content.

(a) One approach suggested last meeting was to perform this decision on a per cell basis

(b) Another approach suggested that (the shared part) of the MBS Service Area information of an Area Session ID as provided by the participating 5GC has to be identical, otherwise one cannot deduce identical content provided.

There are possible MBS Service Area configurations that would make it impossible to follow approach (a)

Figure: possible Area Session ID configurations associated with an assumed “blue” and “green” 5GC.

In the above example, Cells 2 and 3 of Area Session ID X (“blue” 5GC) would correspond to two different Area Session ID, A and B (“green” 5GC).

Observation 1: A unique association between cells and Area Sessions of different 5GC is not (always) possible if the gNB tries to identify identical content of location based MBS sessions delivered by different 5GCs on a per-cell basis only.

Proposal 2: 	Agree on the requirement that (the shared part of) MBS Service Area Information of Area Session IDs from different 5GC have to be identical and on the respective proposed text for 38.300 and 38.401 in the respective Annexes.

## 2.2	Where to ultimately decide whether to share RAN resources in case of disaggregated gNBs - F1AP design

In case of RAN sharing with multiple cell-ID broadcast, it should be common understanding that the entity controlling the per-5GC gNB-DU is the place to finally decide whether RAN resources can be shared for the MBS sessions. It has to be ensured that appropriate information is provided via F1AP to enable this decision.

Proposal 3: 	For RAN sharing with multiple cell-ID broadcast F1AP shall support provision of the Associated Session ID and the MBS Service Area as received by the gNB over NG.

In case of MOCN, there ware 2 options possible how broadcast MBS Session context are handled on F1AP:

- For each MBS Session (from each participating 5GC) a separate Broadcast MBS Session Context is established in the gNB-DU

- For those MBS Sessions which may perform MBS session resource sharing, a single Broadcast MBS Session Context is established at the gNB-DU.

(pro (i)): Aiming at reducing options, it seems beneficial to select an option that serves both, MOCN and RAN sharing with multiple cell-ID broadcast.

(pro (i)): In case a gNB-DU serves shared and non-shared cells, will a Broadcast Session Context be associated with set of MBS sessions for the shared resources and the MBS sessions for the non shared resources? Or will there be separate Broadcast Session Contexts established for the shared and the non shared parts of the MBS sessions? It appears that keeping Broadcast session context on F1 separate, as they are kept separate on NG, is the less complex concept.

(pro (i)): Option (ii), at least in the proposals seen so far, seems to aim at reusing existing Broadcast Session F1AP signalling. Each “legacy” F1AP Broadcast Context is associated with a single MBS Session. Extending the existing signalling would end up in a “main” MBS Session (ID) and “associated” MBS Session (ID(s)). We have be seriously looking at the possibility that one of the participating 5GC stops the MBS Session, which would mean that if the 5GC that is associated with the “main” MBS Session (ID) stops the MBS Session, it would still remain – in a way – continuing with the MBS Session. Extending the existing Broadcast Context F1AP signalling to harbour more than one MBS Session in a single F1AP Elementary Procedure instance doesn’t seem to be a good protocol design approach and should be ruled out.

Proposal 4:	Agree, for MOCN and RAN sharing with multiple cell-ID broadcast, to establish separate Broadcast MBS Session Contexts in the gNB-DU, one per participating 5GC.

Following the above proposal it is obvious that it is the gNB-DU that finally makes the decision on whether and how to share MBS Session Resources – on a per cell basis, in both RAN sharing options: MOCN and RAN sharing with multiple cell-ID broadcast.

Observation 5:	With the above approach, it is the gNB-DU that finally makes the decision whether to share MBS Session resources, for both sharing scenarios, multiple Cell-ID and MOCN.

Proposal 6:	Also for MOCN, the gNB-DU needs to the same information as proposed for RAN sharing with multiple cell-ID broadcast, as proposed in Proposal 3.

For MOCN, there is the possibility to share F1-U resources among Broadcast MBS Session Contexts. For RAN sharing with multiple cell-ID broadcast this is not possible due to the lack of information exchange between the various logical gNBs.

Within a single gNB it is indeed not necessary to establish multiple F1-U resources providing identical content. It should be possible to associate multiple Broadcast MBS Session contexts with the same F1-U resources, however, explicit signalling should be introduced to control the F1-U resource sharing explicitly, not only by deducing it from comparing F1-U TNL addresses.

For MOCN, the gNB-CU could indicate whether sharing of F1-U resources is possible and request the gNB-DU to re-use an already established F1-U resource at the gNB-CU. The DU would need to respond successfully otherwise the procedure would fail (new resources to be allocated at the gNB-CU). If, during the lifetime of a broadcast MBS session, a Broadcast MBS Session context is released, the gNB-DU and the gNB-CU would not release that shared F1-U resource if one Broadcast MBS Session context remains using it.

Proposal 7:	Allow the gNB-CU to request the gNB-DU to share already established F1-U resources with other Broadcast MBS Session Contexts, if appropriate. The established shared F1-U resource would not be released while at least one Broadcast MBS Session Context remains using it.

The above proposal to share F1-U resources proposals above would require E1AP changes to be considered as well. The gNB-CU-UP would need to be informed explicitly whether F1-U terminations at the gNB-CU-UP/gNB-DU are shared among multiple Broadcast MBS Session Contexts / Broadcast Bearer Contexts.

The possibility to share resources would depend on whether the gNB-CU-UP terminates the shared NG-U resources. The gNB-CU-CP intending to apply F1-U sharing would first need to select a gNB-CU-UP that terminates NG-U for the BC session. In Annex G an approach is hinted, where the request to share resources and the subsequent grant is associated to both, NG-U and F1-U resources. Termination of the broadcast session of one PLMN would of course also terminate the provision of user data via F1-U. It is however an implementation matter to ensure sufficient redundancy, if so required by the service.

Further, E1AP does not provide any MBS Service Area information for a Area Session ID. So, at least for E1, an indication would need to be provided that indicates to the gNB-CU-UP which content is meant to provided to the F1-U tunnels associated to a certain (TMGI specific) Area Session ID. The Associated Session ID and the location dependent session associated via the Content ID is provided at BC Bearer Context Setup.

Proposal 8:	To allow the gNB to select a subset of shared NG-U resources, the gNB-CU-CP provides to the gNB-CU-UP the Associated Session ID and Content IDs, associated to the Area Session IDs in case of location dependent BC MBS sessions at BC Bearer Context Setup.

Proposal 9:	At Broadcast Session Setup of a BC MBS Session from another PLMN, the gNB may request “UP sharing” for both NG-U and F1-U resources.

## 2.3	Furthe impact to NGAP protocol functions along agreed SA2 content for TS 23.247

Status of discussions from last meeting:

Define a new class 1 procedure to allow the gNB/CU-CP requesting the setup of shared NG-U resources during an ongoing broadcast MBS session for unicast transport.

For the new procedure, FFS on the name of the message

Option 1:Broadcast Session Modification Required/Confirm/failure

Option 2:Broadcast Distribution Request/Response/failure or Broadcast session transport request/response/failure

Option 1 seems to be most appropriate, as it does not seem appropriate to introduce a concept from multicast as of option 2. This would result in something like the following: (only successful case illustrated, more in one of the Annexes.)

#### 8.17.x.2	Successful Operation

Figure 8.17.x.2-1: Broadcast Session Modification Required, successful operation.

The NG-RAN node initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUIRED message to the AMF. If the requested modification was successfully performed, the AMF shall respone with the BROADCAST SESSION MODIFICATION CONFIRM message.

Editor’s Note:	further specification text is FFS.

Proposal 10:	Define a new class 1 procedure ”Broadcast Session Modification Required” to allow the gNB requesting the setup of shared NG-U resources during an ongoing broadcast MBS session.

# 3	Conclusion and Proposals

We have been discussing information received from SA2 on current status of Rel-18 discussions on resource efficiency for MBS reception in RAN sharing scenarios and concluded with the following:

Observation 1: A unique association between cells and Area Sessions of different 5GC is not (always) possible if the gNB tries to identify identical content of location based MBS sessions delivered by different 5GCs on a per-cell basis only.

Proposal 2: 	Agree on the requirement that (the shared part of) MBS Service Area Information of Area Session IDs from different 5GC have to be identical and on the respective proposed text for 38.300 and 38.401 in the respective Annexes.

Proposal 3: 	For RAN sharing with multiple cell-ID broadcast F1AP shall support provision of the Associated Session ID and the MBS Service Area as received by the gNB over NG.

Proposal 4:	Agree, for MOCN and RAN sharing with multiple cell-ID broadcast, to establish separate Broadcast MBS Session Contexts in the gNB-DU, one per participating 5GC.

Observation 5:	With the above approach, it is the gNB-DU that finally makes the decision whether to share MBS Session resources, for both sharing scenarios, multiple Cell-ID and MOCN.

Proposal 6:	Also for MOCN, the gNB-DU needs to the same information as proposed for RAN sharing with multiple cell-ID broadcast, as proposed in Proposal 3.

Proposal 7:	Allow the gNB-CU to request the gNB-DU to share already established F1-U resources with other Broadcast MBS Session Contexts, if appropriate. The established shared F1-U resource would not be released while at least one Broadcast MBS Session Context remains using it.

Proposal 8:	To allow the gNB to select a subset of shared NG-U resources, the gNB-CU-CP provides to the gNB-CU-UP the Associated Session ID and Content IDs, associated to the Area Session IDs in case of location dependent BC MBS sessions at BC Bearer Context Setup.

Proposal 9:	At Broadcast Session Setup of a BC MBS Session from another PLMN, the gNB may request “UP sharing” for both NG-U and F1-U resources.

Proposal 10:	Define a new class 1 procedure to allow the gNB requesting the setup of shared NG-U resources during an ongoing broadcast MBS session.

Final Proposal 11: Agree on Text Proposals for 38.300, 38.410 and 38.413 as proposed in Annexes B-D.

Final Proposal 12: Agree on Text Proposals for 38.401, 38.473 and 37.483 as proposed in Annexs E-G.

# 4	References

[1]	reference

# Annex A	Text Proposal TS 38.000 [changes shown against latest endorsed text in R3-233782]

Not modified

#### 16.10.6.x	Support of Resource Sharing across multiple Broadcast MBS sessions in RAN Sharing Scenario

Editor’s Note: Support for Enhancement to improve the resource efficiency for broadcast reception in RAN sharing scenarios to be covered here.

NGAP supports resource sharing efficient scheme for broadcast delivery in RAN sharing. Such scheme enables the gNB to identify broadcast MBS sessions from different PLMNs providing identical content. The identification is based on information provided by the involved 5GCs in the Associated Session ID as specified in TS 23.247 [x]

If in the MBS Broadcast Setup Request message an Associated Session ID is received from a 5GC participating in RAN sharing, the gNB uses it to determine whether MBS Session resources can be shared with one or several broadcast MBS sessions associated with the same Associated Session ID requested from another 5GC participating in RAN sharing.

For location dependent MBS sessions, the gNB uses in addition the MBS service area information provided per Area Session ID per 5GC to decide whether MBS Session resources are shared. MBS Session resources for a location dependent MBS session should be only shared if the shared part of the MBS service area information provided per Area Session ID per 5GC are identical.

The gNB applying this resource efficiency scheme,

-	may decide whether NG-U resources are established towards all involved 5GCs or only some of them.

-	resolves different QoS requirements or different S-NSSAIs received from the participating 5GCs in an implementation specific way.

The gNB may also trigger the NGAP Broadcast Distribution Setup procedure (name FFS) towards one 5GC participating in RAN sharing to set up NG-U resources to maintain NG-U connectivity as specified in TS 23.247 [45].

not modified

# Annex B	Text Proposal TS 38.401 [changes shown against endorsed R3-233791]

------------Start of the First Change-----------------

----Not affected parts omitted----

### 7.7.x	Support of resource efficiency for RAN Sharing

#### 7.7.x.1	General

Editor’s Note:	TBD

#### 7.7.x.2	Support of resource efficiency for MOCN

Editor’s Note:	TBD

F1AP supports resource efficiency for MOCN.

For location dependent MBS sessions, the gNB-DU may apply MBS Session resource sharing for broadcast MBS sessions if the location dependent MBS Service Area information provided for the broadcast MBS session, are identical.

#### 7.7.x.3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

F1AP supports resource efficiency in RAN Sharing with multiple cell-ID broadcast.

gNB-DUs sharing the same physical cell resources receive via F1-C information enabling identifying broadcast MBS sessions providing identical content.

For location dependent MBS sessions, the gNB-DU may apply MBS Session resource sharing for broadcast MBS sessions if the location dependent MBS Service Area information, as received from the 5GC, provided for the broadcast MBS session, are identical.

While applying resource efficiency for RAN Sharing with multiple cell-ID broadcast

-	different QoS requirements received from the participating 5GCs are resolved in an implementation specific way.

-	for local or location dependent MBS sessions the respective MBS service area configurations provided by the participating 5GCs are observed.

------------End of the Change-----------------

# Annex C	Text Proposal TS 38.410

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.23	NR MBS Session Management Procedures

The following list of MBS Session management procedures are used to establish, release, or modify NG-RAN resources for a NR MBS session:

-	Broadcast Session Resource Setup;

-	Broadcast Session Modification;

-	Broadcast Session Modification Required;

-	Broadcast Session Resource Release;

-	Broadcast Session Resource Release Required;

-	Multicast Session Activation;

-	Multicast Session Deactivation;

-	Multicast Session Update;

-	Distribution Setup;

-	Distribution Release;

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex D	Text Proposal TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Modification Required | BROADCAST SESSION MODIFICATION REQUIRED | BROADCAST SESSION MODIFICATION CONFIRM | BROADCAST SESSION MODIFICATION REFUSE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.17.x1		Broadcast Session Modification Required

#### 8.17.x1.1	General

The purpose of the Broadcast Session Modification Required procedure is to trigger the modification of MBS session resources. The procedure uses non-UE associated signalling.

#### 8.17.x1.2	Successful Operation

Figure 8.17.x1.2-1: Broadcast Session Modification Required, successful operation.

The NG-RAN node initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUIRED message to the AMF. If the requested modification was successfully performed, the AMF shall respone with the BROADCAST SESSION MODIFICATION CONFIRM message.

Editor’s Note:	further specification text is FFS.

#### 8.17.x1.3	Unsuccessful Operation

Figure 8.17.2x1.3-1: Broadcast Session Modification Required, unsuccessful operation.

If the requested modification was not successfully performed, the AMF shall respone with the BROADCAST SESSION MODIFICATION REFUSE message.

Editor’s Note:	further specification text is FFS.

#### 8.17.x1.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.16.x2	BROADCAST SESSION MODIFICATION REQUIRED

This message is sent by the NG-RAN node to request the modification of an MBS session resource.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Session Modification Required Transfer | O |  | OCTET STRING | Containing the MBS Session Modification Required Transfer IE specified in subclause 9.3.5.y1 | YES | reject |

#### 9.2.16.x3	BROADCAST SESSION MODIFICATION CONFIRM

This message is sent by the AMF to confirm the request to modify an MBS session resource.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Session Modification Confirm Transfer | O |  | OCTET STRING | Containing the MBS Session Modification Confirm Transfer IE specified in subclause 9.3.5.y2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.16.x4	BROADCAST SESSION MODIFICATION REFUSE

This message is sent by the NG-RAN node to report the unsuccessful outcome of the request from the BROADCAST SESSION MODIFICATION REQUEST message.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Session Modification Refuse Transfer | O |  | OCTET STRING | Containing the MBS Session Modification Refuse Transfer IE specified in subclause 9.3.5.y3 | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.y4	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | OCTET STRING | Editor’s Note: coding is FFS |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.5.y1	MBS Session Modification Required Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session TNL Information NG-RAN | O |  | 9.3.2.17 |  | YES | reject |

#### 9.3.5.y2	MBS Session Setup or Modification Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session TNL Information 5GC | O |  | 9.3.2.15 |  |

#### 9.3.5.y3	MBS Session Modification Refuse Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFCPRelocationIndication,

AMFStatusIndication,

BroadcastSessionModificationFailure,

BroadcastSessionModificationRequest,

BroadcastSessionModificationResponse,

BroadcastSessionModificationRequired,

BroadcastSessionModificationConfirm,

BroadcastSessionModificationRefuse,

BroadcastSessionReleaseRequest,

BroadcastSessionReleaseRequired,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

FROM NGAP-PDU-Contents

id-AMFConfigurationUpdate,

id-AMFCPRelocationIndication,

id-AMFStatusIndication,

id-BroadcastSessionModification,

id-BroadcastSessionModificationRequired,

id-BroadcastSessionRelease,

id-BroadcastSessionReleaseRequired,

id-BroadcastSessionSetup,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionModificationRequired	|

broadcastSessionRelease			|

broadcastSessionSetup			|

distributionSetup				|

distributionRelease				|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

multicastSessionActivation					|

multicastSessionDeactivation				|

multicastSessionUpdate			|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

uEContextModification			|

uEContextRelease				|

uEContextResume								|

uEContextSuspend							|

uERadioCapabilityCheck			|

uERadioCapabilityIDMapping		|

writeReplaceWarning,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

broadcastSessionModification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionModificationRequest

SUCCESSFUL OUTCOME		BroadcastSessionModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionModificationFailure

PROCEDURE CODE			id-BroadcastSessionModification

CRITICALITY				reject

}

broadcastSessionModificationRequired NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionModificationRequired

SUCCESSFUL OUTCOME		BroadcastSessionModificationConfirm

UNSUCCESSFUL OUTCOME	BroadcastSessionModificationRefuse

PROCEDURE CODE			id-BroadcastSessionModificationRequired

CRITICALITY				reject

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-DistributionSetupResponseTransfer,

MBS-DistributionSetupUnsuccessfulTransfer,

MBSSessionModificationRequiredTransfer

MBSSessionModificationConfirmTransfer

MBSSessionModificationRefuseTransfer

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-AMF-TNLAssociationToUpdateList,

id-AMFTrafficLoadReductionIndication,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-AuthenticatedIndication,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MBS-DistributionSetupResponseTransfer,

id-MBS-DistributionSetupUnsuccessfulTransfer,

id-MBSSessionModificationFailureTransfer,

id-MBSSessionModificationRequestTransfer,

id-MBSSessionModificationResponseTransfer,

id-MBSSessionModificationConfirmTransfer,

id-MBSSessionModificationRefuseTransfer,

id-MBSSessionModificationRequiredTransfer,

id-MBSSessionReleaseResponseTransfer,

id-MBSSessionSetupFailureTransfer,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- Broadcast Session Modification Required Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST SESSION MODIFICATION REQUIRED

--

-- **************************************************************

BroadcastSessionModificationRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationRequiredIEs} },

...

}

BroadcastSessionModificationRequiredIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionModificationRequiredTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionModificationRequiredTransfer)		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION MODIFICATION CONFIRM

--

-- **************************************************************

BroadcastSessionModificationConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationConfirmIEs} },

...

}

BroadcastSessionModificationResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionModificationConfirmTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionModificationconfirmTransfer)	PRESENCE optional 	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION MODIFICATION REFUSE

--

-- **************************************************************

BroadcastSessionModificationRefuse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationRefuseIEs} },

...

}

BroadcastSessionModificationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionModificationRefuseTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionModificationRefuseTransfer)	PRESENCE optional	}|

{ ID id-Cause												CRITICALITY ignore	TYPE Cause												PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-ActiveSessionInformation-TargettoSourceList ::= SEQUENCE (SIZE(1..maxnoofMBSSessionsofUE)) OF MBS-ActiveSessionInformation-TargettoSourceItem

MBS-ActiveSessionInformation-TargettoSourceItem ::= SEQUENCE {

mBS-SessionID 						MBS-SessionID,

mBS-DataForwardingResponseMRBList	MBS-DataForwardingResponseMRBList													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBS-ActiveSessionInformation-TargettoSourceItem-ExtIEs} }		OPTIONAL,

...

}

MBS-ActiveSessionInformation-TargettoSourceItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionModificationRequiredTransfer ::= SEQUENCE {

mBS-SessionTNLInfoNGRAN 	MBS-SessionTNLInfoNGRAN															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBSSessionModificationRequiredTransfer-ExtIEs} }		OPTIONAL,

...

}

MBSSessionModificationRequiredTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionModificationConfirmTransfer ::= SEQUENCE {

mBS-SessionTNLInfo5GC	MBS-SessionTNLInfo5GC															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBSSessionModificationConfirmTransfer-ExtIEs} }		OPTIONAL,

...

}

MBSSessionModificationConfirmTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionModificationRefuseTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBSSessionModificationRefuseTransfer-ExtIEs} }	OPTIONAL,

...

}

MBSSessionModificationRefuseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupOrModFailureTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBSSessionSetupOrModFailureTransfer-ExtIEs} }	OPTIONAL,

...

}

MBSSessionSetupOrModFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupResponseList ::= SEQUENCE (SIZE(1.. maxnoofMBSSessions)) OF MBSSessionSetupResponseItem

MBSSessionSetupResponseItem ::= SEQUENCE {

mBS-SessionID			MBS-SessionID,

mBS-AreaSessionID		MBS-AreaSessionID													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBSSessionSetupResponseItem-ExtIEs} }	OPTIONAL,

...

}

MBSSessionSetupResponseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupOrModRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MBSSessionSetupOrModRequestTransferIEs} },

...

}

MBSSessionSetupOrModRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionTNLInfo5GC				CRITICALITY reject	TYPE MBS-SessionTNLInfo5GC				PRESENCE	optional		}|

{ ID id-MBS-QoSFlowsToBeSetupModList		CRITICALITY reject	TYPE MBS-QoSFlowsToBeSetupList		PRESENCE	mandatory	}|

{ ID id-MBS-SessionFSAIDList				CRITICALITY ignore	TYPE MBS-SessionFSAIDList				PRESENCE	optional		},

...

}

MBS-SessionFSAIDList ::= SEQUENCE (SIZE(1.. maxnoofMBSFSAs)) OF MBS-SessionFSAID

MBS-SessionFSAID ::= OCTET STRING (SIZE(3))

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-DistributionSetup 						ProcedureCode ::= 69

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-BroadcastSessionModification				ProcedureCode ::= 99 -- to be allocated

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-AssociatedSessionID									ProtocolIE-ID ::= 998 -- to be allocated

id-SharedNGUNotEstablished								ProtocolIE-ID ::= 999 -- to be allocated

id-MBS-AssistanceInformation							ProtocolIE-ID ::= 1000 -- to be allocated

id-AssociatedSessionID									ProtocolIE-ID ::= 1001 -- to be allocated

id-MBSSessionModificationRequiredTransfer				ProtocolIE-ID ::= 1002 -- to be allocated

id-MBSSessionModificationConfirmTransfer				ProtocolIE-ID ::= 1003 -- to be allocated

id-MBSSessionModificationRefuseTransfer					ProtocolIE-ID ::= 1004 -- to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex F	Text Proposal TS 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.14.1	Broadcast Context Setup

#### 8.14.1.1	General

The purpose of the Broadcast Context Setup procedure is to establish an MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.1.2	Successful Operation

Figure 8.14.1.2-1: Broadcast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending BROADCAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the broadcast MBS Session context, it replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

If the Associated Session ID IE is included in the BROADCAST CONTEXT SETUP REQUEST message the gNB-DU node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.401 [4].

If the gNG-CU includes in the BROADCAST CONTEXT SETUP REQUEST message in the Broadcast MRB To Be Setup List IE the F1-U Sharing Requested IE set to "sharing requested" the gNB-DU shall, if supported re-use, if already allocated, existing F1-U resources as identified by the BC Bearer Context F1-U TNL Info at CU IE and include the F1-U Sharing Granted IE set to "sharing granted" in the Broadcast MRB Setup List IE and the BC Bearer Context F1-U TNL Info at DU IE indicating the existing F1-U resources.

The gNB-DU shall report to the gNB-CU, in the BROADCAST CONTEXT SETUP RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Setup List IE is contained in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be setup with an appropriate cause value for each Broadcast MRB failed to setup.

#### 8.14.1.3	Unsuccessful Operation

Figure 8.14.1.3-1: Broadcast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the requested MBS session context it shall consider the procedure as failed and reply with the BROADCAST CONTEXT SETUP FAILURE message.

If the gNG-CU includes in the BROADCAST CONTEXT SETUP REQUEST message in the Broadcast MRB To Be Setup List IE the F1-U Sharing Requested IE set to "sharing requested" and the gNB-DU decides to not share F1-U resources, it shall fail the Broadcast Context Setup procedure.

#### 8.14.1.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.14.4	Broadcast Context Modification

#### 8.14.4.1	General

The purpose of the Broadcast Context Modification procedure is to modify an established MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.4.2	Successful Operation

Figure 8.14.4.2-1: Broadcast Context Modification procedure. Successful operation

The BROADCAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and, if successful, report the update in the BROADCAST CONTEXT MODIFICATION RESPONSE message.

If the Associated Session ID IE is included in the BROADCAST CONTEXT MODIFICATION REQUEST message the gNB-DU node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.401 [4].

If the gNG-CU includes in the BROADCAST CONTEXT MODIFICATION REQUEST message in the Broadcast MRB To Be Setup List IE the F1-U Sharing Requested IE set to "sharing requested" the gNB-DU shall, if supported re-use, if already allocated, existing F1-U resources as identified by the BC Bearer Context F1-U TNL Info at CU IE and include the F1-U Sharing Granted IE set to "sharing granted" and the BC Bearer Context F1-U TNL Info at DU IE indicating the existing F1-U resources in the Broadcast MRB Setup List IE.

If the gNG-CU includes in the BROADCAST CONTEXT MODIFICATION REQUEST message in the Broadcast MRB To Be Modified List IE the BC Bearer Context F1-U TNL Info at CU IE and the F1-U Sharing Requested IE set to "sharing requested" the gNB-DU shall, if supported re-use, if already allocated, existing F1-U resources as identified by the BC Bearer Context F1-U TNL Info at CU IE and include the F1-U Sharing Granted IE set to "sharing granted" and the BC Bearer Context F1-U TNL Info at DU IE indicating the existing F1-U resources in the Broadcast MRB Modified List IE.

If the Broadcast MRB To Be Setup List IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall setup the corresponding resources for the requested MRB(s), and report to the gNB-CU, in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Be Setup List IE is contained in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the setup of the indicated MRB(s) as failed and indicate the reason for the failure with an appropriate cause value for each MRB failed to be setup.

If the Broadcast MRB To Be Modified List IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall update the corresponding context and resources for the requested MRB(s), and report to the gNB-DU, in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the modification result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully modified shall be included in the Broadcast MRB Modified List IE;

-	A list of MRBs which failed to be modified shall be included in the Broadcast MRB Failed To Be Modified List IE;

If the Broadcast MRB Failed To Be Modified List IE is contained in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be modified with an appropriate cause value for each Broadcast MRB failed to modify.

#### 8.14.4.3	Unsuccessful Operation

Figure 8.14.4.3-1: Broadcast Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the broadcast context can be successfully performed, the gNB-DU shall respond with the BROADCAST CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

If the gNG-CU includes in the BROADCAST CONTEXT MODIFICATION REQUEST message in the Broadcast MRB To Be Setup List IE the F1-U Sharing Requested IE set to "sharing requested" and the gNB-DU decides to not share F1-U resources, it shall fail the Broadcast Context Setup procedure.

#### 8.14.4.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.13.1	BROADCAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a broadcast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| >>F1-U Sharing Requested | O |  | 9.3.1.x3 |  | YES | ignore |
| Associated Session ID | O |  | 9.3.1.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.13.2	BROADCAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| >>F1-U Sharing Granted | O |  | 9.3.1.x4 |  | YES | ignore |
| CHOICE BroadcastAreaScope | M |  |  |  | YES | ignore |
| >CompleteSuccess |  |  | NULL |  |  |  |
| >PartialSuccess |  |  |  |  |  |  |
| >>Broadcast Cell List |  | 1 |  |  |  |  |
| >>>Broadcast Cell Item |  | 1 .. <maxcellingNBDU> |  |  |  |  |
| >>>>Cell ID | M |  | NR CGI 9.3.1.12 | Identifier of the cells that establish the broadcast service successfully |  |  |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxcellingNBDU | Maximum no. of cells which establish the MRBs successfully in one DU, the maximum value is 512 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.13.6	BROADCAST CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to modify broadcast context information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 | Overwrites any previously received MBS Service Area information | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| >>F1-U Sharing Requested | O |  | 9.3.1.x3 |  | YES | ignore |
| Broadcast MRB To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | O |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 0 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | Updated gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| >>F1-U Sharing Requested | O |  | 9.3.1.x3 |  | YES | ignore |
| Broadcast MRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.13.7	BROADCAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| >>F1-U Sharing Granted | O |  | 9.3.1.x4 |  | YES | ignore |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | Updated gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| >>F1-U Sharing Granted | O |  | 9.3.1.x4 |  | YES | ignore |
| Broadcast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.222	MBS Service Area

This IE contains the MBS service area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Session Type | M |  |  |  | - |  |
| >location independent |  |  |  |  | - |  |
| >>MBS Service Area Information | M |  | 9.3.1.223 |  | - |  |
| >location dependent |  |  |  |  | - |  |
| >>MBS Service Area Information Location Dependent List |  | 1..maxnoofMBSServiceArea Information |  |  | - |  |
| >>>MBS Area Session ID | M |  | 9.3.1.221 |  | - |  |
| >>>MBS Service Area Information | M |  | 9.3.1.223 |  | - |  |
| >>>MBS Service Area Information from 5GC | O |  | 9.3.1.x2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSServiceAreaInformation | Maximum no. of MBS Service Area Information elements in the MBS Service Area Information Location Dependent List IE. Value is 256 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | OCTET STRING | Editor’s Note: coding is FFS |

#### 9.3.1.x2	MBS Service Area Information from 5GC

This IE contains MBS Service Area information for location dependent MBS sessions as received by the gNB from 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Information from 5GC | M |  | OCTET STRING | Editor’s Note: coding is FFS |

#### 9.3.1.x3	F1-U Sharing Requested

This IE is used by the gNB-CU to indicate that F1-U resource sharing is requested. In the present version of this specification this is used for broadcast MBS session resources in the context of RAN sharing only.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| F1-U Sharing Requested | O |  | ENUMERATED (sharing requested, ...) |  |

#### 9.3.1.x4	F1-U Sharing Granted

This IE is used by the gNB-DU to indicate that the request for sharing F1-U resources was granted. In the present version of this specification this is used for broadcast MBS session resources in the context of RAN sharing only.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| F1-U Sharing Granted | O |  | ENUMERATED (sharing granted, ...) |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex G	Text Proposal TS 37.483

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.5.1.1	BC BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup MBS session resources for a broadcast MBS session.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.106 |  | YES | reject |
| Global MBS Session ID | M |  | 9.3.1.108 |  | YES | reject |
| BC Bearer Context To Setup | M |  | 9.3.3.26 |  | YES | reject |
| Associated Session ID | O |  | 9.3.1.x1 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.112	BC Bearer Context NG-U TNL Info at 5GC

This IE contains TNL information for an MBS Session as provided by the 5GC for both, shared NG-U multicast and unicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHOICE MBS Session Type |  |  |  |  |  |  |  |  |  | - |  |  |  |
| >location independent |  |  |  |  |  |  |  |  |  | - |  |  |  |
| >>MBS NG-U Information at 5GC |  | M |  |  |  | 9.3.1.113 |  |  |  | - |  |  |  |
| >location dependent |  |  |  |  |  |  |  |  |  | - |  |  |  |
| >>Location dependent MBS NG-U Information at 5GC |  |  |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |  |  |  | - |  |  |  |
| >>>MBS Area Session ID |  | M |  |  |  | 9.3.1.111 |  |  |  | - |  |  |  |
| >>MBS NG-U Information at 5GC |  | M |  |  |  | 9.3.1.113 |  |  |  | - |  |  |  |
| UP Sharing Requested |  |  | 9.3.1.x3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.116	BC Bearer Context NG-U TNL Info at NG-RAN

This IE contains NG-RAN NG-U TNL information for an MBS Session for both, shared NG-U unicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHOICE MBS Session Type |  | M |  |  |  |  |  |  |  | - |  |  |  |
| >location independent |  |  |  |  |  |  |  |  |  | - |  |  |  |
| >>MBS NG-U Information at NG-RAN |  | M |  |  |  | 9.3.1.117 |  |  |  | - |  |  |  |
| >location dependent |  |  |  |  |  |  |  |  |  | - |  |  |  |
| >>Location dependent MBS NG-U Information at NG-RAN |  |  |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |  |  |  | - |  |  |  |
| >>>MBS Area Session ID |  | M |  |  |  | 9.3.1.111 |  |  |  | - |  |  |  |
| >>MBS NG-U Information at NG-RAN |  | M |  |  |  | 9.3.1.117 |  |  |  | - |  |  |  |
| UP Sharing Granted |  |  | 9.3.1.x4 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.118	BC Bearer Context F1-U TNL Info at CU

This IE contains gNB-CU UP F1-U TNL information for an MBS Session. It may also contain per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS F1-U Information at CU | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS F1-U Information at CU |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.111 |  |
| >>MBS F1-U Information at CU | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.119	BC Bearer Context F1-U TNL Info at DU

This IE contains CU F1-U TNL information for an MBS Session. It may also contain per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS F1-U Information at DU |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.111 |  |
| >>MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | OCTET STRING | Editor’s Note: coding is FFS |

#### 9.3.1.x3	UP Sharing Requested

This IE is used by the gNB-CU-CP to indicate that UP resource sharing is requested. In the present version of this specification this is used for broadcast MBS session resources in the context of RAN sharing only.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UP Sharing Requested | O |  | ENUMERATED (sharing requested, ...) |  |

#### 9.3.1.x4	UP Sharing Granted

This IE is used by the gNB-CU-UP to indicate that the request for sharing UP resources was granted. In the present version of this specification this is used for broadcast MBS session resources in the context of RAN sharing only.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UP Sharing Granted | O |  | ENUMERATED (sharing granted, ...) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.26	BC Bearer Context To Setup

This IE contains MBS session resource related information used to request BC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  |
| BC Bearer Context NG-U TNL Info at 5GC | M |  | 9.3.1.112 |  |
| BC MRB To Setup List | M |  | BC MRB Setup Configuration 9.3.1.114 |  |
| Requested Action for Available Shared NG-U Termination | O |  | 9.3.1.115 |  |

#### 9.3.3.27	BC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm BC Bearer Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.116 |  |
| BC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >BC Bearer Context F1-U TNL Info at CU | M |  | 9.3.1.118 |  |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.114 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

#### 9.3.3.28	BC Bearer Context To Modify

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at 5GC To Setup or Modify | O |  | BC Bearer Context NG-U TNL Info at 5GC 9.3.1.112 |  |
| BC MRB To Setup List | O |  | BC MRB Setup Configuration 9.3.1.114 |  |
| BC MRB To Modify List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >BC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.119 |  |
| >SDAP Configuration | O |  | 9.3.1.39 |  |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 |  |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |
| BC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

#### 9.3.3.29	BC Bearer Context To Modify Response

This IE contains MBS session resource related information used to confirm a BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.116 |  |
| BC MRB Setup or Modify Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MBS QoS Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >BC Bearer Context F1-U TNL Info at CU | O |  | 9.3.1.118 |  |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.114 | In case the shared MBS NG-U termination had a different MRB Configuration applied. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>