3GPP TSG-RAN WG3 Meeting #121	R3-234237

Toulouse, France, 21st – 25th August 2023

Agenda Item:	31.2

Source:	Ericsson

Title:	On MRB PDCP COUNT wrap around including [draft CRs for several TSs]

Document for:	Discussions & Approval

# 1	Introduction

We had discussions in the past on issues related to proper initialisation of MRB PDCP entities with regards to the PDCP RX-DELIV state variable.

The non-availability of initial PDCP RX-DELIV for MRBs has been agreed at RAN#120 to be solveable by means of existing protocol functions. MRB PDCP COUNT wrap around is still to be discussed along TEI18.

This document continues discussions from last meeting  and in the Annexes you can see a potential implementation on stage 2 and stage 3 level how to implement the approach in RAN3 specifications

# 2	Discussion

## 2.1	General

For NR MBS Rel-17 has introduced a scheme that allows synchronisation of of MRB PDCP state variables in neighbouring cells/gNBs in order to allow minimisation of data loss and duplication avoidance.

One can describe this scheme by a single UP entity placed in a central place in 5GS that performs the allocation of sequence numbers to multicast user data from which PDCP state variables are derived. There are two schemes introduces in Rel-17:

The UP entity allocating sequence number from which PDCP state variable are derived is placed either

(a)	In the MB-UPF:

-	The MB-UPF assignes to each multicast user data packet a per (MBS) QFI generated sequence number, the NG-U DL MBS QFI SN (see TS 38.415) and distributes this information to all gNBs to which a shared NG-U transport is established.

-	The gNBs map the MBS QoS flows to MRBs and associates each PDCP PDU with a COUNT value resulting from the accumulated NG-U DL MBS QFI SNs of the QoS flows mapped to the MRB.

(b)	In a UP entity shared by all gNBs (“shared NG-U termination”) in the region within which minimisation of data loss and duplication avoidance shall be applied

-	The gNBs sharing the NG-U termination are provided with the same PDCP COUNT value assigned by the that shared NG-U termination.

(a/b)	Or a combination of both.

We discussed scenarios where vertical and horizontal synchronisation of PDCP state variables is not possible.

-	“vertical“ de-synchronisation:

-	this is the case for which discussions were closed at RAN3#120, i.e. it is possible, if the UE cannot be configured appropriately based on user data packets received from the MB-UPF, the network can reconfigure the UE once that information is available.

-	“horizontal” de-synchronisation:

-	if neighbouring gNBs (allowing NG/Xn mobility in between the served cells) have initialised their PDCP entities and configured with different PDCP state variables, e.g. due to availability of initialRX-DELIV related information in one gNB, unavailability in another gNB, mobility between these two gNBs cannot support minimisation of data loss and duplication avoidance. It is important to deal with this situation properly.

## 2.2	Proposed Strategies

### 2.2.1	General

One of the fundamental aspects we would like to see being followed is to avoid the 5GC to deal with RAN internal aspects, especially when it comes to very peculiar details of RRC signalling and setting of PDCP state variables. We therefore propose to keep solutions for dealing with PDCP de-synchronisation aspects completely NG-RAN internal.

Proposal 1:	We propose to keep solutions for dealing with desynchronisation of MRB PDCP state variables completely within NG-RAN.

As highlighted during the RAN3#119bis-e discussions, de- and re-synchronising of MRB PDCP state variables among gNBs deriving MRB PDCP state variables directly form SN info provided by the MB-UPF can be achieved by temporarily establishing two sets of MRB PDCP entities at the involved gNBs. For dealing with PDCP COUNT wrap around this does not necessarily mean that gNBs that lower layer resources for these MRBs have to be duplicated.

### 2.2.2	Strategy for avoiding COUNT wrap around:

The proposed strategy to avoid COUNT wrap around is is as follows:

-	establish a second (new) set of MRBs once the PDCP COUNT has exhausted the (first) half of its value range, i.e. the Most Significant Bit (MSB) has changed from 0 to 1, and to

-	initialize the PDCP entities of the second(new) set of MRBs with PDCP COUNT values derived from the “DL MBS QFI Sequence Number” with the appropriate modulo operation so that the MSB of the derived PDCP COUNT equals 0.

-	After all UEs have been re-configured, one by one, with the second (new) MRBs the old set of MRBs is released.

## 2.3	Resulting impacts to RAN3 specifications

Without going into details, which can be seen in the message flows in Annex A (TP for 38.401)  the following main aspects need to be translated into protocol functions on NG, Xn, F1 and E1:

1.	Retrieving progress information needs to allow to distinguish between SN information stemming from MB-UPF generated information and the COUNT value actually used in the PDCP entity. (E1)

2.	Allowing the gNB-CU(-CP) to request the setup of new MRB resources within the same Multicast Context, even if MRBs for the same MBS QoS flows have been established already. (E1, F1)

3.	Allowing the gNB at MRB setup to set the PDCP state variables (COUNT) to either an absolute value or to a value relative to the MB-UPF derived sequence numbers.

4.	Exchange of COUNT values (instead of PDCP SNs) at Xn/NG based mobility to allow the new/target gNB determining whether the PDCP state variable are synchronised (NG, Xn)

5.	Allowing the gNB-CU to request from the gNB-DU to associated existing lower layer resources for the new MRBs (typically the case for ptm resources) or establishing new lower layer resources (typically the case for ptp resources); the gNB-CU should be able to determine which resources to request.

# 3	Conclusion and Proposal

It is proposed to follow the following guiding principles:

Proposal 1:	We propose to keep solutions for dealing with desynchronisation of MRB PDCP state variables completely within NG-RAN.

Proposal 2:	Agree on the following protocol additions identified during the discussion and depicted in the draft CRs in the Annexes:

2.1.	E1 changes to retrieve progress information allowing to distinguish between SN information stemming from MB-UPF generated information and the COUNT value actually used in the PDCP entity.

2.2.	E1 and F1 changes allowing the gNB-CU(-CP) to explicitly indicate the request the setup of new MRB resources, even if MRBs for the same MBS QoS flows have been established already.

2.3.	E1 changes allowing the gNB-CU-CP at MRB setup to set the PDCP state variables (COUNT) to either an absolute value or to a value relative to the MB-UPF derived sequence numbers.

2.4.	Enhance NG and Xn to enable exchange of COUNT values (instead of (only) PDCP SNs) at Xn/NG based mobility to allow the new/target gNB determining whether the PDCP state variable are synchronised

Final Proposal:	It is proposed to discuss the strategies and basic protocol impacts in section 2 and take the draft CRs in the Annexes as a starting point for discussions.

# Annex A:	Draft CR for TS 38.401

3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | <CR#> | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction stage-2 description how to support handling of MRB PDCP COUNT wrap around for a multicast MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It has been acknowledged that MRB PDCP COUNT wrap around shall be avoided for multicast MBS sessions by means of standardised protocol functions on Xn, F1 and E1 interfaces. This CR introduces stage-2 specification how MRB PDCP COUNT wrap around can be handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It has been acknowledged that MRB PDCP COUNT wrap around shall be avoided and handled by means of standardised functions on Xn, F1 and E1 interfaces. This CR introduces stage-2 specification how MRB PDCP COUNT wrap around can be handled. During the operation of MRB resources established for a multicast MBS session the status of MRB PDCP COUNT values are observed. When the PDCP COUNT value of an MRB reaches a certain value, e.g. 231 (the most significant bit is set to “1”, i.e. half of the numbering space has been exhausted), the gNB(-CU-CP) establishes (at the gNB-CU-UP) additional ("new") UP resources for the QoS flows mapped to the "old" MRB, initialised with a COUNT value with HFN set to a low initial value. ( In case the MRB PDCP COUNT is derived from information received via NG-U, the new MRB could be initialised to a value [(NG-U derived COUNT) modulo 231]. In case the MRB PDCP COUNT is derived from local gNB information, the gNB could wait for COUNT to be almost exhausted and set the count of the new MRB to 0. ) The Multicast Session Context at the gNB-DU is modified to add the "new" MRB. The UE Contexts at the gNB-DU is modified to release the "old" MRB and add the "new" MRB. F1-U resources are added, if necessary and then the UE is RRC reconfigured to release the "old" MRB and add the "new" MRB. F1-U are released, and resources for the "old" MRB are finally released. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of MRB PDCP COUNT wrap around would not be possible by means of standardised protocol functions on Xn, F1 and E1 interfaces. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.483 CRxxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 8.15.1.y	Handle of COUNT wrap around by Releasing an old MRB and adding a new MRB to during ongoing Multicast MBS Session

Figure 8.15.1.y-1 illustrates an examplified interaction of NGAP, E1AP, F1AP and RRC protocol functions for handling of COUNT wrap around:

Figure 8.15.1.y-1: Multicast MBS Session Context establishment

1.	E1AP MC Bearer Context Modification allows the gNB-CU-CP to keep track of the current MRB PDCP COUNT. At a certain point in time, the gNB-CU-CP decides  to reconfigure all UEs with new MRBs with a low PDCP COUNT.

NOTE 1:	It is assumed that the gNB triggers MRB COUNT wrap around handling once the MRB PDCP COUNT reaches the value of  231 (the most significant bit is set to “1”, i.e. half of the numbering space has been exhausted)

2.	New UP entities (new MRB IDs, low PDCP COUNT value) are requested from the gNB-CU-UP.

NOTE 2:	It is assumed that the gNB assigns for the new MRBs an PDCP COUNT value that differs only in the most significant bit (set to “0”) from the PDCP COUNT values of the old MRBs.

3.	The Multicast Session Context in the gNB-DU is modified accordingly, indicating to add “new” MRBs and the necessity to also allocate lower layer resources, if applicable.

4.	The UE Context in the gNB-DU are modified accordingly.

5.	New F1-U resources are setup for the new MRBs.

6.	The UE is reconfigured with the new MRBs being added and the old MRBs released.

7.	Old F1-U resources are removed and MRB resources in the gNB-CU-UP removed.

8.	The Multicast Session Context in the gNB-DU is modified to remove the respective MRB resources.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex B:	Draft CR for TS 38.413

3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | <CR#> | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of de-synchronised PDCP COUNT of MRBs at NG based mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Exchange of MRB progress information over NG at NG based mobility currently only contains PDCP Sequence Numbers, i.e. not including the PDCP HFN. If gNBs are re-configuring UEs with a new MRB for which the PDCP COUNT is different than the one for the old MRB, the PDCP SN might not be sufficient to allow minimisation of data loss during NG based mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the option to provide the full COUNT value within the MRB progress information. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it corrects the MRB progress information to include the possibility providing the full PDCP COUNT value as well. The impact can be considered isolated because the change affects the NG function enabling minimisation of data loss during NG based mobility in case the UE is configured with MRBs only. The CR has impact from protocol and functional point of view and is realised in a backwards compatible way. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG based mobility would not support minimisation of data loss for MRBs if gNBs are re-configuring UEs with a new MRB for which the PDCP COUNT is different than the one for the old MRB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.2, 9.3.1.219, 9.3.1.x1, 9.3.1.x2, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.4.1	Handover Preparation

#### 8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the 5GC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE. The procedure uses UE-associated signalling.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source NG-RAN node initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving AMF. When the source NG-RAN node sends the HANDOVER REQUIRED message, it shall start the timer TNGRELOCprep. The source NG-RAN node shall indicate the appropriate cause value for the handover in the Cause IE.

Upon reception of the HANDOVER REQUIRED message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transparently transfer the Handover Required Transfer IE to the SMF associated with the concerned PDU session.

In case of intra-system handover, the information in the Source to Target Transparent Container IE shall be encoded according to the definition of the Source NG-RAN node to Target NG-RAN node Transparent Container IE.

If the DL Forwarding IE is included for a given QoS flow in the PDU Session Resource Information Item IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message and it is set to "DL forwarding proposed", it indicates that the source NG-RAN node proposes forwarding of downlink data for that QoS flow.

If the UL Forwarding IE is included for a given QoS flow in the PDU Session Resource Information Item IE within the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE of the HANDOVER REQUIRED message and it is set to "UL forwarding proposed", it indicates that the source NG-RAN node proposes forwarding of uplink data for that QoS flow.

If the DRBs to QoS Flows Mapping List IE is included in the PDU Session Resource Information Item IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message, it implicitly indicates that the source NG-RAN node proposes forwarding of downlink data for those DRBs.

If the QoS Flow Mapping Indication IE for a QoS flow is included in the Associated QoS Flow List IE within the DRBs to QoS Flows Mapping List IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message, it indicates that the source NG-RAN node has mapped only the uplink or downlink of the QoS flow to the DRB.

The source NG-RAN node shall, for each MRB of each MBS session contained in the MBS Session Information Target to Source List IE, start data forwarding to the TNL address contained in the DL Forwarding UP TNL Information IE. If the MRB Progress Information IE is contained for an MRB in the Data Forwarding Response MRB List IE in the MBS Session Information Target to Source List IE, the source NG-RAN node may use this information

-	to decide whether and how to apply data forwarding for that MRB and whether to establish respective resources

-	to determine when to stop data forwarding.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.219	MRB Progress Information

This IE contains the MRB progress information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE MRB Progress Information | M |  |  |  | - |  |
| >PDCP SN 12bits |  |  |  |  | - |  |
| >>PDCP SN Length 12 | M |  | INTEGER (0..4095) |  | - |  |
| >PDCP SN 18bits |  |  |  |  | - |  |
| >>PDCP SN Length 18 | M |  | INTEGER (0..262143) |  | - |  |
| >COUNT SN 12bits |  |  |  |  | YES | ignore |
| >>COUNT Value for PDCP SN Length 12 | M |  | 9.3.1.x1 |  | - |  |
| >COUNT SN 18bits |  |  |  |  | YES | ignore |
| >>COUNT Value for PDCP SN Length 18 | M |  | 9.3.1.x2 |  | - |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	COUNT Value for PDCP SN Length 12

This information element indicates the 12-bit long PDCP sequence number and the corresponding 20 bits long Hyper Frame Number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP-SN Length 12 | M |  | INTEGER (0..4095) |  |
| HFN for PDCP-SN Length 12 | M |  | INTEGER (0..1048575) |  |

#### 9.3.1.x2	COUNT Value for PDCP SN Length 18

This information element indicates the 18-bit long PDCP sequence number and the corresponding 14 bits long Hyper Frame Number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP-SN Length 18 | M |  | INTEGER (0..262143) |  |
| HFN for PDCP-SN Length 18 | M |  | INTEGER (0..16383) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-COUNT-SN12,

id-COUNT-SN18,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

COUNT-PDCP-SN12 ::= SEQUENCE {

pdcp-SN12						INTEGER (0..4095),

hfn-PDCP-SN12					INTEGER (0..1048575),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN12-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN12-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

COUNT-PDCP-SN18 ::= SEQUENCE {

pdcp-SN18						INTEGER (0..262143),

hfn-PDCP-SN18					INTEGER (0..16383),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN18-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN18-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MRB-ProgressInformation ::= CHOICE {

pDCP-SN-Length12				INTEGER (0..4095),

pDCP-SN-Length18				INTEGER (0..262143),

choice-Extensions		ProtocolIE-SingleContainer { { MRB-ProgressInformation-ExtIEs} }

}

MRB-ProgressInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ID id-COUNT-SN12  CRITICALITY ignore  TYPE COUNT-PDCP-SN12		PRESENCE mandatory}|

{ID id-COUNT-SN18  CRITICALITY ignore  TYPE COUNT-PDCP-SN18		PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-COUNT-SN12											ProtocolIE-ID ::= 998 -- to be allocated

id-COUNT-SN18											ProtocolIE-ID ::= 999 -- to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex B:	Draft CR for TS 38.423

3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | <CR#> | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of de-synchronised PDCP COUNT of MRBs at Xn based mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Exchange of MRB progress information over Xn at Xn based mobility currently only contains PDCP Sequence Numbers, i.e. not including the PDCP HFN. If gNBs are re-configuring UEs with a new MRB for which the PDCP COUNT is different than the one for the old MRB, the PDCP SN might not be sufficient to allow minimisation of data loss during Xn based mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the option to provide the full COUNT value within the MRB progress information. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it corrects the MRB progress information to include the possibility providing the full PDCP COUNT value as well. The impact can be considered isolated because the change affects the Xn function enabling minimisation of data loss during Xn based mobility in case the UE is configured with MRBs only. The CR has impact from protocol and functional point of view and is realised in a backwards compatible way. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Xn based mobility would not support minimisation of data loss for MRBs if gNBs are re-configuring UEs with a new MRB for which the PDCP COUNT is different than the one for the old MRB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.4.2, 9.2.3.147, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the MBS Session Information List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, establish an NG-RAN MBS session resources context as specified in TS 23.247 [46] and TS 38.300 [9], if applicable.

If the HANDOVER REQUEST message includes the MBS Area Session ID IE, the target NG-RAN, if supported, shall use this information as an indication from which MBS Area Session ID the UE is handed over. For each MBS session for which the Active MBS Session Information IE is included in the MBS Session Information Item List IE, the target NG-RAN shall, if supported, use this information to setup respective MBS session resources. The target NG-RAN node shall, if supported, consider that the MBS sessions for which the Active MBS Session Information IE is not included are inactive.

If the HANDOVER REQUEST ACKNOWLEDGE message contains in the MBS Session Information Response List IE the MBS Data Forwarding Response Info IE that the source NG-RAN node shall use the information for forwarding MBS traffic to the target NG-RAN node.

If the MBS Session Associated Information List IE is included in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the information contained in the Associated QoS Flows Information List IE as specified in TS 23.247 [46].

For each MRB indicated in the MBS Mapping and Data Forwarding Request Info from source NG-RAN node IE, the target NG-RAN node shall use the MRB ID IE and, if included, the MRB Progress Information IE which includes the highest PDCP SN or the highest COUNT value of the packet which has already been delivered to the UE for the MRB, to decide whether to apply data forwarding for that MRB and to establish respective resources.

The source NG-RAN shall, for each MRB in the MBS Data Forwarding Response Info from target NG-RAN node IE in the HANDOVER REQUEST ACKNOWLEDGE message, start data forwarding to the indicated DL Forwarding UP TNL Information. If the MRB Progress Information IE is included the source NG-RAN node may use the information to determine when to stop data forwarding.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the MBS Session Information List IE is included in the UE Context Information – Retrieve UE Context Response IE contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use this information to establish an NG-RAN MBS session resources context, if applicable.

For each MRB indicated in the MBS Mapping and Data Forwarding Request Info from source NG-RAN node IE, the new NG-RAN node shall use the MRB ID IE and, if included, the MRB Progress Information IE which includes either the highest PDCP SN or the highest COUNT value of the packet which has already been delivered to the UE for the MRB, to decide whether to apply data forwarding for that MRB and to establish respective resources.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.147	MRB Progress Information

This IE contains the MRB progress Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE MRB Progress Information | M |  |  |  | – |  |
| >PDCP SN 12bits |  |  |  |  | – |  |
| >>PDCP SN Length 12 | M |  | INTEGER (0..4095) |  | – |  |
| >PDCP SN 18bits |  |  |  |  | – |  |
| >>PDCP SN Length 18 | M |  | INTEGER (0..262143) |  | – |  |
| >COUNT SN 12bits |  |  |  |  | YES | ignore |
| >>COUNT Value for PDCP SN Length 12 | M |  | 9.2.3.36 |  | – |  |
| >COUNT SN 18bits |  |  |  |  | YES | ignore |
| >>COUNT Value for PDCP SN Length 18 | M |  | 9.2.3.37 |  | – |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-COUNT-SN12,

id-COUNT-SN18,

maxEARFCN,

maxnoofAllowedAreas,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MRB-ProgressInformation ::= CHOICE {

pdcp-SN12			INTEGER (0..4095),

pdcp-SN18			INTEGER (0..262143),

choice-extension		ProtocolIE-Single-Container { { MRB-ProgressInformation-ExtIEs} }

}

MRB-ProgressInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

{ID id-COUNT-SN12  CRITICALITY ignore  TYPE COUNT-PDCP-SN12		PRESENCE mandatory}|

{ID id-COUNT-SN18  CRITICALITY ignore  TYPE COUNT-PDCP-SN18		PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-COUNT-SN12																						ProtocolIE-ID ::= 998 -- to be allocated

id-COUNT-SN18																						ProtocolIE-ID ::= 999 -- to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex C:	Draft CR for TS 38.473

3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | <CR#> | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for handling of MRB PDCP COUNT wrap around |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the MRB PDCP COUNT value is derived from sequence number information provided via the NG-U on a per MBS QoS flow basis, there are scenarios where UEs cannot be RRC configured with a proper initialRX-DELIV for MRBs and hence RRC configuration of UEs would have to be delayed which could cause in loss of data or congestion due to potentially many UEs have to be RRC configured in a short time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces support for configuring the Multicast Session Context in the gNB-DU with a new MRB even if an MRB exists with the same MBS QoS flows mapped to it. It also introduces the possibility for the gNB-CU to indicate that new lower layer resources are to be established for this new MRB. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it afffects the Multicast Session Context modification procedure only. The CR has impact from protocol and functional point of view and is realised in a backwards compatible way. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gNB-CU would not be able to contol the setup of new MRBs and related lower layer resources if the UEs would need to be configured with MRBs mapped to the same MBS QoS flows as already existing MRBs to handle the case where the MRB PDCP COUNT value is derived from sequence number information provided via the NG-U on a per MBS QoS flow basis and the UEs cannot be RRC configured with a proper initialRX-DELIV for MRBs, which would result in RRC configuration of UEs to be delayed, causing loss of data or congestion due to potentially many UEs have to be RRC configured in a short time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2.2.2, 9.2.14.7, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR.... TS 37.483 CR.... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.14.9	Multicast Context Modification

#### 8.14.9.1	General

The purpose of the Multicast Context Modification procedure is to modify an established MBS session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

#### 8.14.9.2	Successful Operation

Figure 8.14.9.2-1: Multicast Context Modification procedure. Successful operation

The MULTICAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and, if successful, report the update in the MULTICAST CONTEXT MODIFICATION RESPONSE message.

If the Multicast MRB To Be Setup List IE is contained in the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall setup the corresponding resources for the requested MRB(s), and report to the gNB-CU, in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Multicast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Multicast MRB Failed To Be Setup List IE;

If for one MRB in the Multicast MRB To Be Setup List IE the MULTICAST CONTEXT MODIFICATION REQUEST message contains the New MRB Indication IE set to "true", the gNB-DU shall allocate resources for that MRB even if an MRB is already established with the same MBS QoS flows mapped to it.

If the Multicast MRB Failed To Be Setup List IE is contained in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the setup of the indicated MRB(s) as failed and indicated the resource for the failure with an appropriate cause value for each MRB failed to be setup.

If the Multicast MRB To Be Modified List IE is contained in the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall update the corresponding context and resources for the requested MRB(s), and report to the gNB-CU, in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the modification result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully modified shall be included in the Multicast MRB Modified List IE;

-	A list of MRBs which failed to be modified shall be included in the Multicast MRB Failed To Be Modified List IE;

If the Multicast MRB Failed To Be Modified List IE is contained in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the Multicast MRB(s) failed to be modified with an appropriate cause value for each Multicast MRB failed to modify.

#### 8.14.9.3	Unsuccessful Operation

Figure 8.14.9.3-1: Multicast Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the multicast context can be successfully performed, the gNB-DU shall respond with the MULTICAST CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.14.9.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.14.7	MULTICAST CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to modify multicast context information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| Multicast MRB To Be Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| >New MRB Indication | O |  | ENUMERATED (true, ...) | Indicates that resources for the MRB are requested even if an MRB exists with the same MBS QoS flows mapped to it. | YES | ignore |
| Multicast MRB To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | O |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 0 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | O |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-Source-MRB-ID,

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-NewMRBIndication,

maxNRARFCN,

maxnoofErrors,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MulticastMRBs-ToBeSetupMod-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

mBS-DL-PDCP-SN-Length			PDCPSNLength,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeSetupMod-Item-ExtIEs} },

...

}

MulticastMRBs-ToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NewMRBIndication					CRITICALITY ignore EXTENSION NewMRBIndication								PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

NeighbourNR-CellsForSON-List ::= SEQUENCE (SIZE(1.. maxNeighbourCellforSON)) OF NeighbourNR-CellsForSON-Item

NeighbourNR-CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

nR-ModeInfoRel16								NR-ModeInfoRel16												OPTIONAL,

sSB-PositionsInBurst							SSB-PositionsInBurst											OPTIONAL,

nRPRACHConfig									NRPRACHConfig													OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { NeighbourNR-CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

NeighbourNR-CellsForSON-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NewMRBIndication ::= ENUMERATED {true, ...}

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-NewMRBIndication									ProtocolIE-ID ::= 999 -- to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex E:	Draft CR for TS 37.483 – initialRX-DELIV issue

3GPP TSG-RAN WG3 Meeting #120	R3-23xxxx

Incheon, Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | <CR#> | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to handle unavailability of information to set initialRX-DELIV for MRBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It has been acknowledged that MRB PDCP COUNT wrap around shall be avoided for multicast MBS sessions by means of standardised protocol functions on Xn, F1 and E1 interfaces. This CR introduces protocol functions on E1AP to support handling of MRB PDCP COUNT wrap around. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rel-17 has introduced the possibility to configure a UE with new MRBs malled to the same already existing MRBs but with the PDCP entity set to a different PDCP COUNT value. As described in CRxxxx for TS 38.401, for the case that MRB PDCP COUNT is derived from information received via NG-U, the gNB-CU-CP would wait until the MRB PDCP COUNT as exhausted half of its numbering space, i.e. the most significant bit is set to “1” and then starts to re-configure all UEs with MRBs that derive the MRB PDCP COUNT from NG-U by modulo division of [the accumulated NG-U DL MBS QFI SNs of the QoS flows mapped to the MRB] with 231.(231 = (Max (COUNT)+1)/2). This CR introduces E1AP signalling support for that approach. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the MRB PDCP COUNT value is derived from sequence number information provided via the NG-U on a per MBS QoS flow basis and the UEs cannot be RRC configured with a proper initialRX-DELIV for MRBs, RRC configuration of UEs would have to be delayed which could cause in loss of data or congestion due to potentially many UEs have to be RRC configured in a short time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.35b (new), 9.3.1.35c (new), 9.3.1.35d(new), 9.3.1.35e (new), 9.3.1.120, 9.3.1.130, 9.3.1.131, 9.3.3.33, 9.3.3.34, 9.3.3.35, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR.... TS 38.473 CR.... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.35	PDCP Count

This IE include the PDCP Count information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| >PDCP SN | M |  | INTEGER (0 .. ..2PDCP_SN_Size-1) | The PDCP SN Size is provided in the PDCP Configuration IE. |
| >HFN | M |  | INTEGER (0 .. 232-PDCP_SN_Size-1) | The PDCP SN Size is provided in the PDCP Configuration IE. |

#### 9.3.1.35a	MBS PDCP COUNT

This IE includes the MBS PDCP Count information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS PDCP COUNT | M |  | BIT STRING (32) | Corresponds to information provided in the initialRX-DELIV contained in the PDCP-Config IE and to be taken into account to configure the UE, as specified in TS 38.331 [10]. |

#### 9.3.1.35b	MBS PDCP COUNT – PDCP SN Length 12

This IE includes the MBS PDCP Count information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP-SN Length 12 | M |  | INTEGER (0..4095) |  |
| HFN for PDCP-SN Length 12 | M |  | INTEGER (0..1048575) |  |

#### 9.3.1.35c	MBS PDCP COUNT – PDCP SN Length 18

This IE includes the MBS PDCP Count information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP-SN Length 18 | M |  | INTEGER (0..262143) |  |
| HFN for PDCP-SN Length 18 | M |  | INTEGER (0..16383) |  |

#### 9.3.1.35e	MBS PDCP COUNT Initialisation

This IE provides information how the gNB-CU-UP shall set the COUNT value at the setup of the MBS PDCP UP entiy.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS PDCP COUNT Initialisation | M |  |  |  |
| >Relative COUNT Initialisation Information |  |  |  |  |
| >>Relative COUNT Initialisation Information | M |  | ENUMERATED (equal to (NG-U MRB COUNT) modulo 231), any other to-be-specified strategy) | "equal to (NG-U MRB COUNT) modulo 231)": The MRB is derived from the accumulated NG-U DL MBS QFI SNs of the QoS flows mapped to the MRB with the MSB set to "0". "any other to-be-specified strategy ": FFS |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.120	MC MRB Setup Configuration

This IE contains MRB configuration information for a MC Bearer Context Context.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC MRB To Setup List |  |  |  | 1..<maxnoofMRBs> |  |  |  |  |  | - |  |  |  |
| >MRB ID |  | M |  |  |  | 9.3.1.16a |  |  |  | - |  |  |  |
| >SDAP Configuration |  | M |  |  |  | 9.3.1.39 |  |  |  | - |  |  |  |
| >MBS PDCP Configuration |  | M |  |  |  | PDCP Configuration 9.3.1.38 |  |  |  | - |  |  |  |
| >MBS QoS Flows Information To Be Setup |  | M |  |  |  | QoS Flow QoS Parameters List 9.3.1.25 |  |  |  | - |  |  |  |
| >MRB QoS |  | O |  |  |  | QoS Flow Level QoS Parameters 9.3.1.26 |  | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |  | - |  |  |  |
| >New MRB Indication | O |  | ENUMERATED (true, ...) | Indicates that resources for the MRB are requested even if an MRB exists with the same MBS QoS flows mapped to it. | YES | ignore |
| >MBS PDCP COUNT Initialisation | O |  | 9.3.1.35e |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.130	MRB Progress Information

This IE contains the MRB progress information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE MRB Progress Information SNs | M |  |  |  | - |  |
| >12bits |  |  |  |  | - |  |
| >>PDCP SN Length 12 | M |  | INTEGER (0..4095) |  | - |  |
| >18bits |  |  |  |  | - |  |
| >>PDCP SN Length 18 | M |  | INTEGER (0..262143) |  | - |  |
| >MBS PDCP COUNT |  |  |  |  | YES | ignore |
| >>COUNT Value for PDCP SN Length 12 | M |  | 9.3.1.35b |  | - |  |
| >COUNT SN 18bits |  |  |  |  | YES | ignore |
| >>COUNT Value for PDCP SN Length 18 | M |  | 9.2.3.35c |  | - |  |
| MRB Progress Information Type | M |  | 9.3.1.131 |  | - |  |

#### 9.3.1.131	MRB Progress Information Type

This IE contains the MRB progress information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MRB Progress Information Type | M |  | ENUMERATED (oldest available PDCP SN, last delivered PDCP SN, ..., oldest available COUNT, last delivered COUNT) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.34	MC Bearer Context To Modify

This IE contains MBS session resource related information used to request a modification of a multicast MC Bearer Context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at 5GC | O |  | 9.3.1.122 |  | - |  |
| MC Bearer Context NG-U TNL Info at NG-RAN Request | O |  | 9.3.1.123 | To request NG-U TNL information from the gNB-CU-UP, if not yet available at gNB-CU-CP | - |  |
| MBS Multicast F1-U Context Descriptor | C-ifSetupOrRemove |  | 9.3.1.125 |  | - |  |
| Requested Action for Available Shared NG-U Termination | O |  | 9.3.1.115 |  | - |  |
| MC MRB To Setup or Modify List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >MC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.124 |  | - |  |
| >SDAP Configuration | O |  | 9.3.1.39 |  | - |  |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 |  | - |  |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. | - |  |
| >MBS PDCP COUNT Request | O |  | ENUMERATED (true, ...) | Indicates that the MBS PDCP COUNT is requested. | - |  |
| >New MRB Indication | O |  | ENUMERATED (true, ...) | Indicates that resources for the MRB are requested even if an MRB exists with the same MBS QoS flows mapped to it. | YES | ignore |
| >MBS PDCP COUNT Initialisation | O |  | 9.3.1.35e |  | YES | ignore |
| MC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| MC Forwarding Resource Request | O |  | 9.3.1.134 | Requests MC Forwarding Resource related information for the peer node | YES | ignore |
| MC Forwarding Resource Indication | O |  | 9.3.1.135 | Provides MC Forwarding Resource related information from the peer node | YES | ignore |
| MC Forwarding Resouce Release | O |  | 9.3.1.137 | Requests the release of the MC Forwarding Resource | YES | ignore |
| MBS Session Associated Information Non-Support-to-Support | O |  | 9.3.1.140 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifSetupOrRemove | This IE shall be present if either the MC MRB To Setup or Modify List IE or the MC MRB To Remove List IE or both IEs are included. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-MaxCIDEHCDL,

id-ignoreMappingRuleIndication,

id-EarlyDataForwardingIndicator,

id-QoSFlowsDRBRemapping,

id-SecurityIndicationModify,

id-DataForwardingSourceIPAddress,

id-M4ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-PDUSession-PairID,

id-SurvivalTime,

id-UDC-Parameters,

id-SecurityIndication,

id-SecurityResult,

id-SDTindicatorSetup,

id-SDTindicatorMod,

id-DiscardTimerExtended,

id-MCForwardingResourceRequest,

id-MCForwardingResourceIndication,

id-MCForwardingResourceResponse,

id-MCForwardingResourceRelease,

id-MCForwardingResourceReleaseIndication,

id-PDCP-COUNT-Reset,

id-MBSSessionAssociatedInfoNonSupportToSupport,

id-NG-U-SNInformation,

id-COUNT-SN12,

id-COUNT-SN18,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs,

maxnoofMBSSessionIDs,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofExtNRCGI,

maxnoofECGI,

maxnoofSMBRValues

FROM E1AP-Constants

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

COUNT-PDCP-SN12 ::= SEQUENCE {

pdcp-SN12						INTEGER (0..4095),

hfn-PDCP-SN12					INTEGER (0..1048575),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN12-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN12-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

COUNT-PDCP-SN18 ::= SEQUENCE {

pdcp-SN18						INTEGER (0..262143),

hfn-PDCP-SN18					INTEGER (0..16383),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN18-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN18-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MCMRBSetupModifyConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupModifyConfiguration-Item

MCMRBSetupModifyConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

f1uTNLatDU						MCBearerContextF1UTNLInfoatDU		OPTIONAL,

sdap-config						SDAP-Configuration				OPTIONAL,

mbs-pdcp-config					PDCP-Configuration				OPTIONAL,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List		OPTIONAL,

mrbQoS							QoSFlowLevelQoSParameters		OPTIONAL,

mbs-PDCP-COUNT-Req				MBS-PDCP-COUNT-Req				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupModifyConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupModifyConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-New-MRB-Indication  				CRITICALITY ignore  EXTENSION New-MRB-Indication					PRESENCE optional }|

{ID id-MBS-PDCP-COUNT-Initialisation 	CRITICALITY ignore  EXTENSION MBS-PDCP-COUNT-Initialisation		PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MCMRBSetupConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupConfiguration-Item

MCMRBSetupConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

sdap-config						SDAP-Configuration,

mbs-pdcp-config					PDCP-Configuration,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-New-MRB-Indication  				CRITICALITY ignore  EXTENSION New-MRB-Indication					PRESENCE optional }|

{ID id-MBS-PDCP-COUNT-Initialisation 	CRITICALITY ignore  EXTENSION MBS-PDCP-COUNT-Initialisation		PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-PDCP-COUNT	::= BIT STRING (SIZE (32))

MBS-PDCP-COUNT-Initialisation ::= CHOICE {

relative-COUNT-InitialisationInfo		ENUMERATED {equal-to-NG-U-MRB-COUNT-mod-2p31, ffs-strategy, ...},

choice-extension						ProtocolIE-SingleContainer	{{MBS-PDCP-COUNT-Initialisation-ExtIEs}}

}

MBS-PDCP-COUNT-Initialisation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MRB-ProgressInformationSNs ::= CHOICE {

pdcp-SN12			INTEGER (0..4095),

pdcp-SN18			INTEGER (0..262143),

choice-extension		ProtocolIE-SingleContainer { { MRB-ProgressInformationSNs-ExtIEs} }

}

MRB-ProgressInformationSNs-ExtIEs E1AP-PROTOCOL-IES ::= {

{ID id-COUNT-SN12  CRITICALITY ignore  TYPE COUNT-PDCP-SN12		PRESENCE mandatory}|

{ID id-COUNT-SN18  CRITICALITY ignore  TYPE COUNT-PDCP-SN18		PRESENCE mandatory},

...

}

MRB-ProgressInformationType ::= ENUMERATED {oldest-available-PDCP-SN, last-delivered-PDCP-SN, ..., oldest-available-COUNT, last-delivered-COUNT}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

New-MRB-Indication		::= ENUMERATED {true, ...}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MCForwardingResourceResponse									ProtocolIE-ID ::= 181

id-MCForwardingResourceRelease									ProtocolIE-ID ::= 182

id-MCForwardingResourceReleaseIndication						ProtocolIE-ID ::= 183

id-PDCP-COUNT-Reset												ProtocolIE-ID ::= 184

id-MBSSessionAssociatedInfoNonSupportToSupport					ProtocolIE-ID ::= 185

id-VersionID													ProtocolIE-ID ::= 186

id-New-MRB-Indication											ProtocolIE-ID ::= 996 -- to be allocated

id-MBS-PDCP-COUNT-Initialisation								ProtocolIE-ID ::= 997 -- to be allocated

id-COUNT-SN12													ProtocolIE-ID ::= 998 -- to be allocated

id-COUNT-SN18													ProtocolIE-ID ::= 999 -- to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>