3GPP TSG-RAN3 Meeting #121	R3-234256

# France, Touluse, Aug 21st – 25th 2023

Agenda item:	23.2.3

Source:	Xiaomi, Samsung, ZTE

Title:	(TP for BL CR to TS 38.455) Support of SRS bandwidth aggregation

Document for:	Discussion and Decision

1	Introduction

Agreement

For SRS bandwidth aggregation between SRS in two or three carriers, the following is needed for the aggregated SRS resources

- The same periodicityAndOffset, and slotOffset

- The configuration of pathloss RS, Po and alpha to ensure the same Tx PSD (power per subcarrier)

  - The same configuration of Po and alpha.

  - Note: UE may either perform pathloss RS measurement across CCs and form a single path loss value to apply across CCs or perform pathloss RS measurement in a single CC and apply across CCs

Working assumption

For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate:

- SRS resource set(s) in one or two or three of three aggregated carriers

- SRS resource set(s) in one or two of two aggregated carriers.

Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources

Agreement

With regard to support of aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, at least the existing Rel-17 DCI framework (i.e. use multiple DCIs schedule SRSs in multiple carriers) can be reused

- FFS: whether Rel-18 DCI framework for multi-cell PDSCH/PUSCH scheduling with a single DCI (i.e. single DCI schedules SRSs in multiple carriers) can also be reused with or without specification work in RAN1.

Agreement

For SRS bandwidth aggregation across carriers, support

- Single RSRP or RSRPP is reported

  - FFS: the single RSRP/RSRPP is based on aggregated SRS resources across aggregated carriers

- The used SRS resource IDs for the aggregated measurement are shared for RSRP/RSRPP and/or timing measurement results

SRS bandwidth aggregation has been discussed in RAN1 in several meetings, and according to the status report [1], RAN1 already have enough agreements on this topic to guide RAN3 discussion to support SRS bandwidth aggregation.

In this contribution, we’d like to discuss the following aspects from RAN3 perspective to support SRS bandwidth aggregation.

- SRS bandwidth aggregation configuration

- SRS bandwidth aggregation activation/deactivation

- SRS bandwidth aggregation measurement

2	Discussion

2.1 SRS bandwidth aggregation configuration

For configuration, according to RAN1 agreement below, SRS bandwidth aggregation is per SRS resource set basis across two or three carriers, which means existing SRS configuration can be reused, but the enhancement should be focused on how to decide and link the SRS resource set(s) that can be aggregated.

Agreement

For SRS bandwidth aggregation across two or three carriers, support

- Option 2: Per SRS resource set basis.

  - Support new signaling to indicate which SRS resource sets across carriers are linked.

It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied

The first issue is which entity decide which SRS resource set(s) can be aggregated, as some conditions need to be satisfied, and those conditions are more like physical layer parameters (e.g. same periodicityAndOffset, and slotOffset, same Tx PSD), so we think the serving gNB should decide which SRS resource set(s) can be aggregated with the instruction from LMF.

With above understanding, the positioning information exchange procedure can be enhanced to support SRS bandwidth aggregation, LMF can indicate its request on SRS bandwidth aggregation, gNB determine which SRS resource set(s) can be aggregated according to the request from LMF and resource condition in its own, and then indicate the SRS resources set(s) that can be aggregated to LMF.

The second issue is how to let LMF know the linked SRS resource set(s), a simple way is to re-use existing SRS resource configuration IE structure, just to introduce a new identity for SRS bandwidth aggregation (e.g. aggregation identity), e.g. if the SRS resource set 1, SRS resource set 2 and SRS resource set 3 can be aggregated, the aggregation identity can be included in the configuration information for SRS resource set 1, 2 3, thus LMF can know SRS resource set 1, 2, 3 are linked.

Proposal 1, serving gNB should decide which SRS resource set(s) can be aggregated with the instruction from LMF.

Proposal 2, positioning information exchange procedure can be re-used to obtain the bandwidth aggregation SRS configuration.

Proposal 3, introduce new request information in Requested SRS Transmission Characteristics IE for LMF to request the serving gNB to provide the SRS resource set(s) that can be aggregated.

Proposal 4, introduce new identity in SRS resource set configuration to link the SRS resource set(s) that can be aggregated

2.2 SRS bandwidth aggregation (de)activation

Regarding the activation/deactivation for Semi-persistent or Aperiodic SRS bandwidth aggregation, the existing positioning activation and deactivation procedure can be re-used.

However, according to the following IE, the existing SRS activation procedure can only activate one SRS resource set.

<start of the reference from TS 38.455>

9.1.1.17	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - | - |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | YES | ignore |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED(true,…) |  | - | - |
| >>SRS Resource Trigger | O |  | 9.2.35 |  | - | - |
| Activation Time | O |  | Relative Time 1900 9.2.36 | Indicates the start time when the SRS activation is requested | YES | ignore |

<omit the unnecessary part>

9.2.35	SRS Resource Trigger

This information element indicates a DCI code point according to a SRS resource set configuration.

<end of reference from TS 38.455>

Similarly, the deactivation procedure can only deactivate either one SRS resource set or all resources.

To support (de)activate bandwidth aggregation SRS, i.e. multiple SRS resource set(s), enhancements are needed. Two options can be considered:

- Option 1, introduce a new aggregation identity in Positioning activation request and positioning deactivation

- Option 2, introduce a list of SRS resource set in Positioning activation request and positioning deactivation

Observation 1, existing procedure cannot support (de)activate linked SRS resource set(s)

Proposal 5, RAN3 discuss how to support (de)activation linked SRS resource set(s) and consider following options.

-	Option 1, introduce a new aggregation identity in Positioning activation request and positioning deactivation

-	Option 2, introduce a list of SRS resource set in Positioning activation request and positioning deactivation

2.3 SRS bandwidth aggregation measurement

For SRS bandwidth aggregation measurement, if the aggregation info is introduced in SRS configuration IE, if aggregation ID is included in SRS resource sets, it implicitly indicates that LMF request the TRP to measure the aggregated SRS resource sets. the gNBs receives the SRS resource can know which SRS resource sets are linked, so no additional enhancement is needed for measurement related procedure.

Observation 2, no additional specification impacts on measurement procedure to support SRS bandwidth aggregation measurement

2.4 overall procedure

The following is the overall procedure for understanding.

In the annex, we provide the TP for TS 38.455 to show the possible spec impacts to support SRS bandwidth aggregation.

Proposal 6, agree the TP in the annex for supporting SRS bandwidth aggregation.

3	Conclusion

In this contribution, we discussed the potential procedure and spec impacts to support SRS bandwidth aggregation, and had the following observations and proposals.

Configuration

Proposal 1, serving gNB should decide which SRS resource set(s) can be aggregated with the instruction from LMF.

Proposal 2, positioning information exchange procedure can be re-used to obtain the bandwidth aggregation SRS configuration.

Proposal 3, introduce new request information in Requested SRS Transmission Characteristics IE for LMF to request the serving gNB to provide the SRS resource set(s) that can be aggregated.

Proposal 4, introduce new identity in SRS resource set configuration to link the SRS resource set(s) that can be aggregated

Activation and deactivation

Observation 1, existing procedure cannot support (de)activate linked SRS resource set(s)

Proposal 5, RAN3 discuss how to support (de)activation linked SRS resource set(s) and consider following options.

-	Option 1, introduce a new aggregation identity in Positioning activation request and positioning deactivation

-	Option 2, introduce a list of SRS resource set in Positioning activation request and positioning deactivation

Measurement

Observation 2, no additional specification impacts on measurement procedure to support SRS bandwidth aggregation measurement

TP to support SRS bandwidth aggregation

Proposal 6, agree the TP in the annex for supporting SRS bandwidth aggregation

4	References

[1] RP-231092 - SR - NR_pos_enh2, RAN1

5	Annex (TP for TS 38.455)

<Start of the change>

8.2.9	Positioning Activation

8.2.9.1	General

The Positioning Activation procedure is initiated by the LMF to request the NG-RAN node to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE. This procedure applies only if the NG-RAN node is a gNB.

8.2.9.2	Successful Operation

Figure 8.2.9.2-1: Positioning Activation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the NG-RAN node.

For semi-persistent UL SRS, the POSITIONING ACTIVATION REQUEST message includes an indication of the UL SRS resource set to be activated and may include the spatial relation for the semi-persistent UL SRS resource to be activated. For aperiodic UL SRS, if the SRS Resource Trigger IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the value of this IE into account when triggering aperiodic SRS transmission by the UE.

If the Activation Time IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the indicated value as the LMF’s requested time for activation of the UE’s SRS transmission.

Following successful activation of UL SRS transmission in the UE, the NG-RAN node shall respond with a POSITIONING ACTIVATION RESPONSE message. If the POSITIONING ACTIVATION RESPONSE message includes the System Frame Number and/or the Slot Number IEs, the LMF shall consider that the respective information indicates the activation time of SRS transmission by the UE.

If the Aggregation ID IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall activate the SRS resource set(s) that are linked with this Aggregation ID.

<next change>

8.2.10	Positioning Deactivation

8.2.10.1	General

The Positioning Deactivation procedure is initiated by the LMF to indicate to the NG-RAN node that UL SRS transmission should be deactivated in the UE. This procedure applies only if the NG-RAN node is a gNB.

8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Positioning Deactivation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DEACTIVATION message to the NG-RAN node. This message shall include an indication of the UL SRS resource set to be deactivated or release all the related resources or an indication of the linked UL SRS resource set(s) for bandwidth aggregation.

<next change>

9.1.1.17	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - | - |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | YES | ignore |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | YES | ignore |
| >>Aggregation ID | O |  | INTEGER(0..FFS) |  |  |  |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED(true,…) |  | - | - |
| >>SRS Resource Trigger | O |  | 9.2.35 |  | - | - |
| Activation Time | O |  | Relative Time 1900 9.2.36 | Indicates the start time when the SRS activation is requested | YES | ignore |

<next change>

9.1.1.20	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE Abort Transmission | M |  |  |  | YES | Ignore |
| >Deactivate SRS Resource Set |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - |  |
| >Release ALL |  |  | NULL | the NG-RAN node configures UE to stop transmitting SRS for the positioning purpose |  |  |
| >Deactivate Bandwidth Aggregation SRS |  |  |  |  |  |  |
| >>Aggregation ID | M |  | INTEGER(0..FFS) |  |  |  |

<next change>

9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |  |  |
| CHOICE Bandwidth | M |  |  |  |  |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ...) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |  |  |
| >>Periodicity List |  | 0.. 1 |  |  |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |  |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. |  |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  |  |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  |  |  |
| SSB Information | O |  | 9.2.54 |  |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| Bandwidth Aggregation Request information | O |  | FFS |  |  |  |

<next change>

9.2.31	SRS Resource Set

This information element indicates an SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| SRS Resource ID List |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Resource Set Type | M |  |  |  |
| >periodic |  |  |  |  |
| >>periodicSet | M |  | ENUMERATED(true,…) |  |
| >semi-persistent |  |  |  |  |
| >>semi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >aperiodic |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  |
| >>Slot offset | M |  | INTEGER(0..32) | Offset in number of slots, where value 0 indicates no offset. |
| Aggregation ID | O |  | INTEGER(0..FFS) |  |

<end of change>