3GPP TSG-RAN WG3 Meeting #121	R3-234278

Toulouse, France, August 21 – 25, 2023

Title: 	Further discussions on common issues and Stage 2 updates on the introduction of RAN AI/ML

Source: 	Huawei

Agenda item:	12.2.1

Document Type:	Discussion and decision

# 1. Introduction

In last RAN3#120 meeting Rel-18 WI on RAN AI/ML was further discussed and some agreements for both Stage 2 and Stage 3 were reached [1][2][3][4][5]. In this paper, further general considerations on the remaining open issues and further Stage 2 descriptions are discussed with some suggestions being proposed.

# 2. Background

In RAN3#120 meeting, as part of the Stage 2 discussions in [2], the timing information of AI/ML predictions have been discussed and the following was agreed:

The requested prediction time includes the specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future. The encoding needs to be further discussed.

FFS on the encoding and exact values of the specific time point. Down-select the options:

- INTEGER TYPE

- ENUMERATED TYPE

FFS on the benefits of the predicted information over a period of time.

The specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future, can be configured for both one-time reporting and periodic reporting.

Another issue that still needs to be solved is related to the following FFS from the RAN3#117bis-e meeting [6]:

It’s FFS on whether more new procedures needed to transfer different types of AL/ML data (e.g., feedback, measurements for training/inference).

In the rest of this paper we will analyze further the above listed open points and provide additional proposals for the definitions of the needed timing information of the new agreed class 1/2 procedures for RAN AI/ML.

# 3. Discussion

## 3.1 “Requested prediction time” and “reporting periodicity” of the AI/ML prediction information

Prior discussing the FFS points from the last meeting, we should reach a common understanding in RAN3 on some additional aspects that are worth to be analyzed, which are based on the agreements from RAN3#120 meeting.

Figure 1 shows a graphical representation of the “requested prediction time” expressed as a specific point in time in the future (as per the current definition) in relation to its usage within the AI/ML Information Reporting Initiation (FFS on the name) and AI/ML Information Reporting (FFS on the name) procedures in case of one-time reporting, where the “requested prediction time” has been indicated with t1. Note that the other time instants reported in Figure 1, i.e., t0 and t2, are indicated for the sake of completeness of the graphical representation and do not need to be specified in RAN3 specifications: they represent the time instant at which the requested node (Node 2) receives the AI/ML INFORMATION REQUEST (FFS on the name) message, and the time instant at which the requesting node (Node 1) receives the AI/ML INFORMATION UPDATE (FFS on the name) message, respectively.

Figure 1 - "Requested prediction time", one-time reporting of prediction information.

Let’s now focus on the case of periodic reporting as in Figure 2 where, in addition to the “requested prediction time” t1, the “reporting periodicity” T needs to be also introduced and indicated in the AI/ML INFORMATION REQUEST (FFS on the name) message. Two possible interpretations about the use of the “reporting periodicity” are provided:

- all predictions periodically sent in the different AI/ML INFORMATION UPDATE (FFS on the name) messages of the periodic reporting are generated at the same “requested prediction time” t1 – this is shown in Figure 2, Option a)

- the prediction in each AI/ML INFORMATION UPDATE (FFS on the name) message is generated at a “requested prediction time” by shifting t1 by a certain “reporting periodicity” T – this is shown in Figure 2, Option b)

In case of Option a), the “reporting periodicity” T is only used to determine the time instant which corresponds to receiving a certain AI/ML INFORMATION UPDATE (FFS on the name) message, but the prediction within this specific AI/ML INFORMATION UPDATE (FFS on the name) message is always generated at the same time instant in the future t1, i.e., the same “requested prediction time”. While, in Option b), the “reporting periodicity” T is used not only to determine the time spacing between two adjacent reporting occasions – i.e., between two AI/ML INFORMATION UPDATE (FFS on the name) messages received one after the other – but it is also used to determine the “requested prediction time” (t1 + nT), with n = 0, 1, …, N, which refers to the specific point in time in the future for which the (n+1)th prediction is produced and then sent within the (n+1)th AI/ML INFORMATION UPDATE (FFS on the name).

In our view Option a) is the correct interpretation of how to use the “reporting periodicity” T in case of periodic reporting: let’s refer for instance to the weather forecast and assume that, at 9am, a request to know the weather forecast corresponding to 12am (hence “requested prediction time” t1 = 12am) is received, and this weather forecast is requested to be provided periodically every 30min (hence the “reporting periodicity” T = 30min). Therefore, the first weather forecast will be received at 9.30am, the second at 10am, the third at 10.30am, etc. but the content of these predictions is always referred to the weather forecast evaluated at 12am, not at 12am for the first prediction, at 12.30am for the second, at 1pm for the third, and so on.

Figure 2 - "Requested prediction time", periodic reporting of prediction information.

Based on the above we propose:

Proposal 1: RAN3 to agree on the definition of “reporting periodicity” as the reporting interval of the requested prediction information, which is indicated in the AI/ML INFORMATION REQUEST (FFS on the name).

Proposal 1bis: For the case of periodic reporting, the “reporting periodicity” within the AI/ML INFORMATION REQUEST (FFS on the name) message is only used to determine the time spacing between two adjacent reporting occasions, i.e., between two AI/ML INFORMATION UPDATE (FFS on the name) messages received one after the other.

Another aspect that needs to analyzed is related to the fact that currently the “requested prediction time” t1 indicated in the AI/ML INFORMATION REQUEST (FFS on the name) message applies to all the items within the Report Characteristics IE for which the requesting node (Node 1 in Figure 1 and Figure 2) asks for predictions. And this is valid regardless of the reporting type, i.e., one-time or periodic.

The question to be answered is: how to address the issue of requesting predictions for the items listed in the Report Characteristics IE of the AI/ML INFORMATION REQUEST (FFS on the name) message, where different predictions need to be generated for different “requested prediction times”? For instance, assume that if Node 1 indicates in the AI/ML INFORMATION REQUEST (FFS on the name) message that it asks for predictions for the following items: “Energy Cost” and “Predicted RRC connections” items, but such predictions need to be generated for different “requested prediction times”, say tEC for the “Energy Cost” and tRRC for the “Predicted RRC connections”. In our view, this scenario can be addressed by initiating multiple agreed new class1/2 procedures, each procedure with a specific “requested prediction time” for the specific item, hence without specifying multiple “requested prediction times” in the same AI/ML INFORMATION REQUEST (FFS on the name) message. Therefore, we propose:

Proposal 2: In case different “requested prediction times” need to be considered for different items for which a prediction is requested, multiple agreed new class1/2 procedures can be initiated, each procedure with a specific “requested prediction time” for the specific item.

From the last RAN3 meeting (RAN3#120) there is an FFS on the benefits of configuring the “requested prediction time” also as a time interval, other than as a time instant (as per the current definition) [1]:

FFS on the benefits of the predicted information over a period of time.

As a general statement, we should provide enough flexibility for the “requested prediction time” configuration in the AI/ML INFORMATION REQUEST (FFS on the name), in the sense that the “requested prediction time” can be signalled either as a time instant or a time interval (e.g., start time plus end time, or start time plus duration) as represented in Figure 3 (for both reporting types), depending on the prediction information needed for the specific use case.

Figure 3 - "Requested prediction time" as a time interval, for both one-time and periodic reporting.

As an example, let’s consider the Load Balancing (LB) use case: here it could be useful for the requesting node to request a neighbor node to provide an estimation of e.g. the Predicted Number of Active UEs within a time interval in the future, e.g. from 9.00am to 9.30am, so to have an indication of the average number of active UEs over a specific time period in the future which could be exploited to better plan an offloading action towards the requested node. The same information could be achieved by initiating multiple requests towards the requested node, each request with a specific time instant within the interval [9.00 – 9.30] indicated in the “requested prediction time”, however we think that this creates a significant signalling overhead, with subsequent waste of signalling resources. Hence, based on the above, we propose the following:

Proposal 3: For both one-time and periodic reporting, the “requested prediction time” can also be configured in the AI/ML INFORMATION REQUEST (FFS on the name) message as a time interval in the future for which the prediction information is requested.

Another FFS from the last RAN3 meeting concerns the encoding of the “requested prediction time” and it is reported below for convenience [1]:

FFS on the encoding and exact values of the specific time point. Down-select the options:

- INTEGER TYPE

- ENUMERATED TYPE

In our view, the “requested prediction time” should be encoded as an INTEGER type in order to provide enough flexibility and granularity for its configuration in the AI/ML INFORMATION REQUEST (FFS on the name) message, either as a time instant or as a time interval in the future, depending on the use case. Therefore, we propose:

Proposal 4: Regardless of whether the “requested prediction time” is either a time instant or a time interval in the future, it is encoded as an INTEGER type.

## 3.2 “Validity time” of the AI/ML prediction information

Even though the “validity time” of the prediction information has not been discussed in the last meeting, and referring to the discussion in RAN3#119bis-e, there were some draft FFS regarding the “validity time” copied below from the RAN AI/ML Stage 2 SoD in [7] – see text highlighted in yellow:

To be continued:

FFS whether the Requested Prediction time consists of a time interval.

FFS whether validity time needs to be defined, e.g. as follows:

Validity time: time period within which the requested prediction information in the AI/ML INFORMATION UPDATE (FFS on the name) is considered valid

Since the definition of the “requested prediction time” has already been agreed, RAN3 should now discuss and agree on definition of the “validity time”, whose graphical representation is indicated in Figure 4 as τ for the case of one-time reporting and, for the sake of simplicity, the “requested prediction time” configured as a time instant in the future (t1).

Figure 4 - "Validity time" of the prediction information (one-time reporting case).

By referring to Figure 4, in our view the “validity time” of the prediction information is an information optionally provided along the prediction which indicates to the requesting node that it is allowed to use the prediction received at time t2 and beyond (i.e., until t2 + τ ); if the requested node does not provide the “validity time” τ of the prediction along with the prediction itself, then it means that the prediction information is only valid (and hence needs to be used) at the time the prediction is received by the requesting node (i.e., at the time instant t2). By providing the “validity time” of the prediction information the requesting node can potentially use the prediction information over a longer period of time, not only at the time instant t2 when the prediction is received. The “validity time” of the prediction is determined by the requested node (i.e., by the node that makes inferences) and hence it is specified in the AI/ML INFORMATION UPDATE (FFS on the name) message. Therefore, we propose:

Proposal 5: RAN3 to agree to the definition of “validity time” as the time period within which the requested prediction information is considered valid for being used by the requesting NG-RAN node.

Proposal 6: In case of one-time reporting, the “validity time” of the prediction information is optionally indicated in the AI/ML INFORMATION UPDATE (FFS on the name) along with the prediction information.

For the periodic reporting, and regardless of whether the “requested prediction time” is configured as either a time instant or a time interval in the future, we need to discuss the relation between “reporting periodicity” and “validity time” of the prediction information in the following two cases – refer to Figure 5 for a graphical representation of these cases, where, for the sake of simplicity, the “requested prediction time” has been configured as a time instant in the future (t1):

- Case 1: The “reporting periodicity” (e.g. 60s) is shorter than the “validity time” (e.g. 120s) of the prediction information, refer to the left part of Figure 5. In this case, the prediction information in the “reporting periodicity” can be considered as valid by default and used within the whole time period T, i.e., until the next AI/ML INFORMATION UPDATE (FFS on the name) message with a new prediction is received. Hence the “validity time” is not needed to be signalled by the requested node in the AI/ML INFORMATION UPDATE (FFS on the name) message along with the prediction information.

- Case 2: The “reporting periodicity” (e.g. 60s) is greater than the “validity time” (e.g. 30s) of the prediction information, refer to the right part of Figure 5. To address this scenario, we think that the same behavior as in the one-time reporting applies, i.e., the “validity time” of the (n+1)th prediction information (n = 0, 1, …, N) can be optionally signalled by the requested node in the (n+1)th AI/ML INFORMATION UPDATE (FFS on the name) message along with the prediction information to allow the requesting node to keep on using the prediction information also beyond the time instant (t2+nT) where it received the prediction itself.

Figure 5 - Relation between "validity time" and "reporting periodicity" of the prediction information.

Based on the analysis above we propose the following:

Proposal 7: In case of periodic reporting, if the “reporting periodicity” is greater than the “validity time” of the prediction information, the same behaviour of the one-time reporting applies, i.e., the “validity time” can be optionally indicated in the AI/ML INFORMATION UPDATE (FFS on the name) message along with the prediction information.

Proposal 7bis: In case of periodic reporting, if the “reporting periodicity” is shorter than the “validity time” of the prediction information, the “validity time” is not needed to be signalled along with the prediction information.

## 3.3 AI/ML prediction accuracy

In RAN3#119bis-e meeting, whether the prediction accuracy should be provided by the requested node along with the prediction information was discussed again in two separate CBs [7][8], and eventually the following was captured in [9]:

Accuracy:

No consensus on whether the accuracy information is necessary to transfer between requested node and requesting node.

First of all, we need to clarify whether the purpose of transmitting prediction accuracy is to measure the performance of the AI/ML model or to provide some information on the quality of the prediction information (i.e., the likelihood that the prediction will be true).

If the purpose is to measure the performance of the AI/ML model, it does not seem necessary. Because as long as the AI/ML model is not modified, the performance of the AI/ML model that inferences the prediction information should be constant. Moreover, there is no unified standard for measuring the performance of the AI/ML model when different nodes have different AI/ML models or the same AI/ML model is (re-)trained based on different datasets.

If the purpose is to provide some information on the quality of the prediction information, also in this case we think this is not needed. In general, we believe that it is not useful to provide the prediction’s accuracy/confidence (along with the prediction information) since accuracy is not a deterministic information by default but a prediction itself. Moreover, it is not clear to us which could be the behavior of the receiving NG-RAN node when it is provided with such kind of information, will it trust the prediction or not? Eventually, the prediction accuracy can only be calculated with obtaining the ground truth in the “requested prediction time”. However, the prediction information needs to be transferred before the requested node obtains the ground truth. But there could also be other means to evaluate a prediction’s accuracy, e.g. via the UE performance feedback or via other use case specific means.

Therefore, based on the above, we propose the following:

Proposal 8: Accuracy/confidence of a prediction is not signalled along with the prediction itself.

## 3.4 On the need of more new procedures for RAN AI/ML

For the agreed new class1/2 procedures used for RAN AI/ML over Xn there were intensive discussions in RAN3 which led to the definition of these procedures to be both use case agnostic (agreement from RAN3#117bis-e meeting [6]) as well as data type agnostic (agreement from RAN3#119bis-e meeting [9]). There is still a remaining open issue from RAN3#117bis-e meeting [6], reported in Section 2 and copied below for convenience:

-	It’s FFS on whether more new procedures needed to transfer different types of AL/ML data (e.g., feedback, measurements for training/inference).

The main issue here is what kind of information could be included/transferred using these potential new procedures for RAN AI/ML, in addition to the already agreed new class 1/2 procedures.

In the RAN3#119bis-e meeting, as part of the discussion on the Stage 3 aspects for enabling the inference-based Energy Saving (ES) use case [10], there was a discussion on how to signal a request for an estimation of the Energy Cost for an “Additional Load”. On top of the option of re-using the already introduced AI/ML Information Reporting Initiation procedure (Class 1 – AI/ML INFORMATION REQUEST/RESPONSE) to signal to the requested node a description of the “Additional Load” (whose definition is still pending on final consensus) and the AI/ML Information Reporting procedure (Class 2 – AI/ML INFORMATION UPDATE) to allow the requested node to report the estimation of the Energy Cost, it was proposed to introduce a new Class 1 procedure where the requesting node requests an estimation of the Energy Cost for an “Additional Load” and where the requested node responds with such estimation. Motivations for introducing such new additional class 1 procedure are multi-fold:

- it is a simple request/response procedure suitable for one-time request/response of a piece of information imminently needed, as already done in the UE context Retrieve procedure (if a node needs the UE context, it sends a request and receives the context in a response message);

- the procedure can be triggered when the conditions for an ES offloading arise and it employs the minimum amount of signalling (two signalling messages) to retrieve the estimated Energy Cost;

- the procedure is state-less, in the sense that there is no measurement context that needs to be stored at source and target nodes after the response is received.

In our understanding, for the sake of reducing the complexity and aiming at a unified AI/ML framework which is applicable to all the inference-enabled use cases, it would be better to consider re-using the already agreed new class 1/2 procedures as much as possible, which also provide enough flexibility in configuring the request and reporting of the inferred/actual/current Energy Cost values depending on the ES scenario to address. Moreover, the approach of re-using the already agreed new class 1/2 procedures is in line with the agreements of having use case and data type agnostic procedures. Based on these arguments, we do not see the need to introduce additional new procedures for requesting and reporting different types of AI/ML data. Hence, we propose:

Proposal 9: The already agreed new class 1/2 procedures could be used for transferring predicted information, UE performance feedback information as well as use case specific current/actual/measured metrics. Details should be discussed on a use case basis.

## 3.5 Naming of AI/ML procedures and messages

In the last RAN3 meeting there were some proposals in [11] and [12] to remove the «FFS» on the procedures and messages’ names introduced in the context of the RAN AI/ML WI. Such proposals basically remove references to the “AI/ML INFORMATION” part of the agreed new class 1/2 procedures and messages’ names in order to remain as much general as possible, so that such procedures (and related messages) can be re-used in the future for all those functionalities that might leverage them.

Now we see some other proposals in [11] and [12] use “DATA COLLECTION”, hence a more generic name which does not directly point to AI/ML. This is technically not wrong since predicted information does not have to come necessarily from an AI/ML model’s inference, and such information could also be used for legacy algorithms. With this understanding, we are open to discuss the name updating, and we propose to use “DATA ACQUISITION” because we think that the term “collection” encompasses not only the gathering of data but also other operations such as measuring and analysing of data which are not strictly related to the purpose of new agreed procedures for AI/ML in NG-RAN, but more with the AI/ML models.

Proposal 10: RAN3 to discuss and agree to update the names of the new agreed class 1/2 procedures and messages to be more generic, e.g. “DATA ACQUISITION” and remove the “(FFS on the name)”.

## 3.6 Stage 2 update

In the last RAN3 meeting a Stage 2 TP was agreed in [13] which captures an important agreement from the RAN3#119bis-e meeting [9]

Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, which means that the intended use of the data (e.g., input, output, feedback) shall not be indicated.

on top of the descriptions on general principles and mechanisms for the support of the RAN AI/ML function as per the BLCR to TS 38.300 in [14] which was endorsed in RAN3#120.

Moreover, we think that there are further agreements (both Stage 2 and Stage 3) which could be further captured in the Stage 2 specification, including:

--- Stage 2 agreements ---

Legacy information that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures)

--- Stage 3 agreements ---

Predicted Resource Status Information reported in the new procedure for AI/ML Related Information can be predicted radio resources, predicted number of active UEs, and predicted number of RRC Connections.

UE Trajectory Prediction is transferred to the target gNB via the Handover Request.

Introduce the metric of Energy Cost (EC) as the AI/ML metric to be shared over the Xn interface among gNBs.

Proposal 11: RAN3 to agree Stage 2 updates to TS 38.300 (also capturing Stage 3 agreements relevant for Stage 2) achieved up to RAN3#120.

The corresponding TP could be seen in our contribution paper in [15].

# 4. Conclusion

Based on the discussion in this paper, we have the following proposals.

Proposal 1: RAN3 to agree on the definition of “reporting periodicity” as the reporting interval of the requested prediction information, which is indicated in the AI/ML INFORMATION REQUEST (FFS on the name).

Proposal 1bis: For the case of periodic reporting, the “reporting periodicity” within the AI/ML INFORMATION REQUEST (FFS on the name) message is only used to determine the time spacing between two adjacent reporting occasions, i.e., between two AI/ML INFORMATION UPDATE (FFS on the name) messages received one after the other.

Proposal 2: In case different “requested prediction times” need to be considered for different items for which a prediction is requested, multiple agreed new class1/2 procedures can be initiated, each procedure with a specific “requested prediction time” for the specific item.

Proposal 3: For both one-time and periodic reporting, the “requested prediction time” can also be configured in the AI/ML INFORMATION REQUEST (FFS on the name) message as a time interval in the future for which the prediction information is requested.

Proposal 4: Regardless of whether the “requested prediction time” is either a time instant or a time interval in the future, it is encoded as an INTEGER type.

Proposal 5: RAN3 to agree to the definition of “validity time” as the time period within which the requested prediction information is considered valid for being used by the requesting NG-RAN node.

Proposal 6: In case of one-time reporting, the “validity time” of the prediction information is optionally indicated in the AI/ML INFORMATION UPDATE (FFS on the name) along with the prediction information.

Proposal 7: In case of periodic reporting, if the “reporting periodicity” is greater than the “validity time” of the prediction information, the same behaviour of the one-time reporting applies, i.e., the “validity time” can be optionally indicated in the AI/ML INFORMATION UPDATE (FFS on the name) message along with the prediction information.

Proposal 7bis: In case of periodic reporting, if the “reporting periodicity” is shorter than the “validity time” of the prediction information, the “validity time” is not needed to be signalled along with the prediction information.

Proposal 8: Accuracy/confidence of a prediction is not signalled along with the prediction itself.

Proposal 9: The already agreed new class 1/2 procedures could be used for transferring predicted information, UE performance feedback information as well as use case specific current/actual/measured metrics. Details should be discussed on a use case basis.

Proposal 10: RAN3 to discuss and agree to update the names of the new agreed class 1/2 procedures and messages to be more generic, e.g. “DATA ACQUISITION” and remove the “(FFS on the name)”.

Proposal 11: RAN3 to agree Stage 2 updates to TS 38.300 (also capturing Stage 3 agreements relevant for Stage 2) achieved up to RAN3#120.

# 5. Reference

- 3GPP RAN3#120 Chairman’s notes (RAN3_120_agenda_202300530_EOM1.doc)

- R3-233341, Summary of email Discussion on CB: # AIRAN1_stage2, ZTE

- R3-233343, Summary of Offline Discussion on CB: # AIRAN2_LB, Ericsson

- R3-233348, Summary of Offline Discussion on AIRAN3_ME, Lenovo

- R3-233349, SoD on CB: # AIRAN4_ES, CMCC

- 3GPP RAN3#117bis-e Chairman’s notes (RAN3_117bis-e_agenda_20221018_EOM1.doc)

- R3-232122, Summary of CB: # AIRAN1_Stage2, CMCC

- R3-232023, Summary of email Discussion on CB: # AIRAN2_LB, ZTE

- 3GPP RAN3#119bis-e Chairman’s notes (RAN3_119bis-e_agenda_202304026_EOM1.doc)

- R3-232028, SoD on CB: # AIRAN4_ES, Ericsson

- R3-233032, (TP for TS38.423 BLCR) On the naming of the new procedures, Lenovo

- R3-233127, Discussion on AI/ML Procedures Names, Ericsson

- R3-233342, (TP for AI&ML BLCR to TS38.300) Stage 2 updates on the new procedures of AI/ML for RAN, CMCC, CATT, Nokia, Nokia Shanghai Bell, Lenovo, Huawei, Intel, Samsung, ZTE, Ericsson

- R3-232521, (BLCR to 38.300) Draft CR to 38.300 on AI/ML for NG-RAN, CMCC, ZTE, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, CATT, Samsung, Lenovo, Intel Corporation

- R3-234279, (TP for AI/ML BL CR for TS 38.300) Stage 2 updates on the introduction of RAN AI/ML, Huawei, CATT, China Unicom