3GPP TSG-RAN WG3 Meeting #121	R3-234279

Toulouse, France, August 21 – 25, 2023

Title: 	(TP for AI/ML BL CR for TS 38.300) Stage 2 updates on the introduction of RAN AI/ML

Source: 	Huawei, CATT, China Unicom

Agenda item:	12.2.1

Document Type:	Discussion and decision

# 1. Introduction

In last RAN3 meetings several agreements concerning the RAN AI/ML function were achieved by RAN3 [1], both at Stage 2 and at Stage 3 level, and some of the latter ones could also be relevant for the Stage 2 description.

--- Stage 2 agreements ---

Legacy information that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures)

--- Stage 3 agreements ---

Predicted Resource Status Information reported in the new procedure for AI/ML Related Information can be predicted radio resources, predicted number of active UEs, and predicted number of RRC Connections.

UE Trajectory Prediction is transferred to the target gNB via the Handover Request.

Introduce the metric of Energy Cost (EC) as the AI/ML metric to be shared over the Xn interface among gNBs.

This revised TP aims at capturing the above agreements into the Stage 2 specification (TS 38.300).

# 2. TP to TS 38.300 (based on R3-233780)

*****************************************Start of 1st Change***************************************

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

<Unchanged Text Omitted>

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

ML	Machine Learning

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

*****************************************End of 1st Change***************************************

*****************************************Start of 2nd Change***************************************

## X.X AI/ML for NG-RAN

### X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

### X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

Signalling procedures used for the exchange of AI/ML related information are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

The Predicted Resource Status Information (i.e., predicted radio resources, predicted number of active UEs, and predicted number of RRC Connections), which can be used e.g. for the Load Balancing use case, is exchanged between NG-RAN nodes for subsequent UEs’ mobility decisions.

The cell-based UE trajectory prediction, which can be used e.g. for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Request message to provide information for subsequent mobility decisions.

The metric of Energy Cost (EC), which can be used e.g. for the Network Energy Saving use case, is introduced as the AI/ML metric to be exchanged with the neighbouring NG-RAN nodes upon request.

*****************************************End of 2nd Change***************************************

# 3	Reference

- 3GPP RAN3#120 Meeting minutes (RAN3_120_agenda_20230530_EOM1.doc)