3GPP TSG-RAN WG3 Meeting #121	R3-234283

Toulouse, France, August 21 – 25, 2023

Title: 	(TP for AI/ML BL CR for TS 37.483 and 38.473) On the impact of RAN AI/ML over E1 and F1 interfaces

Source: 	Huawei

Agenda item:	12.2.2.4

Document Type:	Discussion and decision

# 1. Introduction

In RAN3#117b-e meeting, the impact of RAN AI/ML over E1 and F1 interfaces was discussed and the following agreements were reached [1]:

UE traffic metric takes the data volume for a UE as the starting point.

The cell-level UE trajectory prediction function is located in gNB CU-CP.

The location for resource status prediction in split architecture:

- For current resource status input data from gNB DU, the resource status prediction function is located in gNB CU-CP.

- For current resource status input data from gNB CU-UP, the resource status prediction function is located in gNB CU-CP.

Also, there are still open issues that need to be further discussed [1]:

Discussion to be continued on the following:

- FFS on detailed E1/F1 impact for the exchange of current/predicted UE traffic after the sufficient work for non-split architecture.

- FFS on detailed E1/F1 impact for the exchange of current/predicted UE trajectory after the sufficient work for non-split architecture.

- FFS on detailed energy efficiency metric and corresponding E1/F1 impact after the sufficient work for non-split architecture.

In this paper, we discuss the above listed left issues.

# 2. Discussion

## 2.1 On whether AI/ML-related information need to be reported over E1 and F1 interfaces

LB and ME use cases

For LB and ME use cases, RAN3 agreed to introduce prediction information (Predicted Radio Resource Status, Predicted Number of Active UEs, Predicted RRC connections, UE trajectory prediction) and UE performance feedback information (Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss) over Xn interface. In addition, for the split architecture scenario, RAN3 agreed that AI/ML Model Training is located in CU/CU-CP and AI/ML Model Inference function is located in CU/CU-CP.

Before discussing the standard impact of F1 and E1, we should clarify how NG-RAN nodes get and use the agreed prediction information and feedback information agreed so far to be signalled over Xn.

For prediction information, it is generated by AI/ML model in CU/CU-CP, so prediction information does not need to be transferred from DU/CU-UP to CU/CU-CP. Next, we should discuss how to use prediction information, to ensure whether it needs to be transferred from CU/CU-CP to DU/CU-UP. We think the UE trajectory prediction could be useful to be transferred in order to help resource allocation in those cells that are predicted to be visited by the UE. For other prediction information, we have not seen any benefit of transferring it from CU/CU-CP to DU/CU-UP. Concerning UE performance feedback information, instead, its main usage is for AI/ML model training or monitoring in CU/CU-CP, so it does not need to be transferred from CU/CU-CP to DU/CU-UP. On the contrary, the CU/CU-CP needs to collect UE feedback information from DU/CU-UP, so we think there is a need to introduce UE performance feedback information over the F1/E1 interface.

- Proposal 1: RAN3 to discuss which prediction information is needed to transfer from CU/CU-CP to DU/CU-UP.

- Proposal 2: RAN3 to agree that there is a standard impact of transferring UE performance feedback over the F1/E1 interface.

ES use case

For ES use case, it was agreed that the measured Energy Cost (EC) per node could be transferred between NG-RAN nodes, as below [2].

It is agreed to include the measured Energy Cost in the AI/ML Information Reporting Initiation and AI/ML Information Reporting procedures (name of the procedures to be further discussed)

It is agreed that the Energy Cost is a node level parameter. Further EC granularities are out of scope of Rel18.

In the split architecture, when a gNB-CU/gNB-CU-CP is requested by its neighbouring nodes to report the measured node-level EC metrics, it needs to collect the measured EC(s) of gNB-DU(s)/gNB-CU-UP(s) in order to derive the overall EC of the node. There would be two options for gNB-CU/gNB-CU-CP collecting the EC value of gNB-DU(s)/gNB-CU-UP(s):

- Option 1: via OAM, i.e., OAM collects the measured EC of gNB-DU/gNB-CU-UP and indicates the value to the gNB-CU/gNB-CU-CP;

- Option 2: via direct reporting, i.e., the gNB-DU/gNB-CU-UP reports its measured EC directly to the gNB-CU/gNB-CU-CP over F1/E1.

Generally, both options could work. The first one allows data gathering in a relatively longer time granularity, e.g. 5 min and above, while the second allows data gathering in a shorter time granularity. Note that it was agreed that the cell-level ES strategy is considered as a starting point for the ES use case, as follows [3]:

RAN3 focus on the cell-level energy saving strategy as a start point, to avoid overlapped discussion for network energy saving SI.

Generally, a cell-level ES strategy, e.g. cell switch-on/off, is not expected to be dynamic. In this regard, Option 1 (i.e., via OAM) seems enough. RAN3 can discuss whether to support some dynamic ES strategies in Rel-18, e.g. dynamic cell switch-on/off and, if dynamic strategies are to be supported, whether to enable the transfer of measured EC over F1 and E1 interfaces (i.e. the second way).

Proposal 3: RAN3 to discuss whether to support dynamic ES strategies in Rel-18, and whether there is a need to transfer the measured EC over E1 and F1 interfaces.

## 2.2 Procedures for AI/ML information reporting over E1 and F1 interfaces

In previous meetings, it was agreed to introduce new Class 1 and Class 2 procedures over Xn for exchanging AI/ML information (e.g., prediction information, UE performance feedback and measured Energy Cost), as below [1-5]:

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information.

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

The new procedure over Xn used for AI/ML related information should be non-UE associated as a start point.

Introduce a new Class 1 procedure for initiating the reporting of AI/ML Related Information and a Class 2 procedure for Data Reporting of AI/ML Related Information.

Reporting options for the new procedure used for AI/ML Related Information to be evaluated on a case-by-case basis. Possible reporting options are one-time and periodic reporting.

The new procedure is non-UE associated procedure. If needed, the procedure can be used to capture UE-associated information.

The response message of the new procedure for AI/ML Related Information indicates if the requested information can be provided.  Introduce the failed measurement in the response message to indicate partial reporting result. T

The Failed Report Characteristics shall be split into two fields:

- Failed Report Characteristics for per cell measurements, reported with per cell granularity

- Failed Report Characteristics for per node measurements, reported with per node granularity

For E1 and F1 interfaces, as discussed in the previous section, only the UE performance feedback and measured Energy Cost metrics (if needed) could be transferred, while no prediction information seems needed to be sent to DU/CU-UP. Considering that UE performance feedback and Energy Cost metrics are general information which are not strictly related to AI/ML, it seems reasonable to use the legacy procedures (e.g., Resource Status Reporting Initiation and Resource Status Reporting in F1AP/E1AP) to transfer such information over E1 and F1.

If any other prediction information is foreseen to be transferred over E1 and F1, we are open to discuss about the introduction of new procedures over these interfaces.

Proposal 4: RAN3 to discuss whether to reuse the legacy procedures or to introduce new procedures for transferring the UE performance feedback and measured EC metrics (if needed) over E1 and F1 interfaces.

The corresponding TP to TS 37.483 and TS 38.473 (assuming the legacy procedures are used) is provided in Annexes.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Proposal 1: RAN3 to discuss which prediction information is needed to transfer from CU/CU-CP to DU/CU-UP.

- Proposal 2: RAN3 to agree that there is a standard impact of transferring UE performance feedback over the F1/E1 interface.

Proposal 3: RAN3 to discuss whether to support dynamic ES strategies in Rel-18, and whether there is a need to transfer the measured EC over E1 and F1 interfaces.

Proposal 4: RAN3 to discuss whether to reuse the legacy procedures or to introduce new procedures for transferring the UE performance feedback and measured EC metrics (if needed) over E1 and F1 interfaces.

# 4. Reference

- 3GPP RAN3#117b Chairman’s notes RAN3_117bis-e_agenda_20221018_EOM1.doc)

- 3GPP RAN3#119b Chairman’s notes (RAN3_119bis-e_agenda_20230426_EOM1.doc)

- 3GPP RAN3#117 Chairman’s notes (RAN3_117-e_agenda_20220825_EOM1.doc)

- 3GPP RAN3#119 Chairman’s notes (RAN3_119bis-e_agenda_20230426_EOM1.doc)

- 3GPP RAN3#120 Chairman’s notes (RAN3_120_agenda_20230530_EOM1.doc)

# Annex 1: TP to 37.483 (v17.5)

<<<<<< Start of CHANGES >>>>>

### 8.2.9	Resource Status Reporting Initiation

#### 8.2.9.1	General

This procedure is used by an gNB-CU-CP to request the reporting of load measurements to gNB-CU-UP.

The procedure uses non UE-associated signalling.

#### 8.2.9.2	Successful Operation

Figure 8.2.9.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from gNB-CU-CP to gNB-CU-UP to start a measurement or stop a measurements.

If gNB-CU-UP is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU-CP, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the RESOURCE STATUS UPDATE message:

-	the HW Capacity Indicator IE, if the second bit, "HW Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the TNL Available Capacity Indicator IE, if the first bit, " TNL Available Capacity Ind Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Energy Cost IE, if the third bit, "Energy Cost" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1";

-	the UE Performance IE, if the fourth bit, "UE Performance" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, this indicates the periodicity for the reporting of periodic measurements. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.2.9.3	Unsuccessful Operation

Figure 8.2.9.3-1: Resource Status Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-CU-UP shall send a RESOURCE STATUS FAILURE message with an appropriate cause value.

#### 8.2.9.4	Abnormal Conditions

If the initiating gNB-CU-CP does not receive either RESOURCE STATUS RESPONSE message or RESOURCE STATUS FAILURE message, the gNB-CU-CP may reinitiate the Resource Status Reporting Initiation procedure towards the same gNB-CU-UP, provided that the content of the new RESOURCE STATUS REQUEST message is identical to the content of the previously unacknowledged RESOURCE STATUS REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the RESOURCE STATUS REQUEST message then gNB-CU-UP shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a RESOURCE STATUS REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU-CP Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-CU-UP shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

<<<<<< Next CHANGE >>>>>

#### 9.2.1.19	RESOURCE STATUS REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = TNL Available Capacity Ind Periodic, Second Bit = HW Capacity Ind Periodic, Third Bit = Energy Cost, Fourth Bit = UE Performance. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

<<<<<< Next CHANGE >>>>>

#### 9.2.1.22	RESOURCE STATUS UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| TNL Available Capacity Indicator | O |  | 9.3.1.72 |  | YES | ignore |
| HW Capacity Indicator | O |  | 9.3.1.73 |  | YES | ignore |
| Energy Cost | O |  | 9.3.1.xx | It represents the actual measurement of the Energy Cost (FFS) | YES | ignore |
| UE Associated Info Result |  | 0..1 |  |  | YES | ignore |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >>gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  |  |  |
| >>gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  |  |  |
| >>UE Performance | O |  | 9.3.1.yy |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<<<<<< Next CHANGE >>>>>

#### 9.3.1.xx	Energy cost

The Energy Cost IE indicates the energy cost.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Cost | M |  | INTEGER (0..FFS) | Value 0 represents the minimum EC value. Value FFS represents the maximum EC value. |

#### 9.3.1.yy	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | FFS |  |
| Average UE Throughput UL | O |  | FFS |  |
| Average Packet Delay | O |  | FFS |  |
| Average Packet Loss | O |  | FFS |  |

<<<<<< End of CHANGES >>>>>

# Annex 2: TP to 38.473 (v17.5)

<<<<<< Start of CHANGES >>>>>

### 8.2.10	Resource Status Reporting Initiation

#### 8.2.10.1	General

This procedure is used by an gNB-CU to request the reporting of load measurements to gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Resource Status Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the RESOURCE STATUS REQUEST message to gNB-DU to start a measurement, stop a measurement, or add cells to report for a measurement. Upon receipt, gNB-DU:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the RESOURCE STATUS REQUEST message and the Report Characteristics IE indicates cell specific measurements, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the RESOURCE STATUS REQUEST message, the Cell To Report List IE shall be included.

If gNB-DU is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-DU shall perform measurements on. For each cell, gNB-DU shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource IE for such cell shall only include the SSB Area Radio Resource Status List IE; If the cell for which Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE if the cell for which Radio Resource Status IE is requested to be reported supports MIMO the Radio Resource Status IE for such cell shall include the MIMO PRB usasge Information IE;

-	the TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List IE for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value IE. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE providing the SSB area capacity with respect to the Cell Capacity Class Value. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

-	the Hardware Load Indicator IE, if the fourth bit, " HW LoadInd Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Number of Active UEs IE, if the fifth bit, "Number of Active UEs Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the NR-U Channel List IE, if the sixth bit, " NR-U Channel List Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

The gNB-DU shall include the following node level measurements in the RESOURCE STATUS UPDATE message:

-	the Energy Cost IE, if the seventh bit, "Energy Cost" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

-	the UE Performance IE, if the eighth bit, "UE Performance" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

If the Reporting Periodicity IE in the RESOURCE STATUS REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. The gNB-DU shall report once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.2.10.3	Unsuccessful Operation

Figure 8.2.10.3-1: Resource Status Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-DU shall send the RESOURCE STATUS FAILURE message with an appropriate cause value.

#### 8.2.10.4	Abnormal Conditions

If the initiating gNB-CU does not receive either RESOURCE STATUS RESPONSE message or RESOURCE STATUS FAILURE message, the gNB-CU may reinitiate the Resource Status Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new RESOURCE STATUS REQUEST message is identical to the content of the previously unacknowledged RESOURCE STATUS REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the RESOURCE STATUS REQUEST message then gNB-DU shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

If the gNB-DU receives a RESOURCE STATUS REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-DU shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

<<<<<< Next CHANGE >>>>>

#### 9.2.1.20	RESOURCE STATUS REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW LoadInd Periodic, Fifth Bit = Number of Active UEs Periodic, Sixth Bit = NR-U Channel List Periodic, Seventh Bit = Energy Cost, Eighth Bit=UE Performance. Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | - |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) |  |  |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | - |  |
| >>>Slice To Report Item |  | 1..< maxnoofBPLMNsNR> |  |  |  |  |
| >>>>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN |  |  |
| >>>>S-NSSAI List |  | 1 |  |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to “add”. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

<<<<<< Next CHANGE >>>>>

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| Energy Cost | O |  | 9.3.1.xx | It represents the actual measurement of the Energy Cost (FFS) | YES | ignore |
| UE Associated Info Result |  | 0..1 |  |  | YES | ignore |
| > UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  |  |  |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |  |  |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |  |  |
| >>UE Performance | O |  | 9.3.1.yy |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofUEReports | Maximum no. UE that can be served by a NG-RAN node. Value is FFS. |

<<<<<< Next CHANGE >>>>>

#### 9.3.1.xx	Energy cost

The Energy Cost IE indicates the energy cost.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Cost | M |  | INTEGER (0..FFS) | Value 0 represents the minimum EC value. Value FFS represents the maximum EC value. |

#### 9.3.1.yy	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | FFS |  |
| Average UE Throughput UL | O |  | FFS |  |
| Average Packet Delay | O |  | FFS |  |
| Average Packet Loss | O |  | FFS |  |

<<<<<< End of CHANGES >>>>>