3GPP TSG-RAN WG3 Meeting #121	R3-234286

Toulouse, France, August 21 – 25, 2023

Agenda item:	23.2.3

Source: 	Qualcomm Incorporated

Title: 	Integrity of NR Positioning Technologies

Document for:		Discussion and Decision

# 1.	Introduction

The Work Item on "Expanded and Improved NR Positioning" includes the following objective for "RAT-dependent Integrity" [1]:

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

At RAN2#121 and RAN2#121bis-e, the following agreements potentially impacting RAN3 were made [2],[3]:

Agreements:

Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope.  Other aspects of determining the TRP error sources are left to deployment and implementation.

Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Agreement:

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

Agreement:

LS to RAN1 to include a request for confirmation that the beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources for DL-AoD positioning.

At RAN2#121bis, an LS was sent to RAN1 (cc: RAN3) with some remaining questions [4]. RAN1 replied in [5] as follows:

RAN1 would like to provide the answers to the following questions: Working assumption: It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN. Q0: RAN2 would like to kindly request RAN1 to confirm whether they have any concern on the above working assumption. Reply to Question 0: From RAN1’s perspective, no concerns are identified for the RAN2 working assumption “It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.” Q1: Are beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) error sources for DL-AOD positioning? Reply to Question 1: Regarding whether beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources or not, RAN1 made the following conclusion in RAN1#111, “RAN1 could not reach consensus on whether beam information (NR-TRP -BeamAntennaInfo) and boresight direction of DL PRS (NR-DL -PRS -BeamInfo) are error sources or not for DL-AoD for UE -based positioning integrity mode.”, where the definition of “UE-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857. Q2: Are DNU flag(s) for TRP/UE positioning measurements needed or not? Reply to Question 2: As agreed in RAN1#110b-e, from RAN1 perspective, study of the application of DNU flag for determination of positioning integrity is within the scope of RAN2 discussion. Specification impact(s) of DNU flag(s) can be discussed in RAN2.

A background of integrity was provided in our previous contribution [6]. In this contribution, we discuss the aspects from the RAN2 agreements above.

# 2.	Discussion

##### Network Integrity Monitoring

Integrity is based on the computation of "Protection Levels". The Protection Level defines the value that, with a very high probability, bounds the error of the navigation solution.

The protection levels are computed based on the expected behaviour of the error sources encountered in a positioning system. The error sources for the NR Positioning Methods were determined by RAN1 during the Study Item phase and are summarized in Table 6.1.1-1 of TR 38.859 [7]. The error sources include:

- UE measurement errors:

-	RSTD measurement

-	UE Rx-Tx time difference measurement

-	DL-PRS RSRP/RSRPP

- TRP measurement errors:

-	gNB Rx-Tx time difference measurement

-	RTOA measurement

-	Angle of arrival measurement

- Network/System errors:

-	TRP/ARP location

- 	Inter-TRP synchronization

-	Boresight Direction of DL-PRS Resource (FFS)

-	Beam Antenna Information of DL-PRS (FFS)

RAN1 confirmed that the error sources can be overbounded by a Gaussian distribution [8]. The parameters for the overbound Gaussian distribution are the mean and standard deviation of each error source [6] (like in GNSS integrity).

The individual error bounds could be derived from real-time reference network measurements. For GNSS, such reference networks are typically maintained by GNSS service providers and how an LMF obtains the data is left to deployment in Rel-17. For NR positioning technologies, a similar reference receiver network would be required to monitor the system to ensure its integrity. For NR, this could for example be realized using PRUs, but is also proposed to be left to deployment/implementation.

Proposal 1:	How an LMF obtains the network/system error bounds (e.g., TRP/ARP location, synchronization error, etc.) – e.g., using a reference receiver network/PRUs, etc. – should be left to implementation/deployment (similar to Rel-17 GNSS integrity).

##### LMF-based PL calculation

To determine the PL by an LMF for UE-assisted mode or network-based positioning methods, the LMF requires the overbounding standard deviation of the UE and TRP measurement error (e.g., RSTD, RTOA, Rx-Tx, AoA, DL-PRS RSRP, DL-PRS RSRPP), which provides an upper bound on the standard deviation of the nominal measurement error σx,i.  That is, under fault-free condition, the standard deviation of the measurement error is no greater than σx,i. In addition, a bias bound may be needed, which provides the mean value for an overbounding model that bounds the measurement mean error.

The UE and TRP measurement errors are not observable and need to be approximated/estimated via e.g., standard error of the mean which estimates the variability across multiple samples of a population:

| σ=(xi-x)2N | (1) |
| --- | --- |

This standard deviation may serve as a measure of uncertainty and requires a number N of samples. Therefore, periodic measurements are required, where either the UE/TRP determines the sample statistics or the LMF uses the periodic reports from the UE/TRP to determine sample statistics.

The sample statistics may be derived or used by an e.g., Kalman Filter whose details depends on LMF implementation/algorithms used. Instead of deriving the measurement statistics at the UE/TRP and reporting to an LMF, it seems more flexible and implementation-independent if an LMF determines any statistics from the (already specified) UE/TRP measurement report directly. I.e., a LMF could request periodic reporting from UEs and TRPs and could derive any required statistics from the  measurement reports.

This would not require specifying a particular method/equation for determining the error statistics/bounds and therefore, could be up to LMF implementation. For example, the LMF could decide which measurement items are used to determine the bounds (e.g., multiple paths, measurement quality, LOS/NLOS Information, etc.), or how many samples are used or needed (e.g., N in equation (1), etc.). Therefore, details could be implementation defined and would be most flexible (i.e., no "hard coding" of the method/definition would be needed).

A corresponding working assumption was made at RAN2#121bis (see section 1):

Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Agreement:

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

As summarized in section 1, RAN1 did not raise any concerns on the Working Assumption [5]:

From RAN1’s perspective, no concerns are identified for the RAN2 working assumption “It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.”

Hence, it is proposed:

Proposal 2: 	Confirm the RAN2 Working Assumption: "It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN."

# 3.	Summary

In this contribution, we discussed the impacts of integrity support to RAN3. The following Proposals were made.

Proposal 1:	How an LMF obtains the network/system error bounds (e.g., TRP/ARP location, synchronization error, etc.) – e.g., using a reference receiver network/PRUs, etc. – should be left to implementation/deployment (similar to Rel-17 GNSS integrity).

Proposal 2: 	Confirm the RAN2 Working Assumption: "It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN."

# References

[1]		RP-231460, "Revised WID on Expanded and Improved NR Positioning".

[2]		R2-2301904, "Report from session on positioning and sidelink relay", Session Chair (MediaTek) 						(RAN2#121).

[3]		R2-2304204, "Report from session on positioning and sidelink relay", Session Chair (MediaTek) 						(RAN2#121bis-e).

[4]		R3-232576, "LS to RAN1 on RAT-dependent positioning integrity", RAN2.

[5]		R3-233706, "LS reply on the RAT-dependent positioning integrity", RAN1.

[6]		R3-231280, "Integrity of NR Positioning Technologies", Qualcomm Incorporated.

[7]		3GPP TR 38.859: "Study on expanded and improved NR positioning".

[8]		R1-2304147, "Reply LS to RAN2 on error source distributions", To: RAN2.