3GPP TSG-RAN3 Meeting #121	R3-234300

Toulouse, France - August 21st – August 25th 2023

Agenda Item:	12.3

Source:	Ericsson, InterDigital, Deutsche Telekom, AT&T

Title:	Improved UE selection granularity

Document for:	Discussion and Decision

# 1	Introduction

During RAN3-120 a discussion on AI/ML related improvements involving MDT has been taken. During the discussion, support was given to improvements on the granularity of UE selection for management based MDT. As a consequence, the following is captured form the Chair notes on the introductory text for AI12.3:

Potential MDT enhancement related issues as follows, need more time to discuss the details and potential standard impacts, coordination with RAN2/SA5 if needed:

- enhance the mdt procedure to solve the issue how to support the consecutive ai/ml data collection for the certain time-series ai/ml model.

- how the source ng-ran node obtains logged ue trajectory information when ue enters rrc connected state and reports to the new ng-ran node.

- how to enable a more granular selection of ues based on enhanced mdt configuration information in management based MDT

- how to map ai/ml feedback information to ai/ml actions and report them over MDT

Study the scenarios, issues and solutions to support the continuous data collection within a period for AI/ML via MDT.

More clarification on granularity of UE selection are needed in the next meeting.

In this contribution, we focus on further discussing Improved UE selection granularity.

# 2	Discussion

In previous RAN3 meetings, we discussed MDT enhancements to support AI/ML based processes. More specifically, we proposed to enhance the current methods of activating management based MDT and collecting data from the UEs to enable a more granular selection of relevant UEs by the RAN [1].

Our proposal enables the OAM to have more control on how to select relevant UEs for MDT configurations, based on selection criteria passed to the RAN. Namely, the proposals consist of allowing the OAM to provide to the RAN, as part of the MDT configuration, details on the type of UEs that should be selected for the management MDT configuration process.

This lifts the restrictions to to make a selection totally left to RAN implementation and limited only in terms of MDT area scope, as in management based MDT.

To recap proposals from [1] we suggest a new method that enables setting up data collection for AI/ML performance monitoring, AI/ML model training, and similar purposes, using the trace/MDT framework. We propose to update MDT procedures to enable a more accurate set of criteria for UE selection in addition to the already existing management based and signalling based MDT configurations. In this approach, the RAN is enabled to select relevant UEs for performing the requested measurements and reporting the requested information based on OAM configurations.

The new mechanism we propose for MDT is based on the following steps:

a)	Enhance the management based MDT configuration received by the RAN with information that allows selection of UEs with specific characteristics indicated by the OAM system. As an example, if the OAM system wants to monitor the performance of the network including 4x4 MIMO communication, the criteria for UE selection would indicate that UEs to be selected for MDT measurement collection process shall support 4x4 MIMO. Alternatively, if the OAM wants to exclude from its measurement collection process specific UEs, such as IoT devices, the criteria for selection will exclude IoT capabilities. 
These enhanced UE selection criteria can be provided in a number of ways, for example:

- Include a list of required UE capabilities as part of the Management Based MDT configuration signalled form the OAM to the Ng-RAN. A representation of UE capabilities can be easily achieved from the RAN2 specifications and provided to the RAN as follows:

| UE Selection Criteria | O |  | OCTET STRING | Includes the UE-CapabilityRAT-ContainerList as defined in TS38.331. The IE is used to select UEs for MDT configuration |
| --- | --- | --- | --- | --- |

- Another example could be by including the Masked IMEISV as part of the Management Based MDT configuration. This information allows the NG-RAN to identify a terminal model without identifying an individual Mobile Equipment. It is already used over the NGPA interface and it can be provided to the NG-RAN as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Masked IMEISV | M |  | BIT STRING (SIZE(64)) | Coded as the International Mobile station Equipment Identity and Software Version Number (IMEISV) defined in TS 23.003 [23] with the last 4 digits of the SNR masked by setting the corresponding bits to 1. The first to fourth bits correspond to the first digit of the IMEISV, the fifth to eighth bits correspond to the second digit of the IMEISV, and so on. |

b)	Enable the RAN to select individual UEs or group of UEs to collect data, e.g., for AI/ML model training purposes, according to the criteria indicated in the MDT configuration.

c)	Reporting the MDT data collected to the OAM to enable AI/ML processes such as training, re-training, performance evaluation.

By filtering the UEs that are selected for Management Based MDT according to the criteria indicated by the OAM it is possible to retrieve data that are more relevant and focussed on the AI/ML training process that needs to be supported.

Without this mechanism the alternative is to rely on a UE selection process that may not be consistent with the training process that needs to be supported. As an example, if training data collection via MDT is triggered to train/re-train AI/ML Mobility Optimisation algorithms it is totally pointless to select for such task IoT UEs that do not move. Collecting measurements from such UEs would mean purely to increase the computational load at the Model Training Function without any benefit or indeed with negative impacts given that IoT UEs may provide statistics that would never be reported by mobile UEs, therefore making the AI/ML model diverge from learning of relevant processes.

The following figure summarizes the flow as an example on how the MDT configuration may be carried out utilizing the proposed generic MDT framework functionality for this scenario.

Figure 1 Example for generic MDT based activation in NG-RAN in the case of non-split architecture

Each step in Figure 1 is described below:

1)	The management system sends a Trace Session activation request to the gNB. This request includes a set of MDT Configuration parameters, where, on top of the existing configuration parameters, the following could be provided:

-	Characteristics of the UEs from which measurements need to be collected, e.g. UE capabilities, Masked IMEISV

UE selection (step 3) is carried out on the basis of the criteria received by the RAN. Namely, if the UE fulfils the selection criteria, that UE may be selected for MDT based data collection.

2)	When RAN receives the Trace Session activation request from its management system containing the enhanced MDT Configuration, it shall start a Trace Session and it should save the parameters associated to the Trace Session.

3)	RAN selects suitable UEs for MDT data collection. The selection is based on the relevant input received from the management system, such as: the capabilities UEs should have to be selected for MDT measurement configuration (as indicated by the OAM), the Masked IMSISV, the area where the UE is located, user consent etc.

4)	gNB configures MDT measurements to the selected UE or group of UEs

5)	When a UE receives the MDT measurement configuration it shall start the MDT functionality based on the received configuration parameters and it can start MDT reporting

6) The RAN forwards the measurements results to the TCE

The proposed enhancement is essential for AI/ML as data collection from UEs for the purpose of AI/ML support (e.g. training data collection) may require that the collected data is associated to a specific use cases. For example, the OAM may require data collection from UEs supporting specific MIMO capabilities, so that the performance of the network for such MIMO use case can be monitored and models can be trained accordingly. Similarly, the OAM may need to collect data concerning UEs supporting DC capabilities, so that models that include DC aspects can be properly trained.

It should be noted that in Signalling Based MDT, the OAM configures the RAN to select a specific UE, hence there is no freedom at RAN level in selecting the UE(s) to configure with MDT measurements. On the contrary, management based MDT leaves to the RAN implementation choice on how to select UEs for MDT measurement configuration. While this is feasible for generic MDT measurement collection processes (e.g. revealing the overall performance of the network), it might not be sufficient for MDT measurement collections for AI/ML purposes. The latter requires that MDT measurements are collected for specific use cases and features and not all UEs may be suitable for collecting and reporting such specific AI/ML data.

In light of the above, RAN3 could discuss these aspects and then LS SA5 to communicate the outcomes of this discussion.

On the basis of the above, the following is proposed:

Proposal 1: it is proposed to enhance MDT to enable a more granular selection of UEs based on an enhanced MDT Configuration where information on UE selection criteria, such as the capabilities of the UEs that should be selected for MDT measurements, is added.

Proposal 2: Liaise SA5 to check for feasibility of the updates to the MDT Configuration concerning UE selection granularity.

# 3	Conclusion

In this contribution possible enhancements to the MDT framework that would enable MDT to be used for data collection for AI/ML have been discussed.

The following conclusions and proposals were derived:

Proposal 1: it is proposed to enhance MDT to enable a more granular selection of UEs based on an enhanced MDT Configuration where information on UE selection criteria, such as the capabilities of the UEs that should be selected for MDT measurements, is added.

Proposal 2: Liaise SA5 to check for feasibility of the updates to the MDT Configuration concerning UE selection granularity [2].

# References

- R3-230457, MDT Enhancements for AI/ML, Ericsson, Deutsche Telekom, InterDigital.

- R3-234301, draft_LS_to_SA5, Improved UE selection granularity