3GPP TSG-RAN WG3 Meeting #121	R3-234310

# Toulouse, France, Aug 21–25 2023

Agenda Item:	10.2.1

Source:	Ericsson

Title:	Successful PSCell Report (SPR) for SON rel-18

Document for:	Discussion and Decision

# 1	Introduction

SPR for NR-DC was discussed during previous RAN3 meetings. In this contribution we continue discussing some of the topics which are not yet concluded such as the SPR configuration and root cause analysis, fetching and forwarding mechanisms and the content of the report.

# 2	SPR for NR-DC

## 2.1	MN initiated PSCell change

Last RAN3 meeting did not conclude on the configuration of SPR and the root cause analysis for the MN-initiated (Conditional) PSCell Change, but the remaining options have been captured as a “to be continued” topic:

Which node decides SPR triggers in MN initiated PSCell change? - Option 2：Source SN node decides the T310/T312 triggers and performs root cause analysis - Option 3：MN decides the T310/T312 triggers and performs root cause analysis, and whether and what information from SN as input needs to be further discussed

After a successful, but almost failed, MN-initiated PSCell change, the UE may generate a SPR and (eventually) the SPR gets delivered to the MN. The SPR helps the MN find out what was the reason for this almost-failure. One typical reason could be a too early PSCell change. The background to this almost-failure is typically incorrect timing when configuring measurements in the UE and triggering a PSCell change. The root cause for this is what the SPRs should help optimising.

As the MN has the best knowledge of the history of its initiated PSCell changes, the MN has also the best possibility to know how to configure source SN related SPR triggers, that is, triggers related to timers T310 and T312.

Observation 1: The MN has the best knowledge of the history of its initiated PSCell changes, therefore, the MN has also the best possibility to know how to configure source SN related SPR triggers.

As the MN has the best knowledge of its PSCell change history, and the SN does not have this knowledge, we propose Option 3, i.e. the MN decides the T310/T312 triggers and performs root cause analysis.

Proposal 1: As the MN has the best knowledge of its PSCell change history, and the SN does not have this knowledge, we propose Option 3.

The responsibility for configuring the actual timer values lies with the source SN, and not the MN. The MN is not able to change these values and need to be satisfied with what the source SN has decided. Therefore, the MN is not helped by knowing the values of these timers when it decides on triggers for SPR.

- Observation 2: For optimizing MN initiated PSCell change, the MN is not helped by knowing the values of the source SN T310/T312 timers.

If SPR is needed only when a timer is close to expiration, MN will need to configure a high percentage as trigger, in order to get enough data from UEs, without receiving too many SPR, e.g. 90%. There is no need to know the exact value of the timer. Therefore, information from SN as input to configuring the threshold is not needed.

- Proposal 2: For MN-initiated classic PScell change/CPC, the MN autonomously decides the T310/T312 triggers and performs root cause analysis.

## 2.2	Fetching and forwarding of the SPR

It has been agreed in RAN3#119bis-e that:

Reuse ACCESS AND MOBILTY INDICATION to forward SPR over XnAP and F1AP and use Uplink/Downlink RAN Configuration Transfer for forwarding SPR over NGAP

To identify the UE context in the old source SN/old target SN when SPR is forwarded by old MN for SPR optimization, old MN identifies the UE context and sends the stored respective SN Mobility Information together with SPR to the old source SN/old target SN

And in RAN3#120 that:

If SPR available indication via SN RRCReconfigComplete is received by SN, SN should inform MN that an SPR is available at the UE e.g., a new IE can be added in S-NODE MODIFICATION REQUIRED message.

RAN2 also agreed that SPR will be aligned on SHR, when possible. Also, the RAN2 running CR includes, similar to SHR, the SPR availability in the RRCReconfigComplete message, among others. Therefore, and taking into account last meeting agreement, SN should inform MN that an SPR is available at the UE, with a new IE can be added in S-NODE MODIFICATION REQUIRED message.

Proposal 3: Add a new IE in S-NODE MODIFICATION REQUIRED message to inform the MN that an SPR is available at the UE.

As for all SON reports, the network is not mandated to store the UE Context after the UE has left the node. Therefore, there is a need for a solution allowing the forwarding of the SPR, without using UE Context, or keeping it for a short time only. Fortunately, for SPR, the UE is likely to still be connected to the MN and the target SN just after PSCell Change/Addition and therefore at the time the SPR is generated. In that case the MN, the target SN and possibly the source SN will still have the UE Context.

Because of this, the MN can fetch the SPR while UE is still connected to that MN, and while the UE is still connected to the target SN.

- Observation 3: Typically, when the MN fetches the SPR from a UE, the MN and target SN (and possibly the source SN) have active UE contexts for that UE.

Once the MN has received the indication that the SPR is available in the UE, it could be beneficial to fetch it when the UE is still connected to the PCell and target PSCell. By doing so, the MN can forward the SPR, using the agreed Access and Mobility Indication procedure immediately. If that is the case, UE XnAP IDs are still used in MN and target SN for that UE (and possibly source SN). Similar to RACH Report, this ID could be used in the SNs to identify the UE Context. It is therefore proposed to add the S-NG-RAN node XnAP ID to the Access and Mobility Indication message, together with the SPR. A TP to TS 38.423 is provided in Annex.

- Proposal 4: Include S-NG-RAN node UE XnAP IDs together with SPRs when the MN sends the SPR to the source and/or target SN.

- Proposal 5: Agree TP to TS 38.423 in Annex

## 2.3	Determining whether a PSCell change was MN-initiated or SN-initiated

It has been discussed whether there is a need for the SPR to contain an indication on whether the PSCell change was MN initiated or SN initiated.

The following WA was then captured during RAN3#119bis-e:

WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

Since a SPR is collected possibly up to 48 hours of generation, it is important for the network to understand which node initiated the PSCell change procedure, since that node would possibly be required to optimize its configuration. A feasible solution is to receive such information from the UE in SPR. However, for SRB1 based configuration, it is not possible for the UE to identify if MN or SN initiated the PSCell change procedure. In order to aid the UE, network can include the node initiating PSCell change procedure information in the SPR configuration.

Proposal 6: Network includes a flag in the SPR configuration if the PSCell change procedure is MN initiated or SN initiated.

Proposal 7: UE logs an indication in the SPR that the PSCell change which led to the SPR was MN initiated or SN initiated.

Proposal 8: Send an LS to RAN2 to confirm the feasibility of the above proposals (7 & 8)

# 3	Conclusion

SPR have been discussed further and the following observations and agreements have been made:

Observation 1: The MN has the best knowledge of the history of its initiated PSCell changes, therefore, the MN has also the best possibility to know how to configure source SN related SPR triggers.

Proposal 1: As the MN has the best knowledge of its PSCell change history, and the SN does not have this knowledge, we propose Option 3.

- Observation 2: For optimizing MN initiated PSCell change, the MN is not helped by knowing the values of the source SN T310/T312 timers.

- Proposal 2: For MN-initiated classic PScell change/CPC, the MN autonomously decides the T310/T312 triggers and performs root cause analysis.

Proposal 3: Add a new IE in S-NODE MODIFICATION REQUIRED message to inform the MN that an SPR is available at the UE.

- Observation 3: Typically, when the MN fetches the SPR from a UE, the MN and target SN (and possibly the source SN) have active UE contexts for that UE.

- Proposal 4: Include S-NG-RAN node UE XnAP IDs together with SPRs when the MN sends the SPR to the source and/or target SN.

- Proposal 5: Agree TP to TS 38.423 in Annex

Proposal 6: Network includes a flag in the SPR configuration if the PSCell change procedure is MN initiated or SN initiated.

Proposal 7: UE logs an indication in the SPR that the PSCell change which led to the SPR was MN initiated or SN initiated.

Proposal 8: Send an LS to RAN2 to confirm the feasibility of the above proposals (7 & 8)

# Annex: TP to SON BL CR to TS 38.423

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

### 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

#### 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE CHANGE REQUIRED message may contain

-	the S-NG-RAN node to S-NG-RAN node Container IE.

If the M-NG-RAN node is able to perform the change requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE CHANGE CONFIRM message to the S-NG-RAN node. For DRBs configured with the PDCP entity in the S-NG-RAN node, the M-NG-RAN node may include data forwarding related information in the Data Forwarding Info from target NG-RAN node IE.

If the S-NODE CHANGE CONFIRM message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8].

The S-NG-RAN node may start data forwarding and stop providing user data to the UE and shall stop the timer TXnDCoverall upon reception of the S-NODE CHANGE CONFIRM message.

If the S-NODE CHANGE REQUIRED message includes the SCG UE History Information IE, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

If the S-NODE CHANGE REQUIRED message includes the SN Mobility Information IE, the M-NG-RAN node shall, if supported, store this information and use it as defined in TS 38.300 [9].

If the S-NODE CHANGE REQUIRED message includes the Source PSCell ID IE, the M-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

If the Conditional PSCell Change Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the requirement concerns CPAC, as described in TS 37.340 [8]. The S-NG-RAN node to M-NG-RAN node Container IE within the Conditional PSCell Change Information Required IE contains at least the suggested PSCell list for each candidate target S-NG-RAN node. Accordingly, the M-NG-RAN node may include the Conditional PSCell Change Information Confirm IE in the S-NODE CHANGE CONFIRM message.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Change Information Required IE included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, forward this information to the candidate target S-NG-RAN node, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

If the Multiple Target S-NG-RAN Node List IE is included in the S-NODE CHANGE REQUIRED message, if multiple Target S-NG-RAN nodes are prepared, the M-NG-RAN node may include the Additional List of PDU Session Resource Change Confirm Info – SN Terminated IE in the S-NODE CHANGE CONFIRM message to provide different data forwarding addresses for different Target S-NG-RAN nodes.

If the SPR availability in UE IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall understand that the UE has generated an SPR for the previous PSCell change, and may retrieve the SPR from the UE.

Interaction with M-NG-RAN node initiated S-NG-RAN node Release:

If the M-NG-RAN node receives the S-NODE CHANGE REQUIRED message indicating releasing target S-NG-RAN node(s) and cancelling all prepared PSCells in the target S-NG-RAN node(s), the M-NG-RAN shall, if supported, trigger the M-NG-RAN node initiated S-NG-RAN node release procedure to the target S-NG-RAN node(s) and cancel all the prepared PSCells at the target S-NG-RAN node(s).

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED --

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  | YES | ignore |
| PDU Session Resources To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Required Info – SN terminated IE nor the PDU Session Resource Modification Required Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.4.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Modification Required Info – SN terminated | O |  | 9.2.1.20 |  | – |  |
| >>PDU Session Resource Modification Required Info – MN terminated | O |  | 9.2.1.22 |  | – |  |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUSessions> |  |  | – |  |
| >PDU sessions to be released List – SN terminated | O |  | PDU session List with data forwarding request info 9.2.1.24 |  | – |  |
| >PDU sessions to be released List – MN terminated | O |  | PDU session List with Cause 9.2.1.26 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| Spare DRB IDs | O |  | DRB List 9.2.1.29 | Indicates the list of unnecessary DRB IDs that had been used by the S-NG-RAN node. | YES | ignore |
| Required Number of DRB IDs | O |  | Number of DRBs 9.2.3.78 | Indicates the number of DRB IDs that the S-NG-RAN node requests more. | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) |  | YES | ignore |
| SCG UE History Information | O |  | 9.2.3.151 |  | Yes | ignore |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| CPAC Information Required | O |  |  | This IE may be sent from the target SN. | YES | ignore |
| >Candidate PSCell List |  | 1 |  | Indicates the full list of candidate PSCells prepared at the target S-NG-RAN node. | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| SCG Reconfiguration Notification | O |  | ENUMERATED (executed, ...) |  | YES | ignore |
| SPR availability in UE | O |  | ENUMERATED (spr-available, …) | Indicated if an SPR is available in the UE | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

<<<<<<<<<<<<<<<<<<<< End of 2nd Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED --

<<<<<<<<<<<<<<<<<<<< 3rd Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report List Item |  | 1 .. <maxnoofRAReports> |  |  | EACH | ignore |
| >>RA Report Container | M |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | – |  |
| >>Successful HO Report Container | M |  | OCTET STRING | SuccessHO-Report-r17 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| Successful PSCell Change Report List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report List Item |  | 1 .. <maxnoofSuccessfulPSCellChangeReports> |  |  | – |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | FFS: pending on RAN2 | – |  |
| >>SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Mobility Information in the PSCell of the source SN in case this message is sent from the MN to the source SN; Mobility Information in the PSCell of the target SN in case this message is sent from the MN to the target SN. |  |  |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Corresponds to the S-NG-RAN node UE XnAP ID allocated by the source S-NG-RAN node or the target NG-RAN node to the UE-associated connection of the UE which generated the corresponding Successful PSCell Change Report |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports, the maximum value is FFS. |

<<<<<<<<<<<<<<<<<<<< End of 3rd Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED --

<<<<<<<<<<<<<<<<<<<< 4th Change >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

-- TEXT OMITTED –

HashedUEIdentityIndexValue,

SPRAvailability

-- TEXT OMITTED –

id-HashedUEIdentityIndexValue,

id-SPRAvailability,

-- TEXT OMITTED –

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUIRED

--

-- **************************************************************

SNodeModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequired-IEs}},

...

}

SNodeModificationRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication						PRESENCE optional }|

{ ID id-PDUSessionToBeModifiedSNModRequired		CRITICALITY ignore		TYPE PDUSessionToBeModifiedSNModRequired	PRESENCE optional }|

{ ID id-PDUSessionToBeReleasedSNModRequired		CRITICALITY ignore		TYPE PDUSessionToBeReleasedSNModRequired	PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-SpareDRBIDs								CRITICALITY ignore		TYPE DRB-List										PRESENCE optional }|

{ ID id-RequiredNumberOfDRBIDs					CRITICALITY ignore		TYPE DRB-Number										PRESENCE optional }|

{ ID id-LocationInformationSN					CRITICALITY ignore		TYPE Target-CGI										PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-RRCConfigIndication						CRITICALITY reject		TYPE RRCConfigIndication							PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-ReleaseFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE ReleaseFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore		TYPE SCGIndicator									PRESENCE optional }|

{ ID id-SCGUEHistoryInformation					CRITICALITY ignore		TYPE SCGUEHistoryInformation					PRESENCE optional }|

{ ID id-SCGActivationRequest					CRITICALITY ignore		TYPE SCGActivationRequest						PRESENCE optional }|

{ ID id-CPACInformationModRequired				CRITICALITY ignore		TYPE CPACInformationModRequired				PRESENCE optional }|

{ ID id-SCGreconfigNotification					CRITICALITY ignore		TYPE SCGreconfigNotification					PRESENCE optional }|

{ ID id-SPRAvailability							CRITICALITY ignore		TYPE SPRAvailability						PRESENCE optional },

...

}

-- TEXT OMITTED –

### 9.3.5	Information Element definitions

-- TEXT OMITTED –

SplitSRBsTypes ::= ENUMERATED {srb1, srb2, srb1and2, ...}

SPRAvailability ::= ENUMERATED {spr-available, ...}

SSBAreaCapacityValue-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaCapacityValue-List-Item

SSBAreaCapacityValue-List-Item	::= SEQUENCE {

sSBIndex				INTEGER(0..63),

ssbAreaCapacityValue	INTEGER (0..100),

iE-Extensions						ProtocolExtensionContainer { { SSBAreaCapacityValue-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaCapacityValue-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- TEXT OMITTED –

SuccessfulHOReportContainer	::= OCTET STRING

SuccessfulPSCellChangeReportInformation	::= SEQUENCE (SIZE(1.. maxnoofSuccessfulPSCellChangeReports)) OF SuccessfulPSCellChangeReportList-Item

SuccessfulPSCellChangeReportList-Item	::= SEQUENCE {

successfulPSCellChangeReport		SuccessfulPSCellChangeReportContainer,

sNMobilityInformation				SNMobilityInformation															OPTIONAL,

uEAssistantIdentifier				NG-RANnodeUEXnAPID																OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { SuccessfulPSCellReportList-Item-ExtIEs} }	OPTIONAL,

...

}

SuccessfulPSCellChangeReportList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SuccessfulPSCellChangeReportContainer ::= OCTET STRING

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>